3GPP TSG-RAN WG2 Meeting #131bis						R2-2507617

Prague, Czech, October 13th – 17th, 2025

Agenda Item:	8.0.1

Source: 	OPPO

Title:  	Discussion on cross-WI ASN.1 Issues (O000, O001, O003, O004, O005)

Document for:	Discussion and Decision

- Introduction

This paper is going to discuss the cross-WI ASN.1 Issues.

- Discussion

  - O000

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O000 | SBFD, MOB | 2 | Applicability of SBFD-RACH to LTM procedure | R2-25xxxxx | OPPO (Qianxi) |  | V003 | ToDo |

[Description]: R2 has not concluded the applicability of SBFD to LTM (other than inter-DU LTM), including the procedure for early UL sync, LTM execution (CSC-based) and CLTM execution.

[Proposed Change]: R2 discuss and conclude the applicability of SBFD to LTM (other than inter-DU LTM), including the procedure for early UL sync, LTM execution (CSC-based) and CLTM execution.

[Comments]:

[Analysis]

SBFD session did not conclude this issue, considering conclusion from 131 as follows

Support co-existence of SBFD with intra-DU LTM. Whether to support the co-existence between SBFD and other LTM cases is not discussed in the Rel-19 SBFD WI.

There are some components to consider here

- Applicability to early UL Sync: Since there is no UL band setting for the early UL sync scenario, it is hard to apply SBFD-RACH;

- Applicability to LTM execution (upon CSC): There is UL band setting provided, yet there is no RO-type indication in CSC command, which is the key decisive point for the applicability;

- Applicability to CLTM execution: There seems no R2 impact to support that.

[O000] R2 confirm SBFD-RACH is not applicable to early UL sync.

[O000] R2 discuss SBFD-RACH applicability to LTM execution upon CSC, by considering whether to introduce RO-type in CSC command MAC-CE

[O000] R2 confirm SBFD-RACH applicability to CLTM execution.

  - O001

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O001 | MOB, SLRelay | 1 | LTM, C-LTM applicability to intermediate Relay | R2-25xxxxx | OPPO (Qianxi) |  | V003 | ToDo |

[Description]: Based on the current spec, it is unclear whether LTM, CLTM based handover can be applied to the intermediate SL relay UEs.

[Proposed Change]: R2 to clarify whether LTM, CLTM based handover can be applied to the intermediate SL relay UEs or not.

[Comments]:

[Analysis]

During R18, it is concluded that LTM can be applied to relay but not remote UE, where R18 U2N Relay is limited to single-hop scenario.

For R19, there is a new UE type, i.e., intermediate Relay, due to the support of multi-hop U2N Relay.

Furthermore, legacy conditional handover (CHO) is not applicable for relay UE. For R19 CLTM, similar principle can be followed.

[O001] R2 confirm that LTM/CLTM is not applicable to SL intermediate-relay UE.

[O001] R2 confirm that CLTM is not applicable to SL relay UE.

  - O003

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O003 | SBFD, NES | 2 | Applicability of NES related RACH parameters | R2-25xxxxx | OPPO (Qianxi) |  | V003 | ToDo |

[Description]: NES introduced parameters for additional RACH (addlRACH-Config-Adapt-r19) within RACH-ConfigCommon, it is unclear whether it is applicable to the case when SBFD RACH is configured, for both cases of sbfd-RACH-SingleConfig-r19 and sbfd-RACH-DualConfig-r19.

[Proposed Change]: R2 discuss and conclude the applicability of NES additional RACH (addlRACH-Config-Adapt-r19) within RACH-ConfigCommon, to the case when SBFD RACH is configured, for both cases of sbfd-RACH-SingleConfig-r19 and sbfd-RACH-DualConfig-r19.

[Comments]:

[Analysis]

In R19, NES introduce the additional RACH configuration via the following IE within RACH-ConfigCommon

RACH-ConfigCommon ::=               SEQUENCE {

<Text Removed>

[[

addlRACH-Config-Adapt-r19               RandomAccessAdaptConfig-r19                              OPTIONAL           -- Need R

]]

}

While SBFD introduce two types of RACH configuration, one is to rely on legacy rach-ConfigCommon, and the other is to rely on new sbfd-RACH-DualConfig, which is essentially to introduce a new RACH-ConfigCommon.

BWP-UplinkCommon ::=                SEQUENCE {

<Text Removed>

rach-ConfigCommon                   SetupRelease { RACH-ConfigCommon }                                      OPTIONAL,   -- Need M

<Text Removed>

additionalRACH-ConfigList-r17       SetupRelease { AdditionalRACH-ConfigList-r17 }               OPTIONAL, -- Cond SpCellOnly2

<Text Removed>

sbfd-RACH-Config-r19[RIL]: O000, MOB,SBFD                         CHOICE {

sbfd-RACH-SingleConfig-r19                   NULL,

sbfd-RACH-DualConfig-r19

}                                                                                                        OPTIONAL  -- Need R

]]

}

AdditionalRACH-ConfigList-r17 ::=       SEQUENCE (SIZE(1..maxAdditionalRACH-r17)) OF AdditionalRACH-Config-r17

AdditionalRACH-Config-r17 ::=       SEQUENCE {

rach-ConfigCommon-r17               RACH-ConfigCommon                                                   OPTIONAL,  -- Need R

msgA-ConfigCommon-r17               MsgA-ConfigCommon-r16                                               OPTIONAL,  -- Need R

...,

[[

sbfd-RACH-DualConfig-r19                                                OPTIONAL  -- Cond NoSingleConfig

]]

}

SBFD-RACH-DualConfig-r19 ::=                          SEQUENCE {

sbfd-AdditionalRACH-Config-r19                        RACH-ConfigCommon                           OPTIONAL,  -- Need R

sbfd-RACH-DualConfig-ValidRO-AcrossSymbolTypes-r19    ENUMERATED {enabled}                        OPTIONAL  -- Need R

}

Considering this

- There seems no problem to configure addlRACH-Config-Adapt-r19 within rach-ConfigCommon (no suffix) and rach-ConfigCommon-r17, together with sbfd-RACH-SingleConfig-r19 enabled;

- But it is problematic to introduce another addlRACH-Config-Adapt-r19 within sbfd-RACH-DualConfig-r19, which would lead to additional question like whether / how this extra addlRACH-Config-Adapt-r19 is to be (de)activated.

[O003] R2 confirm that it is feasible to configure addlRACH-Config-Adapt-r19 within rach-ConfigCommon (no suffix) and rach-ConfigCommon-r17, together with sbfd-RACH-SingleConfig-r19 enabled, but it is not supported to configure addlRACH-Config-Adapt-r19 within sbfd-RACH-DualConfig-r19.

  - O004

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O004 | NES, SLRelay | 1 | Applicability of PO bundling to SL Relay | R2-25xxxxx | OPPO (Qianxi) |  | V002 | ToDo |

[Description]: It is not clear whether the paging adaptation (i.e., PO bundling) feature can be applied to SL Relay UE (first/last Relay) and Remote UE.

[Proposed Change]: R2 discuss and conclude the applicability of paging adaptation (i.e., PO bundling) feature to SL Relay UE (first/last Relay) and Remote UE or not.

[Comments]:

[Analysis]

In R18 when PEI was introduced, it was concluded that it is compatible with SL Relay, since Relay UE can always directly monitor PO without reading PEI, regardless of remote UE capability for PEI.

But for R19 PO bundling,

- If remote UE supports PO bundling, but relay UE does not, it would cause paging message loss since relay UE cannot monitor PO for remote UE;

- Even if relay UE is capable of PO bundling, it would waste relay UE power to mandate it to monitor both types of PO (legacy and bundled PO), without message from remote UE to relay UE regarding the PO type that remote UE relies on;

Considering this complexity, it is suggested that R2 does not support co-configuration of PO bundling and SL Relay in a same cell.

[O004] R2 not purse co-configuration of PO bundling and SL Relay in a same cell.

  - O005

| RIL Id | WI | Class | Title | Tdoc | Delegate | Misc | File version | Status |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O005 | NES, LPWUS | 1 | Applicability of PO bundling to LP-SS | R2-25xxxxx | OPPO (Qianxi) |  | V002 | ToDo |

[Description]: It is not clear whether the paging adaptation (i.e., PO bundling) feature can be enabled together with LP-WUS feature or not.

[Proposed Change]: R2 discuss and conclude the applicability of co-configuring paging adaptation and LP-WUS feature.

[Comments]:

[Analysis]

In R19 NES, when PO bundling is introduced, the corresponding PEI adaptation is also introduced, to support PEI for the bundled PO, but there is no corresponding adaptation for LP-WUS which is being introduced at R19.

It is preferred to keep the compatibility of LP-WUS (which helps UE power saving) and PO-bundling (which helps network power saving), by enable co-configuration of both features.

[O005] R2 discuss to support co-configuration of LP-WUS and PO-bundling, i.e., separate LP-WUS configuration for legacy PO and R19 bundled PO.

3. Conclusion

Based on the discussion above, we made the following observations and proposals:

Proposal 1	[O000] R2 confirm SBFD-RACH is not applicable to early UL sync.

Proposal 2	[O000] R2 discuss SBFD-RACH applicability to LTM execution upon CSC, by considering whether to introduce RO-type in CSC command MAC-CE

Proposal 3	[O000] R2 confirm SBFD-RACH applicability to CLTM execution.

Proposal 4	[O001] R2 confirm that LTM/CLTM is not applicable to SL intermediate-relay UE.

Proposal 5	[O001] R2 confirm that CLTM is not applicable to SL relay UE.

Proposal 6	[O003] R2 confirm that it is feasible to configure addlRACH-Config-Adapt-r19 within rach-ConfigCommon (no suffix) and rach-ConfigCommon-r17, together with sbfd-RACH-SingleConfig-r19 enabled, but it is not supported to configure addlRACH-Config-Adapt-r19 within sbfd-RACH-DualConfig-r19.

Proposal 7	[O004] R2 not purse co-configuration of PO bundling and SL Relay in a same cell.

Proposal 8	[O005] R2 discuss to support co-configuration of LP-WUS and PO-bundling, i.e., separate LP-WUS configuration for legacy PO and R19 bundled PO.

4. Reference

[1] xxx