3GPP TSG-RAN WG2 Meeting #131b	R2-2507652

Prague, CZ, 13 – 17, Oct, 2025

Agenda Item:	8.1.2.2

Source:	Xiaomi

Title:	[X003][O301/S045/N114][X004][E040]Discussion on open issues of UE data collection

Document for:	Discussion

Introduction

In this contribution, we discuss the open issues of UE-side data collection and propose the corresponding TP.

Discussion

X-003

In current CR, UE can indicate the dataCollectionStart along with the dataCollectionCandidateConfigList or alone without dataCollectionCandidateConfigList. However, we understand the presence of dataCollectionCandidateConfigList can implicitly indicate UE is about to collect data. There is no need to report separate start indication in addition. On the other hand, if dataCollectionCandidateConfigList is not reported, the meaning of start indication alone is unclear. Following understanding is observed,

- the UE sends just a start indication and then it expects to receive the list of candidates from the network

- the UE sends just a start indication with the meaning that it can accept any data collection configuration that the NW may decide to provide

For the understanding (i), the start indication is used to request candidate data collection configuration, rather than indicate data collection is about to start. However, we understand NW should provide the candidate configuration to UE if UE is capable of UE data collection even without the request. Furthermore, RAN2 didn’t agree such an indication to request candidate configuration list.

For the understanding (ii), we doubt the usefulness of such indication. Because it assumes UE just wants to collect data and doesn’t care about the configuration. We understand UE requested data collection configuration is useful when UE is lack of data under certain specific configuration. If NW provides configuration randomly, the collected data may be useless.

Therefore, we propose to remove the dataCollectionStart. The presence of dataCollectionCandidateConfigList can implicitly indicate UE is about to collect data.

The corresponding TP is provided in section 4.1.

Proposal 1: [X003] is marked as PropAgree. TP in section 4.1 is adopted.

O-301/S-045/N-114

These RILs discuss the meaning/structure of dataCollectionStart. If the dataCollectionStart is removed according to proposal 1, these RILs can be closed.

Proposal 2: [O301/S045/N114] can be closed if the dataCollectionStart is removed according to proposal 1.

X-004

Currently, the candidate data collection configuration is provided via a list in OtherConfig. However, with a list structure, NW has to provide the full list upon modification or release for a certain entry. ToAddMod structure can be used to reduce the signalling upon modification or release for a certain entry. The corresponding TP is provided in section 4.2

Proposal 3: [X004] is marked as PropAgree. TP in 4.2 is adopted

E-040

RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. E040; AIML; 2; Ensure that UE does not request a candidate UE-side data collection configuration for which it already has a UE-side data collection configuration; R2-25xxxx; Ericsson (Andra); V022; ToDo. [Description]: According to the current procedural text in 5.7.4.3, the UE may request a candidate UE-side data collection configuration in UAI (received in otherConfig), even if it already has a corresponding UE-side data collection configuration (received in CSI-ReportConfig) for that candidate configuration. This is not a desired behaviour and it should be fixed.

In our understanding, for UE-side data collection, as long as the RS configuration(s) available at UE side is suitable for UE-side model’s training purpose, no matter what is the original purpose of network configuring such configuration(s), UE can perform UE-side data collection without requesting new configuration for UE-side data collection.

The only case where UE requests a candidate UE-side data collection is that the UE does not have any suitable configuration for its own training purpose. Therefore, we believe that:

- A good network implementation will provide candidate UE-side data collection configuration(s) different from existing configuration(s) that are already provided to the UE.

- Only a wrong UE implementation will request duplicate network configuration(s).

Proposal 4: [E040] is marked as PropReject. Network provides candidate UE-side data collection configuration(s) in OtherConfig that are different from existing configurations in CSI-ReportConfig. UE will not duplicately request for the same UE-side data collection configuration.

Conclusion

Based on the discussion in section 2, we have following proposals:

Proposal 1: [X003] is marked as PropAgree. TP in section 4.1 is adopted.

Proposal 2: [O301/S045/N114] can be closed if the dataCollectionStart is removed according to proposal 1.

Proposal 3: [X004] is marked as PropAgree. TP in 4.2 is adopted.

Proposal 4: [E040] is marked as PropReject. Network provides candidate UE-side data collection configuration(s) in OtherConfig that are different from existing configurations in CSI-ReportConfig. UE will not duplicately request for the same UE-side data collection configuration.

TP

TP for X-003

- 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

…

1>	if transmission of the UEAssistanceInformation message is initiated to report the UE preference to be configured with radio resources to perform UE-side data collection according to 5.7.4.2:

2>	include dataCollectionPreference in this UEAssistanceInformation message;

2>	if the UE prefers to be configured with radio resources to perform data collection:

3>	for each serving cell configured with candidate UE-side data collection configuration(s) in dataCollectionCandidateConfigList and for which the UE has one or more preferred radio resource configuration(s):

4>	include an entry in dataCollectionPreferredConfigurationList and set the content as follows:

5>	set the dataCollectionServCellIndex to the serving cell index of the cell;

5>	include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList;

NEXT CHANGE

- –	UEAssistanceInformation

The UEAssistanceInformation message is used for the indication of UE assistance information to the network.

Signalling radio bearer: SRB1, SRB3

RLC-SAP: AM

Logical channel: DCCH

Direction: UE to Network

UEAssistanceInformation message

-- ASN1START

-- TAG-UEASSISTANCEINFORMATION-START

DataCollectionPreference-r19 ::= SEQUENCE {

dataCollectionPreferredConfigurationList-r19     SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateList-r19      OPTIONAL,

dataCollectionStopConfigurationList-r19          SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionList-r19               OPTIONAL,

...

}

-- TAG-UEASSISTANCEINFORMATION-STOP

-- ASN1STOP

UEAssistanceInformation field descriptions

END OF CHANGE

TP for X-004

#### 5.7.4.3	Actions related to transmission of UEAssistanceInformation message

<Text Omitted>

2>	if the UE prefers to be configured with radio resources to perform data collection:

3>	set dataCollectionStart to start;

3>	for each serving cell configured with candidate UE-side data collection configuration(s) in dataCollectionCandidateConfig-ToAddModList and for which the UE has one or more preferred radio resource configuration(s):

4>	include an entry in dataCollectionPreferredConfigurationList and set the content as follows:

5>	set the dataCollectionServCellIndex to the serving cell index of the cell;

5>	include in dataCollectionCandidateIdList the dataCollectionCandidateConfigId associated with preferred configuration(s) from dataCollectionCandidateConfigParameterList;

NEXT CHANGE

- 6.3.4	Other information elements

<Text Omitted>

- –	OtherConfig

The IE OtherConfig contains configuration related to miscellaneous other configurations.

OtherConfig information element

-- ASN1START

-- TAG-OTHERCONFIG-START

DataCollectionPreferenceConfig-r19 ::= SEQUENCE {

dataCollectionCandidateConfig-ToAddModList-r19   SEQUENCE (SIZE (1..maxNrofServingCells)) OF DataCollectionCandidateConfig-r19   OPTIONAL, -- Need N

dataCollectionCandidateConfig-ToReleaseList-r19   SEQUENCE (SIZE (1..maxNrofServingCells)) OF ServCellIndex   OPTIONAL, -- Need N

...

}

DataCollectionCandidateConfig-r19 ::= SEQUENCE {

dataCollectionServCellIndex-r19                  ServCellIndex,

dataCollectionCandidateConfigParameter-ToAddModList SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfigParameters-r19  OPTIONAL, -- Need N	dataCollectionCandidateConfigParameter-ToReleaseList	SEQUENCE (SIZE (1..maxCandidateConfig-r19)) OF DataCollectionCandidateConfigId-r19 OPTIONAL, -- Need N

...

}

DataCollectionCandidateConfigParameters-r19 ::= SEQUENCE {

dataCollectionCandidateConfigId-r19         DataCollectionCandidateConfigId-r19,

resourcesForChannelMeasurement              CSI-ResourceConfigId                                     OPTIONAL, -- Need R

resourcesForChannelPrediction-r19           CSI-ResourceConfigId                                     OPTIONAL, -- Need R

associatedIdForChannelMeasurement-r19       AssociatedId-r19                                         OPTIONAL, -- Need R

associatedIdForChannelPrediction-r19        AssociatedId-r19                                         OPTIONAL, -- Need R

...

}

LoggedDataCollectionAssistanceConfig-r19 ::= SEQUENCE {

loggedDataCollectionBufferThreshold-r19     ENUMERATED {kB16, kB32, kB48, spare1} OPTIONAL, -- Need R

...

}

-- TAG-OTHERCONFIG-STOP

-- ASN1STOP

OtherConfig field descriptions

aerial-FlightPathAvailabilityConfig Configuration for the UE to indicate the availability of flight path information for Aerial UE operation.

applicabilityConfigCellId Indicates the serving cell that the applicabilitySetConfigList refers to.

applicabilityReportConfig Configuration for the UE to indicate the applicability of configurations subject to the applicability determination procedure.

applicabilitySetConfigId Indicates the ID of a set of prediction related parameters.

applicabilitySetConfigList Indicates for each serving cell the list of sets of prediction related parameters configured for UE applicability reporting.

bfd-RelaxationReportingConfig Configuration for the UE to report the relaxation state of BFD measurements.

btNameList Configuration for the UE to report measurements from specific Bluetooth beacons. NG-RAN configures the field if includeBT-Meas is configured for one or more measurements.

candidateBandwidth Indicates the bandwidth of the candidate frequency range around the center frequency.

candidateCenterFreq Indicates the center frequency of the candidate frequency range.

candidateServingFreqListNR Indicates for each candidate NR serving cells, the center frequency around which UE is requested to report IDC issues.

candidateServingFreqRangeListNR Indicates the candidate frequency range with the combination of the center frequency and the candidate bandwidth, around which the UE is requested to report IDC issues.

connectedReporting Indicates that the UE can report a preference to remain in RRC_CONNECTED state following a report to leave RRC_CONNECTED state. If absent, the UE cannot report a preference to stay in RRC_CONNECTED state.

dataCollectionCandidateConfigId Indicates the ID of a candidate configuration for UE-side data collection.

dataCollectionCandidateConfig-ToAddModList Indicates for each serving cell the list of candidate radio resources configured for UE-side data collection. The UE is not expected to perform measurements solely based on the configurations provided by this IE.

END OF CHANGE