3GPP TSG RAN WG2#132	R2-2508012

Dallas, USA, 17th - 21st, November 2025

3GPP TSG-RAN WG1 Meeting #122bis	R1-2508108

Prague, CZ, 13th – 17th October, 2025

Title:	LS on SBFD subband frequency location configuration

Release:	Release 19

Work Item:	NR_duplex_evo-Core

Source:	RAN1

To:	RAN2

Cc:

Contact person:	Yanping Xing

xingyanping@xiaomi.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

# Overall description

The following agreement was made in RAN1#122bis to capture the assumptions of firstDL-subbandlocationAndBandwidth, secondDL-subbandlocationAndBandwidth and ul-subbandlocationAndBandwidth in TS 38.213 Clause 11.1.

Agreement

Adopt the following TP in principle to Clause 11.1, TS 38.213.

## 11.1 Slot configuration ## *** Unchanged parts are omitted *** A downlink or flexible symbol provided by tdd-UL-DL-ConfigurationCommon can include an UL sub-band provided by ul-SubbandlocationAndBandwidth, a first DL sub-band provided by firstDL-SubbandlocationAndBandwidth and may additionally include a second DL sub-band provided by secondDL-SubbandlocationAndBandwidth, for a SCS configuration of any configured UL BWP or DL BWP, respectively, as provided by scs-SpecificCarrierList [4, TS 38.211]. The downlink or flexible symbol is then referred to as an SBFD symbol; otherwise, it is referred to as a non-SBFD symbol. Unless otherwise stated, the UE considers symbols in a slot indicated as downlink and as SBFD by tdd-UL-DL-ConfigurationCommon to be available for transmissions. Uplink symbols by tdd-UL-DL-ConfigurationCommon are non-SBFD symbols. An SBFD symbol or a non-SBFD symbol provided by tdd-UL-DL-ConfigurationCommon cannot change to a non-SBFD symbol or to an SBFD symbol, respectively, by other information. The UE is not provided coresetPoolIndex and is not configured to receive PDSCH according to more than one TCI states mapped to one TCI codepoint [6, TS 38.214] for a serving cell where the UE is provided SBFD symbols. For an UL sub-band, a first DL sub-band or a second DL sub-band, the frequency location of the sub-band is provided by the corresponding parameters, respectively: a common RB and a number of contiguous RBs provided by ul-SubbandlocationAndBandwidth, or firstDL-SubbandlocationAndBandwidth or secondDL-SubbandlocationAndBandwidth that indicates an offset and a length as RIV according to [6, TS 38.214], setting , and a value provided by offsetToCarrier for the subcarrierSpacing. *** Unchanged parts are omitted ***

Send an LS to RAN2 to inform that the assumptions of firstDL-subbandlocationAndBandwidth, secondDL-subbandlocationAndBandwidth and ul-subbandlocationAndBandwidth highlighted below are described in TS 38.213, clause 11.1 instead of clause 12.

firstDL-subbandlocationAndBandwidth Configures frequency domain location and bandwidth of the first DL subband. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 with assumptions as described in TS 38.213 [13], clause 12. The network does not configure this field for UL carriers.

secondDL-subbandlocationAndBandwidth Configures frequency domain location and bandwidth of the second DL subband. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 with assumptions as described in TS 38.213 [13], clause 12. The network does not configure this field for UL carriers.

ul-subbandlocationAndBandwidth Configures frequency domain location and bandwidth of UL subband. The value of the field shall be interpreted as resource indicator value (RIV) as defined in TS 38.214 with assumptions as described in TS 38.213 [13], clause 12. The network does not configure this field for DL carriers.

# Action

To RAN2

ACTION:

RAN1 respectfully asks RAN2 to take the above agreement into account.

# Dates of the next TSG RAN WG1 meetings

TSG RAN1 Meeting #123 	17th – 21st Nov, 2025	Dallas, US

TSG RAN1 Meeting #124 	9th – 13th Feb, 2026	Gothenburg, SE