3GPP TSG-RAN WG2 Meeting #132	R2-2508033

Dallas, USA, November 17-21, 2025

Agenda item:	10.3.1.2

Source:	Qualcomm Incorporated

Title:	Discussion on QoS and service awareness in 6G

WID/SID:	FS_6G_radio

Document for:	Discussion and decision

# Introduction

In this paper, we discuss how to support application/service awareness and how UE may leverage it to better 6G applications. We also discuss a few new schemes for L2 procedures, to better support QoS/QoE in 6G.

# Discussion

## Application/service awareness

### RAN awareness

#### At RAN2#131bis meeting, the following agreement on service-awareness in RAN was made:

#### Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcomings with the current NR QoS. Aim to look at a general framework.

Since the consensus among companies is to develop a general framework instead of enhancements specific to individual applications/services, we think that RAN2 should focus on aspects that are common among the expected 6G use cases in its initial study. One such commonality can be generalized traffic characteristics (or traffic metadata), for the following reasons.

Study on service-awareness in RAN is largely motivated by new applications such as XR, immersive communications and mobile AI. They are different from the legacy ones (e.g. eMBB) mainly because of their special traffic characteristics. For example, if an application applies some type of differential encoding or compression before transmitting its data, that will result in dependencies between packets within a traffic flow, and packets within the same QoS flow may have different importances. To provide better QoS support for such applications/services, e.g. ensure application-level instead of packet-level performance, RAN/UE needs to have more awareness about those traffic characteristics.

Although these applications (e.g. XR and mobile AI) may seem different in what they do, their traffic characteristics are often quite similar. This similarity stems from the fact that they all apply some types of source coding on the original raw data, albeit for different purposes. For example,

- An XR application may apply a compression algorithm to its video flows to lower their bit rates. Compression algorithms, which is a form of sourcing coding, work on leveraging underlying patterns or correlations between frames, which introduces dependencies among the output packets. Consequently, some packets become more important than others, because the successful reception of other packets depend on them.

- A mobile AI application may tokenize its input to identify the underlying structure within the data, so that the encoded data is easier for the AI model to understand and process mathematically. This translation in essence is also a form of source coding. Hence it is not surprising that there also exists dependency and different importances among tokens, similar to those in compressed videos.

Observation 1. 	Target applications in 6G (e.g. XR, mobile AI) exhibit similar traffic characteristics, such as dependency and different importances among packets.

From a scheduling and resource management standpoint, what matters most are traffic attributes such as bit rate, burst size, correlation between data, and the associated QoS requirements, rather than the type of applications that produce the traffic. In essence, if two traffic flows originate from entirely different applications but exhibit identical traffic characteristics and QoS demands, then there is no technical justification for RAN or UE to treat them differently. This principle underpins the rationale for having QoS, because it provides a simple abstraction of different applications for networks. And this abstraction enables networks to treat them in a unified and consistent way, based on nature of traffic instead of nature of application.

Observation 2. 	From a scheduling and resource management standpoint, what matters most are traffic attributes and QoS requirements, rather than the type of applications that generate the traffic.

We believe this principle remains valid for the study on service awareness in RAN. To apply this principle, RAN2 can start by identifying key traffic characteristics common among the target applications and then discuss how they can be identified or be made available to RAN and UE in a generalized framework. This initial study will help lay the groundwork for the next step, such as identifying whether and what new mechanisms are needed to support them in L2 procedures.

Proposal 1. 	Initial study on application/service awareness in RAN should focus on identifying traffic characteristics common among the target 6G use cases and exploring whether/how it can be made available to RAN and UE in a generalized framework.

### UE awareness

Much has been made of the idea of application/service awareness in the RAN last meeting, and how it can be introduced in RAN2 protocols. This discussion is captured below [2, 3] and emanated from an argument that support of new requirements may need RAN to track traditional QoS metrics as well as possibly some understanding of the application state on top of that.

R2-2507313 Requirements for L2 protocols InterDigital discussion Rel-20 FS_6G_Radio Observation 2.1-1: Interactive AI-based services are uplink heavy, latency sensitive and data sessions are characterized by periodic and/or irregular data bursts with varying volumes and inter-burst interval. Proposal 1: The 6GR QoS framework shall support uplink transfer of delay critical bursts of varying volumes and inter-burst interval in a resource efficient manner (e.g., without overprovisioning of radio resources). - Xiaomi asks what is the delay requirement for these types of services. Interdigital thinks that some of this examples have provided targets of 200ms, but this depends on use cases. - Apple wonders whether we are going to introduce a new scheduling mechanism or if we can use some of the existing solutions can handle these requirements. Interdigital explains the intention is to first understand the requirements and then we can look at solutions on how to achieve this at the end. - Mediatek thinks that this is more related to scheduling and wonders how QoS can help. Interdigital explains that we use QoS similar to XR to understand the priority of the packets and achieve resource efficiency. Observation 2.4.4-1: Advanced immersive applications and/or codecs can adapt in rate and/or modality to implement graceful QoE degradation. Proposal 6: 6GR QoS framework supports QoS ranging for QoE and/or rate-based adaptations e.g., the UE autonomously selects parameters within the NW-configured range of values. - Lenovo asks what type of parameters you have in mind and the motivation. Interidigtla explains it is general but one example can be PDCP discard and the UE can apply a timer within a configure value range based on traffic. - Qualcomm asks what is QoE, what metric is it, will we specify something or is it a subjective metric. Interdigital doesn’t think that we will specify spefic metrics. Proposal 7: 6GR QoS framework support service-based differentiation and sub-flow granularity e.g., based on L2 application-level awareness. - Oppo asks if the AI based service whether we consider all types including tokenized AI. Interdigital explains that we only had limited examples to show that AI application generate a lot more UL traffic but we can study other services. - Fujitsu asks what sub-flow refers to here as it has SA2 relationship. Interdigital thinks that RAN2 should first discuss and define what RAN needs for RAN awareness and then discuss with other groups.

While RAN2, RAN3, SA2, SA4 will need to jointly develop the 6G QoS framework, we would like to address a different shortcoming of NR or rather an unused opportunity that we can leverage in 6G. L2 protocols are mostly designed without taking into account the UE application awareness. In general, the RAN QoS framework involves the NW characterizing the QoS flows then assigning them to DRBs. Packets within the DRB get similar RAN PDCP/RLC/LCH configuration. In Rel-18 XR, this assumption was somewhat revisited with Rel-18 Low Priority PDU Set discard, having the UE classify incoming XR traffic into high and low priority and performing PDCP discard accordingly based on NW configured discard timers.

This principle can be further followed. While the QoS flow and DRB framework is expected to carry over from 5G, there is an opportunity to further -on top of this framework- design L2 protocols taking into account UE tracking of the application state. Considering that UE is both a RAN node and often the end-host running the application and terminating all the L3-L7 protocols, it can in theory adapt some protocol procedures based on this knowledge.

Observation 3. For many cases, a lot of QoE information is available at the UE on account of hosting the application as well as terminating all protocols across the stack. This allows close and straightforward tracking of the application state.

As a running example, let us consider the PDCP reordering timer for multimedia application. Often the application may have a small playback or jitter buffer to buffer frames for a smooth video playback experience. When this buffer is empty a video may stall (rebuffering event) and the experience is degraded. As we know, PDCP reordering timer holds the PDCP window for a configured duration (e.g. a typical value of 200ms is a common configuration). Suppose a PDCP reordering window is held waiting for a HARQ or an ARQ retransmission, and consider those two cases:

- Case 1: Playback buffer occupancy is high: In this case, it is better for the UE to follow t-Reordering and wait until the missing packet, then give the application (or all applications in the bearer) all their packets in-order as the NW intended in configuration.

- Case 2: Playback buffer occupancy is low or zero: In this case, it is better for the UE to advance the PDCP Rx window without the missing SDU before t-Reordering expires to minimize the impact of buffer underflow and user experience degradation, as a missing SDU is a less disruptive event that video stalling, from a user experience pov. This behaviour is not allowed by the spec. in NR.

In this case, what the UE does exactly in terms of reordering would not impact any network procedures or interoperability. The late packet may be quietly discarded (as it likely would be by the playback buffer anyway). Note that the issue in this example is not due to misconfiguration, just normal variations in application and network. Note also, that having the UE report all states of all applications to have quick semi-static or dynamic reconfiguration of t-Reordering is neither scalable nor practical and would just add overhead to the network and likely not be fast enough to be attractive for deployment.

This is just a simple example of how UE application awareness can be leveraged for performance improvement in RAN. Such opportunities can be further explored by RAN2 to better understand where can L2 protocols benefit from such UE application awareness and how protocols can be designed to ensure any procedures based on such awareness does not break interoperability or negatively impact the network otherwise.

Observation 4. Many L2 protocol parameters can be adapted by the UE with little or no impact of NW operation.

Proposal 2.  	RAN2 to discuss how UE application awareness can influence L2 protocols, e.g., UE ability to process traffic out of order when needed.

## QoS/QoE

Throughout 5G studies, user-plane enhancements centered around two performance objectives:

- Maximize throughput, for applications with traffic similar to FTP or file download;

- Guarantee delay target, for applications such as voice or XR.

However, in our observation, what matters for many applications used by smartphone users is a bit different from the above. For example, user experience of these applications depends on end-to-end latency. But typically, there is no specific delay target that is strictly required like in delay-critical applications. Its delay requirement is rather elastic, i.e. if a packet takes longer than average to deliver, the packet is still usable to the application. The only impact of the extra delay is degradation in user’s experience. For this reason, all the past enhancements designed for delay-critical QoS flows do not directly apply.

Merely increasing link rate does not always benefit this type of application. While higher link rate can help reduce transmission time for data packets, end-to-end latency is affected by several different sources of delays, e.g. scheduling delays, delays inherent in higher-layer protocols (e.g. TCP), etc. Once the link rate becomes sufficiently high so that transmission time becomes insignificant to other sources of delay, increasing the link rate further offers little additional benefits. Therefore, simply boosting link rate should not be the main strategy for improving user experience of this type of application in 6G.

Observation 5. 	Legacy mechanisms developed for delay-critical applications are not directly applicable for applications with elastic delay requirement. Simply increasing link rate does not always benefit them either.

The above analysis shows that new approaches need to be studied. Some potential directions that RAN2 may explore can be the following:

- Application with elastic delay requirements typically use transport layer protocols with congestion control such as TCP or QUIC. One characteristic of this type of protocol is that its transmission window keeps growing as long as no packet loss is detected. Growing window results in large buffer size, which causes long delay. Therefore, network and UE need to take a more active role in help higher layers in managing its buffer size – keep it short while avoiding loss in throughput (e.g. TCP needs to keep the end-to-end pipe fully occupied).

- Application-level bursts (e.g. HTTP objects) in these applications can have a wide range of sizes. For bursts with small sizes, the slow-start phase in higher layer protocols can be a non-negligible source of delay. In those cases, getting over the slow-start phase quickly or avoiding it (if possible) is also important to reduce the total delay.

#### These are merely two examples of potential solutions. Additional options can be studied to enable better user experience for applications with elastic delay requirements.

Proposal 3. 	Study L2 mechanisms that improve user experience (e.g. end-to-end delay) for applications with elastic delay requirements (e.g. mobile AI or any HTTP based app).

Traffic profiles of XR and immersive communication applications can be very different from those of legacy applications. For example, many XR applications can dynamically adjust their codec and thereby change the traffic parameters. Another example is that the QoS requirements for packets within the same traffic flow may dynamically change too, e.g. based on application contexts such as user movement, surroundings, etc.

These characteristics pose challenges for the legacy QoS and resource management framework, which assumes that traffic flows have steady profile and QoS requirements and hence UE only needs to be configured semi-statically. As a result, any change or update often requires extensive signaling, involving reconfigurations in both the core network and RAN. Clearly, QoS and resource management in 6G needs to be more dynamic to efficiently support XR and immersive communications.

Observation 6. 	The legacy QoS and resource management framework is not able to efficiently support 6G applications (e.g. XR, immersive communications) that can adapt their traffic and QoS requirements based on application context, link quality, etc.

#### Some potential directions that RAN2 may explore may include the following. For example, a single DRB may be supported by multiple LCHs and/or RLC entities. When packets in traffic flow change their priorities, bit rate or reliabilities (e.g. due to change in application context), they can be quickly shifted between different LCH and/or RLC. Alternatively, network can pre-configure a DRB with a few sets of L2 parameters such as LCH priorities, LCP and RLC parameters. UE autonomously chooses which set of L2 parameters to apply based on assistance information provided by the application (e.g. protocol header fields) or QFI marked by core network based on assistance information from the application.

Proposal 4. 	Study L2 mechanisms to support applications that can dynamically adapt their traffic and QoS attributes based on application context, link quality, etc.

## Enhanced RLC

Both 5G-NR (up to Rel-18) and LTE have had somewhat a similar starting point of allowing DRB traffic to operate in two RLC modes:

- UM mode: Rely on HARQ reliability. RLC only performs segmentation and reassembly. Typically used for voice since this traffic can withstand some L2 loss.

- AM mode: Supplements HARQ reliability with a secondary ARQ mechanism that, as a baseline, can push LCH reliability to 100%. Typically used for data, e.g. TCP traffic.

These protocol design choices have implicitly divided traffic into two classes, one class handled by AM which operates at high levels of reliability (e.g. 10-5 or 10-6) but may suffer from high latency due to repeated retransmissions, segmentation, HOL blocking, etc. and another traffic class handled by UM which minimizes RLC latency but completely relies on HARQ for error correction and does not track HARQ residual errors or attempt to establish reliability guarantees at the protocol level.

Observation 7.	Up to Rel-18, RLC for DRBs operated in two modes: 1) RLC UM: Minimized latency with no reliability guarantees at RLC level. 2) RLC AM: Operates at a 100% reliability with unbounded RLC latency (except for PDCP discard timer).

Throughout Rel-18 XR SI and beyond, RLC AM was revisited with the aim of improving latency (possibly at the cost of reducing reliability) attempting to establishing some balance between per-packet delay and RLC AM high reliability. Recall the objective and justification in Rel-19 XR WID [1]:

Specify the following user plane enhancements [RAN2]: -RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. RLC-AM is useful to limit data loss, however RLC-AM feedback or retransmission triggering mechanisms are not well adapted for short packet delay budgets applicable to XR traffic. Also, for RLC AM, considerable amounts of data may be in-flight, i.e. in the window, and there is no current way to avoid retransmitting this data, even if the data is old.

Subsequently, Rel-19 introduced some RLC AM related latency reduction procedures termed “avoiding unnecessary retransmission”, that relied on applying PDCP discard timer at the RLC level. The end result in was reducing the RLC AM latency by discarding packets that have been buffered longer than PDCP discard timer at the cost of increasing RLC Error rate.

Observation 8.	Rel-19 XR introduced some RLC AM latency reduction procedures, at the cost of RLC reliability.

An important thing to note about such Rel-18/Rel-19 improvements to reduce latency for XR is that they were introduced to an already stable Rel-15 UP design whereby the PDCP/RLC/MAC protocols, state variables, configurable parameters, timers, etc. were already in place, which somewhat limited the design space.

Furthermore, the lesson learned from 5G in that sense was that use cases are becoming more flexible in terms of their granular QoS requirements and L2 is expected to evolve (as it has in XR features) to play a more active role into adapting to diverse QoS requirements, at least in terms of balancing latency and reliability targets taking into account the discussion above on how user experience is affected by elastic delay requirements, packet relative importances, etc..

A day 1 design for L2 taking into account those requirements allow for protocols that are better prepared to support diverse use cases without relying on post-hoc feature development in later releases. For example, a discarding mechanism from Tx to Rx built into RLC AM from day 1 can enable more discarding policies tailored to specific use case needs in terms of latency and reliability trade-offs.

Proposal 5. 	RAN2 to study how RLC/ARQ can operate at different reliability and latency targets depending on diverse QoS requirements, e.g., by introducing Tx-discard/move window signalling to Rx.

# Conclusion

Based on the above analysis, we respectively request RAN2 to discuss and agree to the following proposals:

Application/service awareness at RAN

Observation 1. 	Target applications in 6G (e.g. XR, mobile AI) exhibit similar traffic characteristics, such as dependency and different importances among packets.

Observation 2. 	From a scheduling and resource management standpoint, what matters most are traffic attributes and QoS requirements, rather than the type of applications that generate the traffic.

Proposal 1. 	Initial study on application/service awareness in RAN should focus on identifying traffic characteristics common among the target 6G use cases and exploring whether/how it can be made available to RAN and UE in a generalized framework.

Application/service awareness at UE

Observation 3. 	For many cases, a lot of QoE information is available at the UE on account of hosting the application as well as terminating all protocols across the stack. This allows close and straightforward tracking of the application state.

Observation 4. 	Many L2 protocol parameters can be adapted by the UE with little or no impact of NW operation.

Proposal 2.  	RAN2 to discuss how UE application awareness can influence L2 protocols, e.g., UE ability to process traffic out of order when needed.

QoS/QoE

Observation 5. 	Legacy mechanisms developed for delay-critical applications are not directly applicable for applications with elastic delay requirement. Simply increasing link rate does not always benefit them either.

Proposal 3. 	Study L2 mechanisms that improve user experience (e.g. end-to-end delay) for applications with elastic delay requirements (e.g. mobile AI or any HTTP based app).

Observation 6. 	The legacy QoS and resource management framework is not able to efficiently support 6G applications (e.g. XR, immersive communications) that can adapt their traffic and QoS requirements based on application context, link quality, etc.

Proposal 4. 	Study L2 mechanisms to support applications that can dynamically adapt their traffic and QoS attributes based on application context, link quality, etc.

Enhanced RLC

Observation 7.	Up to Rel-18, RLC for DRBs operated in two modes: 1) RLC UM: Minimized latency with no reliability guarantees at RLC level. 2) RLC AM: Operates at a 100% reliability with unbounded RLC latency (except for PDCP discard timer).

Observation 8.	Rel-19 XR introduced some RLC AM latency reduction procedures, at the cost of RLC reliability.

Proposal 5. 	RAN2 to study how RLC/ARQ can operate at different reliability and latency targets depending on diverse QoS requirements, e.g., by introducing Tx-discard/move window signalling to Rx.

# References

[1] RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3

[2] RAN2#131bis Chair notes

[3] R2-2507313, Requirements for L2 protocols InterDigital	discussion Rel-20 FS_6G_Radio