3GPP TSG-RAN WG2 Meeting #132	R2-2508046

Dallas, USA, Nov. 17th – 21st, 2025

Source:	vivo

Title:	6GR design approaches and new services

Agenda Item:	10.2.3

Document for:	Discussion and Decision

- Introduction

In RAN2 #131bis, the Study on 6G Radio was discussed from the RAN2 point of view, and some principles for 6GR design was discussed as captured in [1]. In this contribution, we discuss some features, 6GR design approaches and new services for Day-1.

- Discussion

  - Features and design approaches for Day-1

    - Support of diverse device types

In the era of 6G, the mobile system will provide services to a wide range of devices, from low-end IoT devices to high-end advanced eMBB devices. The design of 6GR should learn from the lessons of LTE & NR systems, where several IoT device types (e.g., NB-IoT, (e)MTC, (e)Redcap) were introduced in later releases, leading to numerous technical paths serving similar services and causing significant deployment challenges. Therefore, a clear blueprint for 6G IoT should be established in the first release to avoid market fragmentation and compatibility issues in later releases.

Observation 1: Developing single, scalable 6G RAT to serve diverse device types, including eMBB and 6G IoT devices, is necessary for future IoT markets.

As per the 6G RAN SID [2], the 6GR design should natively support various device types using a single 6G-RAT. Although IoT services will be supported from 6G Day-1, eMBB is expected to remain the primary focus for mobile operators. Therefore, one principle for the single 6G RAT design is to minimize impacts on eMBB UE to ensure the optimal user experience.

To support various device types with a single 6G-RAT, a scalable 6GR design is required, and the following agreement on scalable 6GR design was reached at RAN1#122 meeting:

RAN1#122 Agreement Study a scalable 6GR design for diverse device types, considering aspects: - What should be commonly applicable to all 6G device types - FFS: add-on features dedicated to specific device types, if any

In other words, 6G features can be classified into two types: common features applicable for all 6G device types and features dedicated to specific device types. This principle also applies to RAN2. Specifically, RAN2 needs identify common features applicable to all devices and device-specific features from its perspective, particularly those required for both eMBB and 6G IoT devices. For example, IoT technologies for reduced power consumption and enhanced coverage are also critical for eMBB devices to ensure optimal user experience in specific scenarios. To support harmonization of eMBB and 6G IoT within a single 6G RAT, at least the following aspects needs to be considered:

- Power saving features: The 6G radio protocol design shall support technologies to improve energy efficiency that are mutually required by eMBB and 6G IoT devices. Power saving features, such as eDRX with very long cycles, LP-WUS, and UAI on reduced UE capability (e.g., to single Rx, reduced MIMO layers) introduced in 4G/5G, may still be leveraged in 6G network. Additionally, a common 6GR design for power saving features should be introduced, applicable to both eMBB and 6G IoT devices with minimal differences.

- Coverage enhancement features: The 6G radio protocol design shall incorporate native coverage solutions to ensure seamless service continuity across diverse use cases/scenarios. Extended coverage is another common 6G Radio design aspect applicable to both consumer eMBB and IoT applications. Furthermore, RAN2 studies should focus on developing efficient mechanisms to enable resource sharing for common control channels (e.g., for carrying system information) and supporting extended coverage for both initial access and RRC-connected mode.

By supporting common features required for both eMBB and 6G IoT devices, the network gains greater flexibility. For example, with a single 6G RAT deployed to serve both eMBB and 6G IoT, network adaptation can achieve energy savings. During periods of low eMBB data volume, such as nighttime, operator can deactivate the capacity layer BS and keep the coverage layer BS active to provide basic services for both 6G IoT and eMBB devices, as illustrated in right half of Figure 1. In such case, eMBB devices communicate with the network by leveraging relevant techniques from the common feature set, specifically the coverage enhancement features.

Figure 1:  Network energy saving if coverage enhancement features supported for all devices

In addition to common features applicable to all devices, certain configurations/parameters are shared across different device types. In LTE, MIB/SIB transmissions occur without continuous repetition, whereas NB-MIB/NB-SIB with continuous repetition was specified in a later release to enable wide coverage for NB IoT. However, this independent system information transmission mechanism is insufficient, as some common parameters, such as cell selection and cell access information, are configured separately. Therefore, another principle for the single 6G RAT design is to minimize signalling overhead for common configurations across different device types to improve resource utilization. For example, RACH configuration can illustrate this reduction in redundancy. Two types of RACH configurations are provided via SIB1 using different transmission methods: the first, used by both eMBB and IoT devices is provided by common SIB1 transmitted with repetition, while the second, used exclusively by eMBB devices, is provided by additional SIB1 transmitted without repetition. RAN2 should further study on this approach or alternative solutions to achieve signalling efficient system information transmission.

Proposal 1: From RAN2 perspective, study a scalable design for all device types in one network considering following aspects:

- Minimize performance impact on eMBB devices;

- Natively support common features (e.g., power saving, coverage enhancement) applicable to all device types;

- Minimize signalling overhead (e.g., SIBs) for common configurations for different device types.

    - MUSIM

MUSIM devices have become increasingly popular in many countries. A user may have both a personal and a business subscription in one device, or two personal subscriptions in one device for different services (e.g., use one individual subscription and one “family circle” plan). In 5G MUSIM enhancements were introduced starting in R17, Prior to R17, support for MUSIM within a device is handled in an implementation-specific manner without any support from 3GPP specifications, resulting in a variety of implementations and UE behaviours, some of which were not very network friendly (e.g., UE leaving operator network for MUSIM purpose without any notification). The enhancements of MUSIM to solve those issues in 5G are listed in the table below.

Table1: MUSIM function in R17 and R18

| Release | Function |
| --- | --- |
| MUSIM R17 | The UE requests to change UE ID for paging collision between MUSIM |
|  | The UE requests the short leaving (MUSIM gap without priority) for SIB receiving, RRM measurement, etc from another SIM card |
|  | The UE requests the long leaving for phone call from another SIM card |
|  | Adding voice cause in paging for voice response |
| MUSIM R18 | MUSIM gap handling with priority |
|  | UE dynamic capability changes for MUSIM purpose |

In 6G, it is worthwhile to introduce the MUSIM enhancements from Day-1, as MUSIM devices will be common scenarios covering nearly all devices, e.g., smart phone, cars, or robots. Hardware sharing challenges persist across different MUSIM device due to cost or interference constraints. Without standardized solutions from Day-1, diverse implementations and UE behaviours will continue to degrade user experience and negatively impact network vendors and operators.

Proposal 2：In 6G Day-1 deployment, support of MUSIM device to improve user experience and mitigate adverse network impacts, e.g., include paging cause, MUSIM gap, report long leaving and dynamic capability changes function in Day-1 in MUSIM device or all device types.

    - UE aggregation

UE Aggregation, also referred to as multi-path (MP) relay technology, is introduced in Rel-18. By aggregating the Uu connections from multiple terminals, the users benefit from the significant performance improvements in terms of higher data rates, lower latency, and enhanced connection reliability. As illustrated in Figure 2, a Remote UE can connect to a single BS through both a direct path and an indirect path. The indirect path is established through a Relay UE, which communicates with the BS using a non-3GPP connection (N3C). N3C includes, but is not limited to Wi-Fi, Bluetooth, or wired links. In the UE Aggregation architecture, the Remote UE leverages the Relay UE's additional radio resources and transmission power to extend its service coverage and improve peak data rates.

Figure 2. Illustration of Rel-18 UE Aggregation

According to our prototype verification, UE Aggregation technology shows 2-10x increase in uplink data rate, along with a 5-10% improvement in latency and reliability. Even in poor coverage areas, high-quality service can be sustained, fully demonstrating the potential of UE Aggregation technology.

From a network deployment perspective, relying solely on outdoor Base Stations to achieve indoor coverage is often insufficient. In scenarios where outdoor Base Stations are unreachable, indoor distribution system are typically deployed. However, these supplementary solutions require long-term and high-cost investments. Given that 6G technology is expected to help operators reduce CAPEX and OPEX, we believe it is essential to support UE aggregation from Day-1 of 6G deployment. UE aggregation provides a flexible, low-cost alternative. For example, as shown in Figure 3, mobile terminals (i.e., Remote UE and Relay UE) within in a UE aggregation Group may belong to family members or friends in proximity. A Relay UE located in an area (e.g., near windows or external walls) with a direct path to the network can provide on-demand coverage extension to the Remote UE. In another example, such as NTN with limited coverage, the Remote UE can leverage the Relay UE to boost transmission power, significantly improving the success rate of emergency call setup.

Figure 3. Illustration of UE Aggregation Deployment

Proposal 3：In 6G Day-1 deployment, support UE Aggregation to achieve significant gains in higher data rates and connection reliability in weak coverage scenarios, e.g., indoor or NTN case.

    - 6G voice and voice fallback

In our view, support of voice service is critical for 6GR. Voice services require minimum interruption during mobility scenarios, necessitating fast switching between 5G and 6G networks. Additionally, an effective fallback solution may be needed to ensure sufficient coverage and stable user experience, as it remains unclear whether 6G native voice would be lunched commercially in Day-1.

There are two potential approaches to deploy voice services on 6G Day-1:

- Case 1: 6G Native Voice Deployment 6G native voice deployed on Day-1 over 6G radio. If a UE moves out of 6G coverage, inter-RAT handover is required for service continuity for voice.

- Case 2: Voice Fallback to 5G No native 6G voice deployed for Day-1, and voice fallback to 5G.

UE initiates voice services directly in 6G when UE camping on or connected to a 6GS, enabling 6G to fully support all service types and provide better performance, particularly for concurrent voice and other 6G services. To support 6G native voice on Day-1, features such as voice indication in RRC Connection Request, paging priority, flow based QoS and mapping, SM signaling, UE capability, intra/inter RAT handover, suitable L2 configuration and mechanisms.

Table 2: RAN functions to support voice in 6G

| Feature Name | Protocol Layer | Category |
| --- | --- | --- |
| Voice, video, emergency indication in RRC Connection Request | RRC | Call setup success rate optimization |
| Paging priority | NGAP | Call setup success rate optimization |
| SSAC | RRC | Call setup success rate optimization |
| Flow based QoS, GBR, PBR, LCP, DRB combinations | MAC | Basic function |
| SM signaling | NGAP | Basic function |
| UE capability match | NGAP | Basic function |
| Intra/Inter RAT handover | system | Basic mobility |
| SPS | MAC | Power/Overhead optimization |
| RoHC | PDCP | Coverage/Overhead optimization |
| Short L2 SN | L2 | Coverage/Overhead optimization |
| MAC CE based rate adaption | MAC | Reliability and spectral efficiency optimization |

However, supporting 6G native voice alone may not be sufficient for Day-1 deployment. When a UE moves out of 6G coverage due to mobility, an inter-RAT handover for voice transmission would be triggered. Such interworking solution may require N26-like interface to be deployed to ensure acceptable interruption times. In early 6G deployments, limited 6G coverage may lead to frequent inter-CN node handovers, resulting in longer interruption times and increased signaling overhead compared to intra-CN node handovers.

Figure 4 An example procedure of IRAT handover from 6G to 5G

Another issue to consider is the call setup rate. Based on our experience and observations from 5G deployments, it took a long time, even more than one year, for VoNR to mature. According to our survey, statistics on successful call setup rate during early NR SA deployment are illustrated in figure 5. Even in areas with relatively good NR SA coverage, the successful call setup rate remains lower than that in LTE for most of the tested UE models. The deployment and performance of 6G native voice gradually maturing require accumulated time.

Figure 5 successful call setup rate for LTE and NR SA for Mobile-Originated and Mobile-Terminated cases

For Case 2, similar to the 5G voice rollout, 6G native voice may not be deployed from Day-1, and voice fallback to 5G would be enabled through IRAT handover or redirection.

Observation 2: 6G shall support natively voice from Day-1, however deploying 6GR voice across the entire network on Day-1 is challenging due to 6G limited coverage. Without voice fall back to 5G, voice may not be adequately supported during early 6G deployments.

Hence, we propose that

Proposal 4：In 6G Day-1 deployment, as a complement to 6G native voice in RAN, voice fallback to 5G should be enabled to prevent voice interruptions caused by frequent handovers and ensure a seamless user experience.

    - Energy efficiency (UE PowSav / NES)

Power saving and network energy efficiency have always been among the most critical design objectives of mobile communication systems. With the increasing diversity of 6G services and the ambition toward carbon neutrality, both UE power saving and network energy saving have become even more essential in the 6G RAN system design.

One of important lessons from 5G show that multiple features were introduced for the same target. For example:

On UE power saving, different features for the same purpose are introduced in different releases, while target different scenarios:

- PEI and LP-WUS in RRC IDLE for paging;

- DCP and LP-WUS in RRC CONNECTED for PDCCH monitoring,

On network energy saving, different features for similar purpose are specified:

- OD-SSB on SCell and SSB adaptation on SCell;

However, in practice, operators faced significant confusion on: which features should be deployed, and which combinations of features could bring actual gains. This complexity limited the commercial deployment of several standardized features. For 6G Day-1 system design, it would be beneficial to consolidate features serving the same purpose into an integrated framework, and evaluate their combined gains. Meanwhile, RAN1/2/4 should coordinate to avoid developing different solutions for the same purpose. This approach will provide clearer deployment guidance to operators and increase the likelihood of commercial adoption.

Proposal 5: From 6G Day-1, RAN2 aims to avoid duplicated functionalities with the same purpose for UE power saving or network energy saving by:

- Consolidation of the features with the same purpose into a unified framework, and jointly evaluate their deployment gains and corresponding impacts;

- Coordination among WGs to not define different approaches for the same energy efficiency target.

  - New services in Day-1

    - ISAC

Twenty-four sensing use cases (UCs) are captured in the SA1 requirements TR22. 870, such as low-altitude unmanned aerial vehicle (UAV) supervision, environment object reconstruction, sensing assisted communication in industrial park, etc. 3GPP RAN#109 reached agreement on sensing modes as follows:

Agreement： 6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic.

Among the six sensing modes, bistatic sensing modes including TRP-UE DL and UE-TRP UL can enhance coverage compared to monostatic sensing modes due to higher SNR gains, since UEs are distributed over the cell thus closer to the sensing target. Furthermore, communication links can be reused for Uu-based bistatic sensing without requiring dedicated signal/resources or hardware modifications, while monostatic sensing, however, would require full duplex capability.

Day-1 features shall include UE-related sensing. Specifically, UE-TRP bistatic and TRP-UE bistatic configurations shall be prioritized for support.​

Studies shall be conducted on UE-UE bistatic and UE monostatic configurations. Subsequent decisions on whether to include these two configurations in Day-1 features will be made based on the outcomes of the aforementioned studies.

- Proposal 6: Day-1 features shall include UE-involved sensing. Specifically, TRP-UE DL and UE-TRP UL sensing modes should be prioritized in 6GR.

    - Other emerging services

As for 6GR, it is foreseen that eMBB will remain a typical service scenario that would be widely commercial deployed. In the following part, we will discuss traffic characteristics and requirements for some identified new emerging services in 5G-A that will be further supported in 6G [4] and expected new types of services in 6G [5][6].

- Voice with AI codec

3GPP SA WG2 has been studying the support for IMS voice over NB-IoT NTN connected to EPC. Meanwhile, SA4 has also initiated studies on AI-based audio codecs. In satellite communication systems, the limited bandwidth constrains the data rate of voice service, which may result in degraded audio quality at the receiver side. To study the impact and explore potential optimization approaches, we first consider a geostationary satellite communication scenario.

In this scenario, the satellite remains nearly stationary relative to the ground base station, and the channel variation is relatively slow. Therefore, it can be assumed that the physical layer channel state information (CSI) can be accurately conveyed to the application layer, without the problem of outdated channel information. At the same time, due to the large propagation delay of the satellite link, it is difficult for the physical layer to perform fast adaptive adjustments. Based on these characteristics, it may be feasible to move the channel coding mechanism to the application layer for design and optimization. In addition, the system does not employ a hybrid automatic repeat request (HARQ) mechanism, nor does it introduce channel quality indicator (CQI) feedback.

In the following figure, we provide some evaluation result about the comparison between joint source-channel coding and transmission scheme and the baseline scheme. The baseline scheme is the current NTN physical layer scheme. In the joint coding scheme, the raw audio signal is processed by the encoder (removing the quantizer) to directly produce an encoded complex sequence.

The evaluation metric used is the Perceptual Evaluation of Speech Quality (PESQ). PESQ is an objective speech quality assessment method defined by the International Telecommunication Union (ITU-T) in the ITU-T P.862 standard (2001) [12]. PESQ produces a score (ranging from approximately 1.0 to 4.5), with the following interpretation:

4.5 ≈ Original speech quality (close to lossless)

4.0 ≈ Excellent quality (close to subjective rating of "Good" on a 5-point scale)

3.0 ≈ Acceptable quality

2.0 ≈ Noticeable distortion

1.0 ≈ Very poor

Figure 6 Evaluation result

Based on the above evaluation result, when the SNR is above 5 dB, the PESQ scores of both the baseline scheme and the joint coding scheme remain above 3. When the SNR drops below 5 dB, the PESQ of the baseline scheme rapidly falls below 3, whereas the joint coding scheme can still maintain a PESQ above 3. Notably, when the SNR falls below 0 dB, the joint coding scheme shows a significant PESQ improvement compared to the baseline scheme.

Observation 3: For voice with AI codec，joint coding scheme shows a significant PESQ improvement without any retransmission mechanism(i.e., HARQ and ARQ) comparing to the baseline scheme.

- Immersive communication services

Immersive communication known as eXtended Reality service was identified in 5G-A and in NR R18/R19, some functionalities have been defined in RAN, such as PDU set awareness, delay awareness transmission handling etc. Later on, Tactile and multi-modal communication service has been identified and it is concluded that synchronization between different media components is critical in order to avoid having a negative impact on the user experience [3] In Rel-19, MMSID is defined for RAN to aware different flows belong to one Multi-Modal service but detailed transmission co-ordination between different flows has not been defined. In 6G, we have a better chance to revisit such service requirements.

Besides, the haptics in 5G media services have been identified according to IEEE 1918.1 [7] and studied in SA4 in Rel-19[6]. Haptics service has distinct traffic characteristics (Figure 7) compared to traditional XR service. In [3], some extreme low latency requirements are raised the communication delay for haptic modality can be even less than 5 ms. Such stringent requirements were not fully addressed in 5G-A. Considering the presence of haptics media plays a significant role in improving the sense of presence and immersion. And the real-time tactile feedback in response to user interactions could much impact user experience, we should consider such type of services within immersive communication as well in 6G with the new designed advanced radio technology.

Figure 7 Haptic services traffic characteristics

Observation 4: Multi-modal communication is foreseen as a promising emerging application in 6GR, while coordination of the transmission among multiple flows (e.g., haptic, audio and video) within a multi-modal communication session, including intra-UE or inter-UE cases, and the efficient transmission of haptic traffic, are not yet well addressed in NR.

- Generative AI traffic

The following figure shows the traffic characteristics for generative AI services described in TR 22.870 clause 6.25 [5]. A single IP flow may have dynamically changed traffic characteristics according to different AI tasks. The latency and data rate requirements may vary differently pending to different AI tasks or users/computing status. Besides, the DL traffic is busty without fixed periodicity. Such busty traffics may also have different packet size and delay budgets.

Figure 8. traffic characteristic of typical GenAI applications

It can be concluded that one traffic flow for GenAI may have packets with various type and its characteristic change dynamically.

Observation 5: The traffic characteristics and requirements of generative AI service flow may vary significantly across different bursts and over time.

Moreover, for various GenAI services deployed, tokenized data may be transmitted directly through the mobile network in the coming 6G network. Thus, tokens may be visible to CN even RAN. As discussed in last RAN1 meeting [8], token is used as one of the model parameterizations for generative AI and the parameters may include Token size, Token arrival rate and Token delay budget. It is believed that this technical trend will depend on the future development of AI/ML-related technologies in application layer, and further SA4 input may also be required. So far, there is no essential differences between Tokenization and existing PDU/PDU set modelling can be obtained.

- Service on lightweight AI/AR glasses

In addition to smart phone, new types of devices are emerging in 6GR, e.g. light weight AR/AI glasses, robots. They are foreseen to be the promising new mobile terminal types due to emerging widely AI/AR usage especially in outdoor scenarios, which provide better user experience compared with traditional smart phones.  Due to that XR type of traffic and AI capability carried by such devices drain battery faster, the new devices are more sensitive to power consumption considering their size and weight. Thus, trade-off between energy efficiency and performance need to be considered. And it is also beneficial for other types of UE due to the fact that a growing number of terminals will be equipped with AI capabilities.

Observation 6: The emerging services running on lightweight glasses and AI capable UEs impose concurrent requirements for power saving and QoS such as comparably low latency and high data rates.

Proposal 7: RAN2 should study 6G RAN protocol and functional impacts to support emerging services from Day-1:

- Voice with AI codec schemes;

- Immersive communication including multi-modality and haptic traffic;

- Generative AI;

- New service on lightweight AI/AR glasses.

- Conclusion

In this contribution, we discuss some features, 6GR design approaches and new service on Day-1.  we have the following observations and proposals:

Support features and design approaches for Day-1

Observation 1: Developing single, scalable 6G RAT to serve diverse device types, including eMBB and 6G IoT devices, is necessary for future IoT markets.

Proposal 1: From RAN2 perspective, study a scalable design for all device types in one network considering following aspects:

- Minimize performance impact on eMBB devices;

- Natively support common features (e.g., power saving, coverage enhancement) applicable to all device types;

- Minimize signalling overhead (e.g., SIBs) for common configurations for different device types.

Proposal 2：In 6G Day-1 deployment, support of MUSIM device to improve user experience and mitigate adverse network impacts, e.g., include paging cause, MUSIM gap, report long leaving and dynamic capability changes function in Day-1 in MUSIM device or all device types.

Proposal 3：In 6G Day-1 deployment, support UE Aggregation to achieve significant gains in higher data rates and connection reliability in weak coverage scenarios, e.g., indoor or NTN case.

Observation 2: 6G shall support natively voice from Day-1, however deploying 6GR voice across the entire network on Day-1 is challenging due to 6G limited coverage. Without voice fall back to 5G, voice may not be adequately supported during early 6G deployments.

Proposal 4：In 6G Day-1 deployment, as a complement to 6G native voice in RAN, voice fallback to 5G should be enabled to prevent voice interruptions caused by frequent handovers and ensure a seamless user experience.

Proposal 5: From 6G Day-1, RAN2 aims to avoid duplicated functionalities with the same purpose for UE power saving or network energy saving by:

- Consolidation of the features with the same purpose into a unified framework, and jointly evaluate their deployment gains and corresponding impacts;

- Coordination among WGs to not define different approaches for the same energy efficiency target.

Support new services in Day-1

- Proposal 6: Day-1 features shall include UE-involved sensing. Specifically, TRP-UE DL and UE-TRP UL sensing modes should be prioritized in 6GR.

Observation 3: For voice with AI codec，joint coding scheme shows a significant PESQ improvement without any retransmission mechanism(i.e., HARQ and ARQ) comparing to the baseline scheme.

Observation 4: Multi-modal communication is foreseen as a promising emerging application in 6GR, while coordination of the transmission among multiple flows (e.g., haptic, audio and video) within a multi-modal communication session, including intra-UE or inter-UE cases, and the efficient transmission of haptic traffic, are not yet well addressed in NR.

Observation 5: The traffic characteristics and requirements of generative AI service flow may vary significantly across different bursts and over time.

Observation 6: The emerging services running on lightweight glasses and AI capable UEs impose concurrent requirements for power saving and QoS such as comparably low latency and high data rates.

Proposal 7: RAN2 should study 6G RAN protocol and functional impacts to support emerging services from Day-1:

- Voice with AI codec schemes;

- Immersive communication including multi-modality and haptic traffic;

- Generative AI;

- New service on lightweight AI/AR glasses.

- Reference

- RAN2 #131bis chair notes;

- RP-251881 6G RAN SID;

- R2-2205054 Early measurement for EPS Fallback	vivo, China Telecom, CMCC, SoftBank, China Unicom, Vodafone, Ericsson;

- TR22.261 Service requirements for the 5G system;

- Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond". (https://www.itu.int/dms_pubrec/itu-r/rec/m/R-REC-M.2160-0-202311-I!!PDF-E.pdf);

- TR22.870 Study on 6G Use Cases and Service Requirements;

- TR26.854 study on Haptics in 5G Media Services;

- O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019;

- R1-2506863, vivo, Spreadtrum, Discussion on issues related to support of IMS voice over NB-IoT NTN connected to EPC, Oct. 2025;

- S4-251333, vivo, [FS_ULBC] Considerations on measuring ULBC complexity, July 2025;

- https://github.com/descriptinc/descript-audio-codec;

- ITU-T, P.862, Perceptual evaluation of speech quality (PESQ): An objective method for end-to-end speech quality assessment of narrow-band telephone networks and speech codecs.