3GPP TSG-RAN WG2 Meeting#132	R2-2508047

Dallas, USA, Nov. 17th-21st, 2025

Source:	vivo

Title:	Considerations on User Plane functionalities for 6GR

Agenda Item:	10.3.1.1

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis meeting, the following agreements for 6GR User Plane design were made [1]:

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities, companies should focus on identifying the problems being addrest or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcomings with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

The general consensus is that RAN2 should first focus on defining the basic User Plane functionalities for 6GR. Subsequent discussions can then address the protocol layer design and the mapping of functions to sublayers.

In this contribution, we present our views on 6GR User Plane functionalities, covering both legacy features to be inherited from NR and potential new functions or enhancements, based on emerging service scenarios and evolving requirements in the next-generation networks.

- Discussion

  - Legacy functionalities to be inherited

The Layer 2 (L2) functions of NR Rel-15 were built upon LTE L2 as a baseline, primarily targeting eMBB services. Enhancements were introduced to accelerate L2 processing, enable QoS flow-level handling, and provide basic support for new 5G services such as URLLC. Subsequent releases have gradually added L2 features to better support IIoT and URLLC, including mechanisms like EHC and intra-UE prioritization. In 5G-Advanced, further improvements were made to accommodate emerging media types such as XR, with new L2 functions like DSR and PDU set-based handling.

Looking ahead to 6GR, the User Plane is expected to support three key service scenarios: Immersive Communication, Hyper Reliable and Low-Latency Communication (HRLLC), and Massive Communication [2]. As outlined in [2], immersive communication extends eMBB and demands high reliability, low latency, and large-scale connectivity. HRLLC builds on URLLC with even stricter latency and reliability requirements. Massive communication evolves from 5G mMTC, emphasizing ultra-low power consumption for long-life IoT devices.

By NR Rel-19, L2 has accumulated extensive capabilities to support eMBB, URLLC, and mMTC. When defining the baseline L2 functions for 6GR Day-1, we believe all the existing functions up to Rel-19 should be considered. However, directly inheriting the full set may lead to excessive specification complexity and standardization workload.

Therefore, we propose prioritizing L2 functions with strong commercial relevance in the initial 6G release. The following table provides a preliminary assessment of existing L2 functions up to NR Rel-19, indicating our recommendations for 6GR Day-1 inheritance.

For clarity: Yellow indicates functions requiring further discussion or coordination with other WGs. Grey indicates functions not recommended as Day-1 baseline. All other functions are considered suitable for inheritance.

Table 1. Initial analysis of the existing L2 functions in NR until Rel-19

| NR L2 functions | Analysis |
| --- | --- |
| Basic L2 functions in NR Rel-15 QoS flow to DRB mapping, Reflective QoS flow to DRB mapping, RoHC, SN maintenance, Security protection (ciphering, integrity protection), PDCP duplication, Out-of-order deliver, Reordering, timer-based SDU discard, ARQ, (re)Segmentation/assembly, Random Access, DRX, PHR, Timing Advance Maintenance, HARQ operation, Scheduling information reporting (SR/BSR), Uplink Scheduling (Dynamic grant/Configured grant), LCP, Multiplexing/demultiplexing, Downlink Scheduling (Dynamic assignment/SPS) | The L2 functions defined in NR Rel-15 can naturally serve as the baseline for 6GR. However, the following points should be noted: - The 6G QoS concept and framework are still under discussion in SA2. If the QoS flow concept is retained, L2 shall support QoS flow to DRB mapping. - Regarding reflective QoS, its practical usage in live networks is limited, and continuous checking of the RDI field in the SDAP header imposes processing overhead on the UE. Therefore, we recommend excluding reflective QoS flow to DRB mapping from 6G Day-1. - Other L2 functions from SDAP to MAC in NR Rel-15 should be inherited in 6GR. This does not preclude further enhancements, such as potential L2 ARQ improvements, which were agreed for further study in the last meeting. |
| L2 enhancements for IIoT/URLLC in NR Rel-16/Rel-17 Rel-16: Intra-UE prioritization, EHC, Enhanced PDCP duplication with up to 4 legs, Scheduling enhancements (multiplex active CG/SPS, new logical channel restriction) Rel-17: Survival time support, Enhancements to Intra-UE prioritization/multiplexing | Regarding L2 functions and enhancements for IIoT/URLLC: - Intra-UE prioritization/multiplexing, though specified in Rel-16, is complex and its deployment in NR remains uncertain. We suggest not considering it for 6GR Day-1. - EHC and survival time support are tailored for IIoT/TSN traffic. Their inclusion in 6GR Day-1 depends on operator interest in supporting IIoT integration. RAN2 may evaluate these features on a case-by-case basis. - Scheduling enhancements and enhanced PDCP duplication are incremental improvements over Rel-15 baseline features and can be adopted as part of the initial 6GR L2 design. |
| L2 enhancements for XR in NR Rel-18/Rel-19 Rel-18: Refined long BSR, DSR, DRX enhancements (rational DRX cycle, retransmission less CG), CG enhancements (multiple CG PUSCH transmission occasions with a CG period, unused CG occasion indication), PDU set based discard, PSI-based discard Rel-19: Delay-aware LCP, Enhanced DSR, RLC retransmission related enhancements, Uplink XR rate control | Immersive communication is expected to account for a significant portion of future network traffic. Therefore, native support for immersive services in 6GR Day-1 is highly desirable. These services require high data rates, bounded latency, and high reliability, while also posing challenges in supporting large-scale device connectivity over the radio interface. Devices such as XR glasses, constrained by form factor, further demand advanced power-saving mechanisms. In NR, key L2 enhancements for XR can be grouped as follows: - Scheduling information reporting to enable delay-aware scheduling; - Uplink scheduling enhancements for delay-critical transmissions; - Power-saving mechanisms tailored to XR traffic patterns; - XR awareness at RAN to improve user experience under congestion. These XR-related L2 functions can serve as the baseline for immersive communication support in 6GR Day-1. Further enhancements may be considered on a case-by-case basis. |
| Enhanced L2 functions for IAB | The primary impact of IAB and MBS lies in the network architecture, with minimal influence on the L2 User Plane. Therefore, we consider support for IAB/MBS in 6GR Day-1 to be non-urgent. |
| Enhanced L2 functions for MBS |  |
| Other L2 functions Rel-17: UDC | UDC can be beneficial for extending voice service coverage. However, given its limited deployment in NR and associated complexity, we recommend that RAN2 evaluate whether UDC support is necessary for 6GR Day-1. |

Based on the above analysis, we propose using the basic L2 functions from NR Rel-15, along with selected enhancements from later releases (e.g., PDCP enhancements and CG improvements), and XR-related L2 features as the starting point for 6GR L2 design.

To better meet 6G service requirements, reduce power consumption, and simplify L2 processing and overhead, it may be necessary to reorganize L2 functions across sublayers, introduce new functions, remove obsolete ones, or even eliminate certain protocol sublayers. We recommend that any L2 function enhancement, harmonization, or reallocation be evaluated on a case-by-case basis, with clear justification provided for inclusion or exclusion.

Proposal 1: The following L2 functions from NR can be inherited in 6GR Day-1 as the starting point for functions design of 6GR User Plane:

| - QoS flow to DRB mapping (if QoS flow is retained) | - RoHC | - SN maintenance |
| --- | --- | --- |
| - Security protection (ciphering, integrity protection) | - L2 duplication & duplication detection | - In-order/out-of-order delivery |
| - SDU discard (including timer-based, PSI-based), PDU set discard | - ARQ | - (re-)Segmentation/assembly |
| - Random Access | - Timing Advance Maintenance | - DRX, PHR |
| - Multiplexing/demultiplexing | - HARQ | - Scheduling information reporting (SR/BSR/DSR) |
| - Logical channel prioritization (including delay-aware LCP) | - Uplink scheduling (DG/CG) | - Downlink scheduling (Dynamic assignment/SPS) |

The following subclauses present our views on potential new L2 functions and enhancements for 6GR, evaluated on a case-by-case basis. Additional proposals related to scheduling and L2 retransmission are discussed in our companion paper [3].

  - Highly Efficient Security and Processing

In 5G system, a DRB can be configured to enable Integrity Protection function. Meanwhile, RAN2 has discussed issues related to integrity-protected data rate, e.g., by default, a UE has a minimum data rate capability requirement for supporting integrity-protected DRBs but cannot guarantee to support a relatively high or unlimited integrity protection data rate due to processing capability limitation. Consequently, additional integrity protection and verification operations pose challenges to the UE’s processing capability.

In contrast, the peak data rate in 6G will be significantly higher than that in 5G, and the number and complexity of organized PDUs in UE upon UL grant arrival will also be increased significantly.

To improve the processing efficiency of integrity protection operation and the assembly of a large number of L2 sub-PDUs in the case of 6G ultra-high data rate, PDCP SDU concatenation is an effective solution. The following figure gives an example of PDCP SDU concatenation.

Figure 1. PDCP SDU Concatenation

From the above illustration, it is observed that PDCP SDU concatenation can reduce L2 overhead by decreasing the number of L2 sub-header and MAC-I fields, which can also simplify L2 processing and reduce processing delay. The following table provides the number of bytes reduced for each sub-header at typical SN lengths, such as 12-bit and 18-bit SNs. Table 3 analyses the proportional reduction in L2 header overhead for typical SDU sizes with typical numbers of concatenated PDCP SDUs, demonstrating that the reduction is particularly significant for small SDU sizes.

Table 2. Reduced overhead per PDCP SDU

| Reduced | 12-bit SN | 18-bit SN |
| --- | --- | --- |
| PDCP overhead | 2 bytes (Header) | 3 bytes (Header) |
|  | 4 bytes (MAC-I) |  |
| RLC header | 2 bytes | 3 bytes |
| MAC subheader | 1 byte |  |

Note：Increased LI num in concatenated PDCP header equals to the reduced LI num in MAC header. Hence, the overhead impact of LI does not need to be taken into account separately.

Table 3. Reduced overhead in some typical scenarios

| L2 overhead proportion reduction (18-bit SN) |  |  |  |
| --- | --- | --- | --- |
| Num of concat. SDUs\SDU size | 40-byte SDU | 576-byte SDU | 1500-byte SDU |
| 5 | 30% -> 8% | 2.3% -> 0.73% | 0.87% -> 0.28% |
| 10 | 30% -> 5.3% | 2.3% -> 0.54% | 0.87% -> 0.21% |
| 20 | 30% -> 3.9% | 2.3% -> 0.44% | 0.87% -> 0.17% |

In the following, we demonstrate the benefits of PDCP SDU concatenation for improving security processing efficiency with simulation results. For PDCP security configuration of DRB, ciphering is usually mandatory and integrity protection is optional. Integrity protection typically consumes more processing time than ciphering and represents the bottleneck in security processing. We use three typical SDU sizes: 40 bytes, 576 bytes and 1500 bytes, which represent small packets (e.g., TCP feedback), medium packets (e.g., one of typical video packets) and IP frame packets, respectively.

| Figure 2. Processing Time Reduction percent for 40-byte SDU |  |
| --- | --- |
| Figure 3. Processing Time Reduction percent for 576-byte SDU | Figure 4. Processing Time Reduction percent for 1500-byte SDU |

The benefits of processing time reduction are observed across all scenarios with different security algorithms and typical data packet sizes, demonstrating that PDCP SDU concatenation improves processing efficiency for both integrity protection and ciphering. However, as the number of concatenated SDUs increases, the gain does not grow proportionally. Therefore, a moderate number of SDU concatenations is preferred, e.g., no more than 20 for small packets or no more than 10 for other cases.

Furthermore, PDCP SDU concatenation may be avoided in scenarios that data arrival with non-negligible intervals. This means concatenation prioritizes the SDUs already waiting processing in the buffer and does not need to delay the processing of the previous packet(s) to await subsequent packet(s) for concatenation. When PDCP processing is timely, concatenation may be not executed. Otherwise, appropriate concatenation is performed. Additionally, when link quality deteriorates and TB size decreases, PDCP SDU concatenation should be avoided or the number of concatenated SDUs is reduced to minimize the risks of segmentation in lower layer. PDCP SDU concatenation may occur in the following reasonable scenarios, e.g., when security processing causes to queuing and waiting, continuous data arrivals, one PDU set, in case of good link quality, etc. Details can be left to UE implementation and AI algorithms can further enhance the efficiency of PDCP SDU concatenation.

Proposal 2: RAN2 to investigate solution to improve L2 security processing efficiency and also reduce L2 sub-header overhead & processing complexity, e.g., concatenation in PDCP.

  - Single L2 SN design Consideration

In current 5G system, there are two separate SNs in RLC and PDCP layers, which are used for ARQ & (re-)segmentation functions and security & re-ordering functions respectively. As we proposed in subclause 2.1, these functions are suggested for support in 6G. We can investigate the possibility of joint SN design in 6G in order to avoid duplicated header overhead and simplify L2 processing. For example, it can be analyzed and evaluated whether ARQ & (re-)segmentation functions can be merged into the same layer where security and re-ordering functions are located. There are other overlapping parts, e.g., receiving window mechanism, between ARQ and re-ordering functions, which can be jointly considered with fast L2 retransmission mechanisms. (Re-)Segmentation is a real-time processing and should be performed according to UL grant and allocated resource size after LCP. It seems feasible to re-use SN of security for (re)segmentation in the cases of intra-node or non-split functional location.

However, in 5G, there are several architectures, e.g., DC, CU-DU split with F1 interface, split bearer for the cases of CA duplication/ L2 SL Relay/ IAB, which all have impacts on functionality/sub-layer split between two nodes or two entities. If these architectures would be supported in 6G Day-1 or later release, the joint design of L2 SN would also be impacted. We provide one example of functionality split with joint SN design in the following figure: higher part of a common layer locates in one node and lower part of that locates in the other node.

Figure 5. Function split with joint SN

In summary, joint SN design will bring some benefits, e.g., overhead reduction, avoidance of duplicated functions and then L2 efficiency improvement. Meanwhile, one common layer is slightly inconvenient to support the split architectures and inter-working with 5G. RAN2 can further evaluate the feasibility and detailed impacts, e.g., after decision of the whole network architecture by RAN plenary.

Observation 1: Same SN used for both ARQ & (re-)segmentation functions and security & re-ordering functions is beneficial for L2 overhead reduction and efficiency improvement.

Hence, we propose:

Proposal 3: RAN2 to investigate joint sequence numbering in order to improve L2 transmission efficiency in 6G.

  - C-DRX design

Background and Motivation

The Connected Discontinuous Reception (C-DRX) mechanism has played a key role in both LTE and NR systems. It was introduced as a fundamental RAN-side power saving feature that allows the UE to periodically monitor PDCCH while remaining inactive in between. The main motivation of C-DRX is to achieve an optimal balance between power saving and latency, by controlling PDCCH monitoring, retransmission scheduling, CSI/CQI reporting, and other uplink or downlink-related activities through DRX timers and active/non-active periods.

Power saving has always been a central objective in RAN system design. In 5G, the C-DRX framework was retained and further enhanced with additional mechanisms aiming for better UE power saving. For instance, DCP (DCI with CRC scrambled by PS-RNTI) in Rel-16 and LP-WUS (Low Power-Wake Up Signal) in Rel-18/19 were introduced to further extend the UE’s low-power states and minimize the need for frequent PDCCH monitoring. However, even with these advancements, the fundamental principles of C-DRX remain unchanged — including the management of long/short DRX cycles, retransmission control, and associated DRX timers.

Observation 2: C-DRX has been and remains a fundamental framework for balancing UE power consumption and latency, serving as the baseline mechanism in both LTE and NR.

Lessons Learned from 5G

During later 5G releases, it has been observed that while advanced wake-up mechanisms (e.g., LP-WUS) can bring additional power saving benefits, C-DRX remains indispensable for multiple reasons:

- It provides a flexible structure through configurable DRX parameters that can be tailored to diverse service requirements (e.g., eMBB, URLLC, mMTC).

- It ensures backward compatibility and predictable behavior for existing applications with periodic or semi-periodic data patterns.

- It supports robust retransmission control and CSI/CQI reporting timing, which are essential for maintaining link reliability and throughput stability.

However, 5G experience also shows that C-DRX alone may not always be sufficient for achieving optimal UE power saving, especially for bursty or non-periodic traffic patterns typical of new emerging services (e.g., XR, industrial sensing, or 6G-enabled intelligent surfaces). In such cases, DRX timers cannot effectively predict when to activate or deactivate the UE, leading to unnecessary wake-ups or delayed responses.

Observation 3: While C-DRX provides a mature and stable foundation, its efficiency is limited for services with irregular or unpredictable traffic. Lessons from 5G indicate the need for additional wake-up mechanisms that can adapt dynamically to traffic characteristics.

Considerations for 6G C-DRX Design

In 6G system design, the baseline C-DRX functionality can still serve as a starting point, including:

- Switching between short and long DRX cycles;

- Controlling retransmission timing and scheduling;

- Coordinating CSI/CQI reporting based on DRX activity.

For services with strong periodicity (e.g., sensor reporting, deterministic traffic), traditional C-DRX remains the simplest and most efficient solution. However, to meet 6G’s overarching goal of extreme energy efficiency and sustainability, wake-up-based power saving techniques must also be considered.

For non-periodic or random bursty traffic, wake-up mechanisms may be more suitable, possibly allowing the UE to stay in an even deeper sleep mode and wake up only upon detection of a specific wake-up signal. Whether such wake-up control should rely on C-DRX framework or be designed as an independent function remains a topic for further study. The coordination between C-DRX and wake-up mechanisms will be essential to avoid unnecessary complexity or signaling overhead.

Related aspects are further discussed in our contribution in [4], which includes more detailed proposals for wake-up-based operation.

Observation 4: For 6G, C-DRX should continue as the baseline mechanism for power control and retransmission scheduling, while wake-up-based enhancements are needed to efficiently handle non-periodic and bursty services. The relationship between C-DRX and wake-up control mechanisms should be further studied.

Inter-WG and Inter-TSG Considerations

The C-DRX framework directly interacts with:

- RAN1: for physical layer wake-up signal design and timing alignment;

- RAN3: for signaling coordination and control plane implications;

- SA2: for QoS and service-related requirements that may affect DRX configuration or wake-up operation.

Hence, close coordination among these groups is necessary to ensure consistent system behavior and to avoid overlapping functionalities. Any 6G evolution of C-DRX or wake-up-based power saving should be jointly evaluated with RAN1, especially considering service characteristics or mobility management may influence DRX behavior.

Proposal 4: The C-DRX design on short/long DRX switching, retransmission control, CSI/CQI reporting in 5G could be considered as a starting point for 6G. PDCCH monitoring control should be discussed together with other wake-up based mechanism(s).

  - Random Access design

The random access procedure is a fundamental procedure for UE initial access, RRC connection (re-)establishment, and uplink timing synchronization in cellular networks. From 5G RACH framework, we learnt that it fails to meet the flexible and efficient service requirements when targeting diverse service scenarios with varying coverage requirements, traffic loads, and data volume demands within a cell. Primarily, there are four limitations:

- Limitations of cell-specific RACH resource configuration:

Coverage mismatch:

- Compared with UEs located in cell center, UEs in cell edge require more robust RACH resources (e.g., longer preambles, more Msg1 repetitions) to overcome pathloss. Cell-specific configuration cannot well balance the diverse requirements within a cell.

Load imbalance:

- UE spatial distribution within a cell is typically uneven. Some sub-areas (e.g., subway platforms) have high UE density, while others (e.g., park corners) have low UE density. Cell-specific RACH resource configuration cannot allocate additional resources to high-density areas or reduce redundant resources in low-density areas.

- Complexity and rigidity of RACH partition:

5G introduces RACH partition to support feature combinations, such as SDT, CovEnh, (e)RedCap, and network slicing. However, this design brings complex and rigid MAC resource selection. UEs must first identify the appropriate RACH partition based on their applicable features, then select corresponding resources (potentially associated with a matched repetition number), and further determine the RA type (i.e., 2-step vs 4-step). This multi-step process incurs high UE complexity. What’s worse, once a UE selects a RACH partition, it cannot fallback to use other resources even if RACH attempt has failed for couple times. (e.g., due to sudden load surge in the selected partition).

- Insufficient Msg3 TBS granularity:

5G only supports two TBS categories, baseline Group A intends for 56 bits payload and Group B for larger payloads. This binary division cannot efficiently match data transmission scenario needs, especially for 6G advanced services (e.g., Msg3 carrying security information with 128-bit authentication tokens).

- Fixed UE power ramping step:

The fixed UE power ramping step (e.g., 2 dB or 4 dB or 6 dB) for RACH preamble transmission leads to access delay to some extent. If the step is too small: UEs may require multiple power increments to reach the minimum detectable power, causing repeated RACH failures and prolonging access time (e.g., 3–5 retries for cell-edge UEs).

Observation 5: Cell-level uniform configuration fails to capture area-specific load/coverage variations.

Observation 6: Fragmented and dedicated preamble/RO allocation for UE early identification leads to behavioral complexity and less-flexibility in resource reselection.

Observation 7: Insufficient Msg3 TBS granularity cannot efficiently match data transmission scenario needs.

Observation 8: Fixed UE power ramping step is not fitted with random access required shorter latency.

To address 5G’s limitations, 6G RACH must prioritize flexibility, efficiency, and low latency, with the following enhanced considerations. Replace cell-specific RACH with area-specific (e.g., beam-specific) configuration to address uneven UE distribution and intra-cell requirement differentiation. Meanwhile, dynamically adjust resources (e.g., RO adaption for suddenly crowded areas like event venues) via simple control signaling is expected. We envision that a simpler preamble partitioning framework. Enhanced Msg3 TBS with multi-granularity and potentially enable UEs to signal their intended payload size in the RACH preamble. Last but not least, allow UE-based step adjustment: the UE may automatically step the total power ramping value based on the number of RACH re-attempt for low latency traffic.

Proposal 5: RAN2 to investigate solutions to enable random access procedure characterized by flexible resource configuration, simplified UE behaviour, and low latency data transmission. Close collaboration between RAN2 and RAN1 is critical for translating the potential solutions into practical 6G standards.

  - Head of Line blocking consideration

One of the most notable developments in recent Internet evolution is the widespread adoption of the QUIC protocol. QUIC was designed to overcome several performance limitations of traditional TCP, particularly the Head-of-Line (HOL) blocking issue.

TCP enforces lossless transmission and in-order delivery, which can lead to HOL blocking when multiple independent service flows are multiplexed into a single connection. A packet loss in one flow delays the delivery of all subsequent packets, even if they belong to unrelated flows. In contrast, QUIC allows each service flow to be mapped to a separate stream within the same connection. These streams are independently managed, so packet loss in one stream does not affect others. QUIC effectively eliminates HOL blocking at the transport layer.

This lesson from Internet protocol design highlights a broader principle: multiplexing independent traffic flows into a single queue can introduce HOL blocking risks. A closer look at NR Layer 2 reveals similar multiplexing mechanisms that may also suffer from this issue.

- Case a): When multiple independent streams are multiplexed within a single QUIC connection, the current 5G system lacks the capability to distinguish them at the QoS level. As a result, all traffic from the QUIC connection is typically mapped to a single QoS flow, which is further associated with a single DRB in the RAN. In such a setup, if a packet is lost over the radio interface, subsequent packets-regardless of whether they belong to different QUIC streams-must be reordered at the PDCP layer. This leads to unnecessary queuing latency due to HOL blocking across unrelated streams. Although QUIC avoids HOL blocking at the transport layer, its benefit is limited by the current 5G design. SA2 has already raised concerns about QUIC content awareness in the context of 6G. It is recommended that RAN2 monitor SA2’s progress and assess potential impacts on RAN when needed.

- Case b): In NR, multiple QoS flows with similar characteristics can be mapped to a single DRB, which helps simplify RAN resource allocation and reduce signalling overhead. However, this approach may introduce unintended consequences when the QoS flows are logically independent at the upper layers. Specifically, if a packet loss occurs in one QoS flow, the PDCP layer may still perform reordering across all packets within the shared DRB. This can lead to HOL blocking for other unrelated QoS flows, resulting in unnecessary queuing delays and potential congestion. This effect should be carefully addressed in the design of 6GR, particularly for immersive traffic with stringent latency requirements. In such scenarios, minimizing queuing delay is critical to ensure seamless user experience and maintain end-to-end responsiveness.

- Case c): NR MAC supports multiplexing packets from multiple logical channels into a single MAC PDU. As per Layer 1 specification, a transport block can be forwarded to the MAC layer if it passes CRC verification. As a result, decoding failures in partial code blocks (CBs) or code block groups (CBGs) will block timely delivery of other successfully decoded CBs/CBGs within the same TB. Furthermore, in NR RLC, if certain segments of an SDU are lost, the received segments are eventually discarded. This behaviour can lead to inefficient resource utilization, especially when upper layer concatenation is supported and multiple IP packets may reside within a single segment. Enabling partial segment processing rather than dropping the entire SDU could improve resource efficiency and user experience.

Observation 9: PDCP re-ordering among multiple distinct QoS flows may cause unnecessary L2 queueing delay and potential congestion due to HOL blocking issue.

Observation 10: It is better to deliver partial data successfully received at lower layer to upper layer for processing instead of directly dropping, in order to improve user experience and radio resource efficiency.

Based on the above consideration, we recommend RAN2 to investigate potential HOL blocking issues in Layer 2 protocols and study mitigation solutions for 6GR. The design principles of the QUIC protocol may offer valuable insights.

Proposal 6: RAN2 to study potential HOL blocking issues mitigation solutions in Layer 2 protocols for 6GR.

- Conclusion

In this paper, we provide our thoughts on 6GR functionality design, and make the following observations and proposals:

Legacy functionalities to be inherited:

Proposal 1: The following L2 functions from NR can be inherited in 6GR Day-1 as the starting point for functions design of 6GR User Plane:

| - QoS flow to DRB mapping (if QoS flow is retained) | - RoHC | - SN maintenance |
| --- | --- | --- |
| - Security protection (ciphering, integrity protection) | - L2 duplication & duplication detection | - In-order/out-of-order delivery |
| - SDU discard (including timer-based, PSI-based), PDU set discard | - ARQ | - (re-)Segmentation/assembly |
| - Random Access | - Timing Advance Maintenance | - DRX, PHR |
| - Multiplexing/demultiplexing | - HARQ | - Scheduling information reporting (SR/BSR/DSR) |
| - Logical channel prioritization (including delay-aware LCP) | - Uplink scheduling (DG/CG) | - Downlink scheduling (Dynamic assignment/SPS) |

Highly Efficient Security and Processing:

Proposal 2: RAN2 to investigate solution to improve L2 security processing efficiency and also reduce L2 sub-header overhead & processing complexity, e.g., concatenation in PDCP.

Single L2 SN design:

Observation 1: Same SN used for both ARQ & (re-)segmentation functions and security & re-ordering functions is beneficial for L2 overhead reduction and efficiency improvement.

Proposal 3: RAN2 to investigate joint sequence numbering in order to improve L2 transmission efficiency in 6G.

C-DRX design:

Observation 2: C-DRX has been and remains a fundamental framework for balancing UE power consumption and latency, serving as the baseline mechanism in both LTE and NR.

Observation 3: While C-DRX provides a mature and stable foundation, its efficiency is limited for services with irregular or unpredictable traffic. Lessons from 5G indicate the need for additional wake-up mechanisms that can adapt dynamically to traffic characteristics.

Observation 4: For 6G, C-DRX should continue as the baseline mechanism for power control and retransmission scheduling, while wake-up-based enhancements are needed to efficiently handle non-periodic and bursty services. The relationship between C-DRX and wake-up control mechanisms should be further studied.

Proposal 4: The C-DRX design on short/long DRX switching, retransmission control, CSI/CQI reporting in 5G could be considered as a starting point for 6G. PDCCH monitoring control should be discussed together with other wake-up based mechanism(s).

Random Access design:

Observation 5: Cell-level uniform configuration fails to capture area-specific load/coverage variations.

Observation 6: Fragmented and dedicated preamble/RO allocation for UE early identification leads to behavioral complexity and less-flexibility in resource reselection.

Observation 7: Insufficient Msg3 TBS granularity cannot efficiently match data transmission scenario needs.

Observation 8: Fixed UE power ramping step is not fitted with random access required shorter latency.

Proposal 5: RAN2 to investigate solutions to enable random access procedure characterized by flexible resource configuration, simplified UE behaviour, and low latency data transmission. Close collaboration between RAN2 and RAN1 is critical for translating the potential solutions into practical 6G standards.

HOL blocking consideration:

Observation 9: PDCP re-ordering among multiple distinct QoS flows may cause unnecessary L2 queueing delay and potential congestion due to HOL blocking issue.

Observation 10: It is better to deliver partial data successfully received at lower layer to upper layer for processing instead of directly dropping, in order to improve user experience and radio resource efficiency.

Proposal 6: RAN2 to study potential HOL blocking issues mitigation solutions in Layer 2 protocols for 6GR.

- Reference

- R2-250xxxx, Report of 3GPP TSG RAN WG2 meeting #131bis, Prague, Czech Republic.

- Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond". (https://www.itu.int/dms_pubrec/itu-r/rec/m/R-REC-M.2160-0-202311-I!!PDF-E.pdf).

- R2-2508049, Discussion on 6GR scheduling and retransmission.

- R2-2508054, Discussion on 6GR UE Power Saving and NES.