3GPP TSG-RAN WG2 Meeting #132	R2-2508048

Dallas, USA, Nov.17th– 21st, 2025

Source:	vivo

Title:	Considerations on 6G RAN QoS

Agenda Item:	10.3.1.2

Document for:	Discussion and Decision

- Introduction

In RAN#109, Study on 6G Radio was updated in [1]. In last RAN2 meeting, we had initial discussion on 6G user plane and draw the following conclusion. In this paper, we provide some further consideration on 6G RAN QoS/QoE related issues.

Agreements on UP

- 1.	RAN2 first discuss/agree on the required functionalities for the protocol stack, and only afterwards discuss which protocol layer supports each functionality.  Discussions should focus on standalone architecture, until told otherwise by plenary.  When discussing functionalities companies should focus on identifying the problems being address or new requirements.

2.  	UP design should aim to be hardware-processing friendly while keeping memory requirements low and minimize data movements across protocol layer.

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework.

5.	Support at least the following scheduling schemes: dynamic grant and configured grant.  Further study configured grant.

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

7.	Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ.  Study should identify enhancements needed for HARQ and L2 ARQ.

- Discussion

  - New traffic and service requirements in 6G

As for 6GR, it is foreseen that enhanced Mobile Broadband (eMBB) will remain a typical service scenario that would be widely commercial deployed. In addition to this, [2][3] have also studied typical 6G service types and traffic models, covering applications such as immersive communication, tactile Internet, digital twins, and AI-native services. Furthermore in [4], some emerging new traffic characteristics have been studied. From these studies, it can be observed that new types of traffic characteristics have been identified for 6G systems. These characteristics differ from the conventional QoS dimensions used in NR, such as latency, reliability, and throughput. As a result, such new services may require new QoS functionalities or enhancements to current QoS framework in NR to better capture and manage these new traffic requirements.

In 6GR, several typical new services should be extensively studied:

- Immersive communication services

Immersive communication known as eXtended Reality (XR) service was identified in 5G-A as one of the most important emerging services. In [3], it is characterized as services with both high data rate and low latency requirement. And in NR R18/R19, some functionalities have been defined in RAN to support related services, such as PDU set awareness, DRX enhancement, CG enhancement, delay awareness transmission handling etc. Later on, Tactile and multi-modal communication service has also been identified and it is concluded that synchronization between different media components is critical in order to avoid having a negative impact on the user experience, particularly when the synchronization threshold between two or more modalities is less than the latency KPI for the application [3]. The corresponding multi-modal interactive system from TS 22.261 is shown in Fig.1 below.

Fig.1 Multi-modal interactive system from TS22.261

Based on the study in [3], synchronized transmission is considered as the essential requirement for multi-modal services. Traditionally, application layer may guarantee the synchronization requirement, however as requirement grows more stringent and APP is lack of service flow mapping and real-time transmission status information in 5GS, it can hardly fulfil the requirements in adaptive and efficient way. In Rel-19, MMSID is defined for RAN to aware different flows belong to one Multi-Modal service. In 6G we have a better chance to revisit such service requirements when designing a brand-new RAN. For example, when we design the new BSR/DSR or LCP procedure, such synchronised transmission across different QoS flows can be taken into account as well.

Besides, the haptics in 5G media services have been identified according to IEEE 1918.1 [6] and studied in SA4 in Rel-19[7]. Haptics service has distinct traffic characteristics (as shown in Fig. 2 below) compared to traditional XR service and the typical QoS requirements for haptics are summarized in the following Table 1 [7]. Considering E2E 50ms transmission delay, less than 20ms RAN delay budget should be considered. And in [3], some extreme low latency requirements are raised as rendering and hardware introduce some delay, the communication delay for haptic modality can be reasonably less than 5 ms. Such stringent requirements were not fully addressed in 5G-A. Considering the presence of haptics media plays a significant role in improving the sense of presence and immersion. And the real-time tactile feedback in response to user interactions could much impact user experience, we should consider such type of services within immersive communication as well in 6G with the new designed advanced radio technology.

Fig. 2 Haptic services traffic characteristics

Table 1: Typical QoS requirements for haptics in haptic media enhanced services in TR26.854

|  | Haptics |
| --- | --- |
| Jitter (ms) | ≤ 2 |
| Delay (ms) | ≤ 50 |
| Packet loss (%) | ≤ 10 |
| Update rate (Hz) | ≥ 2000 |
| Packet size (bytes) per channel | 60-350 compressed parametric 50-1500 time sampled |
| Throughput (kbit/s) per channel | n*(fe*16)1 bits/s time sampled 16-32kbps for compressed parametric2 |

Table 2: haptic traffic in multi-modal communication service performance requirements in TR22.261

| Use Cases | Characteristic parameter (KPI) |  |  | Remarks |
| --- | --- | --- | --- | --- |
|  | Max allowed end-to-end latency | Service bit rate: user-experienced data rate | Reliability |  |
| Immersive multi-modal VR (UL: device application sever) | 5 ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.9% (without haptic compression encoding) 99.999% (with haptic compression encoding) | Haptic feedback |
| Immersive multi-modal VR (DL: application sever device) | 5 ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.9% (without haptic compression encoding) 99.999% (with haptic compression encoding) | Haptic feedback |
| Remote control robot | 1-20ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.999% | Haptic feedback |
|  | 20-100ms | 16 kbit/s -2 Mbit/s (without haptic compression encoding); 0.8 - 200 kbit/s (with haptic compression encoding) | 99.999% | Haptic feedback |
| Skillset sharing low- dynamic robotics (including teleoperation) Controller to controlee | 5-10ms | 0.8 - 200 kbit/s (with compression) | 99,999% | Haptic (position, velocity) |

Observation 1: Multi-modal communication is foreseen as one of the promising emerging applications in 6GR, while coordination of the transmission among multiple flows (e.g., haptic, audio and video) within a multi-modal communication session, including intra-UE or inter-UE cases, and the efficient transmission of haptic traffic, are not yet well addressed in NR.

- Generative AI traffic

The following Fig.3 shows the traffic characteristics for generative AI services described in TR 22.870 clause 6.25 [5]. A single IP flow may have dynamically changed traffic characteristics according to different AI tasks. The latency and data rate requirements may vary differently pending to different AI tasks or users/computing status. Besides, the DL traffic is busty without fixed periodicity. Such busty traffics may also have vary packet sizes and delay budgets. Table 3 also shows the detail KPIs for generative AI traffic concluded in TR 22.870.

Fig. 3. Traffic characteristic of typical GenAI applications

Table 3: Required KPI for GenAI applications

| Traffic Type | Burst size | Max Allowed latency for a burst | Service bit rate: user-experienced data rate | Delay | Average packet size |
| --- | --- | --- | --- | --- | --- |
| Image based GenAI app | 400 KB | 50 ms | 64 Mbps | 20 ms | > 800B |
| Video based GenAI app | 20 MB | 400 ms | 400 Mbps | 20 ms | > 800B |
| chatbot | 0.5 KB | 20 ms | 200 Kbps | 30 ms | < 800B |
| Note: meanings of some KPIs: max allowed latency for a burst: max latency for sending out the whole packets within a burst delay: transmission latency of a packet |  |  |  |  |  |

From the above services analysis, it can be observed that one traffic flow for GenAI may have packets with various type and dynamical changed characteristics.

Observation 2: The traffic characteristics and requirements of GenAI service flow may vary significantly across different bursts and over time.

Moreover, for various GenAI services deployed, tokenized data may be transmitted directly through the mobile network in the coming 6G network. Thus, tokens may be visible to CN or even to RAN. As discussed in last RAN1 meeting [8], token is used as one of the model parameterizations for generative AI and the parameters may include Token size, Token arrival rate and Token delay budget. It is believed that this technical trend will be implemented on the future development of AI/ML-related technologies in application layer, and further SA4 input may also be required. So far, there is no essential differences between Tokenization and existing PDU/PDU set modelling can be obtained. In other words, the identified traffic characteristics and corresponding requirements, e.g., packet error rate, packet importance, latency, integration, can be achieved based on the current QoS framework and concepts. As a result, a generic traffic characterization should be considered by first investigating issues of mapping token-communication to existing PDU/PDU set transmission concept.

Observation 3: Token based data transmission exhibits similar traffic characteristics and corresponding requirements, which could potentially be addressed within the current QoS framework by leveraging the existing PDU set concept.

- New service on lightweight AI/AR glasses

In addition to smart phone, new types of devices are emerging in 6GR, e.g., light weight AR/AI glasses, robots. They are foreseen to be one of the promising new mobile terminal types due to emerging widely AI/AR usage especially in outdoor scenarios, which provide better user experience compared with traditional smart phones. Table 4 illustrate the comparison between typical AR/AI glasses and smartphone. Since XR type traffic and AI capability embedded in such devices tend to drain the battery more quickly, these new devices are becoming sensitive to power consumption due to their limited size and weight. Thus, trade-off between energy efficiency and performance need to be considered for the new service on light weight AI/AR glasses. Considering the use of advanced coding schemes at the application layer, the transmission requirement may be relaxed in some scenarios to meet extreme power efficiency. Therefore, power consumption could be regarded as new metric that influences or dynamically adjusts the service QoS requirements.

It is also worth noting that considering such a trade-off between UE power saving and QoS fulfilment would be also beneficial for other types of UEs as well, given that a growing number of terminals are expected to be equipped with AI capabilities.

Table 4 weight and battery capacity comparison between typical AR/AI Glasses and smartphone

|  | AR/AI Glasses | Smartphone |
| --- | --- | --- |
| Weight | About 48g-79g | About 200g |
| Battery capacity | 200~260mAh | 3500-5000mAh |

Observation 4: The emerging services running on lightweight glasses and AI capable UEs impose concurrent requirements for power saving and QoS such as comparably low latency and high data rates.

Proposal 1: 6GR should study to support new services, including immersive communication including multi-modality including haptic traffic, generative AI and power sensitive devices or services, from Day-1.

Proposal 2: 6GR should study and identify the following new services with traffic characteristics and corresponding requirements. These characteristics and requirements should be considered in the 6G Day-1 design to ensure that the system can efficiently support and fulfil the needs of such services:

- Immersive communication service: synchronization for multi-modal service, critical latency and vary burst size for haptic traffic;

- GenAI: QoS requirements may vary significantly across different bursts and over time;

- New service on lightweight AI/AR glasses: trade-off between power saving and high performance.

  - Service awareness in RAN

In 5G NR, in order to support advanced traffic types as XR, service awareness relying on PDU set QoS parameters, PDU set information, and assistant information (e.g. periodicity, jitter, or data burst related information) has been defined. Most of the information is obtained from the server via AF and SMF.

While in future 6G, terminals are expected to be AI-capable and possess more contextual information to better understand end user needs and related services. Meanwhile, due to the diversity in UE capabilities, even when using the same application, the QoS requirements and perceived quality may vary among users, leading to increasingly personalized service demands. From RAN perspective, as the UE is close to both the client/service and RAN, it is in a favorable position to provide service related information in addition to that obtained via the AF from server. From UE perspective, direct interaction with RAN would rapidly response to the service variation, thereby improving end user experience.

With service-related information, including traffic characteristic or transmission/processing status from other node (e.g. UE or core network), RAN could further monitor QoS and optimize transmission in a more resource efficient manner. As a result, it is proposed to also study UE involved RAN service awareness in addition to service exposure from server via the core network. The exposed information may include, but is not limited to:

- PDU set/data burst level traffic characteristic variation, such as data rate, error rate, delay budget, etc.

- User preference and personalized service requirements

- Updates or changes in UE capability

In summary, as the UE possesses richer information on service requirements and QoE, UE involved service awareness should be studied and supported in RAN.

Proposal 3: RAN2 to study UE involved service awareness in RAN in order to enable fast and efficient transmission adaptation.

  - QoS monitoring and RAN exposure

Moreover, RAN related information such as radio congestion status, energy consumption or computing load is expected to have increasing impacts on end-to-end service performance. Such information is critical for UE to adjust the application layer behaviour, e.g. codec adaptation or redundancy configuration. In NR, similar mechanisms, like ECN, UL rate control, have been specified. It is noted that all the mentioned mechanisms utilize RAN congestion information to support various UE side adaptations. However, these functionalities were introduced relatively late in 5G (e.g. Rel-18/19), which limited its early adoption and integration into the ecosystem.

In 6GR, there is an opportunity to define a unified framework from Day-1 i.e. a common set of radio related information that can be utilized by the UE for multiple purposes, taking into account the diverse requirements of any future potential services. Moreover, UEs are expected to have enhanced AI capabilities and richer service context, enabling dynamic adjustment of codec, redundancy, or data rate at the application layer based on network status.

- Thus, in addition to the service awareness discussed above, detailed QoS collaboration between UE and RAN also need further study, and coordination with SA2 may also be required. A unified RAN exposure framework allowing UEs to obtain relevant RAN information in a standardized and efficient manner would enable UEs to perform service level adaptation based on real-time RAN conditions, improve resource utilization and energy efficiency, and support QoE enhancement for diverse services, including AI, immersive communication, and other latency-sensitive or energy aware applications.

Proposal 4: RAN2 to study a unified framework for RAN related information exposure to UE, e.g. radio congestion status, energy consumption, and computing load, to support efficient and adaptive service behaviour at the UE side.

  - Dynamic QoS requirements in RAN

As discussed in 2.1, the traffic characteristics and requirements (such as packet size, delay budget, packet error rate etc.) of emerging 6G service e.g. GenAI may vary significantly across different bursts within a service flow. Also, due to energy efficiency requirements of certain type of UE, QoS requirements may also need to be dynamically adjusted as a trade-off to provide better user experience. However, the current AF-centric dedicated QoS framework in 5GS is largely static, and QoS update is relatively slow due to the signaling exchanges among UE, RAN and UPF. Such mechanism poses challenges and inefficiencies in scenarios where traffic characteristics change rapidly between data bursts.

To achieve a better user experience with further service awareness, 6GS could be able to consider dynamic QoS adaptation in response to traffic variations. The potential dynamic QoS control may also require more flexible RAN resource allocation and transmission (re-)configuration in order to improve service adaption efficiency. This dynamic adaption could be performed in a pre-defined or semi-autonomous manner, with both RAN and UE participating while taking into account the efficiency, signaling overhead and implementation complexity.

In addition, most services other than voice are currently mapped to non-GBR QoS flow. However, such non-GBR mechanisms may face challenges in handling sudden high data rate traffic associated with new services, potentially leading to degraded service quality. As a result, how to achieve better QoS assurance than the best effort approach of existing non-GBR services while being less intensive resource than current GBR services in RAN should be considered.

Fig. 4: overall illustration of service awareness and adaption in RAN

Based on the design experience from previous generations, the QoS framework has generally been defined and specified in SA2, since QoS inherently involves cross-layer interactions and requires coordination across different system functions. Traditionally, RAN2 has not directly specified QoS mechanisms, but has provided the necessary support and corresponding hooks from the RAN side.

In the context of 6GR, the concept of dynamic QoS, potentially with UE involvement, as discussed above, introduces stronger dependency on RAN-side requirements and capabilities compared with previous generations. For example, dynamic QoS adaptation may rely on real-time measurement input, resource availability awareness, or latency-sensitive scheduling decisions at the RAN side. Therefore, it is important for RAN2 to identify and document the RAN-related requirements in advance, so that SA2 can take them into account when designing the overall QoS framework for 6GR.

Proposal 5: Study RAN impacts and requirements to support dynamic QoS adaption e.g., 6GR node controlled and UE involved. Coordination with SA2 is needed, e.g., some requirements on QoS design from RAN could be provided to SA2.

  - Multi-Modality traffic support in RAN

Immersive communication including multi-modality and haptic transmission have been studied briefly in 5G-A, however, the corresponding requirements are not fully addressed. In NR Rel-19 MMSID was defined in [9] to identify different QoS flows belonging to one multi-modality services. Such information could be transmitted to SMF and gNB for their implementation [10].

As analyzed in section 2.1 above, coordination of the transmission among related flows is considered as the key requirement. However, without detailed synchronization requirements at the QoS flow level or PDU set/packet level, the SMF and gNB cannot fully fulfill the requirements or optimize user experience by their implementation. This limitation is especially pronounced for UL: even if the UE obtains detailed synchronization information, without corresponding UL scheduling mechanism, UE can do nothing to improve the service transmission.

Furthermore, current CG/SPS transmission scheme is not well suited for haptic packets due to its aperiodic arrival pattern. Similarly, current URLLC solution is not sufficiently resource efficient, which may adversely impact other service data or UEs. Under the 6G radio design from Day-1, further investigation is needed on how to efficiently support haptic traffic and other immersive services.

For multi-modality services, the identification of synchronization requirements between QoS flows or at the packet level, as well as the corresponding study, needs to be conducted in coordination with SA side. At the same time, RAN2 also needs to study the mechanisms required to satisfy the identified synchronization requirements.

Proposal 6: RAN2 to study mechanisms to address transmission coordination between multiple related flows within one Multi-Modal service under 6GR design from Day-1 and efficient transmission for aperiodic haptic data with various packet size and extreme low delay budget taking into account of potential gain and complexity.

Proposal 7: RAN2 to study mechanisms to achieve the identified synchronization requirements between QoS flows and at the PDU set/packet level, in coordination with SA2.

- Conclusion

In this paper, we provide our consideration on 6G RAN QoS and the following observations and proposals:

Observation 1: Multi-modal communication is foreseen as one of the promising emerging applications in 6GR, while coordination of the transmission among multiple flows (e.g., haptic, audio and video) within a multi-modal communication session, including intra-UE or inter-UE cases, and the efficient transmission of haptic traffic, are not yet well addressed in NR.

Observation 2: The traffic characteristics and requirements of GenAI service flow may vary significantly across different bursts and over time.

Observation 3: Token based data transmission exhibits similar traffic characteristics and corresponding requirements, which could potentially be addressed within the current QoS framework by leveraging the existing PDU set concept.

Observation 4: The emerging services running on lightweight glasses and AI capable UEs impose concurrent requirements for power saving and QoS such as comparably low latency and high data rates.

Proposal 1: 6GR should study to support new services, including immersive communication including multi-modality including haptic traffic, generative AI and power sensitive devices or services, from Day-1.

Proposal 2: 6GR should study and identify the following new services with traffic characteristics and corresponding requirements. These characteristics and requirements should be considered in the 6G Day-1 design to ensure that the system can efficiently support and fulfil the needs of such services:

- Immersive communication service: synchronization for multi-modal service, critical latency and vary burst size for haptic traffic;

- GenAI: QoS requirements may vary significantly across different bursts and over time;

- New service on lightweight AI/AR glasses: trade-off between power saving and high performance.

Proposal 3: RAN2 to study UE involved service awareness in RAN in order to enable fast and efficient transmission adaptation.

Proposal 4: RAN2 to study a unified framework for RAN related information exposure to UE, e.g. radio congestion status, energy consumption, and computing load, to support efficient and adaptive service behaviour at the UE side.

Proposal 5: Study RAN impacts and requirements to support dynamic QoS adaption e.g., 6GR node controlled and UE involved. Coordination with SA2 is needed, e.g., some requirements on QoS design from RAN could be provided to SA2.

Proposal 6: RAN2 to study mechanisms to address transmission coordination between multiple related flows within one Multi-Modal service under 6GR design from Day-1 and efficient transmission for aperiodic haptic data with various packet size and extreme low delay budget taking into account of potential gain and complexity.

Proposal 7: RAN2 to study mechanisms to achieve the identified synchronization requirements between QoS flows and at the PDU set/packet level, in coordination with SA2.

- Reference

- RP-252912	Revised SID: Study on 6G Radio

- Recommendation ITU-R M.2160-0: "Framework and overall objectives of the future development of IMT for 2030 and beyond". (https://www.itu.int/dms_pubrec/itu-r/rec/m/R-REC-M.2160-0-202311-I!!PDF-E.pdf).

- TR22.261 Service requirements for the 5G system

- S2-2508347 QoS Framework for 6G

- TR22.870 Study on 6G Use Cases and Service Requirements

- O. Holland et al., "The IEEE 1918.1 "Tactile Internet" Standards Working Group and its Standards," Proceedings of the IEEE, vol. 107, no. 2, Feb. 2019.

- TR26.854 study on Haptics in 5G Media Services

- R1-2508184 LS on traffic model study in RAN1

- TS 23.501

- TS38.413