3GPP TSG-RAN WG2 Meeting #132		R2-2508054

Dallas, USA, Nov. 17th – 21st, 2025

Source:	vivo

Title:	Discussion on 6GR UE PowSav and NES

Agenda Item:	10.3.3.2

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis meeting, network and UE energy efficiency was discussed and design goal has been agreed as below:

1. Strive to develop solutions that consider network energy efficiency and UE power saving 2. For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this contribution, we will provide our views on UE power saving and network energy saving.

- Discussion

  - General

Power saving and network energy efficiency have always been among the most critical design objectives of mobile communication systems. With the increasing diversity of 6G services and the ambition toward carbon neutrality, both UE power saving and network energy saving have become even more essential in the 6G RAN system design.

In 5G, power saving mechanisms were introduced and enhanced across multiple releases. For example:

- C-DRX and I-DRX provided fundamental control over UE monitoring activity;

- RRC Inactive state helped reduce unnecessary signaling and activity;

- DCP in Rel-16 further improved UE sleep opportunities;

- PEI/LP-WUS in Rel-17/19 aimed at enabling ultra-low-power paging monitoring.

These mechanisms achieved notable power reduction in specific use cases and traffic conditions. Similarly, on the network side,  network energy saving techniques have been proposed in NR Rel-18/19 to reduce overall energy consumption in RAN, such as cell DTRX for CONNECTED mode, SSB-less SCell, OD-SIB1, OD-SSB on SCell, etc.

However, the experience from 5G also revealed that fragmentation of power saving features and lack of overall coordination created complexity during commercial deployment.

In 5G, a variety of UE power saving features were introduced at different releases, each targeting specific aspects of the system. Although these features work effectively in isolation, operators often found it difficult to decide which features to deploy and how to combine them for optimal benefit. Similar case happens in Rel-19, where both OD-SSB and SSB-adaptation on SCell are specified. At the end of the release, researchers may easily observe that most of the impact from OD-SSB operation and SSB adaptation are quite similar, and OD-SSB operation basically covers the full function of SSB adaptation.

Because the features were designed independently, the interactions and combined gains between them were not always clear. Some features may overlap in function (e.g., DCP and LP-WUS), while others may require complex parameter coordination. This led to confusion during deployment, as operators had to evaluate the practical trade-offs between energy efficiency, latency, and complexity on a case-by-case basis.

Therefore, one of the lessons learned from 5G is that similar-purpose features should be designed and evaluated together from the beginning. A unified framework that consolidates multiple power saving mechanisms toward a common goal would help simplify implementation and enhance commercial adoption.

Observation 1: Experience from 5G shows that introducing multiple power saving features independently leads to deployment complexity and unclear combined benefits. For 6G, features with similar objectives should be designed in an integrated manner, with unified evaluation of total gain to facilitate commercial deployment.

Another key observation from 5G design is that UE power saving and network energy saving were often treated separately, even though both are tightly interrelated.

Most 5G features were evaluated mainly from a UE perspective, focusing on how much the UE can save in terms of average or instantaneous power consumption. However, such improvements can sometimes increase network complexity or energy usage. Conversely, network side energy saving schemes may impact UE power efficiency, due to longer wake-up times or additional signaling from UE.

For 6G, it is therefore essential to adopt a holistic design principle, where both UE and network energy aspects are evaluated jointly. This means that whenever a new power saving mechanism is introduced, its impact on the other side (UE ↔ Network) should also be quantitatively assessed.

As noted in ongoing RAN1 discussions, 6G evaluation methodologies are expected to consider cross-dimension energy trade-off, where the benefit for one side should not come at an unacceptable cost for the other. A joint evaluation framework between UE and network energy saving is thus highly desirable.

Close inter-WG coordination is therefore required to avoid overlapping efforts and ensure consistent system-level behavior. For example, when RAN1 evaluates power saving gain on one side and corresponding impact on the other side, RAN2 study should focus on the detailed mechanism and signaling design.

Observation 2: In 5G, UE power saving and network energy saving were designed and evaluated mostly in isolation. For 6G, it is important to consider both gains and mutual impacts simultaneously to ensure that improvements on one side do not negatively affect the other.

Proposal 1: 6G design should jointly consider UE power saving and network energy saving. Specifically,

- when assessing gains for one side, the impact on the other side should also be taken into account.

- UE power saving or network energy saving mechanisms should be designed under a unified framework respectively, where features with similar objectives are integrated and jointly evaluated for overall system efficiency and deployability.

  - UE Power Saving

Efficient UE power saving has been a consistent target in mobile system design. With the transition towards 6G, this issue becomes even more prominent due to wider range of device types (e.g. Smart phone, XR, industrial IoT, metaverse, etc.), and the increasingly diverse set of services (e.g. eMBB, XR/immersive, Massive IoT, Sensing, AI, Voice, Regulatory service, etc.), based on the 6G study as shown in TR 38.914 [1], that demand extended device lifetimes, improved energy efficiency, and seamless connectivity.

    - Wake up in RRC CONNECTED state

In RRC connected mode, periodic PDCCH monitoring is a major contributor to UE power consumption. In 5G Day-1, C-DRX was introduced as the first standardized mechanism, where UEs monitor PDCCH during OnDurationTimer running. As discussed in our contribution in [2], we understand the C-DRX design  in 5G could be considered as a starting point for 6G.  However, since UEs must continuously monitor PDCCH during the regardless of traffic presence, this still leads to non-negligible power consumption, especially for some service with no periodic data patterns.

Therefore, additional mechanisms were introduced in later releases, e.g., DCP (DCI with CRC scrambled by PS-RNTI) in Rel-16 and LP-WUS/WUR (Low Power-Wake Up Signal/Wake Up Receiver) in Rel-18/19 to further reduce unnecessary PDCCH monitoring. Considering the increasing diversity of devices and use cases in 6G, a more efficient, unified, and flexible design for PDCCH monitoring optimization is required from Day-1.

In Rel-16, the DCP feature was standardized to allow UEs to wake up before OnDurationTimer to monitor DCP, which could skip subsequent PDCCH monitoring in case there is no scheduling.

In Rel-18/19, the LP-WUS/WUR feature was studied and then standardized, introducing wake-up signals to trigger UE monitoring of PDCCH. Two options were specified, which are shown in Fig.1.

- Option 1-1: Similar to DCP, where LP-WUS is coupled with C-DRX OnDuration

- Option 1-2: More flexible, allowing LP-WUS to be monitored at any time outside active time, which is decoupled from C-DRX.

Fig.1. LP-WUS operation in 5G

While both features target the same issue, i.e. reducing PDCCH monitoring power consumption. The coexistence of DCP and LP-WUS leads to functional duplication. In practice, LP-WUS (especially Option 1-2) achieves better power saving due to a separate wake-up receiver design, and lower latency performance with enough flexibility. For 6G, redundant features should be avoided, and a unified mechanism should be prioritized.

Besides, 5G Rel-19 standardized both Option 1-1 and Option 1-2 for LP-WUS. However, Option 1-2 can fully realize the functionality of Option 1-1 while being more flexible and efficient, as it decouples wake-up signaling from C-DRX PDCCH monitoring. Thus, standardizing both options was unnecessary and led to redundant complexity. For 6G, there is no need to standardize multiple duplicated options for the same functionality. A single, flexible solution (e.g., equivalent to 5G LP-WUS Option 1-2) is sufficient.

Observation 3: In 5G, multiple duplicated features for PDCCH monitoring increased complexity while providing limited incremental benefit. A single unified wake-up solution for PDCCH monitoring reduction should be prioritized in 6G Day-1, avoiding the delays seen in 5G.

The effectiveness of PDCCH monitoring power saving depends on the availability of mechanisms such as LP-WUS/WUR. In 5G, such features only became available in later releases, limiting the overall power saving gain in earlier deployments. Given the importance of UE power saving, PDCCH monitoring optimization (e.g., LP-WUS/WUR) should be supported from Day-1 in 6G, instead of being introduced gradually in later releases.

Finally, the design of PDCCH monitoring optimization requires cross-WG coordination:

- RAN1: Study signalling and receiver design aspects of LP-WUS/WUR, including waveform and detection.

- RAN2: Define procedures for LP-WUS monitoring, and PDCCH monitoring after waking up.

Tight coordination between RAN1 and RAN2 is needed to ensure that both physical layer design (signalling/receiver aspects) and higher-layer procedures for PDCCH monitoring are matched.

Proposal 2: For RRC CONNECTED state in 6GR, RAN2 considers to support wake up mechanism for PDCCH monitoring from Day-1 for UE power saving:

- LP-WUS/WUR is taken as the baseline, and there is no need to couple with C-DRX PDCCH monitoring, i.e. Option 1-2 in 5G is reused as the starting point;

- Coordination with RAN1, especially on the signaling design, is needed.

    - Wake up for paging reception

Paging reception is always one of the dominant contributors to UE power consumption in RRC_IDLE state. Therefore, efficient paging power saving mechanisms are essential from Day-1 in 6G.

In 5G, multiple features were introduced to address paging-related power consumption. Paging Early Indication (PEI) is specified in Rel-17, while Low Power-Wake Up Signal (LP-WUS) was further studied and specified in Rel-18/19. Both were intended to reduce UE power consumption for paging monitoring, but in practice, the introduction of multiple duplicated functionalities created additional complexity. From an implementation and deployment perspective, having several features targeting the same problem has increased the burden on UEs and gNBs, without delivering fully consistent gains across all scenarios.

Besides, between these two functionalities, LP-WUS was shown to provide significant benefits in terms of paging monitoring efficiency, with a dedicated low-power wake-up receiver (WUR). However, LP-WUS was introduced relatively late in 5G (Rel-18/19), which limited its early adoption and integration into the ecosystem. In contrast, PEI provided only limited improvements and became largely overshadowed by LP-WUS. This indicates that a strong, unified, and effective solution is preferable from the beginning of a new generation, rather than incremental and overlapping mechanisms.

Similar as in RRC_CONNECTED state, in 5G, multiple duplicated features for paging power saving (e.g., PEI and LP-WUS) increased complexity while providing limited incremental benefit. A single unified solution for efficient paging power saving should be prioritized in 6G Day-1, avoiding the delays seen in 5G.

For LP-WUS, another important aspect is inter-band operation. In 5G discussion in Rel-19, it was acknowledged that LP-WUS may not be supported in all main radio (MR) bands, but standardization of cross-band solutions (e.g., monitoring LP-WUS on one band while completing paging reception and subsequent RACH procedures on another band) was not completed due to limited time. This gap reduced the applicability of LP-WUS in real multi-band deployment scenarios. For 6G, such cases are expected to be even more common, given the increased diversity of spectrum usage (from sub-6 GHz to higher bands). Addressing inter-band paging and RACH procedures is therefore essential.

Finally, paging reception power saving spans both physical-layer signaling (LP-WUS/SS signaling design, receiver type design, detection reliability) and higher-layer procedures (RRC state, paging reception, RACH procedure). This requires close coordination between RAN1 and RAN2, and potentially with RAN4 on RF performance, e.g., sensitivity. Without such coordination, there is a risk of fragmented solutions, as seen in the split introduction of PEI and LP-WUS in 5G. Besides, whether LP-WUR could support all MR bands highly depends on the device type design in RAN1, i.e., OOK OOK-based or OFDM-based LP-WUR. Thus, the discussion on inter-band operation also needs inputs from RAN1.

Proposal 3: For RRC IDLE state in 6GR, RAN2 considers to support wake up mechanism for paging reception from Day-1 for UE power saving:

- LP-WUS/WUS design design in 5G is reused as the baseline, and inter-band/carrier operation needs to be studied.

- Coordination with RAN1, especially on signaling design, is needed.

    - RRM RLM BFD measurement relaxation

Motivation:

As discussed above, UE power saving is one of the key design targets for 6G, especially given the expected diversity of device types and service requirements. While features such as LP-WUS/WUR can reduce UE power consumption in paging reception in RRC_IDLE state, another major source of UE power consumption in RRC_IDLE state remains the periodic RRM measurement.

Although the RRM measurement framework for 6G has not yet been discussed/determined, it is highly likely that UEs will still be required to perform periodic measurements (most likely on both serving and neighboring cells) to ensure mobility operation, similar to 5G.

To guarantee mobility, UEs are required to perform periodic measurements on the serving cell and neighboring cell (in case the neighboring cell measurement condition on S(non)IntraSearchP and S(non)IntraSearchQ are fulfilled). This requirement applies even when the UE skips paging monitoring, as the UE still needs to wake up to perform RRM measurements. As a result, even if paging related power saving mechanisms are introduced, the UE still consumes significant power due to periodic RRM measurement, which limits the overall power saving gain.

Similarly, in RRC_CONNECTED state, a UE is required to periodically perform cell-level and beam-level measurements for RRM, RLM, and BFD. Without measurement relaxation or offloading, the benefit of power saving features (e.g., PDCCH monitoring relaxation) is significantly reduced.

Observation 4: 6G UEs most likely will continue to require periodic RRM measurements for mobility, and without RRM measurement relaxation/offloading mechanisms, the power saving benefits of paging-related features in RRC_IDLE (e.g., LP-WUS/WUR) or PDCCH monitoring in RRC_CONNECTED state will be significantly reduced.

Lessons learned from 5G:

5G introduced RRM measurement relaxation and offloading mechanisms progressively, with different applicability and varying designs across multiple releases:

- Rel-16: Introduced RRM measurement relaxation on neighboring cells for RRC_IDLE/INACTIVE.

- Rel-17: Introduced RRM measurement relaxation RRC_CONNECTED state, and RLM/BFD relaxation.

- Rel-19: With the introduction of LP-WUS and corresponding LP-WUR measurement on LP-SS, further enhancements on RRM measurement relaxation/offloading were made.

The RRM measurement relaxation mechanisms in RRC_IDLE/INACTIVE generally follow this principle: the network configures criteria for RRM measurement relaxation/offloading, the UE evaluates whether the criteria are met, and if so, performs measurements according to relaxed requirements defined by RAN4. The RLM/BFD measurement relaxation mechanism has a similar design, but the relaxation status needs to be reported to network side. While the RRM measurement relaxation in RRC_CONNECTED state is quite different: the network configures criteria for RRM measurement relaxation/offloading, the UE evaluates whether the criteria are met, and if so, reports to network and leaves the network to reconfigure the measurement.

It could be found that different criteria, relaxation factors, and applicability to different UE types (e.g., all UE, or only (e)RedCap/LP-WUS UEs) were standardized across releases, leading to fragmentation.

Besides, each release in 5G only standardized a subset of potential use cases, leading to incomplete use cases and lack of uniformity. For example:

- Some releases addressed only some RRC states or only on serving/neighboring cell measurement cell, but not both.

- Certain mechanisms applied only to specific UE types (e.g., (e)RedCap/LP-WUS UE only), leaving other device categories without equivalent support.

This piecemeal approach meant that a complete and consistent framework for RRM measurement relaxation was never achieved within 5G. Fragmented standardization of relaxation mechanisms across 5G releases highlights the need for a unified approach in 6G.

Observation 5: In 5G, RRM relaxation was introduced with varying criteria, relaxation factors, and applicable use cases or UE types across releases, resulting in inconsistent designs. This fragmented approach led to inconsistency in applicability and configuration.

Power saving has proven to be a critical feature for UE performance. However, in 5G, the power saving gain from RRM measurement relaxation/offloading and RLM/BFD relaxation only became available in later releases. Without these mechanisms, the power saving functions for paging reception in RRC idle/inactive or for PDCCH monitoring in RRC connected were significantly less effective, as measurement activity dominated UE consumption.

Thus, in 6GR, RRM measurement relaxation/offloading should be considered from Day-1, ensuring that the power saving functionality in all RRC states can achieve its intended benefits.

Finally, designing a unified RRM measurement relaxation/offloading and RLM/BFD relaxation for 6G requires coordinated work across different RAN WGs, e.g.

- RAN1: Study L1 measurement aspects, including timing, measurement filtering, and physical-layer signaling;

- RAN2: Study criteria and procedures for relaxation/offloading, and study the impact on mobility performance;

- RAN4: Study RRM/RLM/BFD measurement framework and corresponding performance requirements.

Proposal 4: The 6GR should study the RRM measurement relaxation/offloading and RLM/BFD relaxation for power saving purpose on serving and neighboring cell for all RRC states with a unified design, including criteria and mechanism, from Day-1.  Coordination with RAN1 and RAN4 is needed.

    - UE assistance information (UAI)

In 5G design, UE Assistance Information (UAI) mechanism represents another important dimension for UE power saving, which allows the UE to explicitly provide its preferred configuration to the network side, enabling the gNB to dynamically adjust configurations in a UE-aware and power efficient manner.

In 5G, UAI for UE power saving was introduced in Rel-16 to allow the UE to report its preferences regarding power saving. Specifically, the following information types were standardized:

- UE’s preference on DRX parameters for power saving, or

- UE’s preference on maximum aggregated bandwidth, or

- UE’s preference on maximum number of secondary component carriers, or

- UE’s preference on maximum number of MIMO layers, or

- UE’s preference on minimum scheduling offset for cross-slot scheduling, or

- UE’s preference on RRC state.

These mechanisms allowed the network to optimize UE configurations adaptively based on UE side constraints or power saving needs, thereby improving energy efficiency without rigid network side assumptions.

Lessons Learned from 5G and Motivation for Enhancement in 6G

Although the UAI mechanism in Rel-16 provided an effective starting point, several limitations and challenges were observed from the 5G design and deployment experience:

- Late Introduction: it was introduced from Rel-16 onwards, and not as part of the 5G Day-1 design. Consequently, its interaction with other power-saving mechanisms was not optimized from a system-level perspective.

- Limited Scope: The standardized UAI for power saving focused primarily on a few radio layer parameters (e.g., DRX, CCs, MIMO layers), without broader evaluation of which configurations or configuration sets most effectively contribute to UE power saving.

- Lack of Cross Evaluation: The Rel-16 design did not involve comprehensive system-level studies based on actual UE implementation and measurements to determine which configurations or combinations yield tangible power saving benefits.

As 6G aims to support much more diverse device types and traffic patterns, as identified in TR 38.914 [1], UE behavior and capability variations will become increasingly significant. Different devices, from lightweight IoT sensors to powerful XR terminals, will have very different power budgets, processing capacities, and data periodicity. Hence, the ability for a UE to assist the network with richer and more dynamic information becomes crucial.

In 6G, terminals are expected to be more intelligent and autonomous. Many UEs will have advanced sensing, AI-based context awareness, and the ability to predict their own traffic patterns and energy states. Therefore, UAI in 6G should not only indicate preferences on single parameters but also allow UEs to indicate aggregated configuration preference, e.g. combination of some parameters, to assist the network with insights into their expected activity or preference. Such extensions would allow the network to make smarter configuration adjustments, achieving joint optimization of performance and power saving.

To make this effective, tight cross-WG coordination is required for such study, e.g. RAN1/RAN4 to identify which parameters most strongly affect power consumption (e.g., number of CCs, MIMO layers, bandwidth).

Besides, as we known, UAI mechanism could also be useful for other purpose, e.g. overheating, IDC, MUSIM, etc. Thus, to ensure unified design and efficient deployment, RAN2 should also consider to define a common UAI architecture applicable across all these use cases from Day-1.

Proposal 5: For UE power saving in 6GR, RAN2 considers to support UAI based mechanism from Day-1:

- A unified framework on UAI should be supported for UE power saving and other purposes.

  - Network Energy Saving

    - Time-domain NES technique

Introduction

In NR Rel-18/19, time-domain NES techniques were studied and specified. The main approaches can be categorized as below:

- Not transmit signals at all: inter-band SSB-less SCell;

- Truncate tx/rx opportunities to enable longer off-duration: cell DTRX for CONNECTED mode;

- Cluster tx/rx opportunities to enable longer off-duration: paging adaptation by clustered PO configuration;

- Perform tx/rx in an on-demand manner: 1) OD-SSB on SCell; 2) OD-SIB1; 3) RACH adaptation (i.e. additional RO is activated by DCI to be available for a duration); 4) SSB adaptation on SCell (i.e. denser SSB can be provided on-demand)

Casting aside the NES techniques lead by other WGs or related with SIB/paging (which is to be discussed in CP AI), we think the cell DTRX operation for RRC_CONNECTED mode specified in NR Rel-18 NES can be a start for discussion on time-domain NES technique in RAN2.

NR Rel-18 Cell DTRX operation

In NR Rel-18 NES, two way-forwards as Figure X1 shows are proposed to realize a cell DTRX pattern:

- Way-forward 1 (explicit way): Introduce explicit cell DTRX configuration, and dynamically activate/deactivated it with group common DCI 2_9 (The activation/deactivation status in RRC configuration is only used upon setup of a cell DTX/DRX configuration);

- Way-forward 2 (implicit way): Rely on per-UE (re-)configuration of the channels/signals to realize cell DTRX;

Figure X1. illustration of two way-forwards to realize a cellDTRX pattern

As a result, the majority of companies agreed on having an explicit cell DTRX configuration and saw a benefit with L1 signalling for Cell DTX/DRX activation/deactivation from saving per-UE reconfiguration signalling’s perspective. During cell DTRX non-active period, tx/rx of CG/SR/SPS/PDCCH are not expected. RACH for PUCCH resource for SR during cell DRX non-active period is not supported. However, as an exception, the UE can initiate RACH on PCell for emergency call is this case. To avoid impact to initial access for IDLE/INACTIVE UE, SSB/RACH were not impacted by cell DTRX operation in CONNECTED mode in Rel-18.

Consideration for 6GR cell DTRX

In 6GR day-1, we observe that some companies propose to have a unified cell DTRX framework for all RRC mode, i.e. IDLE/INACTIVE/CONNECTED mode. From our observation, it is not easy to reach consensus on this issue at such an early stage to conclude whether we can have so-called unified solution, considering that:

- RAN1 has not yet concluded on how SSB/RACH is designed in 6GR day-1, and thus it is not possible at this stage for RAN2 to evaluate the impact to SSB/RACH from cell DTRX operation;

- In 6GR, we should aim to avoid introducing multiple solutions from different aspects for a same NES target. For instance, it is not desired that to achieve NES gain from no or less SSB transmission on PCell, by a lot of potential NES technique, such as SSB adaptation, joint SSB/RO adaptation, OD-SSB, cell DTRX, etc.

As such, we propose RAN2 to first identify the possible channels/signals that may be adapted to enable long duty-cycle operation for achieving NES gain, regardless they are for IDLE/INACTIVE mode operation or for CONNECTED mode operation, and then discuss whether we introduce an explicit cell DTRX configuration to adapt all the channels/signals or only part of them. With this procedural discussion, we can have a better understanding that to what extent can we achive a unified solution.

We understand that there are several possible way-forwards: 1. Fully rely on (re-)configuration of channels/signals to realize clustered channel/signal pattern; 2. Unified cell DTRX operation for IDLE/INACTIVE/CONNECTED mode, FFS the channels/signals that can be adapted; 3. (somewhere between 1&2) Reuse Rel-18 cell DTRX for CONNECTED mode operation, and the channels/signals for IDLE/INACTIVE mode operation are up to other methods to avoid duplicated NES design for the same channels/signals, e.g. dynamic adaptation for SSB/RACH in RAN1, semi-persistent configuration in SIB (e.g. Clustered PO configuration);

Proposal 6: Study time-domain NES technique for 6GR and identify the channels/signals that may be adapted to enable long duty-cycle operation for NES gain, with the following three alternatives:

- Alt. 1: Fully rely on (re-)configuration of channels/signals to realize clustered channel/signal pattern;

- Alt. 2: Unified cell DTRX operation for IDLE/INACTIVE/CONNECTED mode, FFS the channels/signals that can be adapted;

- Alt. 3: Only reuse NR Rel-18 cell DTRX for CONNECTED mode operation, and the channels/signals for IDLE/INACTIVE mode operation are up to other methods to avoid duplicated NES design for the same channels/signals, e.g. dynamic adaptation for SSB/RACH in RAN1, semi-persistent configuration in SIB (e.g. Clustered PO configuration);

    - Frequency-domain NES technique

In 6GR, support of NES has been widely considered as an overall design principle. Currently, when multiple carriers are deployed as independent cells, SSB and SIB are transmitted in each cell. To achieve NES gain, the system information of some carriers/cells (non-anchor carrier/cell) can be saved on these cells and transmitted on an anchor carrier/cell. Considering that the SIBs of the multiple cells may have part of the duplicated information (e.g. they share the same area-specific OSI), the anchor carrier/cell does not need to actually transmit the whole system information set for the non-anchor carriers/cells.

As for SSB, whether the multiple carriers/cells can share a same SSB on anchor carrier/cell for synchronization depends on whether the anchor carrier/cell can be the reference cell of the other non-anchor carrier/cell. These part of study is up to RAN4 for further study. If the SSB can be shared, it means the SSB transmission on non-anchor carrier/cell can be saved, so that NES gain can be achieved.

According to the NR Rel-18 NES SI report [3], SSB/SIB-less operation in multi-carrier scenario can achieve noteworthy NES gain depending on different assumptions:

With both SSB-less and SIB1-less from one of two carriers for non-CA operation, with assumption that UE is able to acquire sync from a carrier from another band, - compared to baseline of 20ms SSB/SIB1 periodicity on both carriers, one source shows BS energy savings by 55.1%~44.4% with RO periodicity of 20ms~160ms at different loads, and one source shows 9.1%~14.8% energy savings at empty load if an anchor carrier carries additional SIB1 for another carrier; - at different loads, compared to baseline of 20ms SSB/SIB1 periodicity, one source shows BS energy savings by 36.0%~13.5% when combined with simplified SSB (i.e. PSS and SSS only); - one source shows that with baseline of SSB+SIB periodicity of {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell, BS energy savings can be 9.4%~98.4% if an anchor carrier carries the SSB and SIB1 for another carrier depending on the traffic load. Meanwhile, the SIB1 carried on another carrier increase the energy of that carrier by 2.3%~17.8%, resulting a total saving across two carries by 7.1%~80.6%.

Therefore, we propose RAN2 to study frequency-domain NES technique with reduced or no always-on common signals (e.g. SSB/SIB) on some of the carriers/cells for multiple-carrier/cell deployment scenario in 6GR.

Proposal 7: Study frequency-domain NES technique with reduced or no always-on common signals (e.g. SSB/SIB) on some of the carriers/cells for multiple-carrier/cell deployment scenario in 6GR.

- Conclusion

In this contribution, we provide our initial consideration on UE power saving and network energy saving. Based on the discussion, we have the following observations and proposals:

General:

Observation 1: Experience from 5G shows that introducing multiple power saving features independently leads to deployment complexity and unclear combined benefits. For 6G, features with similar objectives should be designed in an integrated manner, with unified evaluation of total gain to facilitate commercial deployment.

Observation 2: In 5G, UE power saving and network energy saving were designed and evaluated mostly in isolation. For 6G, it is important to consider both gains and mutual impacts simultaneously to ensure that improvements on one side do not negatively affect the other.

Proposal 1: 6G design should jointly consider UE power saving and network energy saving. Specifically,

- when assessing gains for one side, the impact on the other side should also be taken into account.

- UE power saving or network energy saving mechanisms should be designed under a unified framework respectively, where features with similar objectives are integrated and jointly evaluated for overall system efficiency and deployability.

UE PowSav:

Observation 3: In 5G, multiple duplicated features for PDCCH monitoring increased complexity while providing limited incremental benefit. A single unified wake-up solution for PDCCH monitoring reduction should be prioritized in 6G Day-1, avoiding the delays seen in 5G.

Proposal 2: For RRC CONNECTED state in 6GR, RAN2 considers to support wake up mechanism for PDCCH monitoring from Day-1 for UE power saving:

- LP-WUS/WUR is taken as the baseline, and there is no need to couple with C-DRX PDCCH monitoring, i.e. Option 1-2 in 5G is reused as the starting point;

- Coordination with RAN1, especially on the signaling design, is needed.

Proposal 3: For RRC IDLE state in 6GR, RAN2 considers to support wake up mechanism for paging reception from Day-1 for UE power saving:

- LP-WUS/WUS design design in 5G is reused as the baseline, and inter-band/carrier operation needs to be studied.

- Coordination with RAN1, especially on signaling design, is needed.

Observation 4: 6G UEs most likely will continue to require periodic RRM measurements for mobility, and without RRM measurement relaxation/offloading mechanisms, the power saving benefits of paging-related features in RRC_IDLE (e.g., LP-WUS/WUR) or PDCCH monitoring in RRC_CONNECTED state will be significantly reduced.

Observation 5: In 5G, RRM relaxation was introduced with varying criteria, relaxation factors, and applicable use cases or UE types across releases, resulting in inconsistent designs. This fragmented approach led to inconsistency in applicability and configuration.

Proposal 4: The 6GR should study the RRM measurement relaxation/offloading and RLM/BFD relaxation for power saving purpose on serving and neighboring cell for all RRC states with a unified design, including criteria and mechanism, from Day-1.  Coordination with RAN1 and RAN4 is needed.

Proposal 5: For UE power saving in 6GR, RAN2 considers to support UAI based mechanism from Day-1:

- A unified framework on UAI should be supported for UE power saving and other purposes.

NES:

Proposal 6: Study time-domain NES technique for 6GR and identify the channels/signals that may be adapted to enable long duty-cycle operation for NES gain, with the following three alternatives:

- Alt. 1: Fully rely on (re-)configuration of channels/signals to realize clustered channel/signal pattern;

- Alt. 2: Unified cell DTRX operation for IDLE/INACTIVE/CONNECTED mode, FFS the channels/signals that can be adapted;

- Alt. 3: Only reuse NR Rel-18 cell DTRX for CONNECTED mode operation, and the channels/signals for IDLE/INACTIVE mode operation are up to other methods to avoid duplicated NES design for the same channels/signals, e.g. dynamic adaptation for SSB/RACH in RAN1, semi-persistent configuration in SIB (e.g. Clustered PO configuration);

Proposal 7: Study frequency-domain NES technique with reduced or no always-on common signals (e.g. SSB/SIB) on some of the carriers/cells for multiple-carrier/cell deployment scenario in 6GR.

- References

- 3GPP TR 38.914, Study on 6G Scenarios and Requirements,

- R2-2508047, Considerations on User Plane functionalities for 6GR, vivo

- TR 38.864, Study on network energy savings for NR