3GPP TSG-RAN WG2 Meeting #132		R2-2508056

Dallas, USA, Nov. 17th – 21st, 2025

Source:	vivo

Title:	Discussion on 6GR Mobility

Agenda Item:	10.4

Document for:	Discussion and Decision

- Introduction

In RAN#109 meeting, 6G SI in [1] has been approved with the below objectives related to mobility:

(4) Mobility for 6GR (for all RRC states), including related RRM [RAN2, RAN1, RAN4, RAN3]

In RAN2#131bis meeting, the mobility requirements have been briefely discussed, and the below agreements have been made:

Agreements 1 Study mobility with the following requirements in 6G mobility design: - Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions. - Robustness of mobility procedures - Energy efficiency for both UE and NW 2 Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this contribution, we will continue to discuss the details on 6G Mobility.

- Discussion

  - Scenarios and HO types for intra-RAT

Motivation

Mobility has always been a fundamental characteristic of cellular networks and a key driver in RAN system design. Optimizing mobility procedures not only ensures seamless connectivity and robust service continuity but also directly impacts user experience and network efficiency. Thus, for 6G, mobility design will remain a cornerstone of system performance.

As the endorsed 6G Requirements for architecture and migration in [2], the consideration of DC (dual connectivity) mobility enhancements is postponed until after March 2026, when RAN plenary will study migration options and provide further guidance on whether 6G-6G DC and/or NR-6GR DC should be considered. For now, RAN2’s 6G study should focus on standalone deployment and enhancements.

Observation 1: For 6G Day-1, the mobility study should prioritize standalone deployment scenarios.

Lessons learned from 5G

From Rel-15 onward, every release in 5G has introduced new mobility functions or enhancements. The major items are summarized as follows in Fig.1:

Fig.1 5G mobility evolution

From these mechanisms, several lessons can be learned:

- Only a very small subset of mobility features were actually commercialized.

- Some features introduced in later releases (e.g., LTM) had strong potential but were too late to reach widespread deployment in 5G.

- Some features introduced early (e.g., DAPS) were not deployed, e.g. due to high UE complexity.

- Mobility enhancements specific to DC scenarios were largely not commercialized.

This suggests that for 6G Day-1, RAN2 should focus on mobility mechanisms with realistic commercial potential.

Observation 2: 6G Day-1 mobility design should prioritize mechanisms with high commercial feasibility and deployment potential, rather than features with limited practical adoption in 5G.

Mobility types for 6G

From 5G experience, L3 HO is a universal solution applicable to all cell switch scenarios, including both Xn-based and NG-based HO. It provides a robust and standardized baseline for inter-cell and inter-RAT mobility. However, compared to LTM, L3 HO typically involves longer interruption time and less robustness especially for intra-DU, intra-CU inter-DU, and even inter-CU scenarios.

Thus, L3 HO remains the baseline solution applicable to all scenarios, particularly for inter-RAT and NG-based HO, but may not be optimal for intra-DU or intra-CU cases.

In 5G, LTM was introduced starting from Rel-18 and enhanced in Rel-19 and Rel-20. For inter-CU mobility, LTM provides superior robustness and low latency compared to L3 HO. For intra-DU and intra-CU inter-DU scenarios, LTM is also almost optimal. While for inter-RAT HO and NG-based HO, LTM may bring higher complexity and less motivation for extension. In 5G, LTM was mainly standardized for Xn-based HO.

Thus, LTM provides significant performance benefits for intra-DU and intra-CU inter-DU mobility, making it a strong candidate for 6G Day-1 support.

Of course, the above-mentioned scenarios still need to be further identified and justified in 6G, especially those related to CU-DU, since the CU-DU architecture for 6G requires further discussion and confirmation.

Conditional cell switch (CHO or conditional LTM) is an effective approach to improve mobility robustness. In later 5G releases, subsequent conditional cell switch without requiring RRC reconfiguration was introduced, further improving applicability. For 6G, Conditional LTM (CLTM) is particularly promising, as it can be based on L1 or L3 measurements, offering more flexibility. Compared to conditional L3 HO, CLTM is more motivated due to its better performance in low-latency scenarios.

Thus, Conditional mobility should be supported from 6G Day-1, with CLTM as the primary focus. Whether conditional L3 HO is needed from Day-1 can be left for FFS

The discussion on PSCell change/addition and CPAC are closely tied to the 6G architecture decisions, particularly DC considerations. Therefore, the study on such items should wait until RAN provides clear guidance.

Proposal 1: RAN2 to study the below cell switch types and consider to support all from Day-1: LTM, L3 HO, and conditional cell switch:

- 5G design for LTM, L3 HO, and Conditional LTM is taken as the baseline.

  - Mobility Framework Design

    - Pre-Configuration

Based on the above discussion, it could be observed that there are several cell switch types for PCell in 5G. Regarding the target cell configuration for all these cell switch types, since HO/CHO/(C)LTM are introduced in different release with different features, the designed mechanisms on candidate cell configuration are independent. For L3 HO, it is requested and sent to UE by the source gNB when the gNB determines to change the PCell. And for CHO and (C)LTM, since pre-configuration is supported, several candidate cell configurations are requested and sent to UE before the cell change. Besides,  for (C)LTM,  with the introduction of subsequent cell switch, pre-configuration is further enhanced.

Observation 3: 5G has different cell switch types with seperate target cell configurations due to the functionalities of pre-configuration and subsequent cell switch are introduced in later releases.

In 5G, these cell switch types could be configured to a UE simultaneously. For example, a UE could be configured with (C)LTM and L3 HO, or be configured with HO, CHO and (C)LTM. In all these cases, the duplicated configuration in the same candidate cell for different HO types will occur, in this case the target cell may reserve duplicated resources for the same UE and the UE will have two sets of candidate configurations for same candidate cells of different HO types. This will lead to the further resource occupation.

Observation 4: In current 5G mobility design, if different cell switch types are configured for a UE simultaneously, duplicate candidate/target configuration will be reserved and sent to UE and duplicated HO preparation procedure will be performed.

As we discussed in section 2.1, the L3 HO and LTM would be supported from 6G Day-1, and there is no reason to avoid the coexistence of these different HO types. Thus, similar duplication configuration issue will reapper in 6GN, which is definatly not desired.

Since the HO preparation of all HO types for the target/candidate cell selection is based on the same strategy, e.g. based on network deployments and measurements, it is likely that the network will configure same candidate cells for CHO/HO/(C)LTM. In order to save configuration signaling and network resources, the most straightforward way is to pre-configure a common candidate configuration for different HO types, and the HO could be triggered by using the common candidate configuration, that is the network could use the common candidate configuration with RRC signalling to trigger L3 HO or with MAC CE to trigger LTM, or UE could  use the common candidate configuration to trigger the conditional HO/LTM. A general procedure is provided in Fig.2 as below. In this case, the duplication issue on candidate configuration in 5G could be resolved.

Fig.2: Common pre-configured candidate configuration for all cell switch types

Besides, subsequent cell switch mechanism is flexible for the low mobility UEs or the UEs with regular ranges or regular moving direction. And in 6G, the AI will be used to predict the UE’s moving direction, so the candidate configuration(s) could be configured well suitable for a UE at least within a time duration or area, thus eliminating the need for frequent changes. With this, it is expected that from 6G Day-1, the pre-configured candidate configurations and subsequent cell switch mechanism should be naturally supported for all HO types to avoid duplicated RRC reconfiguration between cell switch.

With the above , for L3 HO in 6G, it could be enhanced with pre-configuration and subsequenct cell switch, while the basic L3 HO similar to 5G should also be reserved in 6G since anyway the RRC signling needs to be sent upon HO execution for L3 HO, target cell configuration could be provided together with the RRC signaling without any additional effort, and it could help to reserve the failure issue in conditional HO.

Proposal 2: RAN2 considers to support cell switch with pre-configured candidate configuration for all cell switch types, and support L3 cell switch without pre-configuration additionally. For cell switch with pre-configuration:

- Pre-configured candidate configuration is shared/appliable for cell switch types, and can be used for subsequent cell switch.

    - Common Modularity

Another aspect on mobility framework design for common modularity should be considered for cell switch of different HO types, to avoid separate functions are defined for different HO types e.g., to accelerate the handover procedure execution or to perform recovery procedure from HO failure or radio like failure.

In fact, one important lesson we learned from 5G is that, the introduced features e.g. RACH-less cell switch, early TCI state activation and early CSI acquisition were introduced in different release and combined with separate cell switch type which leads to duplicated discussion in different WIs, and the fact that they are introduced in the later 5G release rather than in the early 5G stage hampered early commercial succees. For example, in Rel-18, RACH-less handover is introduced for intra-gNB L3 handover procedure in TN and NTN or for inter-gNB handover procedure in the case of NTN or mobile IAB-DU migration procedure, where the UE uses the same TA value at the target cell as in the source cell or TA value of 0. On the other hand, RACH-less handover is also introduced in Rel-18 for LTM based on early UL synchronization procedure with LTM candidate cell(s) before receiving the cell switch command. It can be seen that the similar enhancements are introduced in different work items, which results in that these features are not applicable for other cell switch types.

Observation 5: Separate features are introduced in different releases with similar intentions and are not applicable to all cell switch types, leading to a disunited design for 5G and make it more difficult for commercial success.

As we analyzed in 5G LTM, the early UL/DL synchronization could significantly reduce cell switch interruption, and early CSI acquisition could reduce the first CSI report latency in target cell and increase MCS for data transmission which ensures the throughput during cell switch. And fast recovery could give UE more chance for cell switch to reduce the cell switch latency. We made a good effort in 5G to make the cell switch lossless as much as possible, thus we think these features should be applied to all cell switch types in 6GR to avoid the duplicated discsussion for different cell switch types.

As we proposed in section 2.2.1, in 6GR, RAN2 would consider to support cell switch with pre-configured candidate configuration for all HO types. If that is agreeable, then we could consider to design e.g. the fast recovery mechanism, early UL/DL synchronization and early CSI acquisiton from 6G Day-1 in a more unified way. For example, regardless of which type of cell switch, a UE might use a single fast recovery procedure and then we do not need to re-design/discuss the fast recovery mechanism for each cell switch type.

In a word, in our understanding, different HO types may use the similar or even the same ‘module’ to address specific requirements, such as reducing interruption time, minimizing handover latency, and improving robustness. Currently, these ‘modules’ are often specified within the context of each end-to-end procedure, but we think in 6GR we should consider to study the unified procedure or modularity for all HO types, which would offer several advantages:

    - Specification Efficiency: For future mobility enhancements, instead of defining a new end-to-end procedure from scratch, 3GPP could focus on defining new modules (if needed) or simply specifying new valid combinations of existing modules. This would significantly reduce specification effort and complexity.

    - Simplified UE implementation: From a UE implementation perspective, it is helpful to reduce complexity, because the UE can avoid to implement duplicated function in different types of cell switch.

Therefore, we propose that RAN2 should study the common modularities for all HO types. For example, the early UL/DL sync/RACH-less procedure, early CSI acquisition, fast recovery can be designed in a common way to support L3 HO (with or without preconfiguration), LTM, and Conditional cell switch, while the UE configuration pre-decoding can also support legacy-like L3 HO with pre-configuration, as well as other cell swith types.

Proposal 3: RAN2 to study the common modularity independent of cell switch types, including early UL/DL sync/RACH-less procedure, early CSI acquisition, fast recovery, UE configuration pre-decoding, etc.

  - RRM Measurement Framework

As discussed in Section 2.1, both L3 HO and (C)LTM should be supported from 6G Day-1. Consequently, both L1 measurement and L3 measurements need to be supported as the basic feature of 6GR.

In Rel-15, L3 measurement of NR inherits the RRM measurement framework of LTE with enabling derivation of cell-level results from beam-level measurements. In Rel-18, L1 beam-level measurement for both serving cell and neighbour cell were introduced for LTM by extending the serving-cell beam management framework (CSI-based) to neighbour cells. Due to different introduction releases and different signalling frameworks, the current NR design for L1 and L3 measurement have the following problems:

- Issue #1: Separate and duplicated RS configuration for L1 and L3 measurement;

- Issue #2: Separate and duplicate measurement procedure for L1 and L3 measurement;

Issue #1: Separate and duplicated RS configuration for L1 and L3 measurement

Using SSB configuration as an example, there are SSB-related RS configurations provided in both the MeasObject for L3 measurement and the LTM-SSB-Config for LTM L1 measurement:

Fig.3: Separate and duplicate RS configuration for L1 and L3 measurement

As shown in Fig.3, separate and duplicate SSB configurations for a same target cell may exist for L1 and L3 measurement, including the time and frequency resource of SSB, the SCS and the RS pattern (i.e., ssb-ToMeasure or ssb-PositionsInburst). A similar duplication exists between L1 CSI-RS measurement configuration and CSI-RS configuration for L3 mobility measurement.

Observation 6: In 5G NR, separate and duplicate RS configurations exist for L1 and L3 measurement.

If reusing the 5G RRM measurement framework for L1 and L3 measurement in 6GR, unnecessary duplication on RS configurations will occur as in 5G, which not only introduces signaling overhead but also the UE measurement complexity. Thus, a common RS configuration applicable to both L1 and L3 measurements to reduce signaling overhead and UE measurement complexity should be design target for 6GR measurement.

Issue #2: Separate and duplicate measurement procedure for L1 and L3 measurement

In NR, taking intra-frequency L1-RSRP measurement for neighbour cells as an example, according to TS 38.133 as below, requirements for intra-frequency L1-RSRP measurement are specified only for known cells. A “known cell” means the UE has performed L3 measurement on the target cell within the last 5 seconds before performing the L1-RSRP neighbour cell measurement.

TS 38.133 9.14 NR intra-frequency L1-RSRP measurements for neighbor cell 9.14.2 Requirements Applicability The requirements in the clause 9.14 are applicable to FR1 and FR2-1 for LTM. The requirements in clause 9.14 apply, provided for the SSB from the neighbor cell configured for intra-frequency L1-RSRP measurement, the following conditions are met: - The cell is known - The SSB configured for intra-frequency L1-RSRP measurement is on the same carrier frequency of SSB configured for L3 intra-frequency measurement. - The SSB resources configured for L1-RSRP measurements are measurable. An SSB resource configured for L1-RSRP for neighbor cell shall be considered measurable when for each relevant SSB the following conditions are met: - L1-RSRP related side conditions given in clause 10.1.19D for FR1 and 10.1.20A for FR2-1, respectively, for a corresponding band, - SSB_RP and SSB Ês/Iot according to annex B.2.4.1 for a corresponding band. The cell is considered as known if the following conditions are met in this requirement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2. Otherwise, the cell is unknown.

This implies that before performing L1 measurements, the UE should first perform L3 measurements to ensure the cell is known.

Observation 7: In 5G NR, the UE should perform L3 measurement to ensure the target cell is a known cell before performing L1 measurement.

In addition, according to the measurement model described in TS 38.300 as shown in Fig.4 below, L1 measurement results (i.e., at point A/A1) are actually the intermediate results for L3 measurement results (i.e., at point C/C1/E). This means that if the UE wants to obtain an L1 measurement results for LTM,  it should perform beam-level L1 measurement twice: one for L3 measurement (i.e. L1 measurement before L3 filtering) which confirms the cell is known, the other one for the actual L1 measurement to derive the LTM L1 result.

Fig.4 Measurement Model in TS 38.300

Observation 8: In 5G NR, L1 measurement results serve as intermediate inputs for L3 measurement results.

To avoid duplicate L1 measurements, a unified L1/L3 measurement approach should be studied. For example, the UE would perform a single L1 beam-level measurement and use that directly as the L1 result; the L3 result would then be derived from the same L1 beam-level measurement through L3 filtering. Similarly, the L3 cell-level measurement could be also derived from the L1 beam-level measurement through beam consolidation/selection.

Finally, RRM measurement spans both procedure and modeling in RAN2 and measurement method and requirements in RAN4. This requires close coordination between RAN4 and RAN2, and potentially with RAN1 on L1 RS design. Without such coordination, there is a risk of mismatch design. Thus, inter-WG coordination with RAN4 is critical to ensure coherent design across RAN2 and RAN4.

Proposal 4: RAN2 to study the unified measurement, e.g. common RS configuration for L1 and L3 measurement.

- Coordination with RAN4 on common beam level and cell level measurement, common L1 and L3 measurement.

  - Unified LTM and Beam Switch in MIMO

In Rel-17 MIMO, ICBM (inter-cell beam management) was introduced to facilitate more efficient DL/UL beam management with lower latency and overhead. For ICBM, UE’s serving cell won’t change, but UE can receive or transmit UE dedicated channels/signals via a TRP associated with a PCI different from the PCI of a serving cell based on L1 measurement of additional/neighbor cells. To achieve this purpose, the SSB resource configuration of additional cell is provided within ServingCellConfig via RRC signalling and the inter-cell beams is modelled as TCI states in serving cell configurations.

In Rel-18, LTM which changes the serving cell based on the L1 measurement is introduced. Since there are many commonalities between ICBM and LTM, in the early stage of this release, we discussed whether model LTM with the ICBM similar RRC structure, however, since the LTM candidate configuration includes more information than ICBM additional cells, using the ICBM structure is not flexible. In the end, there are two independent configurations for ICBM and LTM. Similar as the above coexistence scenario, the measurement configuration for neighbor cells will be configured for ICBM and LTM, then the duplicated RRC configuration issue occurs. Correspondingly, the duplicated L1 measurement/report and TCI state activation procedure for ICBM and LTM also occurs.

Observation 9: In 5G, separate and duplicated RRC configuration, L1 measurement/report and TCI state activation procedure will occur in case ICBM and LTM coexist.

In 5G, companies see the benefit to unify the LTM and ICBM, however, since they are introduced in different WID and different release, there is no chance to discuss this topic. Besides, since both MIMO and LTM require beam measurement. Different mechanisms in RAN1 and RAN2 have been specified for the same purpose on beam measurement reporting. The segmented approaches created specification complexity and duplicated functionalities, which bring challenges for the commercial deployment and implementation.

In our understanding, the coexistence between beam switch (like ICBM or beam manamgement in MTRP) and cell switch like LTM should be discussed in 6G early stage, since it could further reduce the data loss and improve the reliability of data transmissions when UE moves from one cell to another cell via firstly applying the beam switch mechanism and then applying the cell switch as shown in the the following Fig.5. And considering one direction of 6G mobility is to simplify system design and minimize the adoption of multiple mechanism for the same functionality, we suggest to discuss the unified design for beam switch like (ICBM or beam management in MTRP) and cell switch like LTM from 6G Day-1.

As the beam switch will be discussed in RAN1, mostly likely in MIMO WI, while cell switch will be discussed in RAN2, most likely in mobility session, inter-WG coordination with RAN1 is critical to ensure coherent design across RAN1 and RAN2. For example, such requirements on unified design could be provided from RAN2 to affect the RAN1 discussion/design.

Fig.5: Scenarios for beam switch and cell switch

Proposal 5: RAN2 assumes commonality is required between beam switch (like ICBM or beam management in MTRP) and LTM, which are applicable for different scenarios. Such requirement is required to inform RAN1.

  - Joint RLM/BFD Design

As for the case that UE moves within the serving cell, the UE experiences dynamic variations in link quality due to quickly changing radio conditions. Hence, UE must maintain continuous monitoring of the serving cell's link quality to prevent communication interruptions caused by connection degradation. The NR system addresses this requirement through a hierarchical monitoring framework consisting of:

- Radio Link Monitoring (RLM): detects cell-level radio link failures resolved through RRC connection reestablishment.

- Beam Failure Detection and Recovery (BFD/BFR): identifies beam-level failures resolved through MAC-layer initiated beam switching.

These complementary mechanisms ensure service continuity across different intra-cell failure scenarios. Through the BFD procedure, the UE promptly detects link quality degradation of the serving beam and performs a beam switch to restore connectivity with the serving cell, thereby avoiding the declare of RLF. Meanwhile, when no acceptable candidate beams are available for beam failure recovery after beam failure is detected, the UE should declare RLF to re-establishes connectivity by switching to another cell.

In NR, RLM and BFD operate independently based on different link quality indicators provided by the PHY layer, even though both rely on monitoring of downlink reference signals (e.g., DM-RS or CSI-RS) to provide upper-layer indications. And they can be configured with different DL RS resources for evaluation. This misalignment may lead to scenarios that deviate from design expectations, such as: RLF being declared while the serving beam quality remains acceptable, i.e. BFR has not been triggered. This could result in unnecessary RRC connection reestablishment and degraded user experience. To address this challenge, a joint design framework for RLM and BFD/BFR mechanisms that coordinates their operation timing sequences should be considered in 6G from Day-1. Specifically, the system should prioritize beam-level recovery by ensuring that RLF declaration occurs until after BFR is triggered whenever BFR is configured.

Proposal 6: RAN2 to study a joint design framework for RLM and BFD/BFR mechanisms.

  - Pain Point Scenario

In 6GR, improved user experience for anybody, anywhere, anytime is one of the most important factors, where seamless mobility is one key aspect. It is expected to improve the mobility experience in scenarios, where the user experience is unsatisfied in current NR deployment, specially “urban mobility scenario” as discussed in this section.

There are many enhancements in 5G NR to improve the performance in high-speed scenario, targeting extremely high moving speed, e.g., 300km/h. However, in the real-field network, there are still many complaints from the end users in urban areas even with moderate moving speed, etc 30~60Km/h. One simple example is depicted in below Fig.6, where a UE is moving from the right side to the left on the road with surrounding buildings in speed of 30Km/h. The figure in the right shows the RSRP fluctuation of measured signals from Cell1, Cell2 and Cell3. It can be noticed that the RSRP of Cell2 goes up temporarily (at the cross-road) while RSRP of Cell3 is steadily increasing. Based on the RSRP report, the network  switches the UE from Cell1 to Cell2, and then from Cell2 to Cell3, which causes multiple service interruption and potentially link failure, leading to bad user experience. This is one “pain point scenario” that has not been addressed in the commercial 4G and 5G deployments.

Fig.6: RSRP fluctuation in urban grid scenario

In the study of 6GR, necessary technical improvements shall be considered to improve overall user experience of pain point scenarios identified from experience of real deployment. For example, based on the UE's location information or the assistance of other devices, pain point scenarios can be identified, appropriate target cell and handover timing can be selected to avoid late or incorrect handover, and the achievable data rate during handover procedure can be improved. To address the pain point scenario discussed above, studies on potential solutions such as UE-initiated reporting, enhanced LTM procedures etc., shall be considered.

Proposal 7: RAN2 to study to improve user experience in pain point scenario (e.g. urban mobility), including reducing service interruption, failure, as well as improving achievable data rate during mobility, e.g., handover.

- Conclusion

In this contribution, we provide our initial consideration on 6G mobility design. Based on the discussion, we have the following observations and proposals.

Observation 1: For 6G Day-1, the mobility study should prioritize standalone deployment scenarios.

Observation 2: 6G Day-1 mobility design should prioritize mechanisms with high commercial feasibility and deployment potential, rather than features with limited practical adoption in 5G.

Observation 3: 5G has different cell switch types with separate target cell configuration due to the functionalities of pre-configuration and subsequent cell switch are introduced in later releases.

Observation 4: In current 5G mobility design, if different cell switch types configured for a UE simultaneously, duplicate candidate/target configuration will be reserved and sent to UE and duplicated HO preparation procedure will be performed.

Observation 5: Separate features are introduced in different releases with similar intentions and are not applicable to all cell switch types, leading to a disunited design for 5G and make it more difficult for commercial success.

Observation 6: In 5G NR, separate and duplicate RS configurations exist for L1 and L3 measurement.

Observation 7: In 5G NR, the UE should perform L3 measurement to ensure the target cell is a known cell before performing L1 measurement.

Observation 8: In 5G NR, L1 measurement results serve as intermediate inputs for L3 measurement results.

Observation 9: In 5G, separate and duplicated RRC configuration, L1 measurement/report and TCI state activation procedure will occur in case ICBM and LTM coexist.

Proposal 1: RAN2 to study the below cell switch types and consider to support all from Day-1: LTM, L3 HO, and conditional cell switch:

- 5G design for LTM, L3 HO, and Conditional LTM is taken as the baseline.

Proposal 2: RAN2 considers to support cell switch with pre-configured candidate configuration for all cell switch types, and support L3 cell switch without pre-configuration additionally. For cell switch with pre-configuration:

- Pre-configured candidate configuration is shared/appliable for all cell switch types, and can be used for subsequent cell switch.

Proposal 3: RAN2 to study the common modularity independent of cell switch types, including early UL/DL sync/RACH-less procedure, early CSI acquisition, fast recovery, UE configuration pre-decoding, etc.

Proposal 4: RAN2 to study the unified measurement, e.g. common RS configuration for L1 and L3 measurement.

- Coordination with RAN4 on common beam level and cell level measurement, common L1 and L3 measurement.

Proposal 5: RAN2 assumes commonality is required between beam switch (like ICBM or beam management in MTRP) and LTM, which are applicable for different scenarios. Such requirement is required to inform RAN1.

Proposal 6: RAN2 to study a joint design framework for RLM and BFD/BFR mechanisms.

Proposal 7: RAN2 to study to improve user experience in pain point scenario (e.g. urban mobility), including reducing service interruption, failure, as well as improving achievable data rate during mobility, e.g., handover.

- References

- RP-252912, Revised SID: Study on 6G Radio

- RP-252909, Summary of Discussion on 6G Requirements for architecture and migration