3GPP TSG RAN WG2 #132		     	R2-2508059
Dallas, U.S., Nov 17th – 21st, 2025

Agenda item:		10.3.2.1

Title:						Discussion on RRC States in 6G Radio

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:		Discussion

# Introduction

RRC states govern how the UEs behave affecting UE energy consumption and latency before establishing communication and mobility. 4G LTE had 2 RRC states, RRC_IDLE and RRC_CONNECTED, while 5G NR introduced the RRC_INACTIVE in addition to those.

The discussion in RAN2#131-bis [1] and supporting contributions revolved around RRC_INACTIVE state, including the experience about, limitations and first discussions of alternatives. The related Control Plane (CP) agreements about RRC states were [1]:

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

In essence, RRC_IDLE and RRC_CONNECTED were taken for granted. They are well established by 4G LTE and they are also always deployed in 5G NR. The RRC_INACTIVE state is in question, and this contribution discusses further aspects on RRC Inactive mode/state.

# Discussion

As discussed in RAN2#131-bis, the RRC_INACTIVE still was not so widely deployed, though some companies see a current uptake. In our view, this is to be expected as 5G deployments ended up relying too much on 5G NSA and only 5G SA is being slowly further deployed. So, the final verdict on whether RRC_INACTIVE is a useful feature on the field should be made later.

In fact, RRC_INACTIVE solves important problems which were experienced in the field in 4G LTE deployments. 4G LTE was not originally designed to support always-on Apps from smartphones contacting servers every 20 to 60 seconds. And such Apps have become completely widespread. It has become an intractable optimization problem in 4G LTE to ensure both reduced latency and conserve UE battery. In one hand, in 4G LTE if the inactivity timer is set large one gets low latency but UE battery consumption suffers. On the other hand, if the inactivity timer is set small the UE battery is conserved but additional latency is experienced. RRC_INACTIVE was designed to improve this trade-off.

Another problem which emerged due to these always-on Apps in smartphones was the constant signaling to the core, from millions and millions of devices. This extra signaling load always does the same: request service, get a new IP address, make some connections and after inactivity timer expiration tear down everything again. Such cycle should be avoided by RRC_INACTIVE deployment freeing the core of a lot of unneeded processing and signaling, keeping stable IP addresses and, therefore, stable Internet connections.

Observation 1: The RRC_INACTIVE state solves important problems experienced in the field in 4G LTE: 1) improving the trade-off between control plane latency and power consumption of UEs which have always-on Apps, 2) reducing the amount signaling to the core network and 3) improving stability of IP connectivity.

As RRC_INACTIVE addresses such important problems our current view is to keep RRC_INACTIVE on 6G, but we need to improve the design addressing all shortcomings.

Proposal 1: The RRC_INACTIVE is kept in 6G, but the design needs to be revisited and improved for 6G

Many shortcomings of RRC_INACTIVE were discussed in RAN2#131-bis. Among them some have been recurring in multiple contributions and are, for example, described in [2]: RNA notification area complexity, necessity of Xn support and short I-RNTI range. These three aspects are all related to mobility in RRC_INACTIVE. Also, one question which was raised was: doesn’t having both RAN paging and core paging contradict the 6G requirement of having a single streamlined solution for a problem? That’s a clear yes. But again, also RAN paging means that RAN is dealing with some large-scale mobility of the UEs. A task traditionally done by the core network. In short, we see mobility in RRC_INACTIVE as the main aspect to be discussed. RAN2 should understand what went wrong with RRC_INACTIVE mobility and how it can be improved for 6G.

Observation 2: The main issues raised about RRC_INACTIVE are related to mobility: RNA notification area complexity, duplication of paging functionality with the core, necessity of Xn support and short I-RNTI range

Proposal 2: RAN2 studies how to deal with mobility in RRC_INACTIVE state in order to avoid the existing shortcomings (RNA notification area complexity, duplication of paging functionality with the core, necessity of Xn support and short I-RNTI range)

# Conclusions

Observation 1: The RRC_INACTIVE state solves important problems experienced in the field in 4G LTE: 1) improving the trade-off between control plane latency and power consumption of UEs which have always-on Apps, 2) reducing the amount signaling to the core network and 3) improving stability of IP connectivity.

Observation 2: The main issues raised about RRC_INACTIVE are related to mobility: RNA notification area complexity, duplication of paging functionality with the core, necessity of Xn support and short I-RNTI range

Proposal 1: The RRC_INACTIVE is kept in 6G, but the design needs to be revisited and improved for 6G

Proposal 2: RAN2 studies how to deal with mobility in RRC_INACTIVE state in order to avoid the existing shortcomings (RNA notification area complexity, duplication of paging functionality with the core, necessity of Xn support and short I-RNTI range)

# References

- “RAN2 Chair Notes”, InterDigital (Chair), 3GPP RAN2 Meeting #131-bis, Prague, Czech Republic, October 13-17, 2025.

- R2-2507072, “Controlling the 6G access stratum”, Ericsson, 3GPP RAN2 Meeting #131-bis, Prague, Czech Republic, October 13-17, 2025.