3GPP TSG RAN WG2 #132		     	R2-2508060

Dallas, U.S., Nov 17th – 21st, 2025

Agenda item:		10.3.1.3

Title:					Addressing the SR/BSR User Plane Latency: Round-Trip Grants

Source:				Fraunhofer IIS, Fraunhofer HHI

Document for:	Discussion

# Introduction

4G LTE and 5G NR share the same basic model for uplink scheduling: the base station (eNB/gNB) is in full control and before allocating more UL resources the UE informs the base station about the need of transmission via a Scheduling Request (SR) and the amount of data for transmission via a Buffer Status Report (BSR). Despite multiple enhancements on UL scheduling latency to address special types of traffic, such as VoIP and URLLC, the lion’s share of UL data, namely mobile Internet (eMBB UL traffic), remains unoptimized in terms of End-to-End (E2E) Round-Trip Time (RTT). However, eMBB traffic performance is also highly affected by E2E RTT and the problem of SR/BSR user plane latency was acknowledged in RAN2#131-bis [1] as something worth further investigation in the 6G Study [2]. In fact, some of the main agreements about User Plane were [1]:

Agreements on UP

…

3.	6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)

…

6.	Study need for scheduling enhancements to address the SR/BSR/DSR latency

…

This contribution addresses exactly that (long standing) problem paving the way for an enhanced user-experience in 6G.

# Problem description and characterization

In RAN2#131-bis, it was widely acknowledged that proposals for the 6G study should address problems which are clearly framed and stated. For this reason, this section reviews the SR/BSR mechanism, characterizes our main optimization target (UL eMBB traffic) and explains why the current mechanism interacts poorly with widely used Internet protocols. Discussion of potential solutions can be found in the subsequent sections.

The SR/BSR mechanism is illustrated in Figure 1. In a typical case where the UE is configured with PUCCH resources for SR, a data arrival in a buffer will trigger a SR which is transmitted on the configured SR resource. The gNB then gives an UL grant so that the BSR which was generated when the data arrived in the buffer can be transmitted. With the knowledge of the BSR the gNB allocates one or more UL grants to serve the traffic. It should be noted that this cycle affects the UL latency when the UE switches from a state of no data in the buffer to finally having data in the buffer. In a longer UL transfer subsequent BSRs can be sent without needing to go over an SR again. However, most UL traffic is bursty and full of periods when there is no data in the buffer. Thus, the latency price of the SR/BSR/Tx must be paid over and over again.

Figure 1 - Illustration of SR/BSR/Tx cycle needed every UL transmission following a period of no data on buffer

Observing Figure 1 one can identify two main sources of increased delay by following an SR/BSR/Tx cycle: 1) waiting for SR resources and 2) multiple signaling steps (data is finally sent on the 5th message).

Observation 1: The delay in a SR/BSR/Tx cycle comes from 1) waiting for SR resources and 2) multiple signaling steps.

Waiting for SR resources can be reduced by providing dedicated PUCCH resources more often. Nonetheless, according to contributions in RAN2#131-bis it is impractical to have dedicated PUCCH resources too often. For example, [3] states that 5-20 ms are typical configurations in the field. Whether removing signaling steps or not is possible is discussed in section 3.

The SR/BSR/Tx cycle is well-established and served for two mobile generations (4G and 5G) of all-IP systems. So, when is it really a problem and when is it acceptable?

If the UL transfer is long and continuous the SR/BSR/Tx cycle is not a significant overhead, but most often this is not the case. UL data is mostly bursty and characterized by short-lived transfers. Some characterization of UL data on mobile Internet (eMBB) usage can help to understand the problem.

The SR/BSR/Tx happens again every time the UE buffer goes down to zero and new data arrives again. When does that happen in practice?

Each application and implementation has its own characteristics, but there are some general principles that can be discussed and characterized. These principles come from widespread Internet protocols and most common usage of Web technologies to implement web pages and Apps. When a user clicks on a web page or starts an app a lot of communication to the Internet happens. First, some initial DNS resolutions are most often necessary as most Internet services rely on people remembering names, not IP addresses. Even for automated services DNS is typically more meaningful because of load balancers and CDNs which can change the IP to serve a certain traffic. So, quite often DNS requests start the communication, and a first SR/BSR/Tx cycle would happen to send the DNS request. As the browser or App needs to wait for the DNS response the UE buffer goes back to zero and a new SR/BSR/Tx cycle will happen for the next UL data. When the DNS response arrives, connections will be established to the addresses on the response. TCP, TCP+TLS, IPSec and QUIC are the most common type of connections that would be made at this point. Modern browsers often open five or more connections to the same IP address. Thus, new UL data traffic may be generated in a row and depending on UE implementation the UE Tx buffer may go multiple times to zero (each triggering a new SR/BSR/Tx cycle). After connections are established, most often resources are downloaded using HTTP. And due to the use of web development frameworks, integration with statistics, ads, etc, most often downloading an HTTP resource will most often cause obtaining more resources from other sites, triggering new DNS requests for these resources, establishing more connections to the obtained IP addresses and so on. Such cascading effect will continue until all data is available at the UE.  Even after a page is fully downloaded or an App is “doing nothing” new UL traffic may be generated sparsely to reload dynamic page elements, keep connections alive (E2E and on stateful devices such as NAT/firewalls) or sync to the server.

Trying to make sense of this complicated pattern of Internet traffic, there are still some common characteristics of UL eMBB traffic that can be highlighted:

- New user sessions may start in random points in time (e.g. with a new DNS request)

- After a user session is started more traffic arrives randomly but there is an increased probability of follow-up UL traffic

- After main transfer is performed further UL traffic may arrive sparsely

- There is a great deal of UL traffic which is generated in consequence of processing DL data.

Observation 2: The lion’s share of mobile UL traffic (eMBB uplink traffic) can be classified as follows:

- Uplink traffic arriving randomly to start new user sessions (e.g. new DNS request)

- Follow-up Uplink traffic with random arrival (e.g. new transport layer connections)

- Uplink traffic arriving very sparsely to keep a session alive or sync with server

- Uplink traffic generated as a consequence of processing Downlink data (e.g. TCP ACKs and QUIC ACKs)

The discussion of solutions to SR/BSR latency should be oriented to the fact that there are sub-problems, as captured in Observation 2. By doing that each proposed solution can be evaluated whether it is suitable for a certain UL traffic pattern or not. Therefore,  we have the following proposal:

Proposal 1: Streamline the discussion on addressing SR/BSR User Plane latency based on the different types of eMBB uplink traffic (see Observation 2)

The larger number of connections needed for downloading a multitude of resources of a web service or page results in the following characteristics of a single connection: most web transactions are short-lived and most connections are finalized still on a ramp-up phase (in case of TCP – on TCP Slow Start). These characteristics are well-documented in a number of sources, e.g. [4].  There is rarely time to “fill the pipe” and having a constant flow of DL and UL packets. Consequently, SR/BSR/Tx cycles will happen over and over. And as described in [4] the E2E RTT is the dominant factor for total transmission time in high-speed links. Not surprisingly, the RAN2#131-bis contribution [3] reported seeing E2E RTT as the main limitation for performance both on simulations and at the field.

In order to understand the SR/BSR/Tx problem with such traffic (most of web traffic), one needs to look at how short-lived transfers behave. Internet protocols which implement flow control and congestion control try to probe the capacity of the link by starting slow and then ramping up more traffic. This is the case, for example, in TCP, SCTP and QUIC. The E2E RTT is the most important metric for such type of traffic and, actually, any throughput limitations can also be seen as something that increases E2E RTT above the minimum. The increase of E2E RTT due to SR/BSR/Tx cycle is illustrated in Figure 2.

Figure 2 – TCP/SCTP/QUIC flows start with few segments and SR/BSR/Tx cycles significantly delay the feedback needed to continue the transfer

Even as the connection advances to have more data in flight, the increased RTT due to SR/BSR/Tx will continue to take its toll. In fact, every time there is a transmission gap (as usual in Slow Start or for any other reason) the UE buffer will go back to zero and a new SR/BSR/Tx will need to be performed again for the next burst of data.

Figure 3 – View of E2E data in-flight just before a transmission gap, when the UE buffer will go to zero. (see also Figure 4)

Figure 4 - View of E2E data in-flight when the UE buffer will go to zero. Next DL data will again experience increased E2E RTT.

As a conclusion of the analysis of UL eMBB traffic and cross-layer interaction with the prevalent Internet traffic, we can also make the following observation:

Observation 3: The existing (NR) MAC protocol only works at minimal end-to-end Round-Trip Time (RTT) when there is a constant flow of downlink and uplink packets. Any gaps in the transmission such as those during TCP Slow Start trigger a new SR/BSR/Tx cycle increasing end-to-end RTT and worsening significantly the end-user experience.

# Discussion of possible protocols and existing solutions

When aiming at reducing latency it is always important to discuss whether alternative protocols with less steps would be possible. In particular, in reference to Figure 1 a reduction of steps would lead to discussion of 3 different protocols to initiate a new UL data transmission:

- 5-step protocol: SR à UL Grant à BSR à UL Grant à Data Tx  (current mainstream solution)

- 4-step protocol: UL Grant à BSR à UL Grant à Data Tx

- 3-step protocol: BSR à UL Grant à Data Tx

- 2-step protocol: UL Grant à Data Tx

- 1-step protocol: Data Tx (directly)

Proposal 2: RAN2 studies the usage of different protocols for UL transmission: direct Tx, Grant+Tx, BSR+Grant+Tx, Grant+BSR+Grant+Tx or SR+Grant+BSR+Grant+Tx

Whether these alternative protocols are possible or not depends on the characteristics of traffic and system design. Understanding which problem is solved by the eliminated steps sheds some light on alternatives:

- A SR step solely informs the base station about the existence of pending UL traffic. It says nothing about the amount of pending data.

- A BSR informs the base station about the amount of pending UL traffic

It is clear that if BSRs can be sent without SR the information of the SR is also given to the base station. Thus, eliminating steps is definitely possible.

The existing protocol (5-step) can be analyzed in comparison to the different types of traffic described in Observation 2 / Proposal 1. In our understanding, dedicated PUCCH resources for SR/BSR work best for follow-up Uplink traffic with random arrival because there is an increased probability of subsequent UL traffic then. However, it is not so well suited for the other types of UL traffic described in Observation 2.

Observation 4: Dedicated PUCCH resources for SR/BSR work best for follow-up Uplink traffic with random arrival.

Data Tx directly (1-step protocol) is only possible by considering either dedicated resources (exactly the Configured Grant solution – but prohibitive as a general solution) or some contention-based protocol (known to be inefficient for high channel occupation – as experienced e.g. on unlicensed bands).

A 3-step protocol starting with BSR would be possible if the initial transmission from a UE is larger than currently (one bit SR indication) so that it fits with a BSR. Dedicated resources for a larger message would be even more prohibitive than dedicated resources for single-bit SRs. Thus, the only realistic alternative for a 3-step protocol is some contention-based solution. However, again, contention-based solutions are prone to be inefficient for high channel occupation and a design based on that should avoid high channel occupation. Because contention-based resources work well at low utilizations it is important that this solution is used for random sparse UL traffic arrival.

Observation 5: Contention-based resources for SR/BSR work best for random and sparse Uplink traffic arrival.

What about 2-step and 4-step protocols? Does it make sense to initiate the UL cycle from the base station when the buffer is on the UE? In fact, it does for a wide majority of cases. As characterized in section 2, there is a very important group of UL traffic: Uplink traffic generated as a consequence of processing Downlink data (e.g. TCP ACKs and QUIC ACKs). Since the goal of such UL traffic is to enable the smooth flow of downlink data, it makes perfect sense to consider 2-step and 4-step protocols initiated by the base station. How could this work? As described in the proposed solution of the next section.

# Proposed Solution – Round-Trip Grants

Among the UL eMBB traffic described in Observation 2, there is one type of traffic which is not well covered by existing solutions or solutions under discussion: Uplink traffic generated as a consequence of processing Downlink data (e.g. TCP ACKs and QUIC ACKs). Such traffic happens very often and in bursts. The generation of such traffic, however, is not random. It is completely mandatory and predictable. It is simply the consequence of implementation of Internet protocols. In fact both TCP [5] and QUIC [6] descriptions mandate that an ACK is generated at least every second packet (with one ACK every packet being also a common implementation). It is worth therefore analyzing how existing solutions or solutions under discussion handle such traffic:

- Dedicated PUCCH resources are sparsely allocated and add the SR/BSR/Tx cycle delay

- Configured grants would be waste for such traffic. The resources are reserved. If there is no DL data flowing or scheduled there should be no UL for ACKs scheduled.

- UL contention-based resources may be overloaded by such traffic which is so frequent.

This leads to the following observation:

Observation 6: No existing mechanism addresses well the most common uplink traffic arrival: Uplink traffic generated as a consequence of processing DL data (e.g. TCP ACKs and QUIC ACKs)

The proposed solution to address this type of traffic in the best possible way is the introduction of a new type of dynamic grant: a Round-trip Grant (RTG). A  Round-trip Grant allocates both DL resources and UL resources for the same UE. The Round-Trip Grant allows to treat E2E flows as they really are – a closed loop system which needs a reliable flow in both directions (DL/UL) to be really performant. The base station sends an RTG for the UE to receive the DL-heavy and in the same grant allocates the resources so that the UL feedback generated by processing this DL data can be timely sent back to the server – without an SR/BSR/Tx cycle.  With the usage of RTGs most SR/BSR/Tx cycles which occur on the field today can be avoided. This is shown in Figure 5. After a transmission gap (as in Figure 3/Figure 4) the system does not need to go back to long SR/BSR/Tx cycle (Figure 2). Instead, the incoming DL data is allocated using an RTG and the UL TCP/QUIC ACK (also allocated by the RTG) flows immediately back to the server.

Further benefits Round-Trip Grants are:

- Reduced number of DCI messages – improved PDCCH capacity/overhead

- Reduced number of UL transmissions – improved UE power consumption

- Reduced E2E session times – improved user experience

- Efficient treatment of small UL packets – as discussed in our accompanying contribution [7].

Figure 5 – By introducing a new type of dynamic grant, a Round-Trip Grant, allocating both DL and UL resources the delay of SR/BSR/Tx cycles can be avoided for most of eMBB UL traffic.

Proposal 3: Study Round-Trip Grants, allocating both downlink and uplink resources, as a solution to avoid SR/BSR cycles for uplink traffic generated as a consequence of processing Downlink data (e.g. TCP ACKs and QUIC ACKs)

# Conclusions

In this contribution, we discussed the SR/BSR User Plane latency with the following observations and proposals:

Observation 1: The delay in a SR/BSR/Tx cycle comes from 1) waiting for SR resources and 2) multiple signaling steps.

Observation 2: The lion’s share of mobile UL traffic (eMBB uplink traffic) can be classified as follows:

- Uplink traffic arriving randomly to start new user sessions (e.g. new DNS request)

- Follow-up Uplink traffic with random arrival (e.g. the user clicking on a new page or large UL transfer)

- Uplink traffic arriving very sparsely to keep a session alive

- Uplink traffic generated as a consequence of processing Downlink data (e.g. TCP ACKs and QUIC ACKs)

Observation 3: The existing (NR) MAC protocol only works at minimal end-to-end Round-Trip Time (RTT) when there is a constant flow of downlink and uplink packets. Any gaps in the transmission such as those during TCP Slow Start trigger a new SR/BSR/Tx cycle increasing end-to-end RTT and worsening significantly the end-user experience.

Observation 4: Dedicated PUCCH resources for SR/BSR works best for follow-up Uplink traffic with random arrival.

Observation 5: Contention-based resources for SR/BSR works best for random and sparse Uplink traffic arrival.

Observation 6: No existing mechanism addresses well the most common uplink traffic arrival: Uplink traffic generated as a consequence of processing DL data (e.g. TCP ACKs and QUIC ACKs)

Proposal 1: Streamline the discussion on addressing SR/BSR User Plane latency based on the different types of eMBB uplink traffic (see Observation 2)

Proposal 2: RAN2 studies the usage of different protocols for UL transmission: direct Tx, Grant+Tx, BSR+Grant+Tx, Grant+BSR+Grant+Tx or SR+Grant+BSR+Grant+Tx

Proposal 3: Study Round-Trip Grants, allocating both downlink and uplink resources, as a solution to avoid SR/BSR cycles for uplink traffic generated as a consequence of processing Downlink data (e.g. TCP ACKs and QUIC ACKs)

# References

- “RAN2 Chair Notes”, InterDigital (Chair), 3GPP RAN2 Meeting #131-bis, Prague, Czech Republic, October 13-17, 2025

- RP-252912, “Revised SID: Study on 6G Radio”, NTT DOCOMO (Moderator), 3GPP TSG RAN Meeting #109 Beijing, China, September 15-18, 2025.

- R2-2507071, “User plane: Let's keep it simple!”, Ericsson, 3GPP RAN2 Meeting #131-bis, Prague, Czech Republic, October 13-17, 2025

- Nandita Dukkipati, Tiziana Refice, Yuchung Cheng, Jerry Chu, Tom Herbert, Amit Agarwal, Arvind Jain, and Natalia Sutin. 2010. An argument for increasing TCP's initial congestion window. SIGCOMM Comput. Commun. Rev. 40, 3 (July 2010), 26–33. https://doi.org/10.1145/1823844.1823848

- W. Eddy. RFC 9293: Transmission Control Protocol (TCP). RFC Editor, USA. 2022.

- J. Iyengar and M. Thomson. RFC 9000: QUIC: A UDP-Based Multiplexed and Secure Transport. RFC Editor, USA. 2021.

- R2-2508661, “Optimizing 6GR for small UL packets”, 3GPP RAN2 Meeting #132, Dallas, USA, November 17-21, 2025