3GPP TSG RAN WG2 Meeting #132	R2-2508066

Dallas, USA, Nov. 17th – 21th, 2025

Agenda Item:	8.2.2

Source:	Xiaomi

Title:	Discussion on remaining open issues for Rel-19 A-IoT

Document for:	Discussion and Decision

# Introduction

This contribution addresses the remaining open issues recorded in [1].

# Discussion

## Issue 3-7: On MDI indication

The status of this open issue is as follows: [1]

In last meeting, RAN2 discussed the new case with no NAS response and achieved the above agreements. The final decision also has dependency with the LS sent to SA3 in the last meeting on the integrity check failure [2]. Whereas the final decision is pending SA3 (and other WGs) reply, CT1 in the last meeting agreed a pCR in [3] which includes the related agreement on how to handle the integrity check failure at the device A-IoT NAS as follows:

Therefore, the current situation is as follows:

- At least in addition to delayed NAS response, there are some other cases where there is no NAS response at all (e.g., unknown message type) and AS will indicate no NAS response to the reader. From reader’s perspective, at least for the purpose of D2R scheduling, it is necessary to distinguish between the “delayed NAS response” and “no NAS response at all” based on the AS response. This is because if it is “delayed NAS response” the reader may send a follow-up R2D message at a later moment to schedule the “delayed NAS response” from the device, while if it is “no NAS response at all”, the reader does not need to schedule the follow-up D2R resource at all.

Observation 1: From resource scheduling point of view, it is necessary for the reader to distinguish between “delayed NAS response” and “no NAS response at all” to avoid unnecessary D2R resource allocation.

- Based on CT1 pCR, it seems CT1 is expecting the device’s AS to do something in the case of NAS integrity check failure. If it can be concluded that the intention is to ask device to provide an AS response to the reader, then the issue discussed in the last meeting comes back concerning whether the AS response needs to differentiate whether the “no NAS response at all” happens due to integrity check failure, unknown message type or some other reasons. However, SA3 is also discussing RAN2 LS in [2] considering also the pCR from CT1.

To this end, we propose that RAN2 awaits SA3 LS response to make the final decision on how to differentiate the “No NAS response at all” vs. “Delayed NAS response”: In case SA3 replies to RAN2 that AS response in the case of integrity check is not feasible, RAN2 can only discuss how to differentiate “No NAS response at all” vs. “Delayed NAS response” for which our preference is to rely on existing MDI field, instead of introducing a new bit (e.g. to use “MDI”, e.g., in case of “SDU=0” due to delayed NAS response, MDI is set to 1 while in case of “SDU=0” due to no NAS response at all, MDI is set to 0); otherwise, RAN2 can conclude that an additional bit is introduced to indicate whether integrity check failure happens, in the case of “No NAS response at all” indicated by MDI.

Proposal 1 [Issue 3-7]: RAN2 concludes Issue 3-7 on MDI indication based on SA3 LS response on integrity check failure to be received:

- If SA3 indicates that AS response is infeasible in the case of integrity check failure, RAN2 agrees to depend on MDI field to distinct “No NAS response at all” vs. “Delayed NAS response”;

- Otherwise, RAN2 agrees to introduce a new bit additionally indicating whether “integrity failure check” happens, in the case of “No NAS response at all” as indicated by MDI.

## New Issue 6-1: On Collision between inventory and command

The status of this open issue is as follows: [1]

According to the LS from CT1 [4], CT1 observed there can be an attack scenario of false A-IoT paging that block valid inventory procedure and command procedure. Hence, CT1 asks RAN2 whether it is possible for the A-IoT device to respond to the new A-IoT paging after completion of an ongoing inventory or command when collision happens.

Actually, this question is related to the parallel service handling. In Rel-19 RAN2 spent a lot of time on it and finally decided in RAN2#130 to not further discuss any forms of parallel service support/handling for Rel-19 A-IoT. In our understanding, the discussion on how to deal with the parallel service triggered would have to be reopened, if the CT1’s proposals in [4] were supported, and many things need to be discussed/resolved, impossibly for RAN2 to resolve them in one meeting. Thus, we think RAN2 can reply to CT1 RAN2 may consider addressing the issue asked by CT1 in Rel-20.

Proposal 2 [New issue 6-1]: RAN2 replies to CT1 that RAN2 may consider to address the issue raised by CT1 in Rel-20.

## New Issue 6-2: On NAS response for inventory response

The status of this open issue is as follows: [1]

Although the issue was raised towards the inventory response msg, we share the Rapporteur’s view that this case can be regarded as the “delayed NAS response case” and thus can handled in the uniform way. This is proposed as follows:

Proposal 3 [New issue 6-2]: Treat the no inventory response msg case in the same way as the “Delayed NAS response” case (i.e. indicated via MDI).

## New Issue 6-3: On Paging ID type

The status of this open issue is as follows: [1].

- Actually, according to [5], the mechanisms to protect A-IoT device identifier privacy during the inventory procedure is based on the use of a Temporary ID (i.e., T-ID). There are two types of T-ID, i.e., concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If concealed type T-ID is included in the paging message, the device needs to generate a T-ID based on the received RANDAIOT_n and the device determines it needs to reply to the NG-RAN if the generated T-ID matches with the received T-ID.

- If the stored type T-ID is included in the paging message, the device determines it needs to reply to the NG-RAN if the stored T-ID matches with the received T-ID.

Thus, from the device’s perspective, if T-ID is contained in the A-IoT paging message, it needs to distinguish the type of the T-ID, in order to determine whether/how it needs to respond to the paging message.

Observation 2: If T-ID is contained in the A-IoT paging message, the device needs to distinguish the type of the T-ID, i.e. conceal type vs. stored type, which impacts how/whether the device needs to respond to the paging message.

In addition, based on the procedure below cited from [5], the device needs to be indicated whether the stored T-ID type shall be updated with or without a command. And in case the stored T-ID update shall be done without a command, the A-IoT Device generates a new Temp_ID_n+1 and stores the new Temp ID_n+1.

Observation 3: If T-ID is contained in the A-IoT paging message, the device needs to be indicated if the stored T-ID update shall be done with or without a command.

Table 1: Citation from [5]

### 5.4.3 Procedure for AIoT Device identifier protection with Temp ID update during Individual inventory For the protection of AIoT device permanent identifier during the inventory procedure with AIoT device identifier described in clause 5.2.2, the following changes shall apply: - In step 1, AIOTF shall retrieve a T-ID in addition to the RANDAIOT_n from ADM. The ADM shall, based on T-ID type, either fetch the stored T-ID in the AIoT device profile or generate the T-ID as specified in Annex B.1. - In step 2, 3 and 4, the T-ID shall be used as a device identification information. - In step 2 and 3 the AIOTF includes indication of type of T-ID handling. T-ID can be either concealed type or stored type. The concealed type can be based on either the stored T-ID or the permanent identifier. If needed the handling also indicates whether the stored T-ID type shall be updated with or without a command. NG-RAN includes the T-ID handling in the paging message. - In step 4, the AIoT device, based on the T-ID handling indication in the paging message, generates the T-ID in the same way as the ADM did in step 1. The AIoT device determines it needs to reply to the NG-RAN if the generated T-ID matches with the received T-ID. In case the stored T-ID update shall be done without a command, the AIoT Device generates a new Temp_ID_n+1 as specified in Annex B.1 and stores the new Temp ID_n+1. - In step 5 and 6, a device identification information is not included in the D2R message and Inventory Report message. - In step 7, the AIoT device permanent identifier is used as a device identification information. AIOTF requests the ADM to derive a new T-ID as specified in Annex B.1 and to store it in the AIoT Device profile.

Regarding how to handle these indications for T-ID handling, i.e. T-ID type indication and the “update with/without command” indication, RAN2 needs to first discuss which of the following ways to go with:

- Way 1: Include these two indications as the fields in the A-IoT paging message, which needs RAN2 impacts to change A-IoT message format;

- Way 2: Assume that these two indications will be included as part of the bits in the “AIoT device Identifier defined” information signaled to the gNB, and thus be directly included within the “Paging ID” field in A-IoT paging message, in which case no RAN2 impacts are needed to change A-IoT Paging message.

Specifically, as per [6] gNB only needs to broadcast the paging message that includes the “AIoT Device Identification Information” whose detail shall be transparent to MAC layer. This enables Way 2 to avoid any RAN2 impact. It should be noted that though it says “... T-ID handling in the paging message” in [5], it does not necessarily mean to specify the above indications for T-ID handling as explicit fields in the A-IoT paging message, for which SA3 discussion never touched.

Therefore, it is proposed that RAN2 adopt Way 2 above. If Way 2 is confirmed, RAN2 needs to send LS to SA4/SA3 on this assumption.

Proposal 4 [New issue 6-3]: RAN2 assumes that the T-ID handling indications are included in the “AIoT device Identifier defined” and are part of the bits in “Paging ID”. No RAN2 impacts are needed.

Proposal 4a [New issue 6-3]: If Proposal 4 is agreed, send LS to SA4/SA3 on the RAN2 assumption.

# Conclusion

Based on the above discussion, the observations and proposals in this paper are listed as follows:

On Issue 3-7

Observation 1: From resource scheduling point of view, it is necessary for the reader to distinguish between “delayed NAS response” and “no NAS response at all” to avoid unnecessary D2R resource allocation.

Proposal 1 [Issue 3-7]: RAN2 concludes Issue 3-7 on MDI indication based on SA3 LS response on integrity check failure to be received:

- If SA3 indicates that AS response is infeasible in the case of integrity check failure, RAN2 agrees to depend on MDI field to distinct “No NAS response at all” vs. “Delayed NAS response”;

- Otherwise, RAN2 agrees to introduce a new bit additionally indicating whether “integrity failure check” happens, in the case of “No NAS response at all” as indicated by MDI.

On New issue 6-1

Proposal 2 [New issue 6-1]: RAN2 replies to CT1 that RAN2 may consider to address the issue raised by CT1 in Rel-20.

On New issue 6-2

Proposal 3 [New issue 6-2]: Treat the no inventory response msg case in the same way as the “Delayed NAS response” case (i.e. indicated via MDI).

On New issue 6-3

Observation 2: If T-ID is contained in the A-IoT paging message, the device needs to distinguish the type of the T-ID, i.e. conceal type vs. stored type, which impacts how/whether the device needs to respond to the paging message.

Observation 3: If T-ID is contained in the A-IoT paging message, the device needs to be indicated if the stored T-ID update shall be done with or without a command.

Proposal 4 [New issue 6-3]: RAN2 assumes that the T-ID handling indications are included in the “AIoT device Identifier defined” and are part of the bits in “Paging ID”. No RAN2 impacts are needed.

Proposal 4a [New issue 6-3]: If Proposal 4 is agreed, send LS to SA4/SA3 on the RAN2 assumption.

# Reference

[1]	R2-2508558	Summary of A-IoT MAC open issues (outcome of [POST131bis][017][AIoT] MAC CR)

[2]	R2-2507915	LS on integrity failure	RAN2

[3]	C1-256631	Clarification on integrity verification failure

[4]	C1-256624	LS on handling of inventory and command collision

[5]	TS 33.369	Security aspects of Ambient Internet of Things (AIoT) services for isolated private networks (Release 19) V19.0.0

[6]	S2-2509655	Reply LS on Structure updates of AIoT Identifiers