3GPP TSG RAN WG2 Meeting #132	R2-2508080

Dallas, USA, Nov. 17th - 21st, 2025

Agenda Item:	10.3.2.2

Source:	Xiaomi

Title:	Discussion on RRC (re)configuration and signalling design

Document for:	Discussion and Decision

Introduction

This contribution discusses RRC (re)configuration and signaling design in 6G, based on the following agreements in the last meeting: [1]

Agreements

  - ASN.1 is used for encoding of RRC signaling for 6G air interface (same as NR).

  - Study overall RRC structure, configuration improvements, improve readability of ASN.1 for RRC signalling

  - Study how to efficiently, reliably and unambiguously configure UEs while keeping signalling size small (e.g. improvements to delta signaling or no delta signaling).

  - RAN2 will study modular design of RRC for 6G.  Further study how to modularize RRC e.g. based on features, functions, verticals, etc.

Discussion

Modularization of RRC (re)configuration

In the last meeting, RAN2 agreed to study the modular design of RRC (re)configuration for 6G. In 5G NR, the SLPP in TS 38.355 [2] already has applied the ASN.1 modularization, and can be taken as a reference for the 6G RRC modularization study.

In [2], the modular ASN.1 is specified as follows:

- There are multiple modules specific to different positioning methods respectively, and each module processes the information specific to the associated positioning methods independently.

These modules correspond to "SLPP PDU SL-AoA Contents", "SLPP PDU SL-RTT Contents", "SLPP PDU SL-TDOA Contents" and "SLPP PDU SL-TOA Contents" in [2]. Upon reception of an SLPP message, the receiver side processes and applies the configuration parameters for each positioning method it supports with the associated ASN.1 modules. Each module processes and applies the information specific to the associated positioning method independently from other modules, meaning that the configuration parameters specific to different positioning methods have no dependency with each other.

- There are also two common module(s), and information specified for these modules is common to all positioning methods, needed by each positioning method specific module for the processing/application of its own information.

These modules correspond to the "SLPP PDU Common Contents" and "SLPP PDU Common SL-PRS Methods Contents" in [2]. The positioning method specific information generally has dependency with the information in the common modules. Specifically, the positioning method specific information typically cannot work alone right upon being processed by the corresponding module, but needs to work together with the information specified in common modules. Furthermore, some positioning method specific information even refers to the related information specified in the common module. As an example shown in the Annex, the "sl-PRS-ResourceId" configured in the common module for “SLPP PDU Common SL-PRS Methods Contents” are referenced by all the positioning method specific modules for AoA, SL-RTT, SL-TDOA and SL-TOA. This typically means that the positioning specific information can only be applied, when the related information is first processes/applied by the common module.

Observation 1 [Lesson learnt from 5G]: Based on modulization design for NR SLPP protocol, two common modules are mandatory for all SLPP UEs, and for positioning method specific modules, UE only needs to support encoding/decoding of a module if the corresponding positioning method is supported by it.

In 6G RRC design, the modularization for sure cannot be done by positioning methods. However, the general principle of the SLPP modular ASN.1 design can be followed. Specifically, for the 6G RRC modularization:

- There should be multiple modules, and each of them processes the RRC configuration parameters independently and has no dependency with others. These modules, based on the last meeting discussion, can be either feature-specific modules or functionality-specific modules. This means that the configuration parameters specified for one feature-/functionality-specific module is specific to the feature/functionality iteself, and should not refer to any parameters specified in another module.

- There should also be a common module(s) that processes the RRC configuration parameters to support UE’s basic communication functions. All above feature-specific or functionality-specific modules have dependency with the common module(s), and can only apply the configuration parameters specified within the module upon successful processing/application of the configuration parameters by the common module(s).

Accordingly, it is proposed that 6G RRC modularization is studied with a modelling (referring to NR SLPP design) as follows:

Proposal 1: 6G RRC modularization is studied with the following modelling on the RRC modules:

- One common module(s), that process RRC configuration parameters for basic communication functionalities which other feature-/functionality-specific modules have dependency with (i.e. system cannot work without the common module(s));

- A number of feature-/functionality-specific modules, each of which processes feature-/functionality-specific parameters and has no dependency with the other feature-/functionality-specific modules (e.g. no cross-module parameter referencing).

- 6G UE only needs to support the encoding/decoding of common module(s), and feature-/functionality- specific modules, if it supports the corresponding features/functionalities.

Then the UE processes/applies the corresponding RRC parameters by the common module(s), and also processes the feature-/functionality-specific parameters it really supported via corresponding modules respectively (applying them together with parameters specified for common module(s)).

Proposal 2: Upon reception of a RRC (re)configuration msg, UE processes the RRC parameters specified for the common module(s), and processes the feature-/functionality-specific parameters it supports with corresponding modules respectively.

Regarding how to categorize the modules, there are following two aspects to be considered:

- Which RRC configuration parameters for a feature should be processed by the common modules and by the feature-/functionality-specific modules, respectively;

- How the modules other than the common module(s) should be categorized, by functionalities or features.

Whereas this discussion needs to depend on the specific features/functionalities to be really specified for 6G, some high-level principles can be considered with respect to the above two aspects:

Principle 1: RRC configuration parameters needed for basic CP/UP communication functionalities need to be covered by the common module(s).

- Principle 1 above is proposed to address above issue 1. Since 3GPP specifies/introduces functionalities to support different features, it is typically the case that the parameters for some functionalities/features are specified to support the basic CP/UP communication functionalities (e.g. Measurement and reporting, mobility, basic L2 configurations, etc.) and thus are mandatorily supported by every UE from 6G Day-1. Such basic RRC configuration parameters should be separated from those parameters specified for certain features/functionalities optionally supported by UEs (mostly for enhancement/optimization). From the implementation point of view, such separation enables the UE to processes/applies these basic RRC parameters in a more efficient way and start normal communication more timely and reliably, without mixing them with those optional feature/functionalities (especially those the UE does not support). As a result, these RRC configuration parameters needed for basic CP/UP communication functionalities need to be covered by the common module(s).

Principle 2: RRC configuration parameters that are not for basic CP/UP communication functionalities, e.g., optional features, can be put in the feature/function specific module(s).

Principle 3: If the RRC parameters of a feature/functionality is not coupled with those of another feature/functionality, they should be put in different modules.

- Principle 2 and 3 are proposed to address above issue 2. Specifically, the feature-/functionality-specific modules should be specified to enable efficient processing/application of the RRC configuration parameters specific to the features/functionalities that are really supported by the UE. Therefore, it is necessary to decouple the RRC parameters of features/functionalities that have no dependency with each other, meaning that if the RRC parameters introduced for a feature/functionality is not coupled with those of another feature/functionality, they should be processed by different modules. This aims to make the UE more efficiently enable a feature/functionality that it really supports, right upon processing/applying the corresponding parameters by the associated module, and avoid the situation that a UE can only enable a its supported feature/functionality after the processing across a number of different modules.

Above principles are proposed as follows:

Proposal 3: Following principles are considered during 6G RRC modulization study:

- Principle 1: RRC configuration parameters needed for basic CP/UP communication functionalities need to be covered by the common module(s).

- Principle 2: RRC configuration parameters are not for basic CP/UP communication functionalities, e.g., optional features, can be put in the feature-/functionality-specific module(s).

- Principle 3: If the RRC parameters of a feature/functionality are not coupled and can work independently with those of another feature/functionality, they should be put in different feature-/functionality-specific modules.

In 3GPP, it is usually the case that a variety of features/functionalities are introduced to support an advanced communication technology, especially for some RAN1/4 introduced features. Take the MIMO related parameters in 5G NR as an example to show how common modules and feature-/functionality-specific modules would be designed based on above principles. The following Table 1 lists the purpose/intended functionalities for which different MIMO related parameters are supported, and the possible modules the MIMO related parameters should apply.

Table 1. Example on the MIMO parameters

| Module types | Features/feature-groups | Purpose/Specific functionality to support | Remarks |
| --- | --- | --- | --- |
| Common module(s) | Basic communication operations | MIMO layer for scheduling | For CSI measurement related parameters, may include also the UE initiated beam reporting introduced in R19 |
|  |  | CSI measurements reporting for BM (introduced in Rel-15) (e.g. NW configured and/or UE initiated beam reporting) |  |
| Feature-/functionality-specific module(s) | mTRP related enhancement | M-TRP (R16) | Depend on whether multi-TRP is supported as essential features to support MIMO in 6G |
|  |  | mTRP-2TA (R18) |  |
|  |  | mTRP, PUSCH, PUCCH, PDCCH (R17) |  |
|  |  | mTRP BM (R17) |  |
|  |  | mTRP-UTCI (R18) |  |
|  |  | Asymmetric MTRP (R19) |  |
|  | CSI related enhancement | MU-CSI (R16) | Part of them may be upgraded to the essential CSI measurement and reporting functionality, pending 6G Day-1 conclusion. |
|  |  | CSI-CJT (R18) |  |
|  |  | CSI-FDD (R17), |  |
|  |  | CSI-mTRP (R17) |  |
|  |  | CSI-Doppler (R18) |  |
|  |  | CSI-CRI (R19) |  |
|  |  | CSI-CJTC (R19) |  |
|  |  | CSI-Type-I//II |  |
|  | SRS related enhancement | SRS (R17) | Part of them may be upgraded to the essential SRS configuration, pending 6G Day-1 conclusion. |
|  |  | 3TX UE (R19) |  |
|  | ICBM related enhancement related | Inter-cell mTRP (R17) | It may be incorporated with above mTRP features pending 6GR discussion. |
|  | Other enhancement | MB1+2 (R16) Low PAPR RS (R16) UL FPTX (R16) |  |

Handling of (re)configuration failure

During last meeting, there was discussion on whether to improve the RRC (re)configuration failure handling, with the proposals to support partial (re)configuration failure and avoid the reestablishment procedure whenever possible. Further study is needed on the reason/use cases on how/why reconfiguration failure happens and the root cause of the problem [1]:

- For next meeting, can study the reasons why these failures happens and understand the root cause of the problem.

Some companies assumed that the NW can always provide proper configuration parameters based on UE capability. However, even if NW always respect the UE capability, there may be several reasons/issues that the UE is unable to comply with the current configurations that are/have been provided by the RAN, as follows:

- Root issue 1 - Network configuration error: Risk for RAN to provide erroneous/improper configuration for individual parameter(s), e.g. ASN.1 encoding/decoding errors, exceeding related UE capability, etc.

In 5G NR, due to the complicated/deep-nesting signalling structures and the inter-dependency across a number of parameters for the value setting (especially for RAN1/RAN4 introduced parameters), the network has to exhaust all the valid combinations for the value setting across these configuration parameters and cope with the coupling across many levels of parameters. These lead to the configurations error-prone, especially for the delta signalling as shown in [4][5].

- Root issue 2 - UE status/capability change: Inapplicability of some of the current configuration parameters by the UE due to its status/capability change.

At the UE side, although it was provided with correct configurations by the NW, there can be some cases that the UE cannot apply some of the configurations based on the its current statue. Such kind of inapplicability of configurations can be related to UE AS capability change as commented by companies during last meeting (e.g. capability sharing in MUSIM, capability reduction due to current form-factor, etc.), and may also be related to other factors like e.g. thermal state or UE energy/power level (which may limit e.g. the UEs’ support of high-throughput configurations/scheduling).

- Root issue 3 - Attack from the fake network: During the discussion in the last meeting, some companies proposed to use the partial configuration to cope with the security attack from the fake base stations, with the argument that the UE can keep the good/feasible configurations it has already had, simply ignores the problematic configurations from the fake base stations and thus unnecessarily performs reconfiguration/reboot operations when facing security attack.

However, for above Root issue 3, the fake base station attack risk is actually a security issue, and it is related to not only disseminating problematic configurations, but also sending erroneous L2/L1 related information. Therefore, not only the fake base station attack but any security risk should be handled by the related security mechanism, but not be associated with the reconfiguration failure handling here (no matter whether the partial configuration is adopted or reestablishment mechanism is inherited). Therefore, the discussion on partial configuration and overall configuration failure handling should be decoupled from any security risk issues.

Observation 2: Fake base station attack and any security related issue should be handled by security mechanism, and should not be considered in the partial (re)configuration discussion.

For above Root issue 1 and 2, although the signalling design can be improved in 6G (e.g. via optimization of delta signalling discussed in the next section), it cannot be assumed that the above configuration failures can be completely avoided by NW implementation. Also, the case that the UE cannot comply with configuration due to UE AS capability change may also need be fully considered.

Proposal 4: For the study on partial reconfiguration w.r.t. the issues that the UE is unable to comply with the (re)configuration by the RAN, following root issues are considered:

- Root issue 1: Configuration errors by the NW

- Root issue 2: Inability to comply caused by the change of UE status/capability

- Note: These issues may not be addressed completely relying on NW implementation to always provide correct parameters.

In 5G NR, the (re)configuration failure is typically handled by the reestablishment procedure in most cases. Specifically, for most of the parameters, if the UE cannot comply, it typically needs to perform RRC reestablishment procedure, possibly selecting another cell to continue its communications. This leads to non-trivial service interruption, considering the CP latency needed for the reestablishment procedure, and should have been avoided if the UE can keep the current correct configurations. In later releases of 5G NR, it was found that the inability to comply happening to some fields/parameters may not impact the basic communication operations, so below NOTE was introduced to avoid unnecessary reestablishment [3]:

NOTE 00: The UE behaviour specified in this clause does not apply to the following, and the UE ignores, i.e. does not take an action on and does not store, the fields that it does not support or does not comprehend: - The fields in ServingCellConfigCommon that are defined in Rel-16 and later. - The fields of searchSpaceMCCH and searchSpaceMTCH in PDCCH-ConfigCommon that are defined in Rel-17 and later.

However, such a way in 5G NR to specify reconfiguration handling is not future proof, as it needs to determine the failure handling for every field/parameter introduced in later releases case by case.

Observation 3 [Lesson learnt from 5G]: In 5G NR, the reconfiguration failure in most cases is handled by the reestablishment procedure. Partial reconfiguration was introduced since Rel-16 by means of informative texts, which is neither future-proof nor a good way for the UE implementation of intended UE behaviour.

To improve the reconfiguration failure handling in 6G, the partial reconfiguration handling should be studied as a candidate solution. Specifically, the UE is allowed to ignore some of the RRC configuration parameters that it is unable to comply, and resumes communication with the current configuration parameters, instead of directly triggering RRC reestablishment. Furthermore, the partial (re)configuration can be discussed in conjunction with the modularization design which is inherently favourable to support the partial (re)configuration at a per module level. For example, failure to comply with some configuration parameters specific to one module does not need to impact the configuration processing/application of another module. But failure to comply with parameters in common module should trigger the reestablishment since it will impact the basic communication function, e.g., the network cannot decode the RRC response message.

Proposal 5: Study the support of partial (re)configuration in 6GR where UE is allowed to apply a portion of(re)configuration parameters and does not trigger reestablishment. The study can be based on the modularization based 6G RRC design.

Optimization for delta-/non-delta-signalling

In the last meeting, it was agreed to further study signalling optimization in 6G regarding how to enable the efficient, reliable and unambiguous configuration to the UE. The discussion was mainly carried out towards delta signalling which has been used as an important tool to reduce signalling size in 4G/5G.

In general, delta signalling is still needed in 6G RRC, in order to avoid unnecessary signalling overhead (e.g. always including full configuration) and thus improve overall system performance. However, it was pointed out (e.g. in [4][5]) that the delta signalling in the real 5G NR deployment is not as well applied as expected, and full configuration remains needed most of the time. This makes the signalling saving expected from delta signalling unable to be sufficiently realized.

Besides the problem in the field as shown in [4][5], another important reason from the signalling design perspective that prevents the delta signalling to play its role is the complicated signalling structure caused by the deep-nesting in 5G NR. With the deep-nesting, one problem is that some configurations specified as child IEs are subject to many nesting levels, so that it cannot be specified as a parameter using delta signalling, due to the restrictions of (any of) its parent IEs. For example, elements in the full list should not be set as Need M or apply ToAddModList structure.  Another problem is the IE/field at each nesting level itself needs to introduce extra overhead w.r.t. the signalling structure it applies, so the more nesting level, the more such overhead introduced. Then, this obviously reduce the benefit of overhead saving, if the delta signalling is applied at the very low nesting level, because all the higher nesting level parent IEs need be present and cost their own overhead, as long as any fields/IE included in the lower-nesting level needs to be present.

Observation 4 [Lesson learnt from 5G]: From signalling design perspective, the deep-nesting signalling structure broadly applied in 5G NR RRC signalling prevents the benefit of applying delta signalling.

Another aspect that leads to the big signalling size in 5G NR attribute to those mandatory configurations. Specifically, there are a big number of mandatory configurations introduced in Rel-15, and as long as a RRCReconfiugration message is provided, those mandatory configurations have to be there, no matter whether they are modified or not compared with the current configurations UE is having. It is possible that these mandatory configurations (which are mostly related to essential functionalities) are not likely to be changed frequently, e.g. within the same cell or across the cells within a given area. In this case, always signalling them would obviously lead to unnecessary big signalling overhead.

Observation 5: In 5G NR, there are many mandatory configuration parameters introduced in Day-1 which have to be signalled every time the UE is (re)configured. This leads to big but unnecessary signalling overhead, especially when most of them are not changed, e.g. within the same cell or across cells within a given area.

To this end, 6GR can study how to optimize the signalling design by minimizing signalling size considering following aspects:

- Rely on the reference/default configuration to avoid duplication of configurations that do not change often. Actually, LTM design in 5G NR already introduce Reference configurations which are common across a number of candidate neighbour cells. When the UE moves within these cells, it is likely that the reference configurations are not changed, so the NW does not need to reconfigure the UE with duplicated parameters. This is just an example to avoid the duplication of common configurations applicable within a certain area. Inspired by this, 6GR can consider to support the reference/default configurations that include those configurations not changed often, in order to avoid duplicating them every time UE is reconfigured.

- Optimize the delta signalling itself. In addition to saving the common/invariant configurations via the default/reference configurations, the delta signalling is still needed to optimize the signalling overhead of the variant configurations. Based on the Observation 4 above, consideration can be taken to avoid the deep-nesting structure. From signalling perspective, this is tightly related to specifying parameters and making extensions in the higher nesting level whenever possible. Also, the modularization may also be helpful to reduce the coupling of features which have no real dependency.

Besides, considering the problem observed for delta signalling in the real deployment in [4][5], it is also helpful if there can be some guidelines/criteria on when to apply the delta signalling. Intuitively, if the reconfiguration signalling is not quite big or is not sent frequently, relying on full configuration should not lead to the signalling overhead concerns, and if the applicability of delta signalling can be well clarified, the potential errors due to delta signalling can also be further avoided.

Therefore, it is proposed that 6GR study an efficient RRC signalling design to optimize signalling overhead, taking into account to avoid duplication of unchanged configurations (e.g. by means of reference/default configurations), improving the delta signalling design (e.g. avoid deep-nesting structure and dependency of parameters whenever possible) and clarifying the delta signalling applicability.

Proposal 6: Study efficient 6G RRC signalling design to optimize signalling overhead, taking into account the following aspects:

- avoiding duplication of unchanged configurations (e.g. by means of reference/default configurations);

- improving the signalling design by avoiding deep-nesting structure and eliminating dependency of parameters whenever possible;

- clarifying the applicability of delta signalling.

Conclusion

In this paper, RRC (re)configuration and signalling design in 6G are discussed. Observations and proposals are listed as follows:

Modularization of RRC (re)configuration

Observation 1 [Lesson learnt from 5G]: Based on modulization design for NR SLPP protocol, two common modules are mandatory for all SLPP UEs, and for positioning method specific modules, UE only needs to support encoding/decoding of a module if the corresponding positioning method is supported by it.

Proposal 1: 6G RRC modularization is studied with the following modelling on the RRC modules:

- One common module(s), that process RRC configuration parameters for basic communication functionalities which other feature-/functionality-specific modules have dependency with (i.e. system cannot work without the common module(s));

- A number of feature-/functionality-specific modules, each of which processes feature-/functionality-specific parameters and has no dependency with the other feature-/functionality-specific modules (e.g. no cross-module parameter referencing).

- 6G UE only needs to support the encoding/decoding of common module(s), and feature-/functionality- specific modules, if it supports the corresponding features/functionalities.

Proposal 2: Upon reception of a RRC (re)configuration msg, UE processes the RRC parameters specified for the common module(s), and processes the feature-/functionality-specific parameters it supports with corresponding modules respectively.

Proposal 3: Following principles are considered during 6G RRC modulization study:

- Principle 1: RRC configuration parameters needed for basic CP/UP communication functionalities need to be covered by the common module(s).

- Principle 2: RRC configuration parameters are not for basic CP/UP communication functionalities, e.g., optional features, can be put in the feature-/functionality-specific module(s).

- Principle 3: If the RRC parameters of a feature/functionality are not coupled and can work independently with those of another feature/functionality, they should be put in different feature-/functionality-specific modules.

Handling of (re)configuration failure

Observation 2: Fake base station attack and any security related issue should be handled by security mechanism, and should not be considered in the partial (re)configuration discussion.

Observation 3 [Lesson learnt from 5G]: In 5G NR, the reconfiguration failure in most cases is handled by the reestablishment procedure. Partial reconfiguration was introduced since Rel-16 by means of informative texts, which is neither future-proof nor a good way for the UE implementation of intended UE behaviour.

Proposal 4: For the study on partial reconfiguration w.r.t. the issues that the UE is unable to comply with the (re)configuration by the RAN, following root issues are considered:

- Root issue 1: Configuration errors by the NW

- Root issue 2: Inability to comply caused by the change of UE status/capability

- Note: These issues may not be addressed completely relying on NW implementation to always provide correct parameters.

Proposal 5: Study the support of partial (re)configuration in 6GR where UE is allowed to apply a portion of(re)configuration parameters and does not trigger reestablishment. The study can be based on the modularization based 6G RRC design.

Optimization for delta-/non-delta-signalling

Observation 4 [Lesson learnt from 5G]: From signalling design perspective, the deep-nesting signalling structure broadly applied in 5G NR RRC signalling prevents the benefit of applying delta signalling.

Observation 5: In 5G NR, there are many mandatory configuration parameters introduced in Day-1 which have to be signalled every time the UE is (re)configured. This leads to big but unnecessary signalling overhead, especially when most of them are not changed, e.g. within the same cell or across cells within a given area.

Proposal 6: Study efficient 6G RRC signalling design to optimize signalling overhead, taking into account the following aspects:

- avoiding duplication of unchanged configurations (e.g. by means of reference/default configurations);

- improving the signalling design by avoiding deep-nesting structure and eliminating dependency of parameters whenever possible;

- clarifying the applicability of delta signalling.

Reference

[1]	RAN2#131bis Meeting minutes

[2]	TS 38.355

[3]	TS 38.331

[4]	R2-2506957	Control plane architecture and 6G RRC protocol design

[5]	R2-2507073	RRC, ASN.1 and other signalling for 6G aspects

- Annex: Example of Common module vs. Positioning method specific module [2]

#### – CommonSL-PRS-MethodsIEsProvideLocationInformation -- ASN1START -- TAG-COMMONSL-PRS-METHODSIESPROVIDELOCATIONINFORMATION-START CommonSL-PRS-MethodsIEsProvideLocationInformation ::= SEQUENCE { sl-POS-ARP-ID-Tx SL-POS-ARP-ID-Tx-InfoList OPTIONAL, sl-PRS-Error SL-PRS-LocationInformationError OPTIONAL, ... } SL-POS-ARP-ID-Tx-InfoList ::= SEQUENCE (SIZE (1..4)) OF SL-POS-ARP-ID-Tx-Info SL-POS-ARP-ID-Tx-Info ::= SEQUENCE { sl-POS-ARP-ID INTEGER (1..4), sl-PRS-ResourceIdList-Tx SEQUENCE (SIZE(1..16)) OF SL-PRS-ResourceId-Tx } SL-PRS-LocationInformationError ::= ENUMERATED { undefined, ... } SL-PRS-ResourceId-Tx ::= SEQUENCE { sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, tx-TimeStamp SL-TimeStamp } -- TAG-COMMONSL-PRS-METHODSIESPROVIDELOCATIONINFORMATION-STOP -- ASN1STOP #### – SL-AoA-ProvideLocationInformation -- ASN1START -- TAG-SL-AOA-PROVIDELOCATIONINFORMATION-START SL-AoA-ProvideLocationInformation ::= SEQUENCE { sl-AoA-SignalMeasurementInformation SL-AoA-SignalMeasurementInformation OPTIONAL, sl-AoA-Error SL-AoA-LocationInformationError OPTIONAL, ... } SL-AoA-SignalMeasurementInformation ::= SEQUENCE { sl-AoA-MeasList SEQUENCE (SIZE(1.. maxNrOfUEs)) OF SL-AoA-MeasElementPerARP-ID-Rx, ... } SL-AoA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-AoA-MeasElement SL-AoA-MeasElement ::= SEQUENCE { applicationLayerID OCTET STRING OPTIONAL, -- Cond FirstElement sl-LCS-GCS-Translation LCS-GCS-Translation OPTIONAL, -- Cond FirstElement los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, -- sl-losNlosIndicator sl-AngleQuality MeasurementAngleQuality OPTIONAL, -- sl-AngleQuality sl-AoA-AdditionalPathList SL-AoA-AdditionalPathList OPTIONAL, sl-AzimuthAoA-Result INTEGER (0..3599) OPTIONAL, -- sl-PRS-AoA sl-POS-ARP-ID-Rx INTEGER (1..4) OPTIONAL, -- sl-pos-arpID-Rx sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, -- sl-PRS-ResourceId sl-PRS-RSRP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRP sl-PRS-RSRPP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRPP sl-TimeStamp SL-TimeStamp OPTIONAL, -- sl-Timestamp sl-ElevationAoA-Result INTEGER (0..1800) OPTIONAL, -- sl-PRS-AoA ... } SL-AoA-AdditionalPathList ::= SEQUENCE (SIZE(1..2)) OF SL-AoA-AdditionalPath SL-AoA-AdditionalPath ::= SEQUENCE { sl-AngleQuality MeasurementAngleQuality OPTIONAL, -- sl-AngleQuality sl-AzimuthAoA-AdditionalPathResult INTEGER (0..3599) OPTIONAL, -- additionalPath-SL-PRS-AoA sl-ElevationAoA-AdditionalPathResult INTEGER (0..1800) OPTIONAL, -- additionalPath-SL-PRS-AoA sl-PRS-AdditionalPathRSRPP-Result INTEGER (0..126) OPTIONAL, -- additionalPath-SL-PRS-RSRPP ... } MeasurementAngleQuality ::= SEQUENCE { azimuthQuality INTEGER (0..255), elevationQuality INTEGER (0..255) OPTIONAL } SL-AoA-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... } -- TAG-SL-AOA-PROVIDELOCATIONINFORMATION-STOP -- ASN1STOP #### – SL-RTT-ProvideLocationInformation -- ASN1START -- TAG-SL-RTT-PROVIDELOCATIONINFORMATION-START SL-RTT-ProvideLocationInformation ::= SEQUENCE { sl-RTT-SignalMeasurementInformation SL-RTT-SignalMeasurementInformation OPTIONAL, sl-RTT-Error SL-RTT-LocationInformationError OPTIONAL, ... } SL-RTT-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-RTT-MeasElement SL-RTT-SignalMeasurementInformation ::= SEQUENCE { sl-RTT-MeasList SEQUENCE (SIZE(1..maxNrOfUEs)) OF SL-RTT-MeasElementPerARP-ID-Rx, ... } SL-RTT-MeasElement ::= SEQUENCE { applicationLayerID OCTET STRING OPTIONAL, -- Cond FirstElement los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, -- sl-losNlosIndicator sl-POS-ARP-ID-Rx INTEGER (1..4) OPTIONAL, -- sl-pos-arpID-Rx sl-PRS-RxTxTimeDiffMeasResult SL-PRS-RxTxTimeDiffMeasResult, ... } SL-PRS-RxTxTimeDiffMeasResult ::= CHOICE { single-SL-PRS-RxTxTimeDiff SL-PRS-RxTxTimeDiffResult, multiple-SL-PRS-RxTxTimeDiff SEQUENCE { sameSL-PRS-TxAndDiffSL-PRS-Rx SEQUENCE (SIZE (2..4)) OF SL-PRS-RxTxTimeDiffResult OPTIONAL, sameSL-PRS-RxAndDiffSL-PRS-Tx SEQUENCE (SIZE (2..4)) OF SL-PRS-RxTxTimeDiffResult OPTIONAL }, ... } SL-PRS-RxTxTimeDiffResult ::= SEQUENCE { sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, -- sl-PRS-ResourceId sl-PRS-RxTxTimeDiffResult CHOICE { k0 INTEGER (0..1970049), k1 INTEGER (0..985025), k2 INTEGER (0..492513), k3 INTEGER (0..246257), k4 INTEGER (0..123129), k5 INTEGER (0..61565) } OPTIONAL, -- sl-PRS-RxTxTimeDiff sl-PRS-RSRP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRP sl-PRS-RSRPP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRPP sl-RTT-AdditionalPathList SL-RTT-AdditionalPathList OPTIONAL, sl-TimeStamp SL-TimeStamp OPTIONAL, -- sl-Timestamp sl-TimingQuality SL-TimingQuality OPTIONAL, -- sl-TimingQuality tx-TimeInfo SL-TimeStamp OPTIONAL, -- tx-Time-Info ... } SL-RTT-AdditionalPathList ::= SEQUENCE (SIZE(1..8)) OF SL-RTT-AdditionalPath SL-RTT-AdditionalPath ::= SEQUENCE { sl-PRS-RxTxTimeDiffAdditionalPathResult CHOICE { k0 INTEGER (0..8191), k1 INTEGER (0..4095), k2 INTEGER (0..2047), k3 INTEGER (0..1023), k4 INTEGER (0..511), k5 INTEGER (0..255) } OPTIONAL, -- additionalPath-SL-PRS-Rx-Tx-TimeDiff sl-PRS-AdditionalPathRSRPP-Result INTEGER (0..126) OPTIONAL, -- additionalPath-SL-PRS-RSRPP sl-TimingQuality SL-TimingQuality OPTIONAL, -- sl-TimingQuality ... } SL-RTT-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... } -- TAG-SL-RTT-PROVIDELOCATIONINFORMATION-STOP -- ASN1STOP #### – SL-TDOA-ProvideLocationInformation -- ASN1START -- TAG-SL-TDOA-PROVIDELOCATIONINFORMATION-START SL-TDOA-ProvideLocationInformation ::= SEQUENCE { sl-RSTD-ReferenceUE-Info SEQUENCE { applicationLayerID OCTET STRING } OPTIONAL, sl-TDOA-SignalMeasurementInformation SL-TDOA-SignalMeasurementInformation OPTIONAL, sl-TDOA-Error SL-TDOA-LocationInformationError OPTIONAL, ... } SL-TDOA-SignalMeasurementInformation ::= SEQUENCE { sl-TDOA-MeasList SEQUENCE (SIZE(1..maxNrOfUEs)) OF SL-TDOA-MeasElementPerARP-ID-Rx, ... } SL-TDOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TDOA-MeasElement SL-TDOA-MeasElement ::= SEQUENCE { applicationLayerID OCTET STRING OPTIONAL, -- Cond FirstElement los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, -- sl-losNlosIndicator sl-POS-ARP-ID-Rx INTEGER (1..4) OPTIONAL, -- sl-pos-arpID-Rx sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, -- sl-PRS-ResourceId sl-PRS-RSRP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRP sl-PRS-RSRPP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRPP sl-RSTD-Result CHOICE { k0 INTEGER (0..1970049), k1 INTEGER (0..985025), k2 INTEGER (0..492513), k3 INTEGER (0..246257), k4 INTEGER (0..123129), k5 INTEGER (0..61565) } OPTIONAL, -- sl-PRS-RSTD sl-TDOA-AdditionalPathList SL-TDOA-AdditionalPathList OPTIONAL, sl-TimeStamp SL-TimeStamp OPTIONAL, -- sl-Timestamp sl-TimingQuality SL-TimingQuality OPTIONAL, -- sl-TimingQuality ..., [[ sl-TDOA-AdditionalMeasurements SL-TDOA-AdditionalMeasurements OPTIONAL ]] } SL-TDOA-AdditionalMeasurements ::= SEQUENCE (SIZE (1..3)) OF SL-TDOA-AdditionalMeasurementElement SL-TDOA-AdditionalMeasurementElement ::= SEQUENCE { sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, sl-PRS-RSRP-Result INTEGER (0..126) OPTIONAL, sl-PRS-RSRPP-Result INTEGER (0..126) OPTIONAL, sl-RSTD-Result CHOICE { k0 INTEGER (0..1970049), k1 INTEGER (0..985025), k2 INTEGER (0..492513), k3 INTEGER (0..246257), k4 INTEGER (0..123129), k5 INTEGER (0..61565) } OPTIONAL, los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, sl-TDOA-AdditionalPathList SL-TDOA-AdditionalPathList OPTIONAL, sl-TimeStamp SL-TimeStamp OPTIONAL, sl-TimingQuality SL-TimingQuality OPTIONAL, ... } SL-TDOA-AdditionalPathList ::= SEQUENCE (SIZE(1..8)) OF SL-TDOA-AdditionalPath SL-TDOA-AdditionalPath ::= SEQUENCE { sl-RSTD-AdditionalPathResult CHOICE { k0 INTEGER (0..1970049), k1 INTEGER (0..985025), k2 INTEGER (0..492513), k3 INTEGER (0..246257), k4 INTEGER (0..123129), k5 INTEGER (0..61565) } OPTIONAL, -- additionalPath-SL-PRS-RSTD sl-PRS-AdditionalPathRSRPP-Result INTEGER (0..126) OPTIONAL, -- additionalPath-SL-PRS-RSRPP sl-TimingQuality SL-TimingQuality OPTIONAL, -- sl-TimingQuality ... } SL-TDOA-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... } -- TAG-SL-TDOA-PROVIDELOCATIONINFORMATION-STOP -- ASN1STOP #### – SL-TOA-ProvideLocationInformation -- ASN1START -- TAG-SL-TOA-PROVIDELOCATIONINFORMATION-START SL-TOA-ProvideLocationInformation ::= SEQUENCE { sl-TOA-SignalMeasurementInformation SL-TOA-MeasElementPerARP-ID-Rx OPTIONAL, sl-TOA-Error SL-TOA-LocationInformationError OPTIONAL, ... } SL-TOA-MeasElementPerARP-ID-Rx ::= SEQUENCE (SIZE(1..4)) OF SL-TOA-MeasElement SL-TOA-MeasElement ::= SEQUENCE { los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, -- sl-losNlosIndicator sl-RTOA-Result CHOICE { k0 INTEGER (0..1970049), k1 INTEGER (0..985025), k2 INTEGER (0..492513), k3 INTEGER (0..246257), k4 INTEGER (0..123129), k5 INTEGER (0..61565) } OPTIONAL, -- sl-PRS-RTOA sl-POS-ARP-ID-Rx INTEGER (1..4) OPTIONAL, -- sl-pos-arpID-Rx sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, -- sl-PRS-ResourceId sl-PRS-RSRP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRP sl-PRS-RSRPP-Result INTEGER (0..126) OPTIONAL, -- sl-PRS-RSRPP sl-TOA-AdditionalPathList SL-TOA-AdditionalPathList OPTIONAL, sl-TimeStamp SL-TimeStamp OPTIONAL, -- sl-Timestamp sl-TimingQuality SL-TimingQuality OPTIONAL, -- sl-TimingQuality ..., [[ sl-TOA-AdditionalMeasurements SL-TOA-AdditionalMeasurements OPTIONAL ]] } SL-TOA-AdditionalMeasurements ::= SEQUENCE (SIZE (1..3)) OF SL-TOA-AdditionalMeasurementElement SL-TOA-AdditionalMeasurementElement ::= SEQUENCE { sl-PRS-ResourceId INTEGER (0..16) OPTIONAL, sl-PRS-RSRP-Result INTEGER (0..126) OPTIONAL, sl-PRS-RSRPP-Result INTEGER (0..126) OPTIONAL, sl-RTOA-Result CHOICE { k0 INTEGER (0..1970049), k1 INTEGER (0..985025), k2 INTEGER (0..492513), k3 INTEGER (0..246257), k4 INTEGER (0..123129), k5 INTEGER (0..61565) } OPTIONAL, los-NLOS-Indicator LOS-NLOS-Indicator OPTIONAL, sl-TOA-AdditionalPathList SL-TOA-AdditionalPathList OPTIONAL, sl-TimeStamp SL-TimeStamp OPTIONAL, sl-TimingQuality SL-TimingQuality OPTIONAL, ... } SL-TOA-AdditionalPathList ::= SEQUENCE (SIZE(1..8)) OF SL-TOA-AdditionalPath SL-TOA-AdditionalPath ::= SEQUENCE { sl-RTOA-AdditionalPathResult CHOICE { k0 INTEGER (0..16351), k1 INTEGER (0..8176), k2 INTEGER (0..4088), k3 INTEGER (0..2044), k4 INTEGER (0..1022), k5 INTEGER (0..511) } OPTIONAL, -- additionalPath-SL-PRS-RTOA sl-PRS-AdditionalPathRSRPP-Result INTEGER (0..126) OPTIONAL, -- additionalPath-SL-PRS-RSRPP sl-TimingQuality SL-TimingQuality OPTIONAL, -- sl-TimingQuality ... } SL-TOA-LocationInformationError ::= ENUMERATED { undefined, assistanceDataMissing, notAllRequestedMeasurementsPossible, ... } -- TAG-SL-TOA-PROVIDELOCATIONINFORMATION-STOP -- ASN1STOP