3GPP TSG-RAN WG2 Meeting #132	R2-2508085

# Dallas, Texas, USA, 17 – 21 November 2025

Agenda item:		10.4

Source:	Xiaomi

Title:	Discussion on 6G mobility

Document for:		Discussion and Decision

# Introduction

In RAN2#131bis meeting, following was agreed regarding 6G mobility:

Agreements

1	Study mobility with the following requirements in 6G mobility design:

-	Minimize interruption time and ensure service continuity (i.e. minimize throughput degradation during mobility). Consider complexity and gains when discussing solutions.

-	Robustness of mobility procedures

-	Energy efficiency for both UE and NW

2	Study aspects related to mobility (e.g. early DL/UL synchronization, UE configuration processing, pre-configurations, conditional handover, early CSI acquisition)

In this contribution, we discuss design target for 6G mobility, intra 6G mobility in different RRC states, and inter-RAT mobility.

# Discussion

## Review of LTE and NR mobility solutions

In LTE and NR, many mobility solutions were standardized. In Table 1 below, a comparison of mobility solutions is shown. Note that DC related solutions (e.g. CPAC) are not shown according to SID [1] “RAN plenary to make a decision on additional migration option(s) (other than standalone, MRSS, and inter-RAT mobility between NR-6G). This includes decision on additional 6G-6G aggregation beyond 6G CA: 6G-6G DC.”

Table 1: Comparison of LTE and NR mobility solutions

| Feature | LTE or NR | First release | Interruption | Robustness | UE complexity | NW complexity | Commercialization |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Layer 3 HO | LTE, NR | - | ~60 ms (calculation based on TS 38.133 clause 6.1.1.2.2) |  | Low | Low | Yes |
| RACH-less | LTE | 14 | Reduced |  | Low No RACH | Low Indicate the uplink timing and provide the pre-allocated uplink grant in the handover command | No |
| Make-Before-Break | LTE | 14 | Reduced |  | Medium UE needs to be capable of two Rx | Low Keep the connection after HO command for source gNB | No |
| DAPS | LTE, NR | 16 | 1 ms in air interface (TS 38.133 clause 6.1.3.2.2) |  | High UE shall support the simultaneous DL reception of PDCCH and PDSCH from source and target cell, and single UL transmission for DAPS | High Keep the connection after HO command for source gNB, handle the data received from source/target | No |
| CHO | LTE, NR | 16 | ~70 ms (calculation based on TS 38.133 clause 6.1.4.2) | Improved: UE-based Mobility can reduce the latency of handover decision. | Medium UE perform condition evaluation, and needs to evaluate up to 8 cells and 16 measId Need to store up to 8 candidate cell configurations | Medium Network needs to reserve resource up to 8 candidate cells） Note: resource efficiency is bad | Yes |
| LTM | NR | 18, 19 | ~20 ms (calculation based on TS 38.133 clause 6.3.1.2) |  | Medium UE need to support L1 measurement and report UE may need to support Early TA acquisition or UE based TA measurement RACH-less can be achieved Need to store up to 8 candidate cell configurations | Medium Network needs to reserve resource up to 8 candidate cells） Network needs to indicate TA and beam Network needs to handle L1 measurement and report Note: resource efficiency is bad | No |
| Conditional LTM | NR | 19 | ~20 ms (calculation based on TS 38.133 clause 6.3.2.2) | UE-based Mobility can reduce the latency of handover decision. | Medium UE need to support L1 measurement and report UE may need to support Early TA acquisition or UE based TA measurement RACH-less can be achieved Need to store up to 8 candidate cell configurations UE perform condition evaluation, and needs to evaluate up to 8 cells and 16 measId (L3) | Medium Network needs to reserve resource up to 8 candidate cells） Network needs to indicate TA Network needs to handle L1 measurement and report Note: resource efficiency is bad | No |

Note that LTE RACH-less mobility is only applicable for small cells or the scenario that UL Tx timing is not changed, while LTE Make-Before-Break mobility is only applicable for intra-frequency mobility scenario.

Above evaluation of interruption time is based on RAN4 TS 38.133 requirements. For example, for LTM, the analysis is based on clause 6.3.1.2 and 6.3.1.3 assuming interruption time equals to Tcmd + TLTM-interrupt, with TLTM-interrupt = TLTM-RRC-processing + TLTM-processing + Tfirst-RS + TRS-proc + TLTM-IU, and TCMD = THARQ + 3 ms, assuming TLTM-RRC-processing = 0 (UE supports ltm-FastProcessingConfig), TLTM-processing=10 ms (for FR1 to FR1 LTM cell switch and UE supports ltm-FastUE-Processing), Tfirst-RS = TRS-Proc = 0, TLTM-IU = 5 ms (for CG resource periodicity), THARQ=1 ms.

Following is the applicability of LTE / NR mobility solutions to 6G:

- The Rel-8/15 legacy HO of PCell change should be supported as baseline, due to low complexity at the UE and the network.

- RACH-less should be supported for reduced interruption time, especially for small/dense cell deployment.

- Make-Before-Break could be dropped, due to the dual Rx requirement for UE and there is no commercial deployment since Rel-14.

- DAPS can be dropped, due to the high complexity at the UE. In TR 38.914, requirements for mobility interruption time are as follows: “Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. The proponent can report the Mobility interruption time in other scenarios, where applicable.” Since 0 ms interruption time is for mobility between TRxPs of a single base station while DAPS is for mobility between gNBs, there is no strong motivation to consider DAPS for 6G.

- CHO should be supported for improvement of mobility robustness, due to the low complexity for the UE RF.

- LTM (including conditional LTM) should be supported for the reduced interruption time and signalling overhead.

Note: We should simplify the mobility solution if multiple options are considered, i.e., unified mobility solution as discussed in section 2.3.

Lesson learnt from 5G (1): Too many mobility solutions were introduced in 5G which increased standards work load, UE/network complexity and also led to market fragment, especially some solutions are too complex to UE implementation which should be avoided.

Proposal 1: In 6G RAN2 study on mobility, compared to 5G, RAN2 should aim to reduce the options for 6G mobility design. Following NR mobility features can be used as the starting point for 6G study: L3 handover and LTM, including sub-features like RACH-less / early sync and conditional mobility (CHO and conditional LTM).

## Design target for 6G moblity

In last meeting, some companies proposed to reduce the interruption time and improve the robustness of mobility procedure in 6G. However, it is unclear what targeted interruption time and robustness should be. As captured in RANP TR, the requirement for mobility interruption time is “sufficiently minimized to support a seamless application performance”. 0ms is the requirement only for mobility between TRxPs of a single base station.

### 5.1.12 Mobility interruption time Mobility interruption time shall be sufficiently minimized to support a seamless application performance. The minimum requirement for mobility interruption time is 0 ms at least for mobility between TRxPs of a single base station. The proponent can report the Mobility interruption time in other scenarios, where applicable.

In Rel-16, to meet 0ms interruption time requirement for URLLC, RAN2 agreed to support DAPS [4].

In Rel-15 NR, 0ms interruption time can be achievable by using intra-cell using beam mobility and addition/release of SCell for CA operation. However, there is demand to achieve 0ms interruption time in more scenarios especially in URLLC type of service which requires 1ms of end-to-end delay in some scenarios.

However, although it can achieve 1ms interruption time requirement from air interface perspective, additional interruption time is expected from network side since source and target need to do the data forwarding. In addition, the DAPS feature has never been deployed due to the complexity, and lack of the commercial requirements. [5] provided the good reason why the strict 0ms interruption time is not needed.

- There is actually no need to target a "strict 0 ms" radio level interruption during mobility, since what matters in practice is the interruption time at service level (by the way, this is the reason why, also outside of the handover phase, networks are not expected to (and do not) always schedule UEs in every slot/ subframe with 0 ms interruption!)

Lesson learnt from 5G (2): There is no commercial need on the requirement “0ms interruption time”, which should be avoided in 6G.

For robustness, there is no clear target in 5G. The only thing mentioned in the WID [4] (as below) is to reduce handover failure and pingpong rate. For 6G, the situation is not worse than 5G, we do not see the need to improve the handover reliability compared with 5G.

In fact, handover in NR high frequency range with beamforming may increase interruption time in compare to LTE due to beam sweep delay. Reliability reduces for the same reason due to beamforming characteristics provides smaller coverage. When the UE moves or rotates, the UE can experience very fast signal degradation. Another challenge is due to very different channel condition between LoS and non LoS in NR. It is observed that pathloss fluctuation can be tens of dB different in signal strength in beams or between LOS and non-LOS. It may result in higher handover failure and large pingpong rate. Therefore, handover becomes extremely challenging in NR environment than in LTE.

Proposal 2: In 6G design, RAN2 should set the reasonable target on interruption requirement and robustness requirement unless there are additional requirements from RANP. The interruption time from 5G “best solution” LTM can be used as the target performance in RAN2, i.e., not worse than 20ms interruption time. For robustness performance, 5G CHO performance can be the target for 6G without the need for further improvement.

## Unified mobility

As analyzed in previous section, 5G supports two independent mobility solution directions: L3 mobility and LTM. Independent RRC IEs and separate procedures are used for L3 mobility and LTM. This incurs complexity at both UE and network side.

Firstly, there is large signalling overhead to support two set of mobility features in NR, e.g., CHO and LTM are configured by two independent RRC IEs. Both the candidate configurations of CHO or LTM are configured through RRCReconfiguration. For the same candidate cell of CHO and LTM, duplicate configurations are provided to UE in CHO config and LTM config separately. The handover command of the L3 HO is also provided to UE via a RRCReconfiguration message.

Secondly, different procedures address similar motivations, such as CHO and CLTM. However, the supported features and the configuration restrictions for L3 mobility and LTM are different:

- LTM supports early UL/DL sync and subsequent mobility which can reduce handover delay and interruption, but L3 mobility does not support these features.

- LTM supports early CSI acquisition on candidate cell(s) to improve the performance of data transmission on target cell, but L3 mobility does not support the feature.

- For LTM, the RRC processing delay depends on whether the UE support fast processing of candidate configuration, but L3 mobility does not consider the feature.

- Conditional LTM can be triggered based on L1 and L3 measurement, but CHO can only be triggered based on L3 measurement.

- In L3 RRM measurement, measurement configuration and L3 mobility configuration are decoupled, but L1 LTM measurement can only be configured for LTM candidate cell.

Lesson learnt from 5G (3): 5G supports two independent mobility solution directions: L3 mobility and LTM. This incurs complexity at both UE and network side, as well as duplicated configuration and UE efforts for mobility.

Therefore, in 6G, it is beneficial to unify L1 and L3 mobility to minimize signalling overhead and complexity from both UE and network side to avoid duplicated mobility configuration and procedure, and all mobility features can be configured for both L1 and L3 mobility to enhance the flexibility and controllability.

Following study areas can be considered:

- A unified candidate configuration list is supported to avoid the same candidate for different mobility types.

- Unified triggers and unified measurement procedure are applicable for all candidate.

- All feature introduced for 5G LTM can be supported for all candidates, and the UE doesn’t need to distinguish between LTM candidates and CHO candidates.

For the above study areas, some details as shown below:

- Unified Mobility Pre-Configuration for L3 mobility and LTM

  - All candidate configuration(s) are configured via the same RRC IE.

- Unified Trigger for L3 mobility and LTM

  - All candidate configuration can be triggered by the following methods:

    - e.g., NW triggered via L2 or L3 signaling, UE triggered based on measurement event (event based on L1 or L3 measurement), etc.

- Unified Feature supported for L3 mobility and LTM

  - The following features can be applicable to all candidates according to the network configuration:

    - Early DL/UL synchronization (e.g., UE based TA measurement, Early RACH based TA acquisition and Candidate Cell TCI States Activation/Deactivation)

    - Early CSI acquisition

    - Fast RRC processing: e.g., early decoding and compliance check.

    - Subsequent mobility

- Unified Measurement for L3 mobility and LTM:

  - Unified measurement configuration for both L1 and L3 measurement:

    - Unified measurement resource configuration

    - Unified measurement report configuration

  - Unified measurement report trigger:

    - e.g. Periodic, aperiodic, semi-Persistent and event triggered.

  - Unified measurement report signalling:

    - Unified report signalling for both L1 and L3 measurement (e.g., L1, L2 or L3 signalling)

For the unified mobility, if the above features are considered, the minimum interruption time is equal to the minimum value of 5G LTM. Because, the unified mobility can support all features of LTM including features used for reducing the handover interruption, e.g., early DL/UL synchronization and fast RRC processing.

Figure 1: Unified L1 and L3 mobility

Proposal 3: In 6G RAN2 study on mobility, RAN2 to study unified mobility framework for both L1 and L3 mobility, using 5G LTM configuration as the starting point, by considering following directions:

- Unified Mobility Pre-Configuration: all candidate configuration(s) are configured via the same RRC IE.

- Unified Trigger: NW triggered via L2 or L3 signaling and UE triggered based on measurement event.

- Unified Feature Set: early DL/UL synchronization, early CSI acquisition, fast RRC processing and subsequent mobility.

- Unified Measurement: unified measurement configuration, unified measurement report trigger and unified measurement report signalling.

## Beam based mobility

In addition to cell based mobility discussed in above section, NR also supports beam based mobility. Beam based mobility can be either between beams from the same Transmission and Reception Point (TRP), or between beams of different TRPs of the same cell.

TCI (Transmission Configuration Indication) is a field in the DCI to indicate the beam configuration used for transmission and the quasi-co-location (QCL) relationship between reference signals and data transmission. Network can switch the TCI state dynamically to point to a different TRP without triggering a handover or LTM procedure.

In RAN2#131bis meeting, there were proposals on “super” cell concept, where seamless handover can be achieved by a combination of TRP and L3 HO/LTM in 6G, e.g. beam-level mobility based on TRP within “super” cell and LTM across “super” cells. It is not clear what is the key difference between the “super” cell and 5G / NR cell. One argument during meeting discussion is that common channels need to change and NR framework requires a full configuration. It should be noted that in RRC_CONNECTED, the common channels to be received include system information (mainly SIB1 and ETWS/CMAS) and DCI carrying Short Message for system information change and ETWS/CMAS notifications. UE monitors PDCCH for system information and paging in common search space searchSpaceSIB1, searchSpaceOtherSystemInformation, and pagingSearchSpace (TS 38.331 clause 5.2.2.2.2). For those search spaces, TCI states are not configured, as from the condition NotSIB-initialBWP of field tci-StatesPDCCH-ToAddList in IE ControlResourceSet, as below. The reason is that beam reception for common channel is only based on SSB, and it is up to UE implementation to select a suitable SSB to receive common channels. From network deployment perspective, it is expected that every TRP should provide common channels otherwise there would be coverage holes. Therefore for inter-TRP mobility within a cell, UE implementation can ensure the reception of common channels needed for CONNECTED state.

| Conditional Presence | Explanation |
| --- | --- |
| NotSIB1-initialBWP | The field is absent in SIB1 and in the PDCCH-ConfigCommon of the initial BWP in ServingCellConfigCommon, if SIB1 is broadcasted. Otherwise, it is optionally present, Need N. |

Observation: Beam level mobility defined in 5G can support the mobility between TRPs of the same cell, and common channel reception during mobility between TRPs is ensured by UE implementation.

Proposal 4: From RAN2 perspective, beam level mobility for mobility between TRPs of the same cell / base station as supported in NR can be applicable to 6G while details can be studied by RAN1.

## Unified RRM

NR RRM model is shown in Figure 2 below. The design of RRM measurement is lack of system-level consideration in 5G life-cycle, and following issues are observed:

- Redundant UE measurement behaviors: there are many similarities between measurement tasks, e.g. L3 mobility measurement and L1 mobility measurement.

- Duplication between measurement tasks, e.g. L1-RSRP measurement for BM and L1-RSRP for mobility.

- Complicated UE measurement behaviors, e.g. sharing scheme between measurement tasks.

Figure 2: NR RRM model

Lesson learnt from 5G (4): The design of RRM measurement is lack of system-level consideration in 5G life-cycle as there are redundant UE measurement behaviors, duplication between measurement tasks, measurement configurations and resource configurations which make UE measurement behaviors complicated.

In section 2.3 above, unified measurement is discussed from perspective of L1 and L3 mobility. In addition, unified measurement should be also consider measurements for MIMO and BFD / RLM purposes.

In RAN4#116bis meeting, WF on RRM [3] was approved with the following on measurement framework:

RRM framework: measurement capability/delay/overhead/accuracy/quantities/unified measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. Note: the title of the main topic can be revised after selection of the sub-topics.

One RRM measurement related topic is measurement gap. RAN2’s main responsibility is to defined measurement gap related signalling based on RAN4 input, e.g. defining IE MeasGapConfig. NR supports a large number of measurement gap patterns and several types of gaps: e.g. per UE gap, per FR gap, Network Controlled Small Gap (NCSG), Pre-configured Measurement Gap (Pre-MG).

The measurement gap work is mainly RAN4 driven. In RAN4#116bis meeting, WF on RRM [3] has the following agreements regarding measurement gap:

Measurement gap, including gap-less measurement, is agreed as part of RAN4 RRM 6G study. The detailed scope will be further decided. - Interruption enhancement due to gap-less measurement will be part of the MG discussion.

RAN4 also identifies potential MG related sub-topics for study. Some sub-topics might have RAN2 signalling impacts, e.g. MG pattern reduction from 5G, Unified MG, Adaptive MG operation, MG sharing, UE assisted MG configuration, Multi-CC measurements in MG.

Given that RAN4 has already stated measurement gap discussion, RAN2 can wait for RAN4’s progress for a few meetings before starting RAN2’s work.

Proposal 5: In 6G RAN2 study on measurements, RAN2 to study simplified RRM architecture considering L1 and L3 measurement and multiple cells, with potential collaboration with RAN1 and RAN4 on unified measurements on L1 and L3 mobility, MIMO, BFD and RLM. For measurement gap related study, RAN2 wait for RAN4’s progress.

## Idle mode mobility

In RAN2#131bis meeting, it was agreed to continue study Idle mode, Connected mode, and Inactive mode or sub-state. Mobility in idle mode is mainly related to cell (re)selection. The basic operation for cell reselection is that UE evaluates and ranks cells based on measurement results as well as network configuration (e.g. frequency priorities, offsets for frequencies / cells). Such basic principle can be the starting point for 6G study.

In 5G, some UE behaviors are specified for setting the highest / lowest priority according to some capabilities / features e.g. HSDN, MBS, NR sidelink, V2X sidelink. An excerpt from TS 38.304 is copied below:

When the HSDN capable UE is in High-mobility state, the UE shall always consider the HSDN cells to be the highest priority (i.e., higher than any other network configured priorities). When the HSDN capable UE is not in High-mobility state, the UE shall always consider HSDN cells to be the lowest priority (i.e., lower than any other network configured priorities). If the UE is configured to perform both NR sidelink communication and V2X sidelink communication, the UE may consider the frequency providing both NR sidelink communication configuration and V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform NR sidelink communication and not perform V2X communication, the UE may consider the frequency providing NR sidelink communication configuration to be the highest priority. If the UE is configured to perform V2X sidelink communication and not perform NR sidelink communication, the UE may consider the frequency providing V2X sidelink communication configuration to be the highest priority. If the UE is configured to perform ranging/sidelink positioning, the UE may consider the frequency providing ranging/sidelink positioning configuration to be the highest priority.

Although feature specific frequency prioritization provides operator some tools to control the cell reselection, it is mainly introduced in 5G in an ad hoc manner and it is difficult for operator to set the preferred frequency when multiple features are supported in the network.

In 5G, Cell Individual Offset (CIO)Neighbor cell offset can be configured for neighbour cells and it will affect cell ranking. Depending on network’s configuration, such CIO can be bonus / punishment (value can be negative or positive) for cell ranking.

Lesson learnt from 5G (5): In 5G cell reselection, there is lack of unified control for feature specific frequency prioritization and cell individual offset which increased the complexity, and also difficult to use.

In 6G, multiple features (including new features like AI / ML and sensing) are expected to be introduced. Existing reselection priority handling is done at the UE side, and it handles features separately. For the case that a cell supporting multiple features, operator has no good control in weighting different features and directing UEs to the mostly wanted carriers. Study is needed on how to have a unified control for feature specific frequency prioritization and cell individual offset.

In RAN2#131bis meeting, following was agreed regarding spectrum aggregation:

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

Consequently, cell reselection operation in a 6G cell with flexible spectrum utilization should be also studied.

Proposal 6: In 6G RAN2 study on “idle mode” mobility, basic operations of 5G cell reselection can be a starting point for idle mode mobility in 6G, e.g., frequency/cell/feature based selection priority. RAN2 to further study how to make cell (re)selection mechanism more efficient and simpler in 6G, especially, take into account spectrum aggregation and multiple features support.

## Inter-RAT mobility and migration

For migration, RAN WG 6G SID RP-252912 [1] has the following objectives:

- Migration from 5G NR to 6GR as well as interworking and mobility between 5G NR and 6GR:

- 5G-6G Multi-RAT Spectrum Sharing for migration [RAN1, RAN2, RAN4, RAN3]

- Study if any additional migration option(s) is needed (other than standalone, MRSS, and inter-RAT mobility between NR-6G). [RAN] [RAN2, RAN1, RAN3, RAN4]
RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s).

- Mobility between 5G NR and 6GR [RAN2, RAN3, RAN4]

Note: Inclusion of LTE/6G interworking/coexistence aspects may be further discussed based on the requirement from RAN plenary

Migration options supported in 5G NR includes inter-RAT mobility / cell reselection, DSS, and MRDC. In 5G study (as from TR 38.801 [1]), various migration options are considered: Option 2, 3/3A, 4/4A, 5, 7/7A. Typical migration options are shown in Table 2 below.

Table 2: Migration options in 5G

| Option | Architecture | Commercialized |
| --- | --- | --- |
| Standalone (Option 2) Note: Inter-RAT mobility and DSS are migration options on top of standalone option. |  | Yes |
| EN-DC (Option 3) |  | Yes |
| eLTE (Option 5) |  | No |
| NG-ENDC (Option 7) |  | No |

The motivation to specify non-standalone architectures in NR (e.g. EN-DC) is to allow early deployment of NR, e.g. to cater for the smooth upgrade of 4G network and inter-operability between 4G and 5G. Consequently, several drops (ASN.1 freeze) were introduced due to the high load in RAN WGs, e.g. for NSA and SA. This causes both specification/network/UE complexity and additional CAPEX for operators. For example, EN-DC was shortly deployed in China, but was quickly phased out for switching to SA, which not only delayed SA deployment but also incurs more CAPEX. This is a lesson to be learned for 6G standardization. It was already agreed in RAN plenary#102 meeting that (in RP-233985 [2]) Release 21 is expected to be delivered with a single drop (i.e., a single code freeze).

Lesson learnt from 5G (6): Too many options (SA, DSS, EN-DC, NG-EN-DC) introduced in NR led significant load to WGs, caused both network/UE complexity and additional CAPEX for operators which should be avoided.

Baseline migration options include standalone, MRSS, and inter-RAT mobility between NR and 6G. For inter-RAT mobility between NR and LTE, three options are supported: inter-RAT HO and redirection when NR UE is in RRC_CONNECTED, and cell reselection when NR UE is in RRC_IDLE or RRC_INACTIVE. The main difference between inter-RAT HO and redirection is that there is larger service interruption for redirection as UE’s RRC connection in NR is released and UE performs cell selection in LTE and establishes connection to LTE. In redirection, there is less interaction between nodes of different RATs. For inter-RAT mobility between NR and 6G, both inter-RAT handover and redirection can be considered in view of performance and flexibility.

RAN WG 6G SID RP-252912 [1] states that “RAN plenary starts this study in March 2026 and will make a decision by September 2026 whether to expand WG SI scope to cover additional migration option(s)”. MRDC, as a migration option, has the issue of complexity of tight interworking at network side and UE side (as discussed above), and also increases the significant WG workload, market fragmentation, hence it is challenging to consider MRDC for migration from 5G NR to 6G.

Considering the supported high data rate in 5G, to avoid the impact on user experience, NR has improved the mobility interruption a lot with LTM, etc. We expect that the similar mechanisms will be introduced in 6G as well to meet the mobility requirements. However, the interruption and the data loss during traditional inter-RAT handover, e.g., 4G-5G cannot be avoided. We do see the requirements to improve the inter-RAT handover as well in order to avoid downgrade of the use experience. To minimize interruption and data loss during mobility between 5G and 6G (which is one drawback of inter-RAT HO), but avoid to introduce high network and UE complexity, inter-RAT LTM can be considered as an high performance inter-RAT mobility solution with low interruption time.

For inter-RAT mobility between NR and LTE, the solutions to support depends on the core network. Table 3 below compares the supported features in different scenarios related to core network, according to TS 38.300:

Table 3: Inter-RAT mobility support in different scenarios

| Features | Intra-5GC mobility | 5GC to EPC mobility |
| --- | --- | --- |
| Xn based inter-RAT HO | Supported | Not supported |
| NG based inter-RAT HO | Supported | Supported |
| In-sequence and lossless HO | Supported | Not supported |
| Data forwarding (control plane) | Supported | Only for DL data |
| Data forwarding (user plane) | Supported | Supported |
| Redirection | Supported | Supported |

There are some differences (e.g. support of Xn based HO, whether to support in-sequence and lossless HO) when the core network for inter-RAT mobility is same or different. Similar discussion could also happen for inter-RAT mobility between NR and 6G. Although the assumption for 6G core network will help the discussion in RAN2 on inter-RAT mobility, given that SA2 also just started discussion on 6G architecture, it might not be possible to get concrete views from SA2 on 6G core network architecture (especially regarding the differences between 5G and 6G core network). Therefore RAN2 can continue study on inter-RAT mobility in high level and only consult SA2 on core network architecture when there are critical issues to resolve and SA2 discuss has progressed sufficiently to provide the answers.

Proposal 7: In 6G RAN2 study on inter-RAT mobility:

- Cell reselection in Idle mode is supported.

- For inter-RAT mobility between NR and 6G in Connected mode, inter-RAT handover and redirection can be the starting point for further study.

- RAN2 to consider solutions for interworking between 5G NR and 6G with similar performance as intra-RAT mobility, e.g. high performance inter-RAT mobility.

- RAN2 can continue study on inter-RAT mobility in high level and only consult SA2 on core network architecture when there are critical issues to resolve and SA2 discussion has progressed sufficiently to provide the answers.

# Conclusion

In this contribution, we discuss design target for 6G mobility, intra 6G mobility in different RRC states, and inter-RAT mobility, and propose the following based on lessons learnt from 5G:

Lesson learnt from 5G (1): Too many mobility solutions were introduced in 5G which increased standards work load, UE/network complexity and also led to market fragment, especially some solutions are too complex to UE implementation which should be avoided.

Proposal 1: In 6G RAN2 study on mobility, compared to 5G, RAN2 should aim to reduce the options for 6G mobility design. Following NR mobility features can be used as the starting point for 6G study: L3 handover and LTM, including sub-features like RACH-less / early sync and conditional mobility (CHO and conditional LTM).

Lesson learnt from 5G (2): There is no commercial need on the requirement “0ms interruption time”, which should be avoided in 6G.

Proposal 2: In 6G design, RAN2 should set the reasonable target on interruption requirement and robustness requirement unless there are additional requirements from RANP. The interruption time from 5G “best solution” LTM can be used as the target performance in RAN2, i.e., not worse than 20ms interruption time. For robustness performance, 5G CHO performance can be the target for 6G without the need for further improvement.

Lesson learnt from 5G (3): 5G supports two independent mobility solution directions: L3 mobility and LTM. This incurs complexity at both UE and network side, as well as duplicated configuration and UE efforts for mobility.

Proposal 3: In 6G RAN2 study on mobility, RAN2 to study unified mobility framework for both L1 and L3 mobility, using 5G LTM configuration as the starting point, by considering following directions:

- Unified Mobility Pre-Configuration: all candidate configuration(s) are configured via the same RRC IE.

- Unified Trigger: NW triggered via L2 or L3 signaling and UE triggered based on measurement event.

- Unified Feature Set: early DL/UL synchronization, early CSI acquisition, fast RRC processing and subsequent mobility.

- Unified Measurement: unified measurement configuration, unified measurement report trigger and unified measurement report signalling.

Observation: Beam level mobility defined in 5G can support the mobility between TRPs of the same cell, and common channel reception during mobility between TRPs is ensured by UE implementation.

Proposal 4: From RAN2 perspective, beam level mobility for mobility between TRPs of the same cell / base station as supported in NR can be applicable to 6G while details can be studied by RAN1.

Lesson learnt from 5G (4): The design of RRM measurement is lack of system-level consideration in 5G life-cycle as there are redundant UE measurement behaviors, duplication between measurement tasks, measurement configurations and resource configurations which make UE measurement behaviors complicated.

Proposal 5: In 6G RAN2 study on measurements, RAN2 to study simplified RRM architecture considering L1 and L3 measurement and multiple cells, with potential collaboration with RAN1 and RAN4 on unified measurements on L1 and L3 mobility, MIMO, BFD and RLM. For measurement gap related study, RAN2 wait for RAN4’s progress.

Lesson learnt from 5G (5): In 5G cell reselection, there is lack of unified control for feature specific frequency prioritization and cell individual offset which increased the complexity, and also difficult to use.

Proposal 6: In 6G RAN2 study on “idle mode” mobility, basic operations of 5G cell reselection can be a starting point for idle mode mobility in 6G, e.g., frequency/cell/feature based selection priority. RAN2 to further study how to make cell (re)selection mechanism more efficient and simpler in 6G, especially, take into account spectrum aggregation and multiple features support.

Lesson learnt from 5G (6): Too many options (SA, DSS, EN-DC, NG-EN-DC) introduced in NR led significant load to WGs, caused both network/UE complexity and additional CAPEX for operators which should be avoided.

Proposal 7: In 6G RAN2 study on inter-RAT mobility:

- Cell reselection in Idle mode is supported.

- For inter-RAT mobility between NR and 6G in Connected mode, inter-RAT handover and redirection can be the starting point for further study.

- RAN2 to consider solutions for interworking between 5G NR and 6G with similar performance as intra-RAT mobility, e.g. high performance inter-RAT mobility.

- RAN2 can continue study on inter-RAT mobility in high level and only consult SA2 on core network architecture when there are critical issues to resolve and SA2 discussion has progressed sufficiently to provide the answers.

# References

[1] RP-252912, “New SID: Study on 6G Radio”, NTT DOCOMO (Moderator)

[2] RP-233985, “High-level Considerations for 6G Timeline”, TSG Chairs

[3] R4-2514644, “WF on [116bis][105] 6G RRM”, Apple

[4] RP-192277, “NR mobility enhancements”, Intel

[5] R2-1907105, “Way forward on minimization of HO interruption time”, ZTE Corporation, Ericsson, Nokia, Nokia Shanghai Bell, Sanechips