3GPP TSG-RAN WG2#132                	                                                             R2-2508100

Dallas, USA, Nov. 17th – 21st, 2025

Source:	CATT

Title:	Considerations on 6GR AS security

Agenda Item:	10.3.3.3

Document for:	Discussion and Decision

- Introduction

This document regards 5G AS security mechanism as the baseline in 6G, and identifies following issues:

- KgNB* exposed to a third node due to cancelled handover preparation;

- Suboptimal DL data pre-processing and LTM failure recovery;

- Unprotected system information.

We propose RAN2 to acknowledge these issues and send an LS to SA3 accordingly for further study.

- Discussion

  - General aspects for AS security

The mechanism of AS security in 5G generally follows the one in LTE. We have not observed any reason to adopt a complete new AS security mechanism in 6G. Therefore, we prefer assuming the 5G AS security mechanism as the baseline of 6G.

Proposal 1: Assume that the AS security mechanism in 5G is the baseline in 6G.

One major change relative to LTE is allowing that the KgNB does not change upon intra-RAN-node handovers, which vastly reduces UP delay/interruption by no longer requiring PDCP reestablishment.

The motivation is persuasive, but some security risks are introduced as well.

The pair {PCI, ARFCN} shall be unique among all neighbouring cells of a given cell due to measurement reasons. However, there is no reason to require the pair {PCI, ARFCN} to be unique among all cells neighbouring at least one served cell of a given RAN node. Considering that one gNB can host up to 16384 cells (in order for centralised deployment of CU or BBU), but the range of PCI is only 1008, such requirement is neither practical.

Observation 1: The pair {PCI, ARFCN} is not required to be unique among all cells neighbouring at least one served cell of a given RAN node.

Figure 1: Node 1 sends entirely the same KgNB* to Node 2 and Node 3

Assume that there are two cells “Cell 2” and “Cell 3”, hosted by “Node 2” and “Node 3” respectively, sharing the same {PCI, ARFCN} value and neighbouring a large given RAN node, namely “Node 1” at the opposite directions. It is thus possible that:

- Step 0: A UE connects to Node 1 using a KgNB, and is near Cell 2.

- Step 1: Node 1 selects Cell 2 as a candidate target cell for the UE.

- Step 2: Node 1 generates a KgNB* from the current KgNB (or NH) and {PCI, ARFCN} of Cell 2, and sends the KgNB* toward Node 2 in a handover preparation procedure.

- Step 3: Node 1 removes Cell 2 from candidate cell lists and cancels the handover preparation.

- Step 4: The UE moves to the opposite side of Node 1 and is near Cell 3, during which it keeps RRC_CONNECTED and keeps using the original KgNB.

- Step 5: Node 1 selects Cell 3 as a candidate target cell for the UE.

- Step 6: Node 1 generates a KgNB* from the current KgNB (or NH) and {PCI, ARFCN} of Cell 3, and sends the KgNB* toward Node 3 in a handover preparation procedure.

- Step 7: The UE is indeed handed over to Cell 3, turning the derived KgNB* into use.

The KgNB* sent in Step 2 is totally the same as the KgNB* sent in Step 6, due to completely the same derivation input values. As the result, the key used between the UE and Node 3 is known not only in Node 1 but also in Node 2.

Observation 2: Including only KgNB/NH and {PCI, ARFCN} of target cell as inputs to calculate KgNB* while allowing no (master) key update during intra-node handover may incur a situation that the KgNB in use is known not only by the source node of the last inter-node handover but also by a third node.

In later 5G releases, AS mobility enhancements makes the mechanism of AS security more troublesome.

One suboptimal case is on Early Data Forwarding. Early Data Forwarding was introduced for CHO and DAPS, and later reused in (C)LTM. Considering that (C)LTM is widely supported as a Day 1 feature of 6G, Early Data Forwarding will probably be supported from 6G Day 1. In CHO/(C)LTM, it is a common case that multiple cell in one (target) RAN node is configured as candidates, and Early Data Forwarding is performed for all of them.

Figure 2: Difficulty in pre-processing packets received from Early Data Forwarding

However, the target node finds it difficult to pre-process the forwarded DL packets. In LTE and 5G, the PCI and ARFCN of the target cell are inputs in calculating KeNB*/KgNB* in inter-node handovers, making that the KeNB/KgNB used for one candidate cell—and consequently, the KUPenc/KUPint used for that cell—is different from the key used for another candidate cell. The target RAN has to make a choice: either to pre-process every packet per candidate cell (using the key for that cell), or to give up pre-processing entirely (i.e. packets are stored only as PDCP SDUs), causing relatively long UP delay/interruption time upon inter-node handovers.

Observation 3: Including {PCI, ARFCN} of target cell as inputs to calculate KeNB*/KgNB* (and consequently KUPenc/KUPint) makes it hard for the target RAN node to pre-process the packets received through Early Data Forwarding.

Another suboptimal case is on intra-node LTM failure recovery through intra-node LTM candidates, which typically requires no key change. The UE sends an RRCReconfigurationComplete message (corresponding to the target cell) over Uu when completing a LTM, and the complete message is encrypted by a “key stream” whose calculation input includes only KRRCenc and PDCP COUNT as variables (other inputs are constant). When it fails, the UE selects another candidate cell and sends another RRCReconfigurationComplete message (corresponding to the recovery cell) over Uu. The two RRC messages shall not use the same “key stream” (otherwise there will be a security risk), so the latter RRC message has to use a PDCP COUNT higher than the former one.

Figure 3: Delay in intra-RAN-node LTM failure recovery

However, the RAN node usually did not receive the former RRC message, so that PDCP COUNT is skipped. Due to this gap, the PDCP layer in the RAN node will ordinarily wait a reordering timer length before submitting the RRC message toward the upper layer, causing an unnecessary (and lengthy) delay on LTM failure recovery.

Observation 4: The delay of intra-RAN-node LTM failure recovery in 5G is typically lengthy partly due to the design that KRRCenc does not change upon intra-RAN-node handover.

We suggest RAN2 to acknowledge the two suboptimal aspects of 5G, and if possible, prevent them from happening again in the 6G.

Proposal 2: Acknowledge the one risk and two suboptimal aspects of 5G shown in the prior observations, and if possible, to send an LS to SA3 for information.

  - Security of system Information

Broadcasting system information (SI) is one of the essential functions of the RRC protocol, defined in 3GPP TS 38.331 ‎[1]. Broadcasted messages are intended for all UEs which are camping on a cell.  In the idle mode or inactive mode, the UE monitors the SI of cells and choose a suitable cell to camp on. The UE typically acquires the SI from the cell and performs initial access to transition to connected state to obtain services. The system information includes information, among others, like cell (re-)selection parameters, neighbouring cell information, frequency priority, common channel configuration information, NAS common information, and public warning system (PWS) messages.

SA3 has studied and investigated if and how a new protection mechanism could be introduced against over-the-air attackers who broadcast rogue SI messages or replay previously captured SI messages as-is (without modification). In general, an integrity and replay protected SIs could add security value by at least making it difficult for over-the-air attackers to succeed in using a rogue SI or a previously captures SI at a later time, e.g., to lure UEs using SI messages with incorrect neighbouring cells, and to send self-crafted or old PWS messages. The Key issue details and the related solutions have been included in TR 33.809 ‎[2].

Observation 5: According to TR 33.809, lack of integrity protection of SIs poses security risks, e.g. the attacker may use a rogue SI or a previously captures SI to lure UEs to incorrect neighbouring cells or to send self-crafted or old PWS messages.

However, the security of broadcasting system information has only been studied in 5G but not specified, as the basic and essential security system design for 5G was finished during the initial discussion phase, making core enhancements difficult in subsequent versions during 5G evolution. ‌

Observation 6: Security of broadcasting system information is studied in 5G but not be specified due to the late introduction timing.

With current and potential future technological advancements, an increasing number of communication networks and UE applications may be subjected to attacks, such as DoS attacks on network or on UE, replay attacks, privacy compromising attacks, .etc.

Therefore, it is reasonable to continue studying the security of broadcasting system information as a native tech in 6G Day1. Since the MIB, SIB1 and SI message(s) are periodically and frequently broadcasted, encryption imposes a significant burden and large delay on data encoding/decoding in both UE and NW sides, but integrity protection ensures that system information cannot be tampered with. The 6G system should enhance the integrity protection of broadcasting system information.

Proposal 3: Integrity protection of broadcasting system information is supported in 6G Day 1.

Since the system information is broadcasted by the cell, and all UEs in the cell could receive it, the traditional symmetric algorithm is hardly applicable. By using asymmetric algorithm, digital signature could be generated in the NW/OAM based on private key, and the public key could be sent to the UE for verification. The asymmetric-algorithm-based digital signature is typically 4 times long of a symmetric-algorithm-based MAC-I, making it kind of costly. Therefore, we propose protecting only essential SI, e.g. those related to mobility (cell reselection) and/or PWS.

Proposal 4: RAN2 to further study how to reduce the overhead of SI integrity protection, e.g., to apply integrity protection only for essential System Information.

- Conclusion

In this contribution, we provide our views on the 6GR AS security:

Proposal 1: Assume that the AS security mechanism in 5G is the baseline in 6G.

Observation 1: The pair {PCI, ARFCN} is not required to be unique among all cells neighbouring at least one served cell of a given RAN node.

Observation 2: Including only KgNB/NH and {PCI, ARFCN} of target cell as inputs to calculate KgNB* while allowing no (master) key update during intra-node handover may incur a situation that the KgNB in use is known not only by the source node of the last inter-node handover but also by a third node.

Observation 3: Including {PCI, ARFCN} of target cell as inputs to calculate KeNB*/KgNB* (and consequently KUPenc/KUPint) makes it hard for the target RAN node to pre-process the packets received through Early Data Forwarding.

Observation 4: The delay of intra-RAN-node LTM failure recovery in 5G is typically lengthy partly due to the design that KRRCenc does not change upon intra-RAN-node handover.

Proposal 2: Acknowledge the one risk and two suboptimal aspects of 5G shown in the prior observations, and if possible, to send an LS to SA3 for information.

Observation 5: According to TR 33.809, lack of integrity protection of SIs poses security risks, e.g. the attacker may use a rogue SI or a previously captures SI to lure UEs to incorrect neighbouring cells or to send self-crafted or old PWS messages.

Observation 6: Security of broadcasting system information is studied in 5G but not be specified due to the late introduction timing.

Proposal 3: Integrity protection of broadcasting system information is supported in 6G Day 1.

Proposal 4: RAN2 to further study how to reduce the overhead of SI integrity protection, e.g., to apply integrity protection only for essential System Information.

- References

- TS 38.331 NR; Radio Resource Control (RRC) protocol specification

- TR 33.809 Study on 5G security enhancements against False Base Stations (FBS)