3GPP TSG-RAN WG2 Meeting #132	R2-2508117

Dallas, USA, 17 – 21 November 2025

Agenda item:	9.2.2

Source:	Nokia

Title:	Topology 2 aspects

WID/SID:	Ambient_IoT_Solutions_Ph2 - Release 20

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses several aspects related to AIoT operation in Topology 2 scenarios. More specifically, proposals on resource allocation and multi-reader operation are made. Concluding remarks concert also UE reader signaling.

# 2	Aspects of Topology 2 Operation

## 2.1	Resource allocation

In RAN2#131-bis, it was agreed that the resources for the UE reader to use for communication on AIoT interface would be allocated by the gNB through RRC signalling, and that the UE reader may provide assistance information to support the allocation:

- gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. - the reader schedules the AIoT interface transmissions within the resources allocated by the gNB

Open issues include questions around the validity of allocated AIoT resources, their distribution in time and the access of the UE reader to them.

As a preliminary remark, we observe from the WID that the UE reader is expected to be in RRC connected during the AIoT communication session, thus it can be expected that the gNB can be fully in control of the resources.

When allocating AIoT resources, it is natural to think of contiguous allocation. Given the finite processing time for simple devices – especially complex or lengthy commands may take non-negligible time, it may be reasonable to also allocate resources in blocks. To simplify signalling and monitoring, it is desirable to use predictable resource allocation such as piece-wise periodic patters. Their usage allows coordination between readers as well as session multiplexing.

Proposal 1: RAN2 assumes contiguous as well as  periodic block allocation of AIoT resources in time to UE readers.

Clearly, the gNB should be in charge of allocated resource and optimized their usage even after initial / coarse allocations are made. One way of implementing this may consists in the gNB indicating the resource usage start and end times to the UE readers.

Proposal 2: The gNB indicates to the UE reader the start and end time for accessing the allocated AIoT resources.

At the same time, it is important to note that it is actually the UE reader that has full view of the progress of the AIoT session. An AIoT session may get delayed due to collisions (eg when a large number of devices are addresses by a broadcast request), bad network conditions, low harvested power, or low power CW for device 1. Similarly, efficient resource usage allows the UE reader to use less resources than envisaged earlier by the gNB, eg thanks to background interference management of parallel reader activity.

For efficient resource management by the network, the UE reader must be able to provide relevant feedback to the gNB. Most importantly, we see the need the signal the request for additional resources as well as the release of unused resource.

Proposal 3: The assistance information of a UE reader on its AIoT transmissions may include

- request for additional AIoT resources (eg as “incomplete transmission” indication),

- release of existing AIoT resources (eg as “end of transmission” indication).

An important question is on what to do when the UE reader loses connectivity to the controlling gNB in the event of a handover or RLF. Loss of synchronization may cause harmful interference and impact other readers. To protect Uu quality and controllability, the simplest approach is to mandate that a UE reader with interrupted / limited control by the gNB pauses or stops entirely its AIoT transmissions or even reader activities.

Proposal 4: To protect Uu, UE readers pause / stop the usage of the AIoT resources in scenarios with interrupted / limited gNB control such as RLF or handover. FFS details.

Even though a UE reader that re-gained connectivity to its serving gNB can automatically resume its AIoT reader activities, it may be desirable to design a system in which gNB must explicitly reactivate once paused / stopped reader in order to avoid uncoordinated scenarios or undefined states, or to permit for global optimization of the underlying conditions.

Proposal 5: RAN2 to discuss whether a resume signal is needed to re-activate a paused / stopped UE reader after scenario with limited gNB control. FFS details.

## 2.2	Multi-Reader Operation

For the Release 19 point of view, it may be possible for the AIoTF to select multiple UE readers for the same AF request, however these will not operate in parallel for the same coverage area.

In Release 20, we would like to permit the selection of at least UE readers. Clearly, this process should be transparent to the device, that is, there should not be any difference for the device between the operation in Topology 1 setting with a gNB reader or Topology 2 setting with a UE reader. Additionally, due to the transparency w.r.t. reader operation from a device point of view, selection between gNB and UE reader could also be transparent.

Observation 1: In Topology 2, UE readers may be mobile and could have mutually overlapping coverage.

Proposal 6: An AIoT device may be offered the same service by multiple readers. FFS details of reader selection by device including time constraints.

Observation 2: An AIoT device is assumed not to (be able to) distinguish between operation in Topology 1 and 2 readers

Proposal 7: An AIoT device may be served transparently by both UE reader and gNB reader.

To assist the device to select the most suitable UE reader, we support the reader ID indication. Without a reader ID, the device may be substantially constraint in its selection options as readers would have to use some convention in order understand who the reader selected by the device is.

For example, the last reader that paged the device prior to its response could be understood as the target reader. This simplistic approach however leaves room for error (e.g., due to the hidden / exposed node problem, slow device processing under multiple paging requests) which can be simply avoided by the reader ID usage.

Observation 3: Reader ID simplifies UE reader operation in Topology 2.

Proposal 8: Reader ID is supported.

In this context, it is noted that RAN1 discussed efficient usage of preambles for implicit ID indication, for example to deliver information associated with paging or transaction ID. The usage of preambles permits quick and simple PHY-level pre-processing of key identification parameters that could help efficiently decide whether to decode the MAC layer payload, a substantially more complex (and energy-demanding) task.

Observation 4: Preamble-based ID indication permits efficient decision-making without energy/time-consuming decoding of MAC payload.

Proposal 9: RAN2 to send an LS to RAN1 whether preambles can be used to indicate paging/reader/transaction ID.

## 2.3	UE Reader Signalling

The usage of UE reader poses particular challenges to AIoT operation. One of them is UE mobility and the dynamic nature of Topology 2 operation in general. Another one consists in efficient multi-reader operation in such settings.

Observations 5: UE reader availability and / or reachability is transient in Topology 2.

Similarly to R19, we do not see the need for devices to report their individual capabilities or specific capability summaries. The only thing that matters in AIoT operation is the ability of a device to harvest energy and use it for communications with a reader. This ability may depend on hardware and operational options of the device but can be summarized in one meta parameter - the device energy status. Tracking of the energy device status then allows better control and management of AIoT requests and re-transmissions in time.

We see therefore the need to report device energy status for efficient AIoT function.

Proposal 10: Device energy status is reported by the device.

# 3	Conclusion

Always echo in the conclusion all proposals made above.

This document has made the following observations:

Observation 1: In Topology 2, UE readers may be mobile and could have mutually overlapping coverage.

Observation 2: An AIoT device is assumed not to (be able to) distinguish between operation in Topology 1 and 2 readers

Observation 3: Reader ID simplifies UE reader operation in Topology 2.

Observation 4: Preamble-based ID indication permits efficient decision-making without energy/time-consuming decoding of MAC payload.

Observations 5: UE reader availability and / or reachability is transient in Topology 2.

And proposed the following:

Proposal 1: RAN2 assumes contiguous as well as  periodic block allocation of AIoT resources in time to UE readers.

Proposal 2: The gNB indicates to the UE reader the start and end time for accessing the allocated AIoT resources.

Proposal 3: The assistance information of a UE reader on its AIoT transmissions may include

- request for additional AIoT resources (eg as “incomplete transmission” indication),

- release of existing AIoT resources (eg as “end of transmission” indication).

Proposal 4: To protect Uu, UE readers pause / stop the usage of the AIoT resources in scenarios with interrupted / limited gNB control such as RLF or handover. FFS details.

Proposal 5: RAN2 to discuss whether a resume signal is needed to re-activate a paused / stopped UE reader after scenario with limited gNB control. FFS details.

Proposal 6: An AIoT device may be offered the same service by multiple readers. FFS details of reader selection by device including time constraints.

Proposal 7: An AIoT device may be served transparently by both UE reader and gNB reader.

Proposal 8: Reader ID is supported.

Proposal 9: RAN2 to send an LS to RAN1 whether preambles can be used to indicate paging/reader/transaction ID.

Proposal 10: Device energy status is reported by the device.