3GPP TSG-RAN WG2 Meeting #132                                                                   R2-2508118

Dallas, USA, Nov. 17th - 21st, 2025

Source:	CATT

Title:	Discussion on Topology-2 for Ambient IoT

Agenda Item:	9.2.2

Document for:	Discussion and Decision

- Introduction

In the last meeting, we started the Topology-2 for A-IoT with the following progress,

- gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information - gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. - The reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

This contribution discusses some further details on Topology-2. Section 2 gives our detailed analysis on the following aspects:

- Inventory

- Command after inventory

- Handover/RLF of UE reader

Section 3 summarizes the contribution of this paper.

2. Discussion

## 2.1 Inventory

As mentioned in the RAN WID [1], only RRC-based solution for D2T2-B is considered in R20. During SI discussion, the inventory procedure for RRC-based architecture in D2T2-B was captured in TR 38.769 [2]. In this chapter, we further analyse some further details on the inventory procedure.

Figure 1: Message flow for inventory in Topology 2 RRC-based solution

### 2.1.1 A-IoT radio resource allocation

Dedicated A-IoT radio resource

On the A-IoT radio resource, it was confirmed the resource allocation can be triggered by the CN request. And it is provided to the UE reader via dedicated signalling.

- gNB provides AIoT resource information to be used for AIoT interface transmissions to the reader via RRC dedicated signaling. The resource allocation can be triggered by CN request. UE reader may provide assistance information related to AIoT transmissions. FFS what the information may be useful to be provided to gNB. It is up to gNB how the information is used. It is understood that assistance information is not mandated for the gNB to determine resource allocation. - The reader schedules the AIoT interface transmissions within the resources allocated by the gNB.

Another thing that needs to clarify is whether the dedicated signalling can configure UE reader with overlapping A-IoT radio resources, i.e., shared resource pool amongst UE readers. On this topic, it was captured in the TR during the SI discussion,

6.3.6 Topology 2 aspects on the interface between UE reader and RAN … The radio resources used by A-IoT radio interface between the A-IoT device(s) and UE reader are controlled by the network. The radio resources, which are dedicated for a UE reader, are only configured to the UE reader via dedicated signalling. The mechanisms for shared resource pool amongst UE readers are not considered in this release. The UE reader in coverage of BS scenario is supported. The UE reader may perform the A-IoT procedure on A-IoT radio interface between the reader and the device(s), only if the radio resource configuration is valid in the cell, which is under network control. The radio resource validity across multiple cells is not supported in this release. Hence, for all options below, the resources remain valid only in the same cell in which the resources were configured.

The intention is to avoid the potential interference in the overlapping coverage caused by multiple UE readers. It’s better to confirm this as a starting point in the WI phase. Therefore, it is proposed that,

Proposal 1: RAN2 to confirm the shared resource pool amongst UE readers is not supported.

Validity of the A-IoT radio resource

In last meeting, it was discussed whether to rely on the explicit release of the A-IoT radio resource or introduce timer-based mechanism for the validity of the A-IoT radio resource. During the online discussion, companies had touched more details in time-domain about one-shot, continuous or periodic-like resource allocation. In the following, we further discuss the validity of the A-IoT radio resource in the time-domain and frequency-domain dimensions.

Time-domain analysis

From our view, since we are aiming for a simple system, we should avoid a large signalling interaction during an inventory procedure. That means upon receiving the CN request, the gNB should allocate a suitable A-IoT radio resource to the UE reader (maybe based on the assistance information visible to the gNB from the CN) to complete the inventory procedure over the A-IoT radio interface (note the “complete” means the UE reader receives all the intended device ID Msg and stores them locally).

Observation 1: To achieve a simple A-IoT system, the gNB should allocate enough A-IoT radio resource to UE reader to complete the inventory procedure over the A-IoT radio interface. One way is based on the assistance information visible to the gNB from the CN.

Even the UE reader receives an unexpected A-IoT radio resource and finds that it is not enough to complete the inventory procedure, it can provide the assistance information to the gNB after exhaustion (at least for the same band case due to	not allowing the simultaneous (time overlapping) operation of Uu and A-IoT at the intermediate UE in the same band in the WID) of the A-IoT radio resource in time domain so that the gNB can configure another resource for further inventory process by the UE reader.

Observation 2: At least for the same band operation of Uu and A-IoT radio interface, if the UE reader receives a not enough A-IoT radio resource to complete the inventory procedure, it can provide the assistance information to the gNB after exhaustion of the A-IoT radio resource in time domain to request the resource for further inventory process.

Then some companies may argue that the gNB may configure UE reader with some redundant time-domain resource. In this case, some companies pointed out that the assistance information can be an indicator that the inventory procedure had been finished, and thus the gNB can release the A-IoT radio resource in advance. From our view, this is a further optimization as gNB can try the best to configure a suitable resource if we make some assistance information visible to the gNB from the CN.

In summary, since the A-IoT radio resource is configured by the gNB, it can be aligned between the gNB and UE reader on when the UE reader will exhaust the A-IoT radio resource. Thus, the UE reader should consider the A-IoT radio resource to be valid in case there is no explicit release of the A-IoT radio resource before the exhaustion of the resource in the time-domain. Therefore, it is proposed that,

Proposal 2a: The A-IoT radio resource remains valid at the UE reader side in case there is no explicit release of the resource before the exhaustion of the resource in the time-domain.

Frequency-domain analysis

However, since all the resources are under the gNB control, it should be allowed for the gNB to explicitly release the resource for some purposes. For example, the gNB does not want this UE to serve as a reader, or the gNB wants to use this A-IoT radio resource for other purpose. Thus, it is straightforward that the UE reader considers the A-IoT radio resource (both time/frequency domains) to be invalid upon being explicitly released by the gNB.

Proposal 2b: The UE reader considers the A-IoT radio resource (including time/frequency domains) to be invalid upon explicit release by the gNB.

UE assistance information to the gNB

Regarding the UE assistance information to the gNB, based on above analysis, we think 1-bit indication to request the gNB to provide a new A-IoT radio resource is enough. We do not want this assistance information so complicated because it’s up to gNB whether to use this information for further resource allocation.

Proposal 3: 1-bit indication can be used by the UE reader to request a new A-IoT radio resource configured by the gNB.

### 2.1.2 RRC inventory request/response

In step 1c, the gNB sends the RRC Inventory Request Msg to trigger the inventory procedure at the A-IoT radio interface. This RRC Msg will provide the A-IoT information to the UE reader.

- gNB will provide AIoT information to the UE reader via RRC dedicated signalling based on NGAP AIoT information received from CN. FFS what information

Firstly, we want to confirm this is a new RRC Msg.

Proposal 4a: Introduce a new RRC Msg (i.e., RRC Inventory Request Msg) to trigger the UE reader to perform the inventory procedure at the A-IoT radio interface.

Then RAN2 needs to further discuss the potential content within the RRC Inventory Request Msg, by taking into account the NGAP AIoT information received from CN. Recalling R19, the NGAP INVENTORY REQUEST message towards the gNB includes the following information:

INVENTORY REQUEST IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; OCTET STRING (SIZE(16)); YES; reject. A-IoT Correlation Identifier; M; INTEGER (0..65535, ...); YES; reject. Inventory Request Transfer; M; OCTET STRING; Containing the Inventory Request Transfer IE specified in subclause 9.3.x.1; YES; reject. 9.3.x.1 Inventory Request Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. A-IoT Correlation Identifier; M; INTEGER (0..65535, ...); YES; reject. A-IoT Device Identification Requested; M; 9.3.3.xx5; YES; reject. Requested Service Area Information; M; 9.3.3.xx3; YES; reject. Inventory Assistance Information; M; 9.3.3.xx4; YES; reject. Follow on Command Indication; O; ENUMERATED (true, ...); This IE indicates that there is command(s) to be transmitted.; YES; reject.

AIOTF Identifier/A-IoT Correlation Identifier

Firstly, UE reader needs to generate the transaction ID based on the A-IoT Correlation Identifier. Secondly, the A-IoT Correlation Identifier jointly with the AIOTF Identifier uniquely identifies an A-IoT service when there are parallel services towards a UE reader (Note that this is not parallel service over the A-IoT radio interface, because it’s up to reader implementation to execute the service one by one). So when performing inventory reporting to the gNB, UE reader needs to use this AIOTF Identifier/A-IoT Correlation Identifier to identify to which service the report corresponds.

A-IoT Device Identification Requested

It is straightforward to be aware by the UE reader, because the UE reader needs to page the targeted devices.

Requested Service Area Information

In R19, the requested service area information is used by the gNB to select the readers co-located within the gNB, because one gNB maybe have multiple RAN readers. When considering R20, this information is useless to UE reader, because whether to perform the UE reader functionality is under the gNB/CN control.

Inventory assistance information

In R19, the CN may provide the assistance information to the reader, for example, the approximate number of target A-IoT devices that may be involved in the inventory procedure for the efficient RACH resource allocation by the reader, or the expected D2R message size to support not segmented D2R transmission of the device ID. Such assistance information should also be applied for UE reader.

Follow-on command indication

For the command after inventory, in R19, the NGAP inventory request message further indicates to the reader that there is a follow-on command. Based on this indication, in topology-1, the reader will allocate the NGAP ID when reporting each device ID to the CN and map the NGAP ID with the AS ID. Upon receiving the follow-on command for that device, the reader can know using which AS ID to send this command to the specific device. The similar functionality should also be applied to UE reader in topology-2.

A-IoT radio resource configuration

Regarding the A-IoT radio resource information, since the trigger of A-IoT radio resource allocation is by the CN request to the gNB, it is straightforward to include the A-IoT radio resource configuration in the inventory request Msg.

Based on above analysis, it is summarized that,

Proposal 4b: The RRC Inventory Request Msg can include at least the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- A-IoT Device Identification Requested information;

- Inventory assistance information;

- Follow-on command indication;

- A-IoT radio resource configuration.

Regarding Step 2a, the intention is to follow the R19 design in topology-1 that the gNB may reject the A-IoT service request from CN due to the potential gNB payload problem. However, for topology-2, it’s up to gNB whether to provide the A-IoT radio resources to the UE reader based on the gNB payload. If it is provided, the UE reader should follow the gNB control to perform the A-IoT service, i.e., there is no need to introduce the inventory response mechanism for the UE reader to reject the A-IoT service.

Proposal 5: The UE reader should execute the A-IoT service under the gNB control if the gNB provides the A-IoT radio resource to the UE reader. No need to introduce the inventory response mechanism from UE reader side.

### 2.1.3 RRC inventory report

Regarding the inventory report process over RRC, it is clear to introduce a new RRC Msg for this purpose.

Proposal 6a: Introduce a new RRC Msg (i.e., RRC Inventory Report Msg) to report the inventory response from the UE reader to the gNB.

Regarding the content of RRC Inventory Report Msg, we can first recall the NGAP INVENTORY REPORT message in R19,

9.2.x.4 INVENTORY REPORT IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; OCTET STRING (SIZE(16)); YES; reject. A-IoT Correlation Identifier; M; INTEGER (0..65535, ...); YES; reject. Inventory Report Transfer; M; OCTET STRING; Containing the Inventory Report Transfer IE specified in subclause 9.3.x.4; YES; reject. 9.3.x.4 Inventory Report Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description. A-IoT Correlation Identifier; M; 9.3.3.xx1. Global gNB ID; M; 9.3.1.6. Reader Report List; 0..1. >Reader Report Item; 1..< maxnoofReaders>. >>Reader Index; M; 9.3.1.aa1. >>Device Report List; 1. >>>Device Report Item; 1..<maxnoofDevices>. >>>>A-IoT NAS PDU; M; OCTET STRING. >>>>RAN A-IoT Device NGAP ID; O; 9.3.3.yy1. Inventory Complete Indication; O; ENUMERATED (true, ...).

AIOTF Identifier/A-IoT Correlation Identifier

The AIOTF Identifier/A-IoT Correlation Identifier need to be included in the RRC inventory report to identify to which service the report corresponds, due to the parallel services executed by the reader (Note that this is not parallel service over the A-IoT radio interface, because it’s up to reader implementation to execute the service one by one).

Global gNB ID/Reader index

The purpose of introducing Global gNB ID/Reader index is to enable the device positioning at the granularity of reader level. This is meaningless for UE reader case, because the UE reader may move anywhere and thus cause the device to be not covered.

A-IoT NAS PDU (Device ID info)

The A-IoT NAS PDU needs to be included in the RRC inventory report, which is the response to the inventory service.

RAN A-IoT Device NGAP-like ID

As analysed in the section 2.1.2, if the follow-on command is included in the RRC inventory request message, similar to topology-1, the UE reader should allocate a UE A-IoT Device Uu ID when reporting each device ID to the gNB. The UE A-IoT Device Uu ID is used to map the AS ID so that upon arrival of the follow-on command for that device, the UE reader can know using which AS ID to address the corresponding device.

Inventory Complete Indication

For topology-2, similar functionality should be supported, i.e., the inventory complete indication needs to be included in the RRC inventory report so that the gNB can forward this information to the A-IoTF. Then the A-IoTF can send the report content to the AF.

Based on above summary, it is proposed that,

Proposal 6b: The RRC inventory report message can include at least the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- Per device report item, with each item including,

- A-IoT NAS PDU (Device ID info);

- UE A-IoT Device Uu ID;

- Inventory Complete Indication.

Proposal 6c: If UE reader receives the follow-on command indication in the RRC inventory request message, it allocates the UE A-IoT Device Uu ID when reporting the device ID information to the gNB and maintains the device context at UE side by mapping the UE A-IoT Device Uu ID with the AS ID.

## 2.2 Command after Inventory

In this chapter, we further analyse the command procedure based on the following procedure captured in TR 38.769.

Figure 2: Message flow for command in Topology 2 RRC-based solution

### 2.2.1 RRC command request

Upon executing the command procedure after inventory, the gNB allocates the A-IoT radio resource for the command procedure over A-IoT radio interface. After that, gNB sends the RRC Command Request Msg to convey the necessary information to the UE reader. Similar as above, we want to confirm this as a new RRC Msg.

Proposal 7a: Introduce a new RRC message (i.e., RRC Command Request Msg) to trigger the UE reader to perform the Command procedure at the A-IoT radio interface.

Regarding the content of RRC Command Request Msg, we can first recall the NGAP COMMAND REQUEST message in R19.

9.2.x.5 COMMAND REQUEST IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; OCTET STRING (SIZE(16)); YES; reject. A-IoT Correlation Identifier; M; INTEGER (0..65535, ...); YES; reject. RAN A-IoT Device NGAP ID; M; 9.3.3.yy1; YES; reject. Command Request Transfer; M; OCTET STRING; Containing the Command Request Transfer IE specified in subclause 9.3.x.5.; YES; reject. 9.3.x.5 Command Request Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. A-IoT Correlation Identifier; M; INTEGER (0..65535, ...); YES; reject. RAN A-IoT Device NGAP ID; M; 9.3.3.yy1; YES; reject. A-IoT NAS PDU; M; OCTET STRING; YES; reject. Command Assistance Information; O; 9.3.3.yy3; YES; reject.

AIOTF Identifier/A-IoT Correlation Identifier

The AIOTF Identifier and A-IoT Correlation Identifier need to be aware by UE reader for which service the follow-on command is, due to the potential parallel services towards the UE reader.

RAN A-IoT Device NGAP-like ID, i.e., UE A-IoT Device Uu ID

As described in section 2.1.3, UE A-IoT Device Uu ID needs to be sent to the UE reader because the UE reader needs to know for which device the command is applied.

A-IoT NAS PDU

This is the upper layer command (i.e., read, write or permanent disable) for UE reader to be sent to the corresponding device addressed by the associated AS ID.

Command Assistance Information

This command assistance information includes the expected D2R message size to assist the efficient scheduling by the reader. So it’s better to be aware by the UE reader.

A-IoT radio resource configuration

Similar to the inventory procedure, the A-IoT radio resource allocation for the following command procedure over the A-IoT radio interface is triggered by the command request from CN. So it is straightforward to include the A-IoT radio resource configuration for the command procedure in the RRC Command Request Msg.

Based on above analysis, it is summarized that,

Proposal 7b: The RRC Command Request Msg can include at least the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- UE A-IoT Device Uu ID;

- A-IoT NAS PDU (upper layer command);

- Command assistance information;

- A-IoT radio resource configuration.

### 2.2.2 RRC command response

After the UE reader receives upper layer command response from the device, the UE reader needs to report the upper layer response to the serving gNB via RRC message, e.g., RRC Command Response message.

Proposal 8a: Introduce a new RRC Msg (i.e., RRC Command Response Msg) to report the command response from the UE reader to the gNB.

Regarding the content of the RRC Command Response, according to the NGAP COMMAND RESPONSE message,

9.2.x.6 COMMAND RESPONSE IE/Group Name; Presence; Range; IE type and reference; Semantics description; Criticality; Assigned Criticality. Message Type; M; 9.3.1.1; YES; reject. AIOTF Identifier; M; OCTET STRING (SIZE(16)); YES; reject. A-IoT Correlation Identifier; M; INTEGER (0..65535, ...); YES; reject. RAN A-IoT Device NGAP ID; M; 9.3.3.yy1; YES; reject. Command Response Transfer; M; OCTET STRING; Containing the Command Response Transfer IE specified in subclause 9.3.x.6.; YES; reject. Criticality Diagnostics; O; 9.3.1.3; YES; ignore. 9.3.x.6 Command Response Transfer IE/Group Name; Presence; Range; IE type and reference; Semantics description. A-IoT Correlation Identifier; M; INTEGER (0..65535, ...). RAN A-IoT Device NGAP ID; M; 9.3.3.yy1. A-IoT NAS PDU; M; OCTET STRING. Criticality Diagnostics; O; 9.3.1.3.

AIOTF Identifier/A-IoT Correlation Identifier

Similar to the RRC inventory report message, this information is also needed to be included in the RRC command response to identify to which service the response corresponds, due to the parallel services executed by the reader.

RAN A-IoT Device NGAP-like ID, i.e., UE A-IoT Device Uu ID

UE A-IoT Device Uu ID needs to be included in the RRC command response to identify which device is responding the command.

A-IoT NAS PDU (upper layer command response)

This is the upper layer command response to be included in the RRC command response message.

Based on above analysis, it is summarized that,

Proposal 8b: The RRC Command Response Msg can include at least the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- UE A-IoT Device Uu ID;

- A-IoT NAS PDU (upper layer response message);

## 2.3 Handover/RLF of UE reader

### 2.3.1 UE reader handover

For the RRC_CONNECTED UE reader, it may be handed over to a new cell due to mobility. From our view, the first issue is whether the UE continues serving as a reader in the target gNB (cell).

To my understanding, this is under the target gNB (cell) control. At least there is an extreme case that the target gNB may be a non A-IoT functionality supported node.

However, from the UE reader perspective, it can follow the NW indication. During a handover, the target gNB can explicitly indicate whether the UE should continue its reader role in the target cell via the HO command. According to the WID, the A-IoT radio resource is only valid within the current serving cell. This implies that if the target cell intends for the UE to continue serving as a reader, it should provide a new A-IoT radio resource configuration.

Proposal 9a: The target gNB can explicitly indicate whether the UE should continue its reader role in the target cell via the HO command.

Proposal 9b: Upon receiving the HO command, the UE continues to serve as a reader in the target cell if the HO command provides the A-IoT radio resource configuration.

### 2.3.2 Cell switch due to HO failure or RLF

The cell change can also occur during HO failure (for example, T304 expires) or RLF, which leads UE reader to initiate RRC re-establishment procedure to a new cell. So the similar issues as UE reader handover scenario are also found.

Upon finishing RRC re-establishment, the NW will send the first RRC Reconfiguration message to provide the configuration for the re-established cell. Therefore, similar to the handover scenario, in the first RRC Reconfiguration message followed by the RRC re-establishment procedure, the NW can either explicitly indicate the UE to continue acting as a reader in the re-established cell by including A-IoT radio resource configuration.

Proposal 10: Upon finishing RRC re-establishment, the UE continues serving as a reader in the re-established cell if the first RRC Reconfiguration message followed by the RRC re-establishment procedure provides the A-IoT radio resource configuration.

Then upon the initiation of RRC re-establishment, it needs to further discuss how to handle the A-IoT radio resource in the source cell. Since the A-IoT radio resource is only valid within the current serving cell, UE reader should release the A-IoT radio resource of the source cell upon initiating RRC re-establishment.

Proposal 11: UE reader releases the A-IoT radio resource of the source cell upon initiating RRC re-establishment.

3. Conclusion

Based on the analysis in the Section 2, the conclusions of this paper are summarized as follows:

Observation 1: To achieve a simple A-IoT system, the gNB should allocate enough A-IoT radio resource to UE reader to complete the inventory procedure over the A-IoT radio interface. One way is based on the assistance information visible to the gNB from the CN.

Observation 2: At least for the same band operation of Uu and A-IoT radio interface, if the UE reader receives a not enough A-IoT radio resource to complete the inventory procedure, it can provide the assistance information to the gNB after exhaustion of the A-IoT radio resource in time domain to request the resource for further inventory process.

Inventory

Proposal 1: RAN2 to confirm the shared resource pool amongst UE readers is not supported.

Proposal 2a: The A-IoT radio resource remains valid at the UE reader side in case there is no explicit release of the resource before the exhaustion of the resource in the time-domain.

Proposal 2b: The UE reader considers the A-IoT radio resource (including time/frequency domains) to be invalid upon explicit release by the gNB.

Proposal 3: 1-bit indication can be used by the UE reader to request a new A-IoT radio resource configured by the gNB.

Proposal 4a: Introduce a new RRC Msg (i.e., RRC Inventory Request Msg) to trigger the UE reader to perform the inventory procedure at the A-IoT radio interface.

Proposal 4b: The RRC Inventory Request Msg can include at least the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- A-IoT Device Identification Requested information;

- Inventory assistance information;

- Follow-on command indication;

- A-IoT radio resource configuration.

Proposal 5: The UE reader should execute the A-IoT service under the gNB control if the gNB provides the A-IoT radio resource to the UE reader. No need to introduce the inventory response mechanism from UE reader side.

Proposal 6a: Introduce a new RRC Msg (i.e., RRC Inventory Report Msg) to report the inventory response from the UE reader to the gNB.

Proposal 6b: The RRC inventory report message can include at least the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- Per device report item, with each item including,

- A-IoT NAS PDU (Device ID info);

- UE A-IoT Device Uu ID;

- Inventory Complete Indication.

Proposal 6c: If UE reader receives the follow-on command indication in the RRC inventory request message, it allocates the UE A-IoT Device Uu ID when reporting the device ID information to the gNB and maintains the device context at UE side by mapping the UE A-IoT Device Uu ID with the AS ID.

Command after Inventory

Proposal 7a: Introduce a new RRC message (i.e., RRC Command Request Msg) to trigger the UE reader to perform the Command procedure at the A-IoT radio interface.

Proposal 7b: The RRC Command Request Msg can include at least the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- UE A-IoT Device Uu ID;

- A-IoT NAS PDU (upper layer command);

- Command assistance information;

- A-IoT radio resource configuration.

Proposal 8a: Introduce a new RRC Msg (i.e., RRC Command Response Msg) to report the command response from the UE reader to the gNB.

Proposal 8b: The RRC Command Response Msg can include at least the following information,

- AIOTF Identifier/A-IoT Correlation Identifier;

- UE A-IoT Device Uu ID;

- A-IoT NAS PDU (upper layer response message);

Handover/RLF of UE reader

Proposal 9a: The target gNB can explicitly indicate whether the UE should continue its reader role in the target cell via the HO command.

Proposal 9b: Upon receiving the HO command, the UE continues to serve as a reader in the target cell if the HO command provides the A-IoT radio resource configuration.

Proposal 10: Upon finishing RRC re-establishment, the UE continues serving as a reader in the re-established cell if the first RRC Reconfiguration message followed by the RRC re-establishment procedure provides the A-IoT radio resource configuration.

Proposal 11: UE reader releases the A-IoT radio resource of the source cell upon initiating RRC re-establishment.

4. References

- RP-252894 Revised WI: Solutions for Ambient IoT (Internet of Things) in NR Phase 2

- 3GPP TR 38.769, “Study on solutions for Ambient IoT (Internet of Things) in NR (Release 19)”, V19.0.0 (2024-12)