3GPP TSG-RAN WG2 Meeting #132	R2-2508126

Dallas, U.S.A., 17th – 21st November 2025

Agenda Item:	10.3.1.3

Source:	MediaTek Inc.

Title:	On latency reduction mechanisms for 6G eMBB traffic

Document for:	Discussion and decision

# Introduction

In the previous meeting [1], we reached the following agreements on the user plane design. This document discusses design considerations for UL scheduling and data retransmissions such that eMBB’s data transfer latency can be reduced for 6G.

- 6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB) - Study need for scheduling enhancements to address the SR/BSR/DSR latency - Study how to improve L2 ARQ latency/efficiency in 6GR based on tight coordination with HARQ. Study should identify enhancements needed for HARQ and L2 ARQ

# Discussion

Figure 1: Typical UL resource acquisition delays in NR

Uplink scheduling latency reduction

When an NR UE has uplink (UL) data and no UL resources available to transmit the data, it transmits a scheduling request (SR) to the base station using pre-allocated dedicated SR resources to indicate its need to acquire UL resources. On SR reception, the base station only knows that this UE has ‘some’ data to transmit but does not know how ‘much’ data is to be transmitted. To avoid wasting radio resources, the base station usually allocates a small grant to the UE for it to indicate the amount of data in its UL buffers using a Buffer Status Report (BSR). On reception of the BSR, the base station can allocate an appropriately sized grant to the UE to transmit its data as illustrated in Figure 1. The current mechanism to acquire UL resources introduces significant latency to the start of an UL transmission. Table 1 provides a typical NR configuration in the field related to UL resource acquisition delays.

| Steps | Values |
| --- | --- |
| 0. UL data arrival | - |
| 1. Average SR delay | 10ms (20 slots) |
| 2. UL DCI | 0.5ms (1 slot) |
| 3. Scheduling delay (K2) | 1ms (2 slots) |
| 4. BSR transmission | 0.5ms (1 slot) |

Table 1: Typical UL scheduling configuration in NR

To counter this delay, NR introduced the configured grant (CG) feature with which the base station pre-allocates periodic UL resources to a UE. While useful for services with periodic traffic of deterministic size such as URLLC or voice calls, it is not spectrally efficient to use such a mechanism to serve aperiodic eMBB traffic, as resources need to be periodically reserved for specific UEs on the off chance that it may be used. The cost of reserving such dedicated resources for a UE is provided in Table 2. Therefore, we seldom observe CG deployed in NR.

Instead, what is often seen is that base stations randomly schedule small UL grants to the UE. Such a mechanism targets the SR delay in step 1 of Table 1. Avoiding step 1 delays would only be possible if the base station can perfectly predict UL data arrival timing. However, if the base station cannot achieve ‘perfect’ prediction, step 1 delay will still exist while radio resources are wasted. Furthermore, if the UE is not configured to skip UL transmissions when there is no data, this mechanism also results in increased power consumption.

Observation 1: Initiating uplink scheduling of eMBB data in NR faces significant delays due to SR unavailability.

|  | Resource Elements (RE) overhead per UE |
| --- | --- |
| SR + BSR (Dedicated to 1 UE) | 180 |
| BSR (Dedicated to 1 UE) | 168 |
| BSR (Shared by N UEs) | 168 / N |

Table 2: Physical layer resource cost of different approaches to UL scheduling

Ideally, the base station should be aware of UL data as soon as it arrives at the UE i.e. steps 1-3 in Table 1 are skipped and step 4 is initiated on data arrival. This would require the presence of UL resources that are always available for the UE to use for BSR transmissions while remaining spectrally efficient (i.e. avoiding the pitfalls of CG). We propose the use of a pool of always-available UL resources that are shared by several UEs. By sharing the resources among different users, the gNB can schedule always-available resources to tackle SR availability delays while keeping the cost per user down as shown in Table 2. When UL data arrives at the UE, it would immediately send a BSR on this shared resource to acquire UL resources early as illustrated in Figure 2.

Figure 2: Proposed mechanism for early UL resource acquisition in 6G

Proposal 1: Always-available contention-based uplink resources are introduced in 6G to enable early BSR transmission to reduce uplink scheduling latency.

Such a shared contention-based resource pool needs to be carefully designed to achieve a balance between low collision probability and keeping the resource overhead per UE to a minimum. When considering sharing resources amongst UEs, an important consideration is the identification of different UEs that are in contention with each other. This could mean differences in the design of the channel used for contention-based uplink transmissions. This is an aspect that would need to be considered by RAN1 as part of their physical channel design and therefore would require liaising with RAN1 early.

## HARQ-failure driven L2 data retransmissions

In past generations, HARQ and ARQ have been specified as independent functions for data reliability at MAC and RLC layers respectively, with their procedures completely decoupled from each other. Although these two mechanisms are designed to operate independently, in practice, L2 data retransmission procedure of ARQ is designed to only engage when HARQ delivery (i.e. initial transmission and subsequent retransmissions) is unsuccessful. Therefore, one can observe that despite being designed as completely independent functions, the trigger for L2 ARQ is a consequence of the outcome of HARQ delivery.

Observation 2: The only cause for L2 retransmissions is unsuccessful HARQ delivery.

As highlighted in the objectives of this 6G study [2] , we should focus on ‘dimensioning an appropriate set of functionalities, minimizing the adoption of multiple options for the same functionality’. A stack that includes two independent mechanisms to ensure reliability, especially when they are so tightly related in practice, would be contrary to this 6G objective. We therefore propose a HARQ-failure-driven L2 data retransmission mechanism where HARQ and ARQ functions are combined into one data reliability function that we refer to as SHARQ. HARQ failure would be identified by HARQ feedback from the receiver, and L2 data retransmissions can be triggered on HARQ feedback reception as illustrated in Figure 3.

Figure 3: SHARQ, a HARQ-failure driven L2 data retransmission mechanism

By combining HARQ and ARQ into a single procedure, we observe the following improvements in the latency and efficiency of data transmission:

- Reduced L2 data retransmission latency

A consequence of ARQ’s timer-driven L2 data retransmissions is that it is only initiated after an RLC SN gap is detected and the configured RLC timer expires. For packets that fall back to L2 retransmission due to HARQ delivery failure, the additional delay is around 40ms based on typical timer values that we observe in the field. However, if the last packet of a traffic burst (e.g. with immersive traffic) gets lost due to HARQ failure as illustrated in Figure 4, we observe an additional delay of traffic inter-arrival time, as the timer is started only upon SN gap detection. The timer-driven ARQ design cannot achieve low latency performance for general eMBB traffic that requires high reliability without trading off resource efficiency.

Figure 4: NR ARQ L2 retransmission latency

If HARQ failure is used as the trigger condition for retransmission instead of a timer-based approach, the additional delay mentioned above is completely avoided. The 6gNB would be in complete control of when HARQ or L2 data retransmissions are needed and can make appropriate scheduling choices without having to deal with procedure-driven delays.

Observation 3: By triggering L2 retransmissions using HARQ-feedback instead of ARQ timers, additional L2 retransmission delays due to ARQ timer expiry and SN gap identification are avoided.

- Improved memory efficiency

Another consequence of the timer-driven ARQ mechanism is the high memory requirement that it imposes, particularly on devices that support high data rates. The timer-driven procedure delays feedback, increasing the retention time of PDUs in the retransmission buffer of the transmitter. The amount of data that needs to be buffered during these delays rises in proportion with data rates, which is not scalable for 6G where higher data rates than previous generations are targeted. By using HARQ feedback to decide on data release/retransmission from L2 buffers, the delays associated with retransmissions come down, resulting in lower memory requirements. Furthermore, unifying the data reliability procedure into a single procedure leads to less data movement across functions/layers.

Observation 4: By reducing L2 retransmission delays, the memory required to buffer L2 data also reduces.

- Reduced processing complexity by dropping AM window management

The window management mechanism with ARQ in NR is such that the receiver needs to track the reception status of every byte of every SDU within the AM window. As illustrated in Figure 5, the ARQ sliding window in NR can grow all the way up to 131072 SDUs, placing significant processing to track SDU status when data goes missing.

Figure 5: Large ARQ sliding window increases data processing complexity

A typical NR configuration in the field is to have a default DRB configured for general data transmission that is configured with AM. Consequently, this window management complexity and processing overhead is exercised all the time for general data delivery, even though ARQ based retransmissions occur rather rarely, i.e. with a probability of ~10-5 when considering four HARQ retransmission attempts. By replacing the window and timer based ARQ function with SHARQ, we avoid requiring complicated window management mechanisms in favour of implementation/need-driven mechanisms to achieve the same outcomes.

Observation 5: The processing overhead to manage the ARQ window is significant, considering how seldom the retransmission mechanism is exercised.

- Improved per-packet handling flexibility by dropping AM window management

In previous NR releases, we have identified a need for the transmitter to drop packets without performing L2 retransmission to handle immersive services in cases where congestion is experienced and delay budgets are exceeded. A lesson learnt from enhancements done to handle XR traffic in Rel-19 is that handling of dropped packets is quite complicated due to the rigid nature of AM window management. What is clear from the discussions on RAN service-awareness is that there remains high interest to consider packet level handling and differentiation in 6G based on the packet’s importance and associated delay budget. By dropping the rigid window management that we have for AM in favour of flexible L2 buffer handling, decisions on per-packet L2 retransmission can be easily done by the transmitter without placing onerous requirements on the receiver.

Observation 6: Delay-based scheduling and per-packet handling flexibility is improved when rigid ARQ window management is dropped.

Based on these improvements to latency performance, memory requirements and processing overhead, we propose combining HARQ and ARQ functions to form a single function for reliability where HARQ-failures drive L2 retransmissions. If this function is well-designed, it can replace the timer and window-based ARQ function which would lead to further gains in processing complexity reduction and  packet handling flexibility.

Proposal 2: Independent HARQ/ARQ procedures are combined into a single procedure for reliability in which HARQ-failures trigger L2 data retransmissions.

Proposal 3: The procedure for reliable data transfer is designed such that it can replace NR’s timer and window-based ARQ mechanism.

For the HARQ-failure driven L2 retransmission mechanism to replace NR’s timer and window based ARQ, we have the following considerations for downlink (DL) and uplink (UL) data transmission:

- DL data transmission

Figure 6: Timer-based ARQ is designed independent of HARQ  in NR in case HARQ feedback is misinterpreted

The main reason to design L2 ARQ independent of HARQ in earlier generations is the HARQ feedback false-positive issue on the downlink. As illustrated in Figure 6 for NR, in case a HARQ feedback of a NACK is misinterpreted as an ACK, further HARQ retransmissions are stopped. NR relies on the RLC status report triggered on the timer expiry for data loss detection and retransmission.  The PUCCH channel used to carry HARQ feedback has been designed to meet a NACK to ACK false-positive rate of 10-3 [3]. On the other hand, the RLC status report sent on PUSCH has a significantly higher false-positive rate due to its longer CRC. If the 6G HARQ feedback false-positive rate is designed to match the false-positive rate that can be achieved by AM status reporting on PUSCH, we will not need an independent timer and window based ARQ mechanism in addition to HARQ and can rely on the HARQ feedback from the UE to trigger L2 retransmissions of downlink data from the 6gNB.

Observation 7: DL data can be reliably transmitted if the HARQ feedback is designed to be robust to misinterpretation.

Proposal 4: HARQ feedback for 6G is designed to be as robust to misinterpretation as a status report would be.

- UL data transmission

Figure 7: Uplink L2 retransmission in NR relies on status reports as HARQ feedback is absent

With NR, the PHICH feedback channel from LTE was dropped and only the DCI is used to inform the UE whether a new UL transmission or UL retransmission is needed. With the shift to DCI in place of PHICH, the misinterpretation issue that exists for DL data is avoided, as the CRC length of the DCI is comparable to PDSCH.

However, by dropping the PHICH channel from LTE, the UE is left unaware of the reception status of the UL data that was transmitted. The NDI bit carried in UL DCI only tells the UE if the UL HARQ buffer should be flushed for new transmission or not. The data in the HARQ buffer to be flushed may have been successfully received or failed to be decoded by the gNB. Without HARQ feedback information, the UE can only rely on RLC status reports to identify successful or unsuccessful UL transmissions as illustrated in Figure 7.

Figure 8: Uplink L2 retransmission latency reduction achieved with UL HARQ feedback

Instead, if the 6gNB provides the UE with UL data reception status for every UL transmission as illustrated in Figure 8, the UE can use this information to trigger UL L2 data retransmission promptly after the 6gNB gives up on HARQ retransmission.

Observation 8: UL data can be reliably transmitted if the UE gets HARQ feedback for UL transmissions.

Proposal 5: The UE is provided with explicit HARQ feedback that indicates the decoding result of UL transmissions.

By resolving the issues raised above for DL and UL data transmissions, we can have reliable data transmissions for 6G while improving latency performance and reducing memory requirements and processing overhead for all 6G users. In order to resolve these issues and realise these benefits, we need to jointly work with RAN1 as it can affect physical channel design. Therefore, we propose sending an LS to RAN1 to consider RAN2’s requirements for HARQ feedback availability and robustness as part of their design.

# Conclusion

In this contribution we have the following proposals based on these observations:

UL scheduling latency reduction

Observation 1: Initiating uplink scheduling of eMBB data in NR faces significant delays due to SR unavailability.

Proposal 1: Always-available contention-based uplink resources are introduced in 6G to enable early BSR transmission to reduce uplink scheduling latency.

HARQ-failure driven L2 retransmissions

Observation 2: The only cause for L2 retransmissions is unsuccessful HARQ delivery.

Observation 3: By triggering L2 retransmissions using HARQ-feedback instead of ARQ timers, additional L2 retransmission delays due to ARQ timer expiry and SN gap identification are avoided.

Observation 4: By reducing L2 retransmission delays, the memory required to buffer L2 data also reduces.

Observation 5: The processing overhead to manage the ARQ window is significant, considering how seldom the retransmission mechanism is exercised.

Observation 6: Delay-based scheduling and per-packet handling flexibility is improved when rigid ARQ window management is dropped.

Proposal 2: Independent HARQ/ARQ procedures are combined into a single procedure for reliability in which HARQ-failures trigger L2 data retransmissions.

Proposal 3: The procedure for reliable data transfer is designed such that it can replace NR’s timer and window-based ARQ mechanism.

Observation 7: DL data can be reliably transmitted if the HARQ feedback is designed to be robust to misinterpretation.

Proposal 4: HARQ feedback for 6G is designed to be as robust to misinterpretation as a status report would be.

Observation 8: UL data can be reliably transmitted if the UE gets HARQ feedback for UL transmissions.

Proposal 5: The UE is provided with explicit HARQ feedback that indicates the decoding result of UL transmissions.

Liaising with other groups

Proposal 6: Send an LS to RAN1 to take the following design requirements into consideration:

- Contention-based UL PUSCH resource designed for early BSR reporting

- HARQ feedback designed to be robust to misinterpretation

- HARQ feedback made available to the UE for UL transmissions

# References

- R2-2508002 - RAN2#131bis Meeting Report (ETSI MCC)

- RP-252912 - Revised SID: Study on 6G Radio (NTT Docomo et al)

- 3GPP TS 38.104 - NR Base Station (BS) radio transmission and reception