3GPP TSG-RAN WG2 Meeting #132	R2-2508137

Dallas, USA, 17th –21st Nov., 2025

Agenda Item:	10.3.1.2

Source:	Huawei, HiSilicon

Title:	Discussion on QoS, QoE and service-awareness in RAN

Document for: Discussion and Decision

1	Introduction

RAN2 has discussed several aspects of 6G User Plane in RAN2#131bis, the following agreement on service-awareness and QoS was reached [1].

Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework.

In this contribution, we focus our attention on characterization of traffic types for better supporting the existing services and accommodating the forthcoming new services. To achieve this, the new attributes introduced by the new services will be analyzed first, and the benefits and shortcomings of the existing protocol stack and the potential requirements on the 6G protocol stack design are discussed for enlightening the QoS-related considerations as well as protocol-related design.

2	New service requirements for 6G

This section intends to progress on the study of immersive communications and AI services as agreed during the last RAN2 meeting. The relationship between studies of R20 mobile AI and the 6G AI service will also be discussed.

2.1	Token Communication

AI services empowered by large AI/ML models are increasingly applied in various scenarios, such as AI agent, chatbot, personal assistance, autonomous driving and so forth [RP-252132]. In conventional AI traffic, typical compression techniques (e.g. LZ77, JPEG, etc.) are applied to compress the raw data prior to its transmission. As a consequence, at the receiver, the decompression is first required to extract the raw data from the compressed bitstream A tokenizer then transforms the raw data to the tokens, a compact data carrying the context information, before feeding them to the AI/ML model for processing. In contrast, token communication transforms the raw data directly into tokens at the AI application, eliminating the need for traditional compression techniques

On the receiver side, these tokens can be directly forwarded to the AI/ML model as soon as they are received. It has been shown that the token-based communication can achieve comparable performance to LLM-based processing while using only 25.8% LLM computational cost and improving inference efficiency by approximately 40% [2].

Observation 1-1:	The processing efficiency of the AIML model can be improved through token communications.

Similar to the other services transmitted over the network, the token communications traffic also exhibits specific traffic characteristics, which can be summarized as below:

2.1.1	Error tolerance

One notable characteristic of token communications is error tolerance. Figure 1 compares image reconstruction quality between the traffic with token prediction (2nd row) and the one without token prediction (1st row). It is clearly evident that with increasing token error rate, the image reconstruction quality with the aid of the token prediction significantly outperformances its counterpart without token prediction.

Figure1, Image reconstruction with/without token prediction

2.1.2	Different importance for different tokens

Furthermore, the tokens serving for different modalities exhibit different importance. It can be observed from the Figure 2 that the loss of text tokens has more significant impacts compared to the loss of image tokens. The reason is that the erroneous image tokens can be predicted and reconstructed more easily than the text tokens, while the latter is more error-sensitive and the accuracy ratio decreases significantly once token error rate is larger than 1%.

Figure2, Accuracy ratios with different error rate for image and text token

Additionally, the tokens may exhibit different importance even from the same modality. As shown in Figure 3, the loss of tokens on the latter place of the sequence of token transmission leads to less L2 error, which means that token positioned near the tail becomes less important.

Figure3, L2 error with different positions of lost tokens

Inspired by such characteristic, we believe that both the processing efficiency and transmission efficiency can be improved with the aid of the token communications.

Observation 1-2:	Token-based communications is error tolerant and different tokens have varying levels of importance depending on their modality and the position of the token within the traffic stream.

During the discussion in RAN1#122, LS R1-2508184 has been sent, capturing the current progress in RAN1 on the study of AI traffic model.

Agreement For the study traffic model(s) for 6GR AI/ML services: - A representative AI/ML service is the generative AI, e.g., as defined in TR22.870. Send LS to SA4 (cc RAN2, SA1, SA2) requesting input if any on traffic characteristics for AI/ML services. Note: RAN1 is discussing the following options for the model: - Option-1a: The model is parameterized by Token, e.g., Token size, Token arrival rate, and Token delay budget. - Token is the minimum unit of data generated in the application layer. - How to associate Tokens to PHY layer packets. - How to reflect the variable importance of tokens. - Whether other parameters are additionally needed when tokens are encapsulated together into a packet, e.g., packet arrival rate, packet success rate, and packet delay. - Option-1b: The model is characterized by the parameters of PHY layer packet, including e.g., packet size, arrival rates, latency requirement, reliability requirement, etc. - Option-1c: reusing or extending the FTP-3/XR traffic model. - FFS other models/options need to be defined for other AI/ML services.

Within the 3 options, Option-1a focuses on a token-parameterized model, leveraging token size, token arrival rate, and delay budget—all of which should be explicitly defined. Under this option, token is considered as the minimum unit of data generated in the application layer. Issues such as how to associated token to packets, how to reflect different importance of different tokens are also being discussed. This means that the RAN needs to be service-aware: token needs to be visible in RAN.

In contrast, Option-1b just treats packets generated by AI services as the normal traffic and Option-1c largely align with legacy traffic models studied in NR (e.g., XR applications and FTP traffic with PDB requirements). However, these options alone do not fully capture the unique characteristics of AI-service traffic, thereby necessitating a more refined modeling approach.

Note that the above option1 only mentioned about the variable importance of the tokens, not reflecting the error tolerant characteristics of token communications now. Hence, based on the discussion above, for the study of AI services in 6G, we propose the following

Proposal 1:	The design of 6G UP functionality should take into account the requirements and characteristics of AI services such as error tolerance and variable importance of different tokens.

2.2	Immersive Communication

In 5G, the traffic characteristics of XR services studied in R17 TR 38.838 is a specific type of immersive communications (IC) service, and the subsequent normative work has been completed in R18 and R19. According to RAN1 LS below [R1-2508184], XR traffic model studied in TR38.838 has been agreed to be the start point for the traffic model of immersive communication services.

Agreement Study traffic modelling for evaluations related to immersive communication services including but not limited to advanced XR [e.g., TR22.870] and haptics services, - XR traffic models (in TR 38.838) are considered as starting point. - FFS the detailed modifications on the parameters to the XR traffic model, e.g., higher packet size, higher packet arrival rate, higher packet size deviation, PDB, etc. - FFS how many models need to be defined and the corresponding representative use cases. - FFS how to incorporate haptics traffic (TR26.854). Send LS to SA4 requesting input if any on the relevant traffic characteristics, RAN1 can continue the study before SA4 potential response.

Observation 2-1:	RAN1 has agreed that XR traffic models (in TR 38.838) are considered as starting point for immersive communication services.

With the advent of AI era, the definition of immersive services has been expanded to include the mobile AI services. Typical use cases of immersive services have been specified in SA1 specification TR 22.870 [7], as shown below:

- Higher resolution XR cloud gaming: Immersive gaming with 8K resolution display defined in Section 9.1

- Intelligent video conferencing on AI phones: Enhanced meeting experience defined in Section 8.3

- Contextual understanding on AI wearable: Awareness of real-time environment, such as real-time translation, navigation, blind assistance, as defined in Section 8.3

- Healthcare for aging population on embodied AI robots: Healthcare for aging population has been defined in Section 9.9

A unified service flow for IC that can be concluded based on the above-mentioned services is shown in the Figure 4 below, as in [RP-252131]:

Figure4, Service flow of immersive communications services

- Step 1: IC device collects necessary data unit for AI inference or pose control information for split rendering in XR cloud.

- Step 2: IC device uploads the collected data unit or the pose control information to the cloud server via 6G network.

- Step 3: Cloud server performs AI inference based on received data unit or split rendering based on the received pose control information.

- Step 4: Cloud server sends the inference/rendered data to the IC device via 6G network.

The IC services bring new requirements on the 6GR system with ultra-high data rate within stringent latency requirement, which consequently also imposes challenges on the system capacity of the 6GR system. The End-to-End (E2E) latency is usually defined as the time from the UE uploading the collected data to receiving the inference results, as shown in Figure 6.

It is worth noting that QoE provided by such interactive services highly relies on the guaranteed end-to-end (E2E) latency, which usually varies even for the same application. For example, for an AI robot, requirement of the sub-task routing planning to the sink can be more relaxed than that of mug washing, as the environment is more dynamic in the latter sub-task.

The varied E2E latency requirement requires flexible delay budget. To be more specific, if the network schedules the data with the fixed E2E latency, the delay budget over air interface must be always configured according to the most demanding latency requirement. Consequently, the performance of the system, e.g., capacity, will decline as data transmission of other traffics or UEs is deprioritized.

Observation 2-2:	The E2E latency is critical for ensuring good user experience in IC services. However, the E2E latency requirement can be time-varying even within a single IC application.

Proposal 2:	Study the E2E latency requirement for 6G immersive communications traffic.

2.3	R20 Mobile AI vs 6G AI services

During the last meeting, RAN2 briefly discussed how to evaluate Mobile AI traffic characteristics, and whether/how this activity should be performed together with R20 Mobile AI SI for 5G-A. In this section, an explanation is provided on the similarities and differences between R20 mobile AI and 6G AI services. But before that, we would like to recall the XR traffic model discussed in 5G.

2.3.1	5G XR services

For the legacy XR services specified in 5G, as discussed above, the conclusions of the study on the traffic characteristics have been summarized in TS 38.838. The traffic is continuous real-time traffic without congestion control in the transport layer as illustrated in figure below.

Figure 5: Uplink XR traffic characteristics

As can be observed in the figure, the transport layer generally utilizes UDP and the application layer uses RTP. Hence, there is no congestion control mechanism like that in TCP. Because of that, the PDUs belonging to a certain PDU set arrive at the AS layer sequentially. Also, the uplink XR traffic are periodic, either for UL pose control information or the UL video traffic. Various enhancements have been specified in 5G assuming the above traffic arrival pattern, e.g., XR service awareness, DSR, PSI/PDU-set based discard, etc.

Observation 3-1:	The XR enhancements in 5G are aimed at the XR traffic patterns, characterized by UDP/RTP, continuous real-time video traffic, and continuous packet arrival in AS.

2.3.2	5G and 6G AI service: similarities and differences

Traffic arrival pattern of mobile AI traffic is different and the following figure illustrates the E2E traffic from the UE to the AI model on the cloud.

Figure 6, E2E mobile AI service

The E2E AI service consist of the following steps:

- The UE’s AI application generates a prompt, which could be video, voice, image or text. In another model, as explained in section 2.1, the traffic generated by the UE may consist of tokens. The prompt (or tokens) is then transmitted to the remote AI server by the wireless transport network (e.g., by 5GS)

- The AI model on the cloud receives the prompt and preforms inference. The amount of time it takes for performing the inference is called the inference time

- The model finishes the inference and the results are sent to the UE in the form of tokens. The tokens are sent under different batches and sent in different packed messages. The intervals between different tokens are called TPT (Time Per Token)

- The packed messages with tokens are sent in the DL to the UE. The time difference between the time when the AI service is triggered and the reception of the first packed message constitute the E2E RTT for the AI service.

The mobile AI service can be generally categorized under two types: (a) real-time traffic including video and continuous voice, and (b) non-real-time traffic including voice message, image or text. Real-time video traffic is characterized by stringent latency requirements and constant high data rate, while non-real-time traffic has the same latency requirement, but the traffic pattern is in general bursty data triggered by AI tasks.

We believe that real-time mobile AI traffic such as video, could be in general supported by the legacy XR enhancements in the RAN side, although we still need to resolve the service-awareness part, which is currently being discussed in SA2 under 6G discussion.

Observation 3-2:	For real-time mobile AI traffic such as video, the previous enhancements specified in 5G XR can be leveraged.

It is important to emphasize that our focus on R20 mobile AI is on the type of non-video non-real-time traffic as summarized above. The figure below illustrates the traffic pattern for this type of service. The generated application layer data is transmitted over TCP/QUIC protocol with congestion control algorithm. Due to the slow start characteristics of the TCP/QUIC, the transmit window gradually increases the data rate after several RTTs with the receiver, as governed by the congestion control algorithm. Hence, the data arrives in the AS modern in batches, spaced by with intervals approximately equal to the RTT of transport layer (TCP/QUIC).

Figure 7, UL traffic arrival pattern for non-real-time mobile AI services

Based on the above, we summarize the three traffic types discussed above and clarify which of these traffic types require analysis and enhancements as part of R20 Mobile AI in 5G-Advance and which should be addressed solely in the context of 6G.

Observation 3-3:	The similarities and differences between the traffic in scope of 5G XR traffic, R20 Mobile AI traffic, and 6G AI traffic are summarized in the following table

|  | R18/19 XR traffic | R20 Mobile AI traffic | 6G AI traffic |
| --- | --- | --- | --- |
| Real-time/non-real-time | Real-time | Non-real-time | Could be both |
| Upper layer transport | UDP | TCP/QUIC | Could be both |
| Uplink Data arrival pattern | Periodic; Continuous data arrival within a frame | Sporadic; Bursty data arrival with RTT of the transport layer | Could be both |
| Service data unit | PDU within PDU set or PDU set | Generic UL data | Token; or Generic UL/DL data |
| Uplink/Downlink applicability | Both | UL | Both |

While some of the features in 5G XR such as report of remaining time, PDU set could be beneficial for mobile AI, these enhancements were primarily designed for real-time video traffic. For non-real-time traffic such as R20 mobile AI traffic, these, features may be less applicable due to differing traffic patterns.

Observation 3-4:	The existing R18/R19 XR enhancements, defined based on real-time video traffic characteristics, cannot be reused for the non-real-time mobile AI traffic which is to be studied in R20.

Based on the discussion above, we propose the following for R20 and 6G mobile AI study.

Proposal 3:	For mobile AI traffic, RAN2 to study the characteristics of both non-token-based and token-based mobile AI traffic.

3	QoS Enhancement in RAN

3.1	5G QoS framework

The NR QoS framework is illustrated in Figure 8, details can be found under TS 23.501 clause 5.7.1. SMF will configure UE, RAN and UPF with QoS rule, QoS profile and Packet Detection Rules (PDR) according to the SLA requirement from 3rd part service provider respectively.

- A QoS rule contains the QFI of the associated QoS Flow, a Packet Filter Set and a precedence value, which indicates UE how to map UL packets into QoS flows and mark QoS flows.

- A QoS Flow may either be 'GBR' or 'Non-GBR' depending on its QoS profile. The QoS profile of a QoS Flow is sent to the RAN and it contains 5QI, GFBR & MFBR for GBR QoS flow and other QoS parameters, to instruct RAN how to schedule packets.

- A PDR contains a Packet Filter Set, used to instruct UPF to classify packets for QoS flow marking and other actions in DL.

Figure 8, NR QoS Framework

For non-GBR QoS flow, network transmits packets in best effort, does not guarantee any QoS. While for GBR QoS flow, specific QoS parameters will be configured to RAN during the QoS flow establishment process.  UE can request to change QoS parameters through the PDU Session Modification procedure shown in Figure 9. In which, UE contains its preferred new QoS parameters in the PDU session modification request, this request will be processed by multiple network functions like AMF, SMF, PCF, and finally RAN will be re-configured with the new QoS parameters. The QoS parameters change procedure takes at least 200 ms.

Figure 9, QoS parameters update procedure in NR QoS framework

3.2	Issues to address in the legacy 5G QoS framework

3.2.1	Dynamic QoS adaptation

The NR QoS framework is technically complete in that it can accommodate various service requirements from the third party, as can be seen in the QoS framework above in Figure 9.  However, under a lot of cases, only two QoS flows are typically utilized per UE: one GBR QoS flow for operator-controlled voice service, while all other services use the default (non-GBR) flow. The reason is simple: third party usually only prefers to improve user experience through OTT solutions rather than relying on operators. Since they don’t share SLA information with operators, the first step of the NR QoS framework is effectively hindered.

Observation 4-1:	One issue of the NR QoS framework is that it relies on third-party to provide the SLA information necessary to initiate the process.

Key Lesson from 5G is that the 6G QoS framework must operate as a closed-loop system within 3GPP, minimizing reliance on external parties. For example, in 6G, a cloud gamer should be able to pay for better service during a 2-hour train ride without requiring prior agreements between game provider and operator.

Observation 4-2:	6GR QoS framework should enable direct user-operator interaction, minimizing reliance on external parties.

UE is the closet entity to users, it owns the entire protocol stack, including the APP layer, OS, NAS, and AS, as well as valuable information (e.g., wireless condition). Leveraging these capabilities, the UE can estimate experience metrics independently even if the 3rd part APP is not willing to provide information. Furthermore, we propose that user experience metrics should be mapped to standardized QoS parameters before providing to network. Otherwise (UE directly reports user experience metrics to network), whenever a new type of service emerges, SA4 needs to discuss and specify user experience metrics for this new service, which typically takes 12–15 months. Then network function(s), e.g., OAM/NWDAF/DCF in field needs upgrade in order to understand the newly specified user experience metrics, which will take even longer time.

In general, the QoS framework in 6GR in our mind is illustrated in Figure 10, following section we will provide more detailed analysis.

Figure 10, QoS framework in 6GR

As discussed above, current QoS flows can be categorized into two types: GBR and non-GBR. However, neither is well-suited for bursty real-time services, such as mobile AI applications (refer to Clause 6.25 in TR 22.870). The non-GBR fails to meet the requirements because it provides no QoS guarantees, whereas bursty real-time services demand strict latency assurance. On the other hand, GBR requires predefined QoS parameters during the QoS flow establishment phase. However, in bursty real-time scenarios, traffic patterns (burst timing and data volume) are highly unpredictable, depend entirely on user behavior.

Take cloud-rendering as an example, as Figure 11 shows, suppose the data size of a photo is 1 M, and the delay budget is 40ms, and we establish a GBR QoS flow with 25 Mbps data rate guarantee. However, the user may trigger multiple actions – such as generating 10 photos during a short period of time - the instantaneous data rate requirement spikes to 250 Mbps. The QoS parameters indicated at the GBR QoS flow establishment procedure are no longer adequate. Alternatively, establishing a GBR QoS flow with the maximum possible data rate requirement, would lead to significant resource wastage.

Figure 11, Example of carrying busty real-time data using traditional GBR QoS flow

As illustrated by example, it is challenging to specific QoS parameters for busty real-time services during the QoS flow establishment procedure, rendering both GBR and non-GBR flows unsuitable. However, it is feasible to define a QoS parameter ranges for such busty real-time services, where the QoS flow is associated with a QoS range, rather than a specific QoS guarantee.

Observation 4-3:	In addition to GBR and non-GBR, a new QoS flow type could be defined which can provide guarantee over a range of QoS parameters.

Within the defined QoS range, the QoS demand for each burst may dynamically fluctuate. As illustrated in Figure 9, the current NAS signaling-based QoS adjustment procedure is overly complex and incurs a latency of at least 200 ms. However, as specified by SA1 in Clause 6.25 of TR 22.870, the delay budget for mobile AI services typically falls within tens of milliseconds, far shorter than the existing QoS adaptation time. Given this constraint, SA2 has agreed to study dynamic QoS mechanisms [S2-2509800]. A key consideration is whether minor QoS adjustments (e.g., within a narrow range) can be handled locally at the air interface bypassing core network signaling. For instance, if the service supports a QoS range guarantee for 10-20 ms, and this time’s adjustment is only from 10 ms to 12 ms. Such a minor 2 ms adjustment could be addressed by appropriate scheduling at air interface. In this case, it might be unnecessary to traverse so many core network functions. Instead, implementing a closed-loop adjustment directly at the air interface could enable faster QoS adjustment.

- Whether and how to enhance QoS mechanisms to: - support QoS targets fulfilling application requirements (e.g., dynamic QoS requirements) in a less resource intensive manner than the existing GBR, Delay Critical-GBR.

Proposal 4:	RAN2 to study the RAN-level mechanisms for dynamic QoS adaptation.

3.3.2	Flexible allocation of QoS budgets

A smooth user experience relies on whether the end-to-end (E2E) latency requirement can be fulfilled or not, rather than if the QoS in a partial network is fulfilled. In last meeting, RAN2 had agreed that “6G user plane should aim to reduce the radio and end-to-end latency for general services (including eMBB)”

Under the current 5G QoS framework, as we illustrated in section 3.1, the core network determines the static QoS parameters according to the service requirement. The parameter for the delay requirement is the PDB (packet delay budget), which is the upper bound of the delay measured between the UE and the N6 termination point at the UP. The PDB can be split into two parts, the Uu part and the N3 part.

Before Rel-20, timestamps in the N3 GTP-U header are supported for packet delay measurement during QoS monitoring procedure. Delay thresholds could be configured to RAN and UPF and when such delay thresholds are exceeded, RAN and UPF could report to the UPF the outage of delay support. This procedure is initiated by the UPF and the Uu delay and the N3 delay are guaranteed independently by the gNB and the core network. While in Rel-20, 3GPP will support “the coordination between gNB and CN to enable/disable N3 interface delay measurement from CN to gNB, for better latency guarantee” [RP-252755].

Even with the enhancements in Rel-20, it would only guarantee the latency within the 3GPP network. From the service flow for immersive services, we can see that the end-to-end latency would further includes the N6 part. In SA4, TS 26.522 has specified some timestamp information contained in the RTP header extension and RTCP payload for E2E delay measurements of RTP streams [3]. However, this delay measurement is performed at the RTP layer and not visible to the 3GPP network.

Figure 12, Delay budget collaboration among different regions

In the real network, the delay can vary, including the Uu delay, N3 delay and N6 delay, which means that no one will be responsible for the E2E delay requirement. We have observed that some applications have dynamic changing environment which leads to dynamic latency in each segment. In order to guarantee the total latency requirement in 3GPP, we should allow different entities to coordinate with each other. As shown in Figure 12, when the real-time Uu interface delay is long, NW should try to ensure short N3 interface latency in order to compensate for the Uu latency.

Based on the discussion above, we think we need to study how to guarantee the E2E delay requirement form the RAN point of view to provide a better user experience in 6G.

Proposal 5:	RAN2 to study on the mechanisms for guaranteeing E2E latency by coordination between different entities.

3.3.3	QoS guarantee granularity

In the NR QoS framework, the finest granularity is QoS flow. All data within a QoS Flow enjoy the same treatment, like the same priority level and the same PDB, and so on. In R18 XR, we have introduced the PDU set discard based on PSI, which denotes different levels of importance for PDU sets transported under a single QoS flow. However, PSI is only used in the case of congestion when PSI based discard is activated by the network.

From the service perspective, data packets within the same QoS flow can have different requirements, such as

- Varying remaining delay budgets: for the IC traffic, the UE uploads the data to the cloud for inference. Since the inference time can depend on the data volume of the uploaded data (the inference time for a video can be larger than the time for a photo) or the task type (simple task or complex task), the remaining delay budgets can be different.

- Different levels of importance as Clause 2.1 explained: In the token communication, the importance is different for different tokens, which means we should consider different levels of reliability for different tokens during transmission.

Rel-18 XR has discussed some solutions including PDU discard based on PSI. However, the requirements of new services mentioned above cannot be directly addressed by this method. We believe 6GR needs to keep on exploring some built-in solutions rather than some patchwork solutions.

Proposal 6:	RAN2 to study the mechanism for differentiated handling of packets within a QoS flow by taking into account of the 6G new service requirements (e.g. AI token-based communication and immersive communication).

4	Framework of Service-awareness in RAN

During the last RAN2 meeting, regarding the service awareness, RAN2 has agreed on the following

4. Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic). Understand the issues and shortcoming with the current NR QoS. Aim to look at a general framework.

We understand that at the current stage of 6G study, we should primarily focus on the discussion on the functionalities, while the discussion on the framework for service awareness could be discussed later. In this section, we offer some of our preliminary views on the framework for application/service awareness.

4.1	Origin of service awareness

In the legacy, the service awareness that is visible to RAN comes from various sources. For example, for Uu uplink, the service awareness for XR service information, such as PDU set information comes from UE implementation. For the downlink, it may come from the NG-AP signalling, e.g., for XR service pattern, or from GTP-U header extension for the PDU set information for the DL XR traffic. In 6G, we also foresee such sources of service awareness.

4.2	Different level of service awareness

In the legacy 5G L2 design, we have multiple enhancements, some are explicitly put under the name of service awareness, e.g., R18 XR enhancements based on PDU set information, others we do not call them service awareness, but their actual functionality is based on the service awareness. We think service awareness could be categorized into the following levels:

- DRB level: The gNB and UE should be aware of the type of upper layer transport protocols, e.g., TCP/UDP/IP/Ethernet such that the proper header compression algorithm could be configured.

- Packet level: In XR, we support PDU set/PDU information for each packet such that PDU set-based/PSI based discard could be supported

- Service level: In XR, for each QoS flow, the traffic patterns, such as periodicity, jitter could be identified and reported to the RAN either from the UE or from the CN

4.3	Different usage of service awareness

As discussed above, different kinds of service awareness are used for different purposes. For example, PDU set information can be used for PDU-set level discard. Jitter and periodicity of the traffic can be used for configured grant configuration, DRX configuration etc. It is also worthwhile to highlight that in the RAN1 progress in the last meeting on the study of traffic model in AI services, RAN1 has mentioned in Option1 that How to associate Tokens to PHY layer packets. This means that this service awareness will also be visible to RAN.

By the discussion above, we could illustrate the framework for service awareness by the following figure.

Figure 13, Service Awareness framework

However, we also think that RAN2 should focus on the discussion on the service traffic characteristics and functionalities first at this stage of study. We need to identify the particularities for each type of traffic and what types of service awareness can be utilized for enhancements. Different functionalities could be defined based on this service awareness. Then, we can discuss on the framework for the service awareness, which is only about how to organize these awareness and functionalities in the specification.

Proposal 7:	RAN2 to further study the framework for service awareness based on the progress on traffic characteristics and functionalities for different services.

5	Conclusion

New service requirements for 6G

Observation 1-1:	The processing efficiency of the AIML model can be improved through token communications.

Observation 1-2:	Token-based communications is error tolerant and different tokens have varying levels of importance depending on their modality and the position of the token within the traffic stream.

Proposal 1:	The design of 6G UP functionality should take into account the requirements and characteristics of AI services such as error tolerance and variable importance of different tokens.

Observation 2-1:	RAN1 has agreed that XR traffic models (in TR 38.838) are considered as starting point for immersive communication services.

Observation 2-2:	The E2E latency is critical for ensuring good user experience in IC services; However, the E2E latency requirement can be time-varying even within a single IC application.

Proposal 2:	Study the E2E latency requirement for 6G immersive communications traffic.

Observation 3-1:	The XR enhancements in 5G are aimed at the XR traffic patterns, characterized by UDP/RTP, continuous real-time video traffic, and continuous packet arrival in AS.

Observation 3-2:	For real-time mobile AI traffic such as video, the previous enhancements specified in 5G XR can be leveraged.

Observation 3-3:	The similarities and differences between the traffic in scope of 5G XR traffic, R20 Mobile AI traffic, and 6G AI traffic are summarized in the following table

|  | R18/19 XR traffic | R20 Mobile AI traffic | 6G AI traffic |
| --- | --- | --- | --- |
| Real-time/non-real-time | Real-time | Non-real-time | Could be both |
| Upper layer transport | UDP | TCP/QUIC | Could be both |
| Uplink Data arrival pattern | Periodic; Continuous data arrival within a frame | Sporadic; Bursty data arrival with RTT of the transport layer | Could be both |
| Service data unit | PDU within PDU set or PDU set | Generic UL data | Token; or Generic UL/DL data |
| Uplink/Downlink applicability | Both | UL | Both |

Observation 3-4:	The existing R18/R19 XR enhancements, defined based on real-time video traffic characteristics, cannot be reused for the non-real-time mobile AI traffic which is to be studied in R20.

Proposal 3:	For mobile AI traffic, RAN2 to study the characteristics of both non-token-based and token-based mobile AI traffic.

QoS Enhancement in RAN

Observation 4-1:	One issue of the NR QoS framework is that it relies on third-party to provide the SLA information necessary to initiate the process.

Observation 4-2:	6GR QoS framework should enable direct user-operator interaction, minimizing reliance on external parties.

Observation 4-3:	In addition to GBR and non-GBR, a new QoS flow type could be defined which can provide guarantee over a range of QoS parameters.

Proposal 4:	RAN2 to study the RAN-level mechanisms for dynamic QoS adaptation.

Proposal 5:	RAN2 to study on the mechanisms for guaranteeing E2E latency by coordination between different entities.

Proposal 6:	RAN2 to study the mechanism for differentiated handling of packets within a QoS flow by taking into account of the 6G new service requirements (e.g. AI token-based communication and immersive communication).

Service-awareness framework

Proposal 7:	RAN2 to further study the framework for service awareness based on the progress on traffic characteristics and functionalities for different services.

6	Reference

- R2_131bis_ChairNotes https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_131bis/Inbox/Chair_Notes/R2_131bis_ChairNotes_25-10-17_17-00_final.zip

- Hao, Zixu, et al. "Hybrid SLM and LLM for edge-cloud collaborative inference." Proceedings of the Workshop on Edge and Mobile Foundation Models. 2024.

- TS 26.522: 5G Real-time Media Transport Protocol Configurations.