3GPP TSG-RAN WG2 Meeting #132	R2-2508141

Dallas, USA, November 17th – 21st, 2025

Agenda Item:	10.3.2.3

Source:	OPPO

Title:	Discussion on 6GR Initial Access and Spectrum aggregation

Document for:	Discussion and Decision

Introduction

In this contribution, we would like to present our consideration regarding spectrum aggregation, initial and system access, including system information, paging etc.

Discussion

Spectrum Aggregation

In RAN2#131bis, it was concluded that

Agreements on Spectrum aggregation 1 Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed. 2 Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc). Understand the deployment scenarios and problems to address

From TS 36.300

- 7.5 Carrier Aggregation When CA is configured, the UE only has one RRC connection with the network. At RRC connection establishment/re-establishment/handover, one serving cell provides the NAS mobility information (e.g., TAI), and at RRC connection re-establishment/handover, one serving cell provides the security input. This cell is referred to as the Primary Cell (PCell). In the downlink, the carrier corresponding to the PCell is the Downlink Primary Component Carrier (DL PCC) while in the uplink it is the Uplink Primary Component Carrier (UL PCC). Depending on UE capabilities, Secondary Cells (SCells) can be configured to form together with the PCell a set of serving cells. In the downlink, the carrier corresponding to an SCell is a Downlink Secondary Component Carrier (DL SCC) while in the uplink it is an Uplink Secondary Component Carrier (UL SCC).

From TS 38.300

- 7.7 Carrier Aggregation When CA is configured, the UE only has one RRC connection with the network. At RRC connection establishment/re-establishment/handover, one serving cell provides the NAS mobility information, and at RRC connection re-establishment/handover, one serving cell provides the security input. This cell is referred to as the Primary Cell (PCell). Depending on UE capabilities, Secondary Cells (SCells) can be configured to form together with the PCell a set of serving cells. The configured set of serving cells for a UE in a cell group always consists of one SpCell and one or more SCells.

It is clear that since 4G, aggregation of carriers has been handled as aggregation of cells. Although it is an easier solution in the sense of keeping the per-CC cell framework.

The 5G CA framework was mainly inherited from 4G CA framework, which handled the CC aggregation in a way of (per-CC-)cell aggregation.

Meanwhile, there are proposals for 6G to adopt a more flexible spectrum aggregation framework, which allows aggregating multiple CCs into a single cell.

Even though the different view of the two frameworks, R2 can firstly progress on the intended functionality, and further conclude on the applicable framework by evaluating the implementation complexity and performance gain following either of the two frameworks.

For spectrum aggregation, R2 discuss the issue by firstly identifying necessary functionality candidates from R2 perspective, and then evaluating the pros/cons of either per-CC cell (as in legacy CA) and multi-CC cell (SCMC) framework.

In our view, there are some aspects of 4/5G CA framework can be considered for improvement in 6G.

Firstly, for RRC_IDLE state, the enhancement is more for initial access procedure.

Issues of 5G CA scheme and potential enhancement in 6G (RRC_IDLE)

| Key Dimension | Detailed Aspect | 5G Status | 6G Improvement | Applicable Scenarios |
| --- | --- | --- | --- | --- |
| Spectrum Efficiency | Reduced per-CC SSB | Although SSB-less/OD-SSB SCell has been defined in 5G, they are limited to RRC_CONNECTED state. | Reduced always-on SSB from Day-1. | At least for intra-band/inter-band and collocated scenario, while on-demand signal can be extended to non-collocated |
|  | Reduced per-CC SIBx and etc. | OD-SIB1 in 5G relies on Cell-A assistance. | Reduced always-on SIBx from Day-1. | Intra-/inter-band scenario and collocated/non-collocated scenario. |
| Load Distribution | Flexible load distribution during initial-access | Dynamic offloading between frequencies is limited to SUL for RACH load balance. Otherwise, static load balance between frequencies is based on cell reselection procedure NOTE: NB-IoT allows balance between anchor and non-anchor carriers. | Dynamic load balance for frequencies other than SUL, not limited to RACH attempts. | Intra-/inter-band scenario and collocated/non-collocated scenario. |
| Coverage Extension | DL/UL decoupling | Cross-band DL/UL coupling (other than SUL band) is not supported | Support cross-band DL/UL coupling | Inter-band and (non-)collocated scenarios |

Secondly, for RRC_CONNECTED state, seems quite some aspects which are highly dependent on R1 effort.

Issues of 5G CA scheme and potential enhancement in 6G (RRC_CONNECTED)

| Key Dimension | Detailed Aspect | 5G Status | 6G Improvement | Applicable Scenarios |
| --- | --- | --- | --- | --- |
| Spectrum Efficiency | Reduced per-CC SSB | Although SSB-less/OD-SSB SCell has been defined in 5G, the deployment is limited since the support of inter-band support is introduced late in R18, and non-collocated scenario (OD-SSB) is introduced late in R19. | Reduced always-on SSB from Day-1. | At least for intra-band/inter-band and collocated scenario, while on-demand signal can be extended to non-collocated |
|  | Cross-CC/Integrated PxSCH design | Per-CC PDCCH and PDSCH/TB design, even though cross-CC scheduling is supported. | Cross-CC/Integrated PDSCH for less-DCI, less PHY resource fragmentation due to resource allocation restriction, and less L12 overhead for larger TB. | For intra-band and collocated scenario |
|  | Cross-CC/Integrated HARQ design | Per-CC HARQ entity | Cross-CC HARQ to explore diversity across CCs | Both intra/inter-band and (non-)collocated scenarios |
|  | Power-efficient & fast CC (de)activation | SCell (de)activation, fast SCell activation and dormant BWP | An improved trade-off between fast UP bit-rate ramp-up upon carrier activation and power saving upon carrier deactivation | Both intra/inter-band and (non-)collocated scenarios |
| Link Robustness | P/SCC Role alleviation | Critical functions are on PCell, including but not limited to security, RACH, RLM and etc. | Alleviate the P/S-CC role, and thus robust to failure of either CC | The benefit is mainly from non-collocated scenario (if assuming lower-frequency is normally set as PCC) |
| Coverage Extension, UL-heavy traffic compatible configuration | DL/UL decoupling | Cross-band DL/UL coupling (other than SUL band) is not supported | Support cross-band DL/UL coupling | Inter-band and (non-)collocated scenarios |

Given the analysis above, the 5G CA framework has quite some drawbacks:

The legacy cell aggregation approach leads to some drawbacks, at least including 
1) unnecessary per-cell overhead, 
2) unnecessary scheduling overhead/restriction, 
3) limited HARQ diversity due to per-cell HARQ entity, 
4) limited trade-off between fast SCell activation and power effiency
4) Static load balance restricted by per-cell camping and access (other than SUL),  
5) Asymmetric cell functionality, and
6) limitation of D/U coupling (intra-band or SUL).

Given the aspects above, R2 can list the potential enhancement candidates first.

And in order to avoid agenda collision, R2 can leave the detailed discussion of RRC_IDLE / initial-access related enhancement to initial-access agenda item.

And for RRC_CONNECTED, R2 can focus on PCC/SCC role alleviation, while waiting for progress from other WGs (e.g., R1) for the other components, and also cross-CC HARQ entity (even though it is related to both R1 and R2).

For spectrum aggregation at RRC_IDLE, R2 study functionality components including 
1) SSB/SIB delivery via anchor-carrier only,
2) RACH/Paging load distribution across anchor-/non-anchor-carrier, and
3) non-SUL band acting as non-anchor-carrier,
and discuss the applicable scenario accordingly (e.g., intra-/inter-band, (non)collocated).

For spectrum aggregation at RRC_CONNECTED, R2 study functionality components focusing on PCell/SCell role alleviation and cross-CC HARQ entity, and discuss the applicable scenario accordingly (e.g., intra-/inter-band, (non)collocated), while waiting for progress by R1 on other related features.

After identifying the solution candidates, the next step is to evaluate the complexity of implementing them via per-CC-cell (legacy CA) framework or via multi-CC-cell (SCMC) framework.

Comparison of per-CC-cell/multi-CC-cell framework (RRC_IDLE)

| Key Dimension | Detailed Aspect | If follow per-CC Cell framework | If follow multi-CC Cell framework | Comment |
| --- | --- | --- | --- | --- |
| Spectrum Efficiency | Reduced per-CC SSB | 6G has to specify the UE behavior in a way that UE keep monitoring Cell-1 of anchor-carrier for SSB/SIB-Rx(/Paging-Rx), and performing RACH-Tx (/Paging-Rx) over Cell-2 of non-anchor-carrier. NOTE: Even in 5G, the aggregation of SUL-band (e.g., SSB/SIB-Rx on NDL, and RACH over SUL) is under a same cell framework, and NUL/SUL switching is modelled as carrier switching in the same cell. | 6G specify the UE behavior over multiple CCs of a same cell, i.e., it can be modelled as switching/operation of multiple CC/BWPs. | Wired multi-cell operation during RRC_IDLE state, and further confusion for cell reselection procedure design (which is supposed to handle UE switching between cells). |
|  | Reduced per-CC SIBx and etc. |  |  |  |
| Load Distribution | Flexible load distribution during initial-access |  |  |  |
| Coverage Extension | DL/UL decoupling |  |  |  |

For RRC_IDLE, enhancing the spectrum aggregation but sticking to per-CC cell framework would lead to strange multi-cell operation during RRC_IDLE state, and further confusion for cell reselection procedure design (which is supposed to handle UE switching between cells).

Comparison of per-CC-cell/multi-CC-cell framework (RRC_CONNECTED)

| Key Dimension | Detailed Aspect | If follow per-CC Cell framework | If follow multi-CC Cell framework | Comment |
| --- | --- | --- | --- | --- |
| Spectrum Efficiency | Reduced per-CC SSB | Same SSB-less/OD-SSB framework as in 5G | 6G allows per-CC SSB configurability | Both framework works |
|  | Cross-CC/Integrated PxSCH design | 6G has to specify PxSCH across Cells. | 6G still restrict the PxSCH within a same Cell. | Across-Cell PxSCH is quite wired concept, which basically ruin the concept of cell, if it cannot define the boundary of physical channel any more. |
|  | Cross-CC/Integrated HARQ design | 6G has to specify coupling of HARQ entity between cells | 6G still limit the HARQ entity definition within a same cell. | Across-Cell HARQ is quite wired concept, which basically ruin the concept of cell, if it cannot define the boundary of HARQ entity any more. |
|  | Power-efficient & fast CC (de)activation | 6G has to specify mechanism for each serving Cell | 6G has to specify mechanism for each serving carrier. | Both framework works |
| Link Robustness | P/SCC Role alleviation | 6G allows RACH, PUCCH, RLM configurability across serving cells. | It is enabled automatically since all CCs are in the same cell. | Higher complexity if following per-CC-cell framework. |
| Coverage Extension, UL-heavy traffic compatible configuration | DL/UL decoupling | 6G specify coupling between two cells | 6G allows inter-band CC within the same cell | Both framework works |

For RRC_CONNECTED, enhancing the spectrum aggregation but sticking to per-CC cell framework would lead to either higher complexity, or ruin the concept of cell, if it cannot define the boundary of Physical channel or HARQ entity any more.

Initial Access

For the 6G design, based on RAN requirements as defined in RP-250810, energy efficiency and energy saving are required for both network and device:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Energy efficiency and energy saving: both for network and device.

For System Information, it was agreed in RAN2 #131bis meeting that energy efficiency should be considered.

Agreements on System Information - System information design should consider energy efficiency and low complexity for both network and UE. - System information design should ensure that the public warning requirements are met. - Study RAN2 aspects of system information designs (e.g. on-demand SIB and SSB, periodicity of SSBs) - Study how to improve flexibility and extensibility of scheduling SI design - Study mechanisms to improve SI update mechanism - Study aspects related to SIB1 size. Understand the issues and desired content/size from RAN2 point of view. - Study areas specific SIB design. Understand 5G pain points and what improvements can be considered. NOTE: this is a starting list

In NR, the on-demand mechanism was introduced to reduce always-on system transmissions in the time domain, aiming to improve power efficiency for both UE and the network. Specifically, on-demand SIB transmission is supported for all SIB messages, including SIB1 and OSI. However, due to backward compatibility constraints, OD-SIB1 is currently only supported in NES cells with assistance from another cell. In 6G, where backward compatibility is no longer a limiting factor, the feasibility of OD-SIB1 for standalone cells can be explored to achieve further power-saving benefits.

SIB transmission schemes in NR

| SIB type | 5G Status | 6G improvement |
| --- | --- | --- |
| SIB1 | Time-domain Non-Standalone Cell (Configuration from SIBx in Cell-A) MSG1 as request SIB1 acquisition based on SIBx setting | Support OD-SIB1 for standalone cell/carrier. |
|  | Frequency-domain Each carrier (for camping by RRC_IDLE UE) needs to broadcast SIB1 | Reduce per-CC overhead by SIB1 broadcasting at coverage layer |
| OSI | Time-domain Standalone Cell MSG-1/3 as request OSI acquisition based on SIB1 setting | NR scheme as baseline |
|  | Frequency-domain Each carrier (for camping by RRC_IDLE UE) needs to broadcast OSI | Reduce per-CC overhead by OSI broadcasting at coverage layer |

On-demand system information is supported in NR for both SIB1 and other system information, reducing the need for always-on transmission and reception in the time domain. This mechanism contributes to power savings for both the network and UE.

From frequency domain, in NR, the system information broadcasting at each cell/carrier that is considered for camping by the non-connected UE is required. This results to the per-carrier overhead, and the per-carrier overhead can be avoided since the configuration maybe same.

In NR, each carrier (for camping by RRC_IDLE UE) needs to broadcast system information , but it can be improved for power efficiency in specific scenarios.

The on-demand mechanism in NR can be inherited in the 6G system information design to improve energy efficiency. As backward compatibility is no longer a constraint in 6G, the on-demand SIB1 can be further enhanced to support both standalone and non-standalone cells/carriers. In addition, RAN2 can study reduce the per-CC SIB broadcasting overhead by rely on the coverage carrier for SIB broadcasting.

For 6G system information delivery, RAN2 study schemes to improve energy efficiency, which includes 1) on-demand triggering of SIB1 and OSI; 2) relying solely on anchor carrier to carry system information.

In NR, Paging-Occasion can be used for the network to

- Reach UEs in RRC_IDLE and in RRC_INACTIVE state upon DL data arrival through paging message, and

- Notify UEs in RRC_IDLE, RRC_INACTIVE and RRC_CONNECTED state of system information change and ETWS/CMAS notifications through short message.

In NR, PO monitoring (PDCCH/PDSCH reading) is needed for 1) DL data arrival, 2) System information change, or 3) ETWS/CMAS notifications.

A UE in RRC_IDLE and in RRC_INACTIVE state may use DRX for paging monitoring/reception for the purpose of power saving. For DL data arrival triggered paging, if UE receives a P-RNTI scrambled PDCCH in its PO,

- If the systemInfoModification bit in short message is set to 1, the UE (re-)acquires SIB1, checks the validity for each SIB, and then acquires the SIB(s) for the one(s) considered as invalid from the start of the next modification period. There may be a case that the SIB related to a specific feature is to be changed, and the UE does not support this feature, but the UE could not know the paging is triggered by the change of SIB for this feature until it (re-)acquires SIB1, which leads to unnecessary UE power consumption.

- If the etwsAndCmasIndication bit in short message is set to 1, the UE immediately (re-)acquires SIB1 and acquires SIBs for PWS according to the si-SchedulingInfo in SIB1.

- The UE further decodes PDSCH scheduled by the PDCCH and processes the paging message. If its UE ID is included in the pagingRecordList in paging message, the UE initiates RRC connection establishment.

In NR, PEI and LP-WUS are introduced, which can be used to wake up UE for paging monitoring. In both features, UE sharing the same PO can be further divided into multiple subgroups, and PEI and LP-WUS can wake up specific UE subgroups so that UE power consumption due to false paging alarms can be reduced. For both PEI and LP-WUS, UE does not know what paging is used for (i.e., whether paging is triggered by DL data arrival or it is intended for system information change and ETWS/CMAS notification) until it receives paging.

5G Short-Message / Paging based notification

| Paging trigger event | UE behaviour if PEI/LP-WUS is not used | UE behaviour if PEI/LP-WUS is used |
| --- | --- | --- |
| DL Data Arrival | Monitor/receive paging > Initiate RRC connection establishment | Monitor/receive PEI/LP-WUS > monitor/receive paging > Initiate RRC connection establishment |
| System Information Change | Monitor/receive paging > (re-)acquires SIB1 > check validity for each SIB > acquires invalid SIB(s) | Monitor/receive PEI/LP-WUS > monitor/receive paging >(re-)acquires SIB1 > check validity for each SIB > acquires invalid SIB(s) |
| ETWS/CMAS Notification | Monitor/receive paging > (re-)acquires SIB1 > acquires SIBs for PWS | Monitor/receive PEI/LP-WUS > monitor/receive paging > (re-)acquires SIB1 > acquires SIBs for PWS |

In NR, for SM triggered by change of the SIB(s) related to a specific feature, UE does not know the triggering SIB(s) until it (re-)acquires SIB1, which leads to unnecessary UE power consumption for a UE who does not support specific feature.

In NR, when a UE is wakened up by PEI/LP-WUS, the UE always monitors PO firstly, regardless of the triggering event.

From network perspective, for DL data arrival triggered paging, it will send paging in the UE’s PO, and if PEI/LP-WUS is used, network will send PEI/LP-WUS to the UE before paging the UE. In case network intends to change system information, it needs to repeat notification for/in all PO in order to inform all UEs, which may cause large signalling overhead and is not beneficial for NES. If PEI/LP-WUS is used, network will send PEI/LP-WUS on all PEI/LP-WUS occasions as well to wake up all UEs camping on this cell.

In NR, if network intends to change system information, it needs to repeat notification for all POs in order to inform all UEs within a modification period, which is harmful for NES.

Based on above, we propose that in 6G, RAN2 studies more efficient mechanism to notify paging/system-information-change/ETWS-CMAS-indication to further reduce UE and Network power consumption, e.g., UE could be aware of whether there is paging and/or system information change and/or ETWS-CMAS-indication via LP-WUS/PEI reception, rather than being mandated to read PO to differentiate the cases.

For 6G system information update, RAN2 study more efficient mechanism to notify events of paging/system-information-change/ETWS-CMAS-indication, to improve UE and Network energy efficiency.

Besides, based on the requirement from 6G SID, the 6G design needs to target diverse device types with diverse bandwidth capability and take into account diverse spectrum allocations:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations.

For diverse devise types, a unified yet scalable initial access framework is essential, one that ensures compatibility across diverse UE types which may have different bandwidth capabilities. Additionally, to support the anticipated increase in device volume, enhanced initial access capacity is needed.

In 6G, there is a strong demand to support unified initial access of different UE types (e.g., based on different max CBW capability).

For spectrum utilization, the first question is about which spectrum would be the main target for 6G

6G Spectrum Candidates

| Spectrum Range | Detailed Info | Analysis |
| --- | --- | --- |
| Major 6G spectrum candidate 6.425-7.125GHz 7.125-7.400/8.400GHz, 14.80-15.35GHz (4.4-4.8GHz) | Identified as major 6G spectrum candidate, based on WRC-23 (6.425-7.125GHz) and WRC-27 (7.125-7.400/8.400GHz, 14.80-15.35GHz) | Mainly as basic contributor to capacity, but limited coverage compared to 4G/5G spectrum |
| dmW-band as coverage booster Sub-2GHz | Potential refarmed spectrum from legacy network, e.g., 700/800/900MHz, 1.8GHz | Mainly as coverage enabler, but not capacity enabler |
| mmW-band as capacity booster 26GHz, 40Ghz | The deployment / commercialization of mmW-band is to be seen | For capacity enhancement, yet rather limited coverage. |

Considering the major candidate spectrum for 6G as summarized above, in our view, what brings real benefit to the consumer is in two aspects

- Robust connection to the network (e.g., carrying control plane data and also maybe essential user plane data like for emergency service), for which a lower frequency band is needed to secure the always-on low-bit-rate link;

- On-demand connection to the network (e.g., carrying high-data-rate traffic data for entertainment service, and/or for load re-distribution), for which temporary available low/medium/high frequency band is required.

Then eventually, what consumer needs is a RAN which is robust and always-on to carry a moderate-bit-rate connection, and also able to take additional load / higher bit-rate traffic when necessary. That is the ultimate goal of efficient usage / aggregation of spectrum.

Spectrum-wise, the candidate spectrum for 6G diverse, and it requires a solution to aggregate spectrum
1) cmW-band (6.425-7.125GHz (WRC-23), 7-15GHz (WRC-27)) and potentially mmW-band acts as basic contributor to capacity, but with limited coverage compared to 5G; 
2) while sub-2GHz spectrum refarming helps coverage, but clearly, they are not capacity enabler.

Considering all these requirements, it is good to re-screen the offloading/re-distribution solutions provided by 5G

5G offload method for initial access

| Granularity | 5G Solutions | Comments | Drawbacks |
| --- | --- | --- | --- |
| BWP | Distribute various device-type across different BWP:s | Via the Redcap-specific BWP, which is to be utilized based on device-type | Currently BWP concept cannot cover inter-carrier/band scenario |
| Carrier, Alt1 | Distribute across different carriers based on channel quality and network slice | Via cell reselection procedure | Existing cell reselection procedure is only based on channel quality and slicing, and is based on static criterion |
| Carrier, Alt2 | Distribute across different UL bands based on channel quality | Via NUL/SUL selection procedure | Existing SUL solution relies on specific SUL type band |

As summarized in the above table, several load distribution mechanisms have been defined in NR:

- Frequency resources for distribution span multiple granularities, including BWP, carrier, and band levels.

- Various factors such as device type, channel quality, and network slice are considered across different solutions.

- Each solution presents specific drawbacks, as outlined in the table.

5G tools for initial-access offloading/re-distribution has drawbacks, e.g., 1) incapable for cross-carrier/band scenario (1st case above), 2) limited to channel quality / slicing and based on static criterion (2nd case above), and 3) limited to SUL type band (3rd case above).

In NB-IoT, multi-carrier operation is supported for Paging and RACH load distribution, allowing both anchor and non-anchor carriers to be utilized within a single cell. More detailed design is summarized as the following table

Table 9 NB-IoT multi-carrier operation

|  | 4G NB-IOT Status | 6G Improvement |
| --- | --- | --- |
| Scenario | Anchor and non-anchor carriers are collocated The total frequency span to not exceed 20MHz | Non-collocated case can be studied Extended to larger than 20M for general inter-band scenario |
| RACH transmission | Carrier selection for PRACH transmission is determined based on coverage level | Additional factors other than coverage can be studied (e.g., diverse device types), and dynamic switching mechanism |
| Paging reception | Carrier selection for Paging reception is based on UE ID / weight (and coverage) | Additional factors other than coverage can be studied (e.g., diverse device types) |

The multi-carrier operation for initial-access offloading/re-distribution is supported in NB-IoT, but its applicability was limited to specific scenarios and use cases.

For 6G, a unified mechanism for load offloading and redistribution is preferred, in line with the requirements of the 6G SID: multiple solutions serving the same functionality should be avoided.

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting.

Therefore, based on the above analysis, the 6G redistribution mechanism should be able to:

- Support diverse device types with varying capabilities (e.g., in terms of channel bandwidth)

- Enable flexible redistribution without relying on cell reselection, thereby reducing per-CC overhead for both UE and network

- Support both intra-band and inter-band scenarios, in alignment with 6G spectrum utilization as analyzed in O12

For 6G initial access, RAN2 study the scheme(s) for flexible offloading/re-distribution for RACH and Paging across frequency-domain resources. FFS on the applicable scenario (intra-/inter-band, and collocated/non-collocated).

Access Control

According to the latest progress from SA1, 6G requirements for UAC have been updated as follows [TR22.870 v0.4.0]:

5.7.6 Unified Access Control (UAC) 5.7.6.1 Description Unified Access Control (UAC) mechanism is specified in [14], enables the operators to control the access to 3GPP network in congestion scenarios by determining which access attempt should be allowed or blocked based on operator's policies, deployment scenarios, subscriber profiles, and available services. 6G will introduce new services and capabilities, such as artificial intelligence, sensing, and computing, which require enhancement of the existing mechanism in [14] to efficiently manage evolving technical demands and service requirements. Therefore, the UAC shall be enhanced to address these new services, enabling network operators to control and manage the network in congestion scenarios. 5.7.6.2 Potential New Requirements [PR 5.7.x.2-1] The 6G system shall support Unified Access Control requirements as specified in [14]. [PR 5.7.x.2-2] The 6G system shall support suitable access categories to manage the access attempt for the new services (such as sensing, AI application, computing) in congestion scenarios

According to SA1 requirement, 6G UAC shall support Access Control for 1) existing categories as specified in 5G system, and 2) additional categories for new services (such as sensing, AI application, computing) in congestion scenarios.

In 5G UAC, multiple access categories are defined based on the type of access attempt, and their mapping relationship is listed in the table below.

Access Category in 5G

| Access Category number | Conditions related to UE | Type of access attempt |
| --- | --- | --- |
| 0 | All | MO signalling resulting from paging |
| 1 (NOTE 1) | UE is configured for delay tolerant service and subject to access control for Access Category 1, which is judged based on relation of UE’s HPLMN and the selected PLMN. | All except for Emergency, or MO exception data |
| 2 | All | Emergency |
| 3 | All except for the conditions in Access Category 1. | MO signalling on NAS level resulting from other than paging |
| 4 | All except for the conditions in Access Category 1. | MMTEL voice (NOTE 3) |
| 5 | All except for the conditions in Access Category 1. | MMTEL video |
| 6 | All except for the conditions in Access Category 1. | SMS |
| 7 | All except for the conditions in Access Category 1. | MO data that do not belong to any other Access Categories (NOTE 4) |
| 8 | All except for the conditions in Access Category 1 | MO signalling on RRC level resulting from other than paging |
| 9 | All except for the conditions in Access Category 1 | MO IMS registration related signalling (NOTE 5) |
| 10 (NOTE 6) | All | MO exception data |
| 11-31 |  | Reserved standardized Access Categories |
| 32-63 (NOTE 2) | All | Based on operator classification |

As can be observed, 5G AC essentially adopted a coarse-grained classification based on service attributes rather than precise control at the level of service type or QoS characteristic. This design leads to some limitations. For instance, most of uplink data-triggered access attempts are mapped to a single access category AC=7 (i.e., MO-Data). In congestion scenarios, traffic of different QoS characteristics typical undergoes the same access control checks, which would result in high-priority traffic being blocked but lower-priority traffic may get passed.

5G UAC is based on Access-Category (AC) where most UP traffic is always categorized as MO-Data, without further differentiation of various QoS characteristic.

To meet the requirements of SA1 for 6G UAC, RAN2 should consider further enhancement of UAC to support access control with finer granularity in coordination with CT1/SA1, e.g., supporting the new services and assessing the feasibility of incorporating QoS information as an input to AC.

For 6G access control, RAN2 study enhancement on access control based on finer granularity, by coordination with SA1/CT1.

Conclusion

Based on the discussion in section2, we have following proposals:

Observation 1	The legacy cell aggregation approach leads to some drawbacks, at least including  1) unnecessary per-cell overhead,  2) unnecessary scheduling overhead/restriction,  3) limited HARQ diversity due to per-cell HARQ entity,  4) limited trade-off between fast SCell activation and power effiency 4) Static load balance restricted by per-cell camping and access (other than SUL),   5) Asymmetric cell functionality, and 6) limitation of D/U coupling (intra-band or SUL).

Observation 2	For RRC_IDLE, enhancing the spectrum aggregation but sticking to per-CC cell framework would lead to strange multi-cell operation during RRC_IDLE state, and further confusion for cell reselection procedure design (which is supposed to handle UE switching between cells).

Observation 3	For RRC_CONNECTED, enhancing the spectrum aggregation but sticking to per-CC cell framework would lead to either higher complexity, or ruin the concept of cell, if it cannot define the boundary of Physical channel or HARQ entity any more.

Observation 4	In NR, for SM triggered by change of the SIB(s) related to a specific feature, UE does not know the triggering SIB(s) until it (re-)acquires SIB1, which leads to unnecessary UE power consumption for a UE who does not support specific feature.

Observation 5	In 6G, there is a strong demand to support unified initial access of different UE types (e.g., based on different max CBW capability).

Observation 6	Spectrum-wise, the candidate spectrum for 6G diverse, and it requires a solution to aggregate spectrum 1) cmW-band (6.425-7.125GHz (WRC-23), 7-15GHz (WRC-27)) and potentially mmW-band acts as basic contributor to capacity, but with limited coverage compared to 5G;  2) while sub-2GHz spectrum refarming helps coverage, but clearly, they are not capacity enabler.

Observation 7	5G tools for initial-access offloading/re-distribution has drawbacks, e.g., 1) incapable for cross-carrier/band scenario (1st case above), 2) limited to channel quality / slicing and based on static criterion (2nd case above), and 3) limited to SUL type band (3rd case above).

Observation 8	The multi-carrier operation for initial-access offloading/re-distribution is supported in NB-IoT, but its applicability was limited to specific scenarios and use cases.

Observation 9	5G UAC is based on Access-Category (AC) where most UP traffic is always categorized as MO-Data, without further differentiation of various QoS characteristic.

Proposal 1	For spectrum aggregation, R2 discuss the issue by firstly identifying necessary functionality candidates from R2 perspective, and then evaluating the pros/cons of either per-CC cell (as in legacy CA) and multi-CC cell (SCMC) framework.

Proposal 2	For spectrum aggregation at RRC_IDLE, R2 study functionality components including  1) SSB/SIB delivery via anchor-carrier only, 2) RACH/Paging load distribution across anchor-/non-anchor-carrier, and 3) non-SUL band acting as non-anchor-carrier, and discuss the applicable scenario accordingly (e.g., intra-/inter-band, (non)collocated).

Proposal 3	For spectrum aggregation at RRC_CONNECTED, R2 study functionality components focusing on PCell/SCell role alleviation and cross-CC HARQ entity, and discuss the applicable scenario accordingly (e.g., intra-/inter-band, (non)collocated), while waiting for progress by R1 on other related features.

Proposal 4	For 6G system information delivery, RAN2 study schemes to improve energy efficiency, which includes 1) on-demand triggering of SIB1 and OSI; 2) relying solely on anchor carrier to carry system information.

Proposal 5	For 6G system information update, RAN2 study more efficient mechanism to notify events of paging/system-information-change/ETWS-CMAS-indication, to improve UE and Network energy efficiency.

Proposal 6	For 6G initial access, RAN2 study the scheme(s) for flexible offloading/re-distribution for RACH and Paging across frequency-domain resources. FFS on the applicable scenario (intra-/inter-band, and collocated/non-collocated).

Proposal 7	For 6G access control, RAN2 study enhancement on access control based on finer granularity, by coordination with SA1/CT1.