3GPP TSG-RAN WG2 Meeting #132	R2-2508142

Dallas, USA, Nov. 17th - 21st

Agenda Item:	8.13.2

Source:	OPPO

Title:	Discussion on the sl-RelayUE-HopType indication (O511)

Document for:	Discussion, Decision

Introduction

This paper will discuss the following RIL issue.

O511 RIL Id; WI; Class; Title; Tdoc; Delegate; Misc; File version; Status. O500; NR_SL_relay_multihop-Core; 1; How to set indication in the measurement report to indicate the hop status of the candidate relay UE; R2-25xxxxx; OPPO (Bingxue Leng); V044. [Description]: The implementation in the running CR has the following further issues to be fixed: 1/ The indication is for the case when the UE is connecting to the network via direct/single-hop relay at the source side, which means the condition “If the remote UE is acting as multi-hop U2N Remote UE” is not correct; 2/ The sl-RelayUE-HopType setting based on the “number of hops contained in the discovery message” is not correct since: - sl-RelayUE-HopType associated with R19 Last Relay should be set to multi-hop since although the hop count=1, it is still a R19 multi-hop Relay which provides multi-hop Relay service; - There is no number of hops contained in the discovery message for R17 single-hop U2N Relay; - There maybe no number of hops contained in the discovery message for Model-B discovery since it is optional field according to TS 23.304 [Proposed Change]: 5> for each L2 U2N Relay UE that is included in the sl-MeasResultsCandRelay: 6> set the cellIdentity to include the cellAccessRelatedInfo contained in the discovery message received from the concerned L2 U2N Relay UE; 6> set the sl-RelayUE-Identity to include the Source L2 ID of the concerned L2 U2N Relay UE; 6> set the sl-MeasResult to include the SD-RSRP of the concerned L2 U2N Relay UE; 6> If the remote UE is acting as multi-hop U2N Remote UE supports remoteUE-MH-OperationL2 7> set the sl-RelayUE-HopType to single-hop if the number of hops contained in the discovery message is 1 single hop discovery or to multi-hop if the number of hops contained in the discovery message is > 1multi-hop discovery;

Discussion

Based on the agreement, the following sentence was added in the running CR:

#### 5.5.5.1 General Figure 5.5.5.1-1: Measurement reporting The purpose of this procedure is to transfer measurement results from the UE to the network. The UE shall initiate this procedure only after successful AS security activation. <Omitted Text> 1> if there is at least one applicable neighbouring cell or candidate L2 U2N Relay UE to report: 2> if the reportType is set to eventTriggered or periodical: 3> if the measurement report concerns the candidate L2 U2N Relay UE: 4> set the sl-MeasResultsCandRelay in measResultNeighCells to include the best candidate L2 U2N Relay UEs up to maxNrofRelayMeas in accordance with the following: 5> if the reportType is set to eventTriggered: 6> include the L2 U2N Relay UEs included in the relaysTriggeredList as defined within the VarMeasReportList for this measId; 5> else: 6> include the applicable L2 U2N Relay UEs for which the new measurement results became available since the last periodical reporting or since the measurement was initiated or reset; 5> for each L2 U2N Relay UE that is included in the sl-MeasResultsCandRelay: 6> set the cellIdentity to include the cellAccessRelatedInfo contained in the discovery message received from the concerned L2 U2N Relay UE; 6> set the sl-RelayUE-Identity to include the Source L2 ID of the concerned L2 U2N Relay UE; 6> set the sl-MeasResult to include the SD-RSRP of the concerned L2 U2N Relay UE; 6> If the remote UE is acting as multi-hop U2N Remote UE 7> set the sl-RelayUE-HopType to single-hop if the number of hops contained in the discovery message is 1 or to multi-hop if the number of hops contained in the discovery message is > 1;

Firstly, the indication is intended to address scenarios where a Remote UE supports R19 multi-hop operation but is connecting to the network via a direct path or an R17 single-hop U2N relay (instead to acting as multi-hop U2N Remote UE), while reporting inter-gNB multi-hop relay candidates. Therefore, the condition for setting the sl-RelayUE-HopType indication should be based on whether the Remote UE supports remoteUE-MH-OperationL2, rather than whether it “is acting as a multi-hop U2N Remote UE,” in order to cover the intended scenario.

To address the inter-gNB direct/single-hop to multi-hop path switching scenario, the condition for setting the sl-RelayUE-HopType indication should be remote UE supports remoteUE-MH-OperationL2 instead of “is acting as multi-hop U2N Remote UE”.

Then, regarding how to set the indication, according to the current running CR, sl-RelayUE-HopType is set based on the number of hops contained in the discovery message. However, this approach does not always accurately reflect the single-hop or multi-hop status.

The first case is the R19 Last Relay, which performs R19 multi-hop U2N Relay discovery with hop number of 1. Under the current CR logic, this would incorrectly result in the sl-RelayUE-HopType being set to single-hop. However, the R19 Last Relay provides multi-hop U2N relay service and should be reported as a multi-hop U2N candidate Relay. As specified in TS 23.304, single-hop and multi-hop U2N relays are distinct services, each with its own discovery mechanism and identified by separate Relay Service Codes.

#### 5.1.4.1a Policy/Parameter provisioning for 5G ProSe Multi-hop UE-to-Network Relay The following information is provisioned in the UE in support of the UE assuming the role of a 5G ProSe Multi-hop UE-to-Network Relay: 1) Authorisation policy for acting as a 5G ProSe Layer-3 and/or Layer-2 Multi-hop UE-to-Network Relay when "served by NG-RAN": - PLMNs in which the UE is authorized to relay traffic for 5G ProSe Multi-hop Layer-3 and/or Layer-2 Remote UEs supporting 5G ProSe Multi-hop UE-to-Network Relay. The authorisation for a UE to act as a 5G ProSe Multi-hop UE-to-Network Relay also authorizes the use of 5G ProSe Multi-hop UE-to-Network Relay Discovery with Model A and Model B. NOTE 1: It is up to UE and application implementation to select a discovery model or whether to perform both models simultaneously. 2) ProSe Relay Discovery policy/parameters for 5G ProSe Multi-hop UE-to-Network Relay: - Includes the parameters that enable the UE to perform 5G ProSe Multi-hop UE-to-Network Relay Discovery when provided by PCF or provisioned in the ME or configured in the UICC: - 5G ProSe Multi-hop UE-to-Network Relay Discovery parameters (User Info ID, Relay Service Code(s), UE-to-Network Relay Layer Indicator per RSC, Multi-hop indicator per RSC, maximum number of hops per RSC). The UE-to-Network Relay Layer Indicator indicates whether the associated RSC is offering 5G ProSe Layer-2 or Layer-3 UE-to-Network Relay service. RSC dedicated for emergency service may also be provisioned. The Multi-hop indicator indicates whether the associated RSC can be supported for 5G ProSe Multi-hop UE-to-Network Relay operation. The maximum number of hops per RSC indicates the maximum number of hops can be supported for the associated RSC. NOTE 2: The RSC used for Multi-hop service is different from the RSC used for single-hop service.

Single-hop U2N Relay and multi-hop U2N Relay are different services with different RSC. Which means the Last Relay with hop-count 1 is still a multi-hop U2N Relay UE and the corresponding sl-RelayUE-HopType should be set to multi-hop.

Another case involves R17 single-hop U2N relay discovery, where the discovery message does not include a “number of hops” field. Consequently, the sl-RelayUE-HopType indication cannot be set to single-hop based on the current criteria.

For R17 single-hop U2N relay, the discovery message does not include a “number of hops” field. However, the sl-RelayUE-HopType associated with an R17 single-hop U2N candidate relay should be set to single-hop, which cannot be achieved based on the current running CR.

In addition, according to TS 23.304, for R19 multi-hop U2N relay discovery Model-B, the “number of hops” field in the discovery message is optionally present. As a result, the sl-RelayUE-HopType indication cannot be reliably set in this case either.

#### 5.8.3.1 Common identifiers for 5G ProSe UE-to-Network Relay The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Announcement message (Model A), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Announcer Info and Relay Service Code are contained in the message: - Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery. - Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1. - Announcer Info: provides information (i.e. User Info ID) about the announcing user. - Relay Service Code: parameter identifying a connectivity service the 5G ProSe UE-to-Network Relay provides to a 5G ProSe Remote UE. The Relay Service Codes are configured in a 5G ProSe UE-to-Network Relay for advertisement. Additionally, the Relay Service Code may also identifies authorized users the 5G ProSe UE-to-Network Relay would offer service to and may be used to select the related security policies or information e.g. necessary for authentication and authorization between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay (e.g. a Relay Service Code for relays for police members only would be different than a Relay Service Code for relays for Fire Fighters only, even though potentially they provided connectivity to same DN e.g. to support Internet Access). The 5G ProSe UE-to-Network Relay Discovery Announcement message (Model A) is extended and modified as follows when used for 5G ProSe Multi-hop UE-to-Network Relay discovery: - Hop-Count: This value reflects the number of PC5 hops for the 5G ProSe Remote UE to reach the network. It is set to 1 by the 5G ProSe UE-to-Network Relay and shall be incremented by 1 every time the 5G ProSe UE-to-Network Relay Discovery Announcement message is forwarded by a 5G ProSe Intermediate UE-to-Network Relay. A 5G ProSe Intermediate UE-to-Network Relay shall only process the 5G ProSe UE-to-Network Relay Discovery Announcement if the Hop-Count is present and the value is less than the (pre-)configured maximum number of hops for the associated RSC and the optional Hop-Limit in the message. - Source Layer-2 ID: the 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay self-selects a Source Layer-2 ID when sending the discovery message. - Destination Layer-2 ID: the 5G ProSe Intermediate UE-to-Network Relay sets the Destination Layer-2 ID of the 5G ProSe UE-to-Network Relay Discovery Announcement message based on configuration described in clause 5.1.4.1. - Announcer Info: identify information (i.e. User Info ID) of the announcing 5G ProSe UE-to-Network Relay or the 5G ProSe Intermediate UE-to-Network Relay. - (optional) Root Relay Info: this is the User Info ID of the 5G ProSe UE-to-Network Relay. A 5G ProSe Intermediate UE-to-Network Relay may store it in the discovery entry and included it in the 5G ProSe UE-to-Network Relay Discovery Announcement message. This information can be used in the relay selection at the 5G ProSe Remote UE or other 5G ProSe Intermediate UE-to-Network Relays. - (optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links to the Root Relay, i.e. the 5G ProSe UE-to-Network Relay, for this RSC. - (optional) Hop-Limit: this is the hop limit set by the announcing 5G ProSe UE-to-Network Relay to a value smaller than the configured maximum number of hops. The following parameters are used for the 5G ProSe UE-to-Network Relay Discovery Solicitation message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoverer Info and Relay Service Code are contained in the message: - Source Layer-2 ID: the 5G ProSe Remote-UE self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery. - Destination Layer-2 ID: the Destination Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery is selected based on the configuration as described in clause 5.1.4.1. - Discoverer Info: provides information (i.e. User Info ID) about the discoverer user. - Target Info: provides information (i.e. User Info ID) about the targeted discoveree user. - Relay Service Code: information about connectivity that the discoverer UE is interested in. The Relay Service Codes are configured in the 5G ProSe Remote UEs interested in related connectivity services. To support Multi-hop 5G ProSe UE-to-Network Relay discovery the following parameters are added: - (Optional) Hop-Count: indicates the number of hops that the message is already relayed. It is increased by 1 per hop. - (Optional) Hop-Limit: an unmodified value that indicates the hop limit of the message. It is set, by the 5G ProSe Remote UE, to a value smaller than the (pre)configured maximum number of hops. - Path information: an (ordered) list of User Info ID(s) and corresponding L2 ID(s) of Intermediate UE-to-Network Relay(s) that indicates the transmitted path of the message. - (optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links starting from the PC5 link between the Remote UE and an Intermediate UE-to-Network Relay in the path information for this RSC. The following parameters are used in the 5G ProSe UE-to-Network Relay Discovery Response message (Model B), where Source Layer-2 ID and Destination Layer-2 ID are used for sending and receiving the message and Discoveree Info and Relay Service Code are contained in the message: - Source Layer-2 ID: the 5G ProSe UE-to-Network Relay self-selects a Source Layer-2 ID for 5G ProSe UE-to-Network Relay Discovery. - Destination Layer-2 ID: set to the Source Layer-2 ID of the received 5G ProSe UE-to-Network Relay Discovery Solicitation message. - Relay Service Code: identifies the connectivity service the 5G ProSe UE-to-Network Relay provides to 5G ProSe Remote UEs that matches the Relay Service Code from the corresponding Discovery Solicitation message. - Discoveree Info: provides information (i.e. User Info ID) about the discoveree. To support Multi-hop 5G ProSe UE-to-Network Relay discovery the following parameters are added: - (Optional) Hop-Count: indicates the number of hops between the 5G ProSe Remote UE and the 5G ProSe UE-to-Network Relay on the path selected by the 5G ProSe UE-to-Network Relay. - Path information: an (ordered) list of User Info ID(s) and corresponding L2 ID(s) of the discoverer user and the Intermediate UE-to-Network Relay(s) on the path selected by the 5G ProSe UE-to-Network Relay. - (optional) Accumulated QoS for PC5 link: this reflects the QoS supported over all the PC5 links along the path from Remote UE to the 5G ProSe UE-to-Network Relay for this RSC.

For R19 multi-hop U2N relay discovery Model-B, the “number of hops” field in the discovery message is optionally present. As a result, if this field is absent, the sl-RelayUE-HopType indication cannot be set.

Therefore, it is proposed to fix the above issues as proposed in RIL O511.

(O511) For sl-RelayUE-HopType indication, update the running CR based on O511 so that: 1) it is set by remote UE supports remoteUE-MH-OperationL2; 2) the value is set to single-hop or multi-hop depending on whether a single-hop U2N Relay discovery message or a multi-hop U2N Relay discovery message is received.

Conclusions

We have the following observations:

Observation 1	To address the inter-gNB direct/single-hop to multi-hop path switching scenario, the condition for setting the sl-RelayUE-HopType indication should be remote UE supports remoteUE-MH-OperationL2 instead of “is acting as multi-hop U2N Remote UE”.

Observation 2	Single-hop U2N Relay and multi-hop U2N Relay are different services with different RSC. Which means the Last Relay with hop-count 1 is still a multi-hop U2N Relay UE and the corresponding sl-RelayUE-HopType should be set to multi-hop.

Observation 3	For R17 single-hop U2N relay, the discovery message does not include a “number of hops” field. However, the sl-RelayUE-HopType associated with an R17 single-hop U2N candidate relay should be set to single-hop, which cannot be achieved based on the current running CR.

Observation 4	For R19 multi-hop U2N relay discovery Model-B, the “number of hops” field in the discovery message is optionally present. As a result, if this field is absent, the sl-RelayUE-HopType indication cannot be set.

We have the following proposals:

Proposal 1	(O511) For sl-RelayUE-HopType indication, update the running CR based on O511 so that: 1) it is set by remote UE supports remoteUE-MH-OperationL2; 2) the value is set to single-hop or multi-hop depending on whether a single-hop U2N Relay discovery message or a multi-hop U2N Relay discovery message is received.