3GPP TSG-RAN WG2 Meeting #132	R2-2508157

Dallas, USA, Nov. 17th - 21st

Agenda Item:	8.8.2

Source: 	CATT

Title: 	Discussion on SMTC enhancements of inter-frequency

Document for: 	Discussion and Decision

Introduction

In RAN2#131bis [1], we achieved the following agreement.

Agreements:

1. 	[V204/H250] We extend the Rel-19 SMTC enhancements to inter-frequency, using as a baseline the Rel-19 agreed solutions for intra-frequency (for both location-based SMTC selection in RRC_IDLE/INACTIVE state and UE-assisted SMTC configuration in RRC_CONNECTED state). If modifications for inter-frequency are needed, Scenarios 1, 2, 3 (adjacent and overlaid inter-frequency neighbor cells of similar size) are prioritized. We introduce a new (per-UE) capability with signaling for the support in RRC connected of 2 periodicities in the inter-frequency case

According to the agreement, the current RRC CR [2] is implemented as follows, in which the refLocList configuration and the closest N reference locations reporting are performed per frequency.

OtherConfig information element -- ASN1START -- TAG-OTHERCONFIG-START Assisted-SSB-MTC-Config-r19 ::= SEQUENCE { closestLocsToReport-r19 INTEGER (1..4), intraFreqConfig-r19 SSB-MTC-LocPerFreqInfo-r19 OPTIONAL, -- Need R interFreqConfigList-r19 SEQUENCE (SIZE (1..maxFreq)) OF SSB-MTC-LocPerFreqInfo-r19 OPTIONAL -- Need R } SSB-MTC-LocPerFreqInfo-r19 ::= SEQUENCE { carrierFreq-r19 ARFCN-ValueNR OPTIONAL, -- Cond InterFreq refLocList-r19 SEQUENCE (SIZE (1..6)) OF ReferenceLocation-r17 OPTIONAL -- Need S } Editor’s note: The exact solution implementation for assisted SMTC and measurement gap configuration for inter-frequency neighbours is to be further discussed. UEAssistanceInformation message -- ASN1START -- TAG-UEASSISTANCEINFORMATION-START SSB-MTC-RefLocReportList-r19 ::= SEQUENCE (SIZE (1..maxFreqPlus1)) OF SSB-MTC-RefLocReport-r19 SSB-MTC-RefLocReport-r19::= SEQUENCE { carrierFreq-r19 ARFCN-ValueNR, referenceLocationReport-r19 BIT STRING (SIZE (6)) } -- TAG-UEASSISTANCEINFORMATION-STOP -- ASN1STOP Editor’s note: The exact solution implementation for assisted SMTC and measurement gap configuration for inter-frequency neighbours is to be further discussed.

During the RRC ASN.1 comment phase, companies have divergence views on whether these procedures needs to be performed per frequency. This contribution gives our views on this issue.

Discussion

The intention we introduced the reference location configuration and closest reference locations reporting in connected mode is to assist the network configuring appropriate SMTCs around the UE. For this purpose, the network only needs to determine the coarse location of the UE.

Firstly, we would like to clarify that the reference locations is not limited to be configured as the cell center of neighbour cells. It could be left to NW implementation to configure suitable reference locations to determines UE’s coarse location for different deployment scenarios, the configured reference locations could be points within the serving cell and/or cell center of neighbour cells. The following gives an example that the reference locations are 6 evenly distributed points within the serving cell.

- Case a): If the network configures UE to report the first closest reference point, the UE reports the reference location #1 to the network. According to the report, the network could determine the UE is within the blue area.

- Case b): If the network configures UE to report the 2 closest reference points, the UE reports the reference location #1 and #2 to the network. According to the report, the network could determine the UE is within the red area.

Case a): report the first closest reference point       Case b): report the 2 closest reference points

Then, according to the determined coarse location, the network configures SMTCs for the neighbour cells. Take case a) as an example (the coarse location of UE is the blue area), the following are examples of configuring SMTCs for the 4 inter-frequency scenarios [3] as discussed in the last meeting.

Figure 1 Potential NTN inter-frequency scenarios

Cell index {a, b, … f} are neighbor cells on f1, cell index {1, 2, … 7} are neighbor cells on f2.

- For Scenario 1, NW may configure the SMTCs f2: {1,2,3} for the UE.

- For Scenario 2, NW may configure the SMTCs f1: {d,e,f}, f2:{1,2,3,7} for the UE.

- For Scenario 3, NW may configure the SMTCs f1: {d,e,f}, f2:{1,6,7} for the UE.

- For Scenario 4, NW may configure the SMTCs f1: {d,e,f}, f2:{1,3,5,7} for the UE.

Therefore, based on the configured reference locations the network can be able to select appropriate measurement on neighbour cells for the UE including both intra-frequency and inter-frequency. That is, per frequency reference locations configuration and per frequency closest reference locations reporting are not needed.

Proposal 1：For UE-assisted SMTC configuration in RRC_CONNECTED state, per frequency reference locations configuration and per frequency closest reference locations reporting are not supported, i.e., frequency is not differentiated in these procedures.

In the current RRC CR, if refLocList is absent in the otherConfig, the UE shall use the refLocList from SIB2 or SIB4 for the spec, as shown below.

refLocList A list of reference locations for assisted SMTC and measurement gap configuration in RRC_CONNECTED state. When this field is absent in intraFreqConfig, the UE shall use the refLocList associated to smtc5list provided in SIB2, if available. When this field is absent in an entry within interFreqConfigList, the UE shall use the refLocList associated to smtc5list provided in SIB4 for the specific carrierFreq, if available.

If proposal 1 is agreed, the refLoclist in dedicate signaling does not distinguish frequencies, while the refLoclist in SIB2/SIB4 are per frequency configured, there may be multiple sets of reference locations in SIB2 and SIB4. It is necessary to clarify the reference locations in which SIB is used when the refLocList does not exist in OtherConfig. The simplest way is to specify UE use the refLocList provided in SIB2. If the network consider the refLocList provided in SIB2 is insufficient to determine the UE’s coarse location used for configuring SMTCs in RRC_CONNECTED state, the network shall configure refLocList in the RRCReconfiguration message.

Proposal 2: If Proposal 1 is agreed, change the field description of refLocList as following:

refLocList A list of reference locations for assisted SMTC and measurement gap configuration in RRC_CONNECTED state. When this field is absent in intraFreqConfig, the UE shall use the refLocList associated to smtc5list provided in SIB2, if available. When this field is absent in an entry within interFreqConfigList, the UE shall use the refLocList associated to smtc5list provided in SIB4 for the specific carrierFreq, if available.When this field is absent and closestLocsToReport is signalled, the UE shall use the refLocList associated to smtc5list provided in SIB2, if available.

Conclusion

In this contribution, the inter-frequency assisted-SMTC enhancement is discussed with the following proposal:

Proposal 1：For UE-assisted SMTC configuration in RRC_CONNECTED state, per frequency reference locations configuration and per frequency closest reference locations reporting are not supported, i.e., frequency is not differentiated in these procedures.

Proposal 2: If Proposal 1 is agreed, change the field description of refLocList as following:

refLocList A list of reference locations for assisted SMTC and measurement gap configuration in RRC_CONNECTED state. When this field is absent in intraFreqConfig, the UE shall use the refLocList associated to smtc5list provided in SIB2, if available. When this field is absent in an entry within interFreqConfigList, the UE shall use the refLocList associated to smtc5list provided in SIB4 for the specific carrierFreq, if available.When this field is absent and closestLocsToReport is signalled, the UE shall use the refLocList associated to smtc5list provided in SIB2, if available.

Reference

- RAN2#131bis ChairNotes.

- R2-2507785  Initial corrections to NR NTN Phase 3 Rel-19 Ericsson	CR	Rel-19

- R2-2507763  Report of [AT131bis][303][NR NTN] RRC open issues	Ericsson  discussion	Rel-19	NR_NTN_Ph3-Core