3GPP TSG-RAN WG2 Meeting #132	R2-2508161

Dallas, US, Nov. 17th-21th

Agenda Item:	9.7.2

Source: 	CATT, CENC

Title: 	Discussion on supporting IMS voice call over GSO for NB-IoT

Document for: 	Discussion and Decision

1	Introduction

This contribution discusses how to support IMS voice call over GSO for NB-IoT in the following aspects: - Analyse the specification impact of CP solution and UP solution and give final decision. - Analyse the RoHC header size and give the expected average RoHC header size in LS reply to SA4. - Discussion on the possible solutions to avoid the potential issues when handling voice packets of different sizes on SPS and dynamic scheduling. - Discussion on some other spec impacts which are needed for NB-IoT supports IMS voice call over GSO.

2	Discussion

2.1	CP vs UP solution

In last meeting, RAN2 discuss CP solution and UP solution and majority of companies in RAN2 prefer the UP over CP solution [1].

RAN2#131bis Agreements regarding the reply LS to SA2: - We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

Additionally, SA2#171 discussed these two solutions and reached conclusions that the voice packets shall be transported over the NB-IoT (GEO) user plane [2].

# 8 Conclusions Editor's note: This clause will list conclusions that have been agreed during the course of the study item activities. ## 8.1 Interim conclusions for Key Issue #1 Editor's note 1: Other interim conclusions for Key Issue#1 is FFS. The following interim conclusions for Key Issue #1 support of IMS voice call over NB-IoT via GEO satellite connecting to EPC are made: - The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

Observation 1: SA2 already achieved conclusion that the voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

Since majority of companies in RAN2 prefer the UP over CP solution and SA2 already agreed to adopt the UP solution, it is proposed that UP solution is adopted for NB-IoT to support MS voice call over GSO.

Proposal 1: UP solution is adopted for NB-IoT to support IMS voice call over GSO.

For UP solution, the IMS signalling and voice data are carried in DRBs over the Uu, which can be supported naturally. One noteworthy issue, if 2 DRBs are used to separately carry the IMS signalling and the voice data to differentiate their QCI requirements, there is no other DRB for other service as the NB-IoT can support at most 2 DRBs currently. One way to solve this issue is to support one additional DRB for NB-IoT, but this method unavoidably increases the complexity of UE. From our perspective, it is not a common case for the NB-IoT having IMS voice call service and another service at the same time. Additionally, the data rate supported by the NB-IoT is difficult to support the IMS voice call services and other services simultaneous. Maybe we can just keep the current NB-IoT capability as it. We can further discuss whether to support one additional DRB so that parallel service of IMS voice call and other service can be supported.

Observation 2: For UP solution, if these 2 DRBs are used to separately carry the IMS signalling and voice data, there is no DRB left for other service during an on-going IMS voice call. Therefore, the necessity for an additional DRB can be discussed.

Proposal 2: Further discusses whether to support one additional DRB to enable parallel service of IMS voice call and other service.

2.2	RoHC header size

In last meeting, the expected average RoHC header size was discussed but not concluded [1].

RAN2#131bis Agreements regarding the reply LS to SA4: - RAN2 will provide further feedback on the expected average RoHC header size.

The RoHC scheme has three modes of operation, called Unidirectional mode (U-mode), Bidirectional Optimistic mode (O-mode), and Bidirectional Reliable mode (R-mode) [3]. In U-mode, the compressor will be less efficient and have a slightly higher probability of loss propagation because the periodic refreshes and the lack of feedback for initiation of error recovery. In R-mode, the compressor bases its confidence only on acknowledgments received from the decompressor. In O-mode, the compressor aims to maximize compression efficiency and sparse usage of the feedback channel. Due to the large RTT delay of GSO satellites, O-mode is more suitable for efficient voice transmission to support IMS voice call over GSO.

The RoHC header is related to different compression state, e.g., Initialization and Refresh (IR) state, First Order (FO) state, or Second Order (SO) state. According to RAN1’s reply LS to SA2 [4], the probability of packets consecutively lost is negligible under typical operating conditions when line-of-sight condition for the duration of the call. Under this condition, the voice data packet and Silence Insertion Descriptor (SID) packet can be compressed while the compressor operates in the SO state usually. Therefore, UO-0 packet for voice data packet and UO-1 packet for SID packet are usually used in SO state under O-mode. For UO-0 packet, the RoHC header is 3 bytes (with 2 bytes UDP Checksum, when UDP Checksum is enabled). For UO-1 packet, the RoHC header is 4 bytes (with 2 bytes UDP Checksum, when UDP Checksum is enabled).

Therefore, RAN2 could reply LS to SA4, indicate if UDP Checksum is enabled, the minimum RoHC header size is 3 bytes, but 4 bytes RoHC header size is also usually used due to alternation of silent period and talk spurt period.

Proposal 3: Reply an LS to SA4 indicate if UDP Checksum is enabled, the minimum RoHC header size is 3 bytes for voice data packet and 4 bytes for SID packet.

2.3	Transmission of voice packets

The IMS voice call service is composed of talk spurt and silent period, and the data packets of IMS voice call service within the talk spurt and silent have different periodic characteristics and packet sizes. The following discusses how to guarantee the data transmission of IMS voice call over GSO for NB-IoT with considering the voice packet characteristics.

- Talk spurt and silent period have different periodicities

Considering the periodic characteristics of voice packets (include voice data packet and SID packet), support SPS for NB-IoT is meaningful to improve the efficiency of scheduling procedure. Taking VoLTE AMR as example, the packet interval of talk spurt is 20ms and the packet interval of silent period is 160ms [5]. To meet the different transmission requirements of talk spurt and silent periods, it is proposed to support configuring multiple sets of SPS resources for NB-IoT, which can be configured with different periodicities.

As for the specific values of the SPS periodicity, it is closely related to the packets interval of IMS voice call service. In case of IMS voice call over GSO, SA4 is considering defining other packets intervals for NB-IoT, e.g., 120ms and/or 240ms [6]. In last meeting, we replied LS to SA4 that 120ms and 240ms periodicities, which do not divide 10240, would require additional specification work in RAN2 to resolve the issues. Wait for SA4’s progress to decide the candidate values of the SPS periodicity.

RAN2#131bis Agreements regarding the reply LS to SA4: - We indicate to SA4 that RAN2 has not started work on SPS for voice but thinks that SPS periodicities like 120ms and 240ms periodicities that do not divide 10240 would require additional specification work in RAN2 to resolve the issues

- Voice packets of different sizes

The voice packets of different sizes may be caused by many reasons, e.g. alternating appearance of silent period and talk spurt period, changing of ROHC compressor/decompressor state, etc. Based on RAN1’s reply LS to SA2 [4], the probability of packets consecutively lost is negligible under line-of-sight condition, that is the size of ROHC header/voice packet will not change frequently during a certain period. From our perspective, silent period and talk spurt period change is the main reason for different size of voice packets. This issue can also be well handled by supporting the configuration of multiple sets of SPS resources for NB-IoT, which can be configured with different resource sizes, so that different SPS resource sizes can be configured for silent period and talk spurt period. Additionally, SR and BSR can be used as supplementary mechanism when the configured UL grant is insufficient.

Proposal 4: Support configuring multiple sets of SPS resources for NB-IoT. Wait for SA4’s progress to decide the candidate values of SPS’s periodicity.

2.4	Other issues

Some other issues are identified for NB-IoT supports IMS voice call over GSO.

- Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN

Since the IMS voice call is supported for NB-IoT, the NB-IoT UE needs to know whether a GSO cell supports the IMS voice call service, so that it could decide whether to initiate the IMS voice call service when camping on this cell. An indication for that use can be introduced in SIB2-NB.

Proposal 5: Introduce an indication in SIB2-NB to indicate network supports IMS voice call over GSO for NB-IoT.

If the UE determines the cell supports the IMS voice call over GSO and a requirement is received from the upper layers, it initiates a RRC connection setup procedure. For network information, an establishment cause for IMS voice call can be added in the establishementCause-NB IE. An example TP is shown as following.

#### – EstablishmentCause-NB The IE EstablishmentCause-NB provides the establishment cause for the RRC connection request or the RRC connection resume request as provided by the upper layers. EstablishmentCause-NB information element -- ASN1START EstablishmentCause-NB-r13 ::= ENUMERATED { mt-Access, mo-Signalling, mo-Data, mo-ExceptionData, delayTolerantAccess-v1330, mt-EDT-v1610, mo-VoiceCall-v20xx, spare1} -- ASN1STOP

Proposal 6: Add an establishment cause for IMS voice call in the establishementCause-NB IE.

- Support of necessary modifications for emergency call for voice over NB-IoT-NTN

If emergency call is supported for NB-IoT, the network should also indicate that to UE. For LTE, the UE can perform cell (re)selection procedure with considering whether the cell supports emergency call, to prevent UE camping on a cell does not support IMS emergency call (shown as the yellow highlight text). For NB-IoT, camp on any cell state is supported in Rel-19, after the emergency call service is supported in Rel-20, the following text could also be applicable to NB-IoT. For this purpose, an indication of cell supports emergency call can be introduced in SIB1-NB.

### 5.2.9 Camped on Any Cell state In this state, the UE shall perform the following tasks: - monitor the paging channel of the cell as specified in clause 7 according to information sent in system information; - monitor relevant System Information as specified in TS 36.331 [3]; - perform necessary measurements for the cell reselection evaluation procedure; - execute the cell reselection evaluation process on the following occasions/triggers: 1) UE internal triggers, so as to meet performance as specified in TS 36.133 [10]; 2) When information on the BCCH or BR-BCCH used for the cell reselection evaluation procedure has been modified; - regularly attempt to find a suitable cell trying all frequencies of all RATs that are supported by the UE. If a suitable cell is found, UE shall move to camped normally state; - if the UE supports voice services and the current cell does not support emergency call as indicated in System information specified in TS 36.331 [3], the UE should perform cell selection/ reselection to an acceptable cell of any supported RAT regardless of priorities provided in system information from current cell, if no suitable cell is found. NOTE: The UE is allowed to not perform reselection to an inter-frequency E-UTRAN cell in order to prevent camping on a cell on which it cannot initiate an IMS emergency call.

Proposal 7: Introduce an indication in SIB1-NB to indicate network supports IMS emergency call over GSO for NB-IoT.

In Rel-19, we support PWS for NB-IoT NTN. On basic of that, to enable the NB-IoT UE camps on cell to obtain PWS service without PLMN limitation, the applicable cell concept [7] is extended to NB-IoT. However, in Rel-19, the NB-IoT does not support emergency call service, that the definition of acceptable cell excludes emergency call for NB-IoT. Since the emergency call service is support for NB-IoT UE in Rel-20, the definition of acceptable cell for NB-IoT can be updated accordingly.

Acceptable Cell: A cell that satisfies certain conditions as specified in 4.3. Except for NB-IoT, Athe UE can always attempt emergency calls on an acceptable cell, but restriction as in 5.3.3 apply. acceptable cell: An "acceptable cell" is a cell on which the UE may camp to obtain limited service (originate emergency calls (except for NB-IoT) and receive ETWS and CMAS notifications), and it is not applicable to RRC_INACTIVE state. Such a cell shall fulfil the following requirements, which is the minimum set of requirements to initiate an emergency call and to receive ETWS and CMAS notification in a E-UTRAN network: - The cell is not barred, see clause 5.3.1; - The cell selection criteria are fulfilled, see clause 5.2.3.2;

Proposal 8: Update the definition of acceptable cell in TS 36.304 for NB-IoT to support emergency call service.

- ROHC enhancement

For UP solution, the ROHC is performed in the PDCP layer for header reduction. In LTE, the IMS voice call data is with RTP header, and the profile 0x0001 or 0x0101 can be used. In current spec, the NB-IoT only supports the following profiles with yellow highlight [8][9]. For UP solution, the NB-IoT supported ROHC profiles may need to be extended. However, whether the profile 0x0001 or 0x0101 can be directly used by NB-IoT is also pending SA2’s selection on IP solution or non-IP solution. RAN2 can postpone the discussion on ROHC enhancement when there is more progress on these aspects.

Table 5.5.1.1: Supported ROHC protocols and profiles

| Profile Identifier | Usage: | Reference |
| --- | --- | --- |
| 0x0000 | No compression | RFC 5795 |
| 0x0001 | RTP/UDP/IP | RFC 3095, RFC 4815 |
| 0x0002 | UDP/IP | RFC 3095, RFC 4815 |
| 0x0003 | ESP/IP | RFC 3095, RFC 4815 |
| 0x0004 | IP | RFC 3843, RFC 4815 |
| 0x0006 | TCP/IP | RFC 6846 |
| 0x0101 | RTP/UDP/IP | RFC 5225 |
| 0x0102 | UDP/IP | RFC 5225 |
| 0x0103 | ESP/IP | RFC 5225 |
| 0x0104 | IP | RFC 5225 |

Observation 3: Whether the ROHC enhancement is needed for NB-IoT depends on SA2’s down selection between IP and non-IP solution.

Proposal 9: Postpone the discussion on ROHC enhancement until the IP vs non-IP solution is finalized by SA2.

3	Conclusion

This contribution discusses how to support IMS voice call over GSO for NB-IoT, the observations and proposals are listed as follows:

CP vs UP solution

Observation 1: SA2 already achieved conclusion that the voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

Proposal 1: UP solution is adopted for NB-IoT to support IMS voice call over GSO.

Observation 2: For UP solution, if these 2 DRBs are used to separately carry the IMS signalling and voice data, there is no DRB left for other service during an on-going IMS voice call. Therefore, the necessity for an additional DRB can be discussed.

Proposal 2: Further discusses whether to support one additional DRB to enable parallel service of IMS voice call and other service.

RoHC header size

Proposal 3: Reply an LS to SA4 indicate if UDP Checksum is enabled, the minimum RoHC header size is 3 bytes for voice data packet and 4 bytes for SID packet.

Transmission of voice packets

Proposal 4: Support configuring multiple sets of SPS resources for NB-IoT. Wait for SA4’s progress to decide the candidate values of SPS’s periodicity.

Other issues

Proposal 5: Introduce an indication in SIB2-NB to indicate network supports IMS voice call over GSO for NB-IoT.

Proposal 6: Add an establishment cause for IMS voice call in the establishementCause-NB IE.

Proposal 7: Introduce an indication in SIB1-NB to indicate network supports IMS emergency call over GSO for NB-IoT.

Proposal 8: Update the definition of acceptable cell in TS 36.304 for NB-IoT to support emergency call service.

Observation 3: Whether the ROHC enhancement is needed for NB-IoT depends on SA2’s down selection between IP and non-IP solution.

Proposal 9: Postpone the discussion on ROHC enhancement until the IP vs non-IP solution is finalized by SA2.

4	Reference

- RAN2#131bis ChairNotes.

- S2-2509293 "Interim conclusions on KI#1 Support of Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC", Wuhan, China, 13th – 17th October, 2025.

- IETF RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP, and uncompressed".

- R2-2508011 "Reply LS on issues related to support of IMS voice over NB-IoT NTN connected to EPC", Prague, Czech Republic, 13th - 17th October, 2025.

- S2-2507021 "Discussion paper on IMS voice packets transportation options", Goteburg, Sweden, 25th - 29th August, 2025.

- R2-2506754 "LS on bundling period and SPS for ULBC", Prague, Czech Republic, 13th - 17th October, 2025.

- R2-2506556 "Introduction of IoT NTN Enhancements", Bengaluru, India, 25th - 29th August, 2025.

- 3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) protocol specification".

- 3GPP TS 36.323: "Evolved Universal Terrestrial Radio Access (E-UTRA); Packet Data Convergence Protocol (PDCP) specification".