3GPP TSG-RAN WG2 Meeting #132                                         R2-2508173

Dallas, USA, 17-21 Nov 2025

Source:        ZTE Corporation

Title:           Discussion on remaining issues for MAC

Agenda item:   8.11.2

Document for:  Discussion and Decision

- Introduction

This contribution proposes our view on the remaining MAC issues for Rel-19 SBFD WI which is provided in [1].

- Discussion on MAC-1

- MAC-1: Whether/how to address the issue in case RO type switching and Msg1 repetition number fallback are triggered simultaneously.

Since RO type switch and Msg1 repetition number fallback use a same counter to determine the satisfy criteria, so it may happen that upon a specific counter value, both criteria of RO type switch and Msg1 repetition number fallback both satisfy.

RAN2 has agreement that when this case happens, UE (firstly) performs RO type switch. The agreement itself does not contain the wording ‘first’ but the consensus was that the agreement aims to resolve the order of Msg1 repetition number fallback and RO type switch.

Technically speaking, it is not reasonable to let UE directly ignore the Msg1 repetition number increase after RO type switch is performed, since RO type switch includes both directions and the reason for such switch is not totally the same as the reason for Msg1 repetition number fallback. So those two should be independent features and UE should examines them both upon each counter value. Furthermore, it is the simplest way from the specification effort perspective.

Observation 1: It is not reasonable to let UE directly ignore the Msg1 repetition number increase after RO type switch is performed, since RO type switch includes both directions and the reason for such switch is not totally the same as the reason for Msg1 repetition number fallback.

When both RO type switch and Msg1 repetition number fallback happen upon one specific counter value, the most reasonable UE behavior should be:

- If UE performs RO type switch to a set with same Msg1 repetition number, UE can determine to further fallback to a higher Msg1 repetition number according to the following branch of Msg1 repetition number increase criteria (on the switched RO type);

- If UE performs RO type switch to a set with higher Msg1 repetition number, UE should NOT try to increase the Msg1 repetition number on the switched RO type again.

However, capturing such UE behavior brings rather heavy MAC specification impact and it is complex for UE implementation. Even if the issue is worth discussing, to simply the effort, we suggest to do nothing on current MAC spec, i.e., let UE check both RO type switch and Msg1 repetition number fallback criteria in sequence.

Observation 2: It is reasonable to further increase the Msg1 repetition number if UE does not re-select a set with higher Msg1 repetition number during RO type switch. However, this brings heavy MAC spec impact.

Proposal 1: For MAC-1, support to do nothing on current MAC spec, i.e., for each specific counter value, let UE check both RO type switch and Msg1 repetition number fallback criteria in sequence.

- Discussion on MAC-2

- MAC-2: Content to be captured in the LS for requesting RAN1 to update the definition of the first PRACH occasions in TS 38.213, to let it inclusively cover the ROs for cases where tdd-UL-DL-ConfigurationCommon is not applied.

RAN1’s specification is as following:

- 8 Random access procedure … Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a random access procedure using: - first PRACH occasions each including only symbols that are indicated as uplink or flexible by tdd-UL-DL-ConfigurationCommon and considered as uplink for the random access procedure, or - second PRACH occasions, that are in RBs that are both in the active UL BWP and in the UL sub-band, and associated either only with SBFD symbols that include at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon when the UE is provided either sbfd-RACHSingleConfig or sbfd-RACHDualConfig, or start from an SBFD symbol and end in a non-SBFD symbols when the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes The procedures in clause 8.1 apply separately for the first PRACH occasions and the second PRACH occasions. Only a Type-1 random access procedure is applicable for the second PRACH occasions.

Our thinking is, once a RO is not the second PRACH occasions, it should belong to the first PRACH occasions. Such description should be correctly captured in RAN1, for example:

- 8 Random access procedure … Prior to initiation of the physical random access procedure, Layer 1 may receive from higher layers an indication to perform a random access procedure using: - first PRACH occasions, including the PRACH occasions that are not defined as second PRACH occasions, or - second PRACH occasions, that are in RBs that are both in the active UL BWP and in the UL sub-band, and associated either only with SBFD symbols that include at least one SBFD symbol indicated as downlink by tdd-UL-DL-ConfigurationCommon when the UE is provided either sbfd-RACHSingleConfig or sbfd-RACHDualConfig, or start from an SBFD symbol and end in a non-SBFD symbols when the UE is provided sbfd-RACHDualConfig and sbfd-RACHDualConfig-ValidROAcrossSymbolTypes The procedures in clause 8.1 apply separately for the first PRACH occasions and the second PRACH occasions. Only a Type-1 random access procedure is applicable for the second PRACH occasions.

So a LS is needed to update the definition of first PRACH occasions.

A draft LS wording can be as following:

LS Content: RAN2 finds inconsistency between MAC spec and PHY spec about the first PRACH occasions. To be specific, TS38.321 specifies if a RO is not a second PRACH occasion, UE should assume it as a first PRACH occasion. However TS38.213 defines first PRACH occasions to be on top of tdd-UL-DL-ConfigurationCommon. So for a carrier where tdd-UL-DL-ConfigurationCommon does not apply (such as SUL or FDD carrier), such carrier will not have a ‘first PRACH occasion’ according to current 38.213. Therefore, RAN2 kindly requests RAN1 to update the definition of first PRACH occasions to cover the case where the tdd-UL-DL-ConfigurationCommon does not apply, for example, to update as: ‘first PRACH occasions, including the PRACH occasions that are not defined as second PRACH occasions, or’. LS Action: RAN2 kindly requests RAN1 to update the definition of first PRACH occasions in TS38.213 to cover the case where the tdd-UL-DL-ConfigurationCommon does not apply on a carrier.

Proposal 2: For MAC-2, RAN2 sends LS to RAN1 with the following action:’RAN2 kindly requests RAN1 to update the definition of first PRACH occasions in TS38.213 to cover the case where the tdd-UL-DL-ConfigurationCommon does not apply on a carrier.’

- Conclusion

In this contribution, we propose the following observations and proposal:

Observation 1: It is not reasonable to let UE directly ignore the Msg1 repetition number increase after RO type switch is performed, since RO type switch includes both directions and the reason for such switch is not totally the same as the reason for Msg1 repetition number fallback.

Observation 2: It is reasonable to further increase the Msg1 repetition number if UE does not re-select a set with higher Msg1 repetition number during RO type switch. However, this brings heavy MAC spec impact.

Proposal 1: For MAC-1, support to do nothing on current MAC spec, i.e., for each specific counter value, let UE check both RO type switch and Msg1 repetition number fallback criteria in sequence.

Proposal 2: For MAC-2, RAN2 sends LS to RAN1 with the following action:’RAN2 kindly requests RAN1 to update the definition of first PRACH occasions in TS38.213 to cover the case where the tdd-UL-DL-ConfigurationCommon does not apply on a carrier.’

- Reference

- R2-2508163, Summary of Rel-19 SBFD MAC open issues for maintenance, Samsung, 2025-11