3GPP TSG-RAN WG2 Meeting #132	R2-2508182

Dallas, USA,  Nov. 17th - 21st

Agenda item:	9.3.3

Source:	LG Electronics Inc.

Title:	Discussion on LCM for UE-side RRM measurement event prediction

Document for:	Discussion and Decision

# 1. Introduction

During R19 AIML SI for NR mobility, there were three use cases: RRM measurement prediction, RRM measurement event prediction and RLF prediction. The former two uses cases were evaluated through simulations.

In this contribution, we discuss the LCM and inference configuration/reporting for UE-sided model for RRM measurement event prediction.

# 2. Discussion

## Functionality and Applicability related reporting

There are two methods to predict measurement event, namely indirect and direct measurement event prediction. The direct measurement event is predicted directly by an AI/ML model, i.e. the output of the model is the likelihood of an event occurrence. This performance metric is less straightforward than the RSRP difference in the indirect event prediction. In WID [4], the feasibility study of core requirement specification for direct event prediction is expected in Q2 2026. Based on this consideration, the indirect case should be the primary focus for work item.

Proposal1: For RRM measurement event prediction, RAN2 to prioritize indirect event prediction discussion over direct event prediction

In the study phase for AI/ML mobility, RRM measurement prediction function receives legacy cell measurement signals as input and utilizes an AI/ML model to predict future signal conditions as output. In RRM measurement event detection, the indirect case refers to the use of the predicted output from the RRM measurement prediction function as input to the existing 5G NR event detection framework, thereby enabling the detection of future handover events. RRM measurement event prediction can be easily made by RRM measurement prediction. This means these two functionalities are closely related in the operation of AI/ML-based handover.

Observation2: The indirect event prediction can be derived from the RRM measurement prediction.

Based on the discussion outcome of RRM measurement prediction in the perspective of LCM, the LCM aspects for RRM measurement event prediction can be discussed more easily. For example, the applicability report process is expected to be very similar to RRM measurement prediction.

Proposal2: RAN2 to postpone the discussion on functionality and applicability report for indirect RRM event prediction until the completion of discussion on functionality and applicability report for RRM measurement prediction discussion

## Inference configuration / reporting

In the agreements in the study item phase, the inference configuration for RRM measurement event prediction include the following elements: The predefined length of the processing window (PW) associated with the AI/ML model used for prediction, event-related information, including the event type and relevant event-specific parameters.

In addition, it is agreed that for the RRM measurement event prediction, the event prediction report may contain RRM measurements, time-related information about the predicted event (e.g., the time/window of occurrence), RRM measurement data, which will be defined during this work item phase to specify the exact types of measurements to be included, Time-related information regarding the predicted event, such as the expected time of occurrence or the occurrence window. For the measurement event reporting procedure, it is noted that when a measurement event is determined, and the UE is configured with temporal domain case B along with frequency domain prediction, the UE shall report the measurement event by following an approach similar to the legacy procedure.

For indirect event prediction, the measurement-related configuration will mostly follow that of RRM measurement prediction. However, the key difference lies in when and how the measurement report is sent. To address this, the meaning of event prediction needs to be clarified, and it may vary depending on the type of prediction domain.

Regarding temporal domain prediction, if future measurement results can be predicted, the event can be predicted for a future time. Depending on how the event prediction actually operates, several approaches to realizing event prediction can be considered.

RAN2 can assume two different methods of event prediction for consideration:

- Method 1. T-ahead prediction: UE at time t predicts RRM measurement results for t+T, and evaluates the Ax event for time t+T based on the predicted RRM measurement results. T can be configurable by the NW

- Method 2. Event satisfaction time prediction: UE at time t predicts when the event is expected to be met during a time duration, e.g., (t, t+T)

Figure 1. Method 1: UE at t predicts whether cell n satisfies event A4 at t+T. In this figure, UE ‘s prediction yields that cell n satisfies event A4 at t+T.

Figure 2. Method 2: UE at t predicts that cell n satisfies event A4 at t+T*.

In the case of Method 1, event prediction for event Ax is executed for a T-ahead time with respect to the current time. This method is useful when the network needs to know the satisfaction of the Ax event at least T time earlier. Upon receiving T-ahead measurement reporting, the network may perform proactive handover preparations to enhance mobility performance. In this sense, the network may need to determine the proper value of T.

In the case of Method 2, event prediction for event Ax is repeated for a time period from t to t+T, rather than for a specific future time t+T. This method can be viewed as multiple operations of the T’-ahead prediction method, with T’ increasing from 0 to T. Since the UE provides collective prediction results on event Ax for the upcoming time period, the network can obtain a more comprehensive prediction of event Ax for that period, which facilitates finer control of mobility.

Proposal3: For RRM event prediction with temporal domain type, RAN2 to discuss the definition and implications of RRM event prediction with T-ahead:

- Method 1. T-ahead prediction: UE at time t predicts RRM measurement results for t+T, and evaluates the Ax event for time t+T based on the predicted RRM measurement results. T can be configurable by the NW

- Method 2. Event satisfaction time prediction: UE at time t predicts when the event is expected to be met during a time duration, e.g., (t, t+T)

In the operational model for RRM measurement event prediction, when the UE successfully predicts a handover, it reports this prediction to the network, enabling the network to initiate preparation for the target cell handover. If it sends incorrect predictive report, network may make a mobility decision with the erroneous information so that it can significantly degrade system performance. This issue may occur due to degradation in model performance, or degradation in measurement data accuracy, e.g. due to noise and interference around the UE. To solve this issue, it is considerable for UE to determine the event based on AIML outputs which are of sufficient level of confidence. Confidence is defined as the probability that the inference output value is correct and AI/ML models can output confidence as well as inference output for a dataset. However, the AI/ML model predicted class probabilities do not faithfully estimate the true correctness likelihood and often lead to overconfident and underconfident predictions. Thus, the reliability of the confidence needs to be taken into account as well.

Proposal4: RAN2 to discuss how to ensure sufficient level of reliability on the predicted measurement event.

# 3. Conclusion

Proposal1: For RRM measurement event prediction, RAN2 to prioritize indirect event prediction discussion over direct event prediction

Proposal2: RAN2 to postpone the discussion on functionality and applicability report for indirect RRM event prediction until the completion of discussion on functionality and applicability report for RRM measurement prediction discussion

Proposal3: For RRM event prediction with temporal domain type, RAN2 to discuss the definition and implications of RRM event prediction with T-ahead:

- Method 1. T-ahead prediction: UE at time t predicts RRM measurement results for t+T, and evaluates the Ax event for time t+T based on the predicted RRM measurement results. T can be configurable by the NW

- Method 2. Event satisfaction time prediction: UE at time t predicts when the event is expected to be met during a time duration, e.g., (t, t+T)

Proposal4: RAN2 to discuss how to ensure sufficient level of reliability on the predicted measurement event.

# 4. References

[1] 3GPP TR 38.744 V19.0.0 (2025-09) Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR;

[2] R2-2506702, Report of 3GPP TSG RAN WG2 meeting #131 Bengaluru, India

[3] R2-2508181, Discussion on LCM for UE-sided RRM prediction

[4] RP-252899 Revision of WID on AI mobility