3GPP TSG-RAN WG2 Meeting #132                                    R2-2508188

Dallas, USA, 17th – 21st November, 2025

Source:	ZTE Corporation, Sanechips

Title:	RAN2 impacts for supporting voice over NB-IoT NTN

Agenda Item:	9.7.2

Document for:	Discussion and Decision

Introduction

In RAN#108, a Rel-20 new WID [1, RP-251867] for IoT over NTN Phase 4 enhancements has been agreed. It has been further revised in RAN#109[2, RP-252473]. The main objective of this WID is to support IMS voice call over GSO case of NB-IoT over NTN, as following:

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

For supporting voice over NB-IoT NTN, SA2 has already had much study and several solutions have been proposed, which can be roughly categorized into two types: one based on Control Plane CIoT EPS Optimization (CP solution), and the other based on User Plane CIoT EPS Optimization (UP solution).

In previous SA2#171 meeting, SA2 has given the following interim conclusions for Key Issue #1 “Support of IMS voice call over NB-IoT NTN via GEO satellite connecting to EPC”:

- The voice packets shall be transported over the NB-IoT (GEO) user plane, i.e. using DRB and S1-U.

Also RAN2#131bis meeting has given the following preference on using UP solution and inform this to SA2 and the relevant WGs via LS [R2-2507784]:

- We add a sentence in the same reply LS saying that majority of companies in RAN2 prefer the UP over CP solution (because of the higher impacts on RAN2 specs of the CP solution). RAN2 also understands that the final decision will be taken at the checkpoint in December plenary.

In this paper, we will initially discuss some RAN2 relevant issues for supporting IMS-based voice service via UP solution for NB-IoT over GSO, and give our observations and proposals.

- Discussion

  - Support for RoHC protocol with UP solution

Based on the common preference from SA2 and RAN2 on using UP solution, the subsequent related issue is whether ROHC protocol can be applied.

    - Discussion on header-to-payload ratio for voice data packet

The ROHC scheme has already been supported in legacy NB-IoT. For UP solution, ROHC is typically implemented between the UE and the eNB as part of the PDCP layer, where header compression is performed before data is transmitted over the radio interface. For CP solution, ROHC is typically implemented between the UE and the MME as part of the NAS layer. ROHC is a key enabling technology for NB-IoT, making IP-based communication feasible and efficient for massive-scale, low-power IoT deployments by minimizing protocol overhead and optimizing radio resource utilization.

Currently legacy NB-IoT only supports header compression for at most two protocol layers involving IP protocol. It does not support header compression for protocol stacks involving RTP protocol as no support for real-time service such as IMS-based voice service, in legacy NB-IoT system.

In LTE/5G, IMS voice service is traditionally carried over user-plane data transmission channels. SIP (Session Initiation Protocol) signaling for establishing the end-to-end voice call tunnel are carried over IP protocol using transport protocols such as UDP or TCP. Meanwhile, voice data packets are encapsulated with RTP, UDP (or TCP), and IP protocol headers sequentially to ensure timing synchronization, lightweight transport-layer communication, reliable delivery and network-layer addressing and routing.

In the SA2 study, several solutions are proposed for supporting voice service in GEO. Among them, Solution#2 seems closest to the traditional implementation of IMS-based voice services. The protocol stack for IMS voice control signaling (SIP) and voice data packets mentioned in Solution #2 are cited as below for reference:

TR 23.700-19 V1.0.0, Solution#2: …………….The PDN connection is based on IP type, therefore, the protocol stack could be as follows: Figure 6.2.2-3: Protocol stack for IMS signalling Figure 6.2.2-4: Protocol stack for IMS voice data

The multi-protocol layer encapsulation of voice packets generally bring the following protocol header overhead:

- RTP Header: 12 bytes

- UDP Header: 8 bytes

- IP Header: 20 bytes for IPv4 or 40 bytes for IPv6

So the total overhead is about: 12 (RTP) + 8 (UDP) + 20 (IPv4) = 40 bytes or 12 (RTP) + 8 (UDP) + 40 (IPv6) = 60 bytes

For IMS-based voice service, voice data packets exhibit characteristics different from those of regular data packets for internet browsing, e.g., small packet size typically. In some typical examples, for 20 ms voice, 160 bytes would be resulted from G.711 audio codec scheme, or about 60 bytes from AMR-WB scheme, or about 30 bytes from AMR-NB scheme. It can be seen that, with relative small raw voice data, after encapsulation with full protocol headers, the total packet size becomes kind of large—often resulting in a high header-to-payload ratio.

Observation 1a: In legacy LTE/5G, IMS voice service is traditionally carried over user-plane data transmission channels. SIP signaling are carried over IP protocol using transport protocols such as UDP or TCP. Meanwhile, voice data packets are encapsulated with RTP, UDP (or TCP), and IP protocol headers sequentially. With relative small raw voice data, after encapsulation with full protocol headers, the total packet size becomes kind of large—often resulting in a high header-to-payload ratio.

Furthermore, in a scenario with large RTT and high Doppler shift like NB-IoT over GEO, smaller voice packet with raw voice data from low bit rate codec may be more expected.

In general, low bit rate codec can represent a segment of speech signal with less data, thereby reducing the bandwidth required for transmitting voice packets. This allows more concurrent calls to be supported within the same bandwidth, thus increasing system capacity. Additionally, low-rate voice signals can have stronger noise resistance, enabling longer coverage range. Therefore, low bit rate codec is suitable for transmission scenarios like NB-IoT over GEO. That’s one of the motivation for SA4 to study Ultra Low Bitrate Speech Codec for supporting voice over NTN.

However, SA4 has not made much progress on this topic, so we roughly estimate the possible packet size based on the target codec bite rate range mentioned in the WID description in below:

RP-252473 Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things(IoT) Phase 4 ………………………………………… SA1 has studied the use case of IMS voice call using GSO satellite access, and the results are documented in TR 22.887. Normative service requirements and KPIs on IMS voice call using GSO satellite access is captured in TS 22.261, as shown in Table 1 below. Table 1: Performance requirements for satellite access Scenario; Experienced data rate (DL); Experienced data rate (UL); Area traffic capacity (DL) (note 1); Area traffic capacity (UL) (note 1); Overall user density; Activity factor; UE speed; UE type; Others. IMS voice call using GEO (note 7); [1-3] kbit/s; [1-3] kbit/s; N/A; N/A; N/A; N/A; TBD; Handheld; Call set up time (note 8) ≤30 s (note 9). Note 7: All the values defined for IMS voice call using GEO satellite access are generic for both regular and emergency calls. Note 8: call set up time refers to [56]; Note 9: 30s is the upper bound that is derived based on the user’s patience suggestions (30s) in [57];. SA has approved a new SA4-led SID on Ultra Low Bitrate Speech Codec in SP-250378. According to the agreed contribution in SA4 in S4-251123, the PHY bite rate is set to 1~5kbps and codec bite rate is set to 0.8~4.8kbps. …………………

The size of the voice data packet to be transmitted is not only influenced by the codec bit rate, but also by the sampling interval length, or called frame duration. For a certain codec bit rate, the frame duration has various impacts:

- Generally, a shorter frame duration will result in lower voice latency, making it more suitable for real-time interaction service (e.g., voice calls). But it’s noted that, when the frame duration is short, it leads to a higher frequency of encoding/decoding per unit time, increasing the computational complexity on the UE. A shorter frame duration also generates smaller packets, and more packets during a certain time period. After adding packet headers, e.g., the protocol headers from RTP/IP/UDP, the smaller packets will result in a higher protocol overhead ratio.

- In contrast, a longer frame duration can mitigates the issues in shorter frame duration, e.g., high computational complexity and high protocol overhead ratio. A longer frame duration can leverage more context information, achieving higher compression ratios. Moreover, a longer frame duration would result in more bits per packet, and further result in larger packet size and less packets during a certain time period. So fewer transmitted packets might be helpful for reducing congestion. However, under a longer frame duration, the impact of losing a single packet is relatively greater. Also a longer frame duration will increases end-to-end delay.

Based on above comparison, we understand a longer frame duration is more suitable to NB-IoT over GSO.

The frame duration for traditional voice service is typically 20ms. Currently it is observed that in SA4 discussion, higher frame duration are used, such as 80ms, 120ms, 160ms, 240 ms etc. We select some examples within this range for subsequent estimations.

Based on the assumption on target codec bite rates 0.8~4.8kbps and possible frame durations, we roughly estimate example voice packet sizes to be supported by NB-IoT over GEO as in the following Table-1.

For the convenience of subsequent discussions, we also provide here the transmission duration of a certain voice data packet under certain assumptions on system transmission data rate. Considering that in GEO scenarios, regular Internet browsing services may reach transmission rates around 10kbps (and may reach tens of kbps if HARQ feedback is disabled), we do not choose the extremely conservative transmission data rate, e.g., 1kbps, but instead use 2kbps.

Table 1.

| Codec bitrates | Frame Duration (Sampling Interval) | Number of voice frames per second | Size of a voice packet (=Codec bitrates * Frame Duration ) | Transmission duration of a voce packet with 2kbps transmission data rate |
| --- | --- | --- | --- | --- |
| 0.8 kbps | 80ms | 12.5 frames/s | 76 bits | 38ms |
|  | 160ms | 6.25 frames/s | 152 bits | 76ms |
| 2.5 kbps | 80ms | 12.5 frames/s | 200 bits | 100ms |
|  | 160ms | 6.25 frames/s | 400 bits | 200ms |
| 3.4 kbps | 80ms | 12.5 frames/s | 272 bits | 136ms |
|  | 160ms | 6.25 frames/s | 544 bits | 272ms |
| 4.8 kbps | 80ms | 12.5 frames/s | 384 bits | 192ms |
|  | 160ms | 6.25 frames/s | 768 bits | 384ms |

Based on the above estimation, we have the following observation:

Observation 1b: For IMS voice service, the size of the voice data packet (voice frame) to be transmitted is primarily influenced by the codec bit rate and the frame duration. With the target codec bite rate set to 0.8~4.8kbps for NB-IoT over GEO WID and also with the example frame duration of 80ms or 160ms, the estimated example sizes of voice data packet in NB-IoT over GEO are given. It can be seen that the range of size of voice frame is about 10 bytes to 100 bytes, which is similar as the raw voice packet size in legacy LTE/5G.

Therefore, similar as that in legacy LTE/5G, the ROHC protocol is conventionally necessary to compress the headers of voice packets, reducing header-to-payload ratio, as well as the signaling load transmitted between the UE and the network.

Observation 1c: The size of raw voice data that needs to be supported in NB-IoT over GEO may be similar as that in LTE/5G, so the ROHC protocol is conventionally necessary to compress the protocol headers overhead of voice data packets, with main intention to reduce header-to-payload ratio, as well as the signaling load transmitted between the UE and the network.

    - Feasibility and advantages for applying ROHC with UP solution

ROHC (Robust Header Compression) can reduce the overhead of IP/UDP/RTP headers by leveraging redundancy and predictability in header fields across packets. It operates in different modes and uses context-based compression to transmit only the changing parts of headers in most of time—significantly reducing the protocol headers overhead and improving bandwidth efficiency—especially beneficial for real-time traffic like voice.

Firstly, the basic ROHC operation modes (U-mode, O-mode and R-mode) and packet format used for each mode are summarized in Annex A.1. Among them, R-mode is most complicated, has the strictest requirements and less used, while the U-mode and O-mode are suitable options for a simplified systems such as NB-IoT.

Observation 2a: U-mode and O-mode of RoHC protocol are with remarkable compression ratio or compression efficiency where in full header compression state, the header overhead is 1 byte (U-mode) or 3 bytes (R-mode) respectively. So they are suitable options for a systems such as NB-IoT over GEO.

Secondly, in previous SA2 discussions, there were arguments that application of ROHC protocol in NB-IoT over GEO scenario is prone to unstable operation, e.g., it might be easy to back to initial IR state from FO/SO states after consecutive packets loss. However, according to the following RAN1 LS [R1-2508096] given in the latest RAN1#122bis meeting, we understand consecutive loss of multiple packets would be rare case in GEO scenario where line-of-sight condition has consensus to be considered. So it is unlikely for RoHC to easily fall back to its initial IR state.

As for the controversial case of line-of-sight not being guaranteed, some previous studies have indicated that the link budget could be very poor, leading to severe packet loss that may damage the entire communication link. Regardless of whether RoHC is used or not, any data transmission schemes may face failures. So merely emphasizing the RoHC state fallback issue in this scenario is meaningless.

R1-2508096 (RAN1#122bis): --------------------------------------------------- Question 2 (To RAN1): Can RAN1 provide any data regarding the probability that such number of consecutive packets e.g. 16 or 64 can be lost or erroneously decompressed? How often such event can occur? RAN1 reply: Assuming line-of-sight condition for the duration of the call, no change in large scale parameters (e.g. fixed shadowing), and using an NTN TDL-C channel model (which is a channel model previously used by RAN1 and SA4 evaluations), the probability of having 16 or 64 packets consecutively lost is negligible under typical operating conditions (e.g. up to 10% packet error rate) For the case of line-of-sight not being guaranteed for the duration of the call, an intermittent blockage (e.g. according to the land mobile satellite channel model in TR 38.811) may still occur in practice, in which case the probability of having 16 or 64 packets consecutively lost may not be negligible. RAN1 has not reached consensus on whether to consider this case.

Observation 2b: According to RAN1 discussion, the probability of having multiple packets consecutively lost is negligible in GEO scenario where line-of-sight condition has consensus to be considered. So it is very unlikely for RoHC to easily fall back to its initial IR state. For the controversial case of line-of-sight not being guaranteed, the link budget could be very poor, leading to severe packet loss that may damage the entire communication link. Regardless of whether RoHC is used or not, any data transmission schemes may face failures. So merely emphasizing the RoHC state fallback issue in this scenario is meaningless.

Thirdly, in the following, we provide some analysis to illustrate that even if occasional packet loss occurs, ROHC fallback will also rarely happen:

- Packet discard issue in IR state: referred to the observation in above, with 2kbps transmission rate for voice in NB-IoT over GEO, transmission duration for a full header packet with total about 500bits size (e.g., 200bits voice data+40bytes RTP/UDP/IP header) is about 250ms. During this period, if Frame Duration is not so small, only 1 or 2 full header packet may be generated and possibly be discard. This is small issue and easy to be addressed.

- Packet loss issue in FO/SO states: RoHC for NB-IoT over GEO can mainly work in U-mode which has numerous design features—periodic refresh, W-LSB, CRC checks and error detection etc.—collectively form an extremely robust fault-tolerant system. These mechanisms enable ROHC to maximize time spent in the efficient FO or SO states in field deployments. Shortly to say, with the "repair mechanisms" or "mitigation techniques" of U-mode or even O-mode, compression state fall back will seldom happened. The following is a brief description of some mechanisms to enhance robustness of ROHC.

  - Periodic Refresh: In U modes, the compressor periodically sends IR or IR-DYN headers. This actively "refreshes" the context in the absence of feedback, preventing context invalidation due to long idle periods or accumulated errors.

  - CRC Checks and Error Detection: In U modes, all compressed headers include a CRC (3-bit, 7-bit, or 8-bit). This allows the decompressor to detect the vast majority of header damage caused by channel bit errors. Damaged packets can be discarded without corrupting the context (preventing "damage propagation"). This protects context integrity and reduces invalid state transitions caused by erroneous packets.

Observation 2c: The "repair mechanisms" of RoHC, such as periodic refresh, CRC checks and error detection, would enable RoHC to perform "self-repair" when a few packet loss or a bit errors occur, and maximize time spent in the efficient FO or SO states. This also makes the RoHC mechanism to be applicable in challenging environments like NB-IoT over GEO (high latency, certain error rate).

Fourthly, there is concern that, due to ROHC state transitions or SID frames during voice call, the SPS scheduling method or dynamic scheduling (which have been agreed to be further studies in RAN2) based on maximum TBS, commonly used for voice data packets, may suffer from over- or under-scheduling issues. We give the following observation for this concern:

Observation 2d: Seldom RoHC state transition enables the transmission of voice packets with compressed header and of consistent size for most of the time, which is beneficial for efficient resource scheduling. Moreover, a bit longer Frame Duration can also be helpful to reduce the number of SID frames, resulting in more consistent voice data packets during the whole call.

Last, in legacy NB-IoT, it not only supports RoHC mechanism, but also support the storage of the ROHC context and its resumption in the next connection. The related spec description is cited as below:

### 5.3.12 UE actions upon leaving RRC_CONNECTED or RRC_INACTIVE Upon leaving RRC_CONNECTED or RRC_INACTIVE, the UE shall: …………………… 1> if leaving RRC_CONNECTED was triggered by suspension of the RRC: …………………… 2> store the UE AS Context including the current RRC configuration, the current security context, the PDCP state including ROHC state, C-RNTI used in the source PCell, the cellIdentity and the physical cell identity of the source PCell, and the spCellConfigCommon within ReconfigurationWithSync of the PSCell (if configured); 2> store the following information provided by E-UTRAN: 3> if the UE connected to 5GC is a BL UE or UE in CE: 4> the fullI-RNTI, if present; 4> the shortI-RNTI, if present; 3> else: 4> the resumeIdentity; 3> the nextHopChainingCount, if present. Otherwise discard any stored nextHopChainingCount that does not correspond to stored key KRRCint; 3> the drb-ContinueROHC, if present. Otherwise discard any stored drb-ContinueROHC; ### 5.3.3.3a Actions related to transmission of RRCConnectionResumeRequest message …………………… 1> if the UE is resuming an RRC connection after early security reactivation in accordance with conditions in 5.3.3.18: 2> if the UE is initiating UP-EDT in accordance with conditions in 5.3.3.1b; or 2> if the UE is initiating UP transmission using PUR in accordance with conditions in 5.3.3.1c: 3> restore the PDCP state and re-establish PDCP entities for all SRBs and all DRBs; 3> if drb-ContinueROHC has been provided in immediately preceding RRC connection release message, and the UE is requesting to resume RRC connection in the same cell: 4> indicate to lower layers that stored UE AS context is used and that drb-ContinueROHC is configured; 4> continue the header compression protocol context for the DRBs configured with the header compression protocol; 3> else: 4> indicate to lower layers that stored UE AS context is used; 4> reset the header compression protocol context for the DRBs configured with the header compression protocol; 3> resume all SRBs and all DRBs; ……………………

Such mechanism allows RoHC to directly operate in its last stable compression state even for a new voice call connection, thereby reducing or even eliminating the delay associated with RoHC context setup, e.g., the transitioning time from the initial IR state to the stable FO/SO compression states.

Observation 2e: NB-IoT also supports the storage of the ROHC context and its resumption in the next connection. Such mechanism allows RoHC to directly operate in its last stable compression state even for a new voice call connection, thereby reducing or even eliminating the delay associated with RoHC context setup or initial state transition, with further benefit of reducing the voice call setup latency.

    - Concerns for using Non-IP data delivery scheme

In SA2 study, some solution propose to use Non-IP‑based voice packet transmission mechanisms.

Unlike supporting normal data service in legacy NB-IoT, where Non-IP data delivery is mainly end-to-end and used in private network environments to carry very simple industrial application data, supporting IMS voice requires enabling service routing across wide coverage areas. So if without UDP/IP protocol header encapsulation, it not only would cause incompatibility with existing network nodes, such as firewalls, NAT (Network Address Translation), load balancers, and QoS systems as they inherently rely on UDP/TCP/IP protocols at the network layer, but also cause some issues as application conflicts caused by lack of port mechanism.

Therefore, if it supports to transmit voice data via Non-IP data delivery scheme, UDP/IP protocol headers cannot be removed end-to-end and can only be omitted over the air interface. These protocol headers still need to be reconstructed at the core network and the UE, for end-to-end data delivery. This introduces additional complexity.

Observation 3a: For supporting wide range voice service in NB-IoT over GSO, UDP/IP protocol cannot be end-to-end removal as it may cause incompatibility with existing network nodes. To reconstruct UDP/IP protocol header at the CN and the UE would introduce additional complexity.

Regarding RTP protocol header, most SA2 solutions agree that it is still required end-to-end inclusion, but some solutions suggest applying a simplified version of the RTP protocol (e.g., only include timestamp information). However, based on our understanding, anyway RTP Fixed Header Fields may need to be included. RTP header extension (such RTP header extension may be able to be defined in our 3GPP scope, e.g., in SA4) can only be done by making use of the extension (X) bit in the existing RTP Fixed Header Fields. So we are highly doubt whether it is really possible to develop a simplified/reduced RTP protocol that can achieve the purpose of reducing RTP protocol header overhead, e.g., from 12 bytes to 1 or 2 bytes.

Moreover, a new simplified/reduced RTP protocol header may also cause inability to leverage standard security mechanisms like DTLS as it depends on the UDP/IP structure.

Finally, current RTP protocol can enable multipoint communication and mixing (e.g., in an audio conference bridge). So to apply a simplified/reduced RTP protocol header, e.g., without such information as SSRC, the receiver cannot separate or manage multiple audio sources in a conference or mixed scenario. This will block the future expansion of voice application in NB-IoT over GSO.

Observation 3b: It’s unclear how to develop a simplified/reduced RTP protocol (e.g., only include timestamp information) as it can only do RTP protocol extension by making use of the extension (X) bit in the existing RTP Fixed Header Fields. Moreover, to apply simplified/reduced RTP protocol may cause inability to leverage standard security mechanisms like DTLS, and also cause inability to future functional expansion of voice application in NB-IoT over GEO, e.g., to support multipoint communication in the future.

With so many unknowns, we cannot even image a picture of the so-called new RTP protocol and also the Non-IP based solution. It’s also unclear where or which working group can do the relative specification work, how long time such work will be needed and what’s kind of impacts will be expected on RAN2. So we think it may be useful to send a LS to SA4 and SA2 to check any possibility of a new RTP protocol and also the Non-IP based solution.

Observation 3c: With so many unknowns on the so-called Non-IP based solution, before any real technical discussion, it may be useful to send a LS to SA4 and SA2 to check any possibility of a new RTP protocol and also the Non-IP based solution.

Based on observations 2a~2e for the feasibility and advantages of applying RoHC protocol and also observations 3a~3c for the concerns for Non-IP based solution, we ensure that applying ROHC with UP solution should be unquestionable.

Proposal 1: RTP/UDP/IP with Robust Header Compression (RoHC) shall be supported for transporting voice data packets via UP solution for NB-IoT over GEO scenario.

  - Support of Semi-Persistent Scheduling function

For voice service—which generally exhibits a periodic nature and with constant packet sizes—Semi-Persistent Scheduling (SPS) is an efficient air interface resources scheduling method. This is also one of the reasons why this aspect has been mentioned as an objective in WID.

However, for legacy NB-IoT, only UL Semi-Persistent Scheduling function is supported and UL SPS configuration is only used for BSR provision. So the functionality and usage of SPS in legacy NB-IoT are very limited.

With reference to SPS function in legacy LTE/5G, if companies will have common thinking that SPS needs to be supported for voice service in NB-IoT over GEO, at least it needs to support SPS function for general user plane data transmission, e.g., DRB transmission. Moreover, not only UL, the DL SPS also needs to be supported.

Observation 4a: As legacy NB-IoT only supports UL Semi-Persistent Scheduling function for sending BSR, if there can be consensus that SPS needs to be introduced for voice service support in NB-IoT over GEO, at least it needs to support SPS function for general user plane data transmission, e.g., DRB transmission. Moreover, not only UL, the DL SPS also needs to be supported.

Furthermore, for SPS in legacy LTE/5G, there are some relevant sub-features for SPS as below:

- Feature according to the configured skipUplinkTxSPS (the UE skips UL transmissions for a configured uplink grant if no data is available for transmission in the UE buffer);

- Feature according to the configured implicitReleaseAfter (Number of empty transmissions before implicit release SPS)

If full SPS function needs to be introduced, the above sub-features may also need to be considered.

Observation 4b: If full SPS function needs to be introduced, some sub-features, e.g., the ones according to the configured skipUplinkTxSPS or implicitReleaseAfter, may also need to be considered.

In legacy NB-IoT, the operation of SPS function is that, the periodicity of UL SPS and the SPS C-RNTI are configured via RRC signaling. Meanwhile, the activation and deactivation/release of SPS resources are indicated by DCI scrambled with the SPS C-RNTI, monitored in the UE-specific search space. The activation of SPS is also along with SPS resources grant.

It can be seen that, although the SPS feature offers the benefit of reducing PDCCH scheduling overhead, it still requires certain signaling exchanges. This is especially relevant when the size of the service data packets changes—a scenario that can occur during a voice call. For example, during silence periods, the UE may stop sending data packets for voice speech and instead transmit silence descriptor (SID) frames, which typically have different sizes from the voice data packets. Such transitions may necessitate reconfiguration, and also release and re-activation of SPS resources.

The following alternatives can be initially considered to addressing the above potential issues:

- Alt1: It’s an enhancement to traditional SPS, e.g., pre-configuration of two SPS resources which may have different periodicities etc. Then the data packets size change may be addressed by enabling rapid switching between two SPS configurations.

  - However, in NB-IoT over GEO system, due to the extremely narrow bandwidth and the fact that DL scheduling for each UE occupies the entire frequency-domain resources, when multiple UEs share the same PRB, it may be difficult to grant radio resources when activating another SPS for a specific UE. Moreover, even if resources can be granted, after the UE receives the DCI for SPS activation, it typically still needs to wait for a timing gap before it can actually start the next PUSCH or PDSCH transmission corresponding to the new SPS—all these things will make such SPS switch less efficient than expected.

- Alt2: It’s also worth considering a variant of traditional dynamic scheduling, e.g., a dynamic scheduling way based on an assumption of fixed data packet size, which may also reduce or even eliminate the need for traditional PDCCH scheduling. Compared to above conventional SPS, this optimized dynamic scheduling scheme may offer better flexibility in adapting to variations in packet size.

  - However, this way may lead to either wasted resources (if a maximum fixed packet size is assumed) or insufficient scheduling resources (if a minimum fixed packet size is assumed).

Proposal 2: Before introducing full SPS function for supporting voice service in NB-IoT over GEO, it’s suggested RAN2 to firstly evaluate and compare the following two alternatives—with consideration on the narrow bandwidth, scheduling restrictions and large RTT in such system as NB-IoT over GEO:

- Alt1: One or two SPS configurations—which may offer advantages in reducing more scheduling overhead but with difficulty on SPS configuration switch;

- Alt2: Conventional or enhanced dynamic scheduling way—which may provide better scheduling flexibility but with possible wasted resources or insufficient resources.

  - Support for more than two DRBs

After introducing the support of voice service for NB-IoT over GEO, as more services can be used by the UE, there may be several possible scenarios on how the UE performs. For example:

- When UE powers on and performs an attach procedure, it may firstly only establish a default bearer to the "internet" APN to facilitate subsequent transmission of regular Internet data—without establishing a default bearer to the "ims" APN as UE may assume voice call is rare case and can be triggered at late stage;

- When UE powers on and performs an attach procedure, it may be also possible to firstly establish a default bearer to the "ims" APN and complete IMS registration—without establishing a default bearer to the "internet" APN. Such process can facilitate UE receive/trigger voice call at any time. After IMS registration, the UE may release or suspend the DRB (DRB for SIP signaling), while still maintaining its registration status at the IMS server;

- When UE powers on and performs an attach procedure, UE may want to establish default bearers to both the "internet" APN and "ims" APN—to facilitate both subsequent transmission of regular Internet data and also at least perform SIP procedure when needed.

For all above registration scenarios, it seem the existing 2 DRBs would be sufficient. However, at least for the third scenario, there is still potential risk that no DRB can be used if UE also want to setup a bearer for voice data in advance.

Also according to the following typical IMS mobile call procedure mentioned in SA2 study, it can be seen that UE may need to establish two DRBs/EPS bearers, even before the voice call tunnel are completely setup, with the intention to reduce the voice call setup latency. For such case, if UE want to simultaneously setup a bearer for transmission of regular Internet data, it’s also no DRB to be used.

TR 23.700-19 V1.0.0 A.2 Example message size and corresponding transmission delay for SIP messages in IMS Figure A.2-1 shows the typical IMS mobile call processing in TN scenario: Figure A.2-1: IMS mobile call processing in TN scenario

- In a summary, based on the above roughly analysis for the possible service performing scenarios, we see the necessity to introduce at least one more DRB for UP solution.

- Proposal 3: It’s suggested to introduce at least one more DRB for UP solution.

  - Enhancements for UP solution for supporting voice service

For using UP solution to support IMS voice service for NB-IoT over GSO, there are still some minor enhancements that need to be considered.

    - To add RTP related ROHC profiles into PDCP configuration

For NB-IoT, if IP header compression for UE solution is supported, the ROHC profiles defined in 3GPP PDCP protocol, e.g., TS 36.323, need to be supported.

As shown below, ROHC has already been supported for UP solution in NB-IOT but we observe that some profiles which involve RTP protocol and applicable to voice service, e.g. 0x0001 and 0x0101, are missing in the ROHC configuration in PDCP configuration.

TS 36.323 Table 5.5.1.1: Supported ROHC protocols and profiles Profile Identifier; Usage:; Reference. 0x0000; No compression; RFC 5795. 0x0001; RTP/UDP/IP; RFC 3095, RFC 4815. 0x0002; UDP/IP; RFC 3095, RFC 4815. 0x0003; ESP/IP; RFC 3095, RFC 4815. 0x0004; IP; RFC 3843, RFC 4815. 0x0006; TCP/IP; RFC 6846. 0x0101; RTP/UDP/IP; RFC 5225. 0x0102; UDP/IP; RFC 5225. 0x0103; ESP/IP; RFC 5225. 0x0104; IP; RFC 5225. TS 36.331 The IE PDCP-Config-NB is used to set the configurable PDCP parameters for data radio bearers. PDCP-Config-NB information element -- ASN1START PDCP-Config-NB-r13 ::= SEQUENCE { discardTimer-r13 ENUMERATED { ms5120, ms10240, ms20480, ms40960, ms81920, infinity, spare2, spare1 } OPTIONAL, -- Cond Setup headerCompression-r13 CHOICE { notUsed NULL, rohc SEQUENCE { maxCID-r13 INTEGER (1..16383) DEFAULT 15, profiles-r13 SEQUENCE { profile0x0002 BOOLEAN, profile0x0003 BOOLEAN, profile0x0004 BOOLEAN, profile0x0006 BOOLEAN, profile0x0102 BOOLEAN, profile0x0103 BOOLEAN, profile0x0104 BOOLEAN }, ... } }, ..., [[ cipheringDisabled-r16 ENUMERATED {true} OPTIONAL -- Cond ConnectedTo5GC ]] } -- ASN1STOP PDCP-Config-NB field descriptions. ……………….. profiles The profiles used by both compressor and decompressor in both UE and E-UTRAN. The field indicates which of the ROHC profiles specified in TS 36.323 [8] are supported, i.e. value true indicates that the profile is supported. Profile 0x0000 shall always be supported when the use of ROHC is configured. If support of two ROHC profile identifiers with the same 8 LSB's is signalled, only the profile corresponding to the highest value shall be applied.. ----------------

According to above observation, the ROHC profiles 0x0001 and 0x0101 need to be included in the “headerCompression-> PDCP-Config-NB”.

- Proposal 4a: Support ROHC for voice over NB-IoT NTN and include RTP related ROHC profiles for PDCP configuration for UP solution.

    - New RRC establishment cause

Based on the following comparison between NB-IoT and LTE, it can be seen that the establishment cause for voice and emergency call is not supported in legacy NB-IOT and need to be addressed if we decide to support voice and emergency call.

| Establishment Cause for NB-IoT (Not including voice nor emergency) | Establishment Cause for LTE (Including voice and emergency) |
| --- | --- |
| EstablishmentCause-NB-r13 ::= ENUMERATED {mt-Access, mo-Signalling, mo-Data, mo-ExceptionData, delayTolerantAccess-v1330, mt-EDT-v1610, spare2, spare1} | EstablishmentCause ::= ENUMERATED {emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1} |

- Proposal 4b: Introduce establishment cause for voice call in EstablishmentCause-NB-r13.

- Conclusion

Based on the above discussion, the following observations are given for several aspects:

1.1 Support for RoHC protocol with UP solution

Observation 2a: U-mode and O-mode of RoHC protocol are with remarkable compression ratio or compression efficiency where in full header compression state, the header overhead is 1 byte (U-mode) or 3 bytes (R-mode) respectively. So they are suitable options for a systems such as NB-IoT over GEO.

Observation 2b: According to RAN1 discussion, the probability of having multiple packets consecutively lost is negligible in GEO scenario where line-of-sight condition has consensus to be considered. So it is very unlikely for RoHC to easily fall back to its initial IR state. For the controversial case of line-of-sight not being guaranteed, the link budget could be very poor, leading to severe packet loss that may damage the entire communication link. Regardless of whether RoHC is used or not, any data transmission schemes may face failures. So merely emphasizing the RoHC state fallback issue in this scenario is meaningless.

Observation 2c: The "repair mechanisms" of RoHC, such as periodic refresh, CRC checks and error detection, would enable RoHC to perform "self-repair" when a few packet loss or a bit errors occur, and maximize time spent in the efficient FO or SO states. This also makes the RoHC mechanism to be applicable in challenging environments like NB-IoT over GEO (high latency, certain error rate).

Observation 2d: Seldom RoHC state transition enables the transmission of voice packets with compressed header and of consistent size for most of the time, which is beneficial for efficient resource scheduling. Moreover, a bit longer Frame Duration can also be helpful to reduce the number of SID frames, resulting in more consistent voice data packets during the whole call.

Observation 2e: NB-IoT also supports the storage of the ROHC context and its resumption in the next connection. Such mechanism allows RoHC to directly operate in its last stable compression state even for a new voice call connection, thereby reducing or even eliminating the delay associated with RoHC context setup or initial state transition, with further benefit of reducing the voice call setup latency.

Observation 3a: For supporting wide range voice service in NB-IoT over GSO, UDP/IP protocol cannot be end-to-end removal as it may cause incompatibility with existing network nodes. To reconstruct UDP/IP protocol header at the CN and the UE would introduce additional complexity.

Observation 3b: It’s unclear how to develop a simplified/reduced RTP protocol (e.g., only include timestamp information) as it can only do RTP protocol extension by making use of the extension (X) bit in the existing RTP Fixed Header Fields. Moreover, to apply simplified/reduced RTP protocol may cause inability to leverage standard security mechanisms like DTLS, and also cause inability to future functional expansion of voice application in NB-IoT over GEO, e.g., to support multipoint communication in the future.

Observation 3c: With so many unknowns on the so-called Non-IP based solution, before any real technical discussion, it may be useful to send a LS to SA4 and SA2 to check any possibility of a new RTP protocol and also the Non-IP based solution.

1.2 Support of Semi-Persistent Scheduling function

Observation 4a: As legacy NB-IoT only supports UL Semi-Persistent Scheduling function for sending BSR, if there can be consensus that SPS needs to be introduced for voice service support in NB-IoT over GEO, at least it needs to support SPS function for general user plane data transmission, e.g., DRB transmission. Moreover, not only UL, the DL SPS also needs to be supported.

Observation 4b: If full SPS function needs to be introduced, some sub-features, e.g., the ones according to the configured skipUplinkTxSPS or implicitReleaseAfter, may also need to be considered.

Furthermore, the following proposals are given:

1.1 Support for RoHC protocol with UP solution

Proposal 1: RTP/UDP/IP with Robust Header Compression (RoHC) shall be supported for transporting voice data packets via UP solution for NB-IoT over GEO scenario.

1.2 Support of Semi-Persistent Scheduling function

Proposal 2: Before introducing full SPS function for supporting voice service in NB-IoT over GEO, it’s suggested RAN2 to firstly evaluate and compare the following two alternatives—with consideration on the narrow bandwidth, scheduling restrictions and large RTT in such system as NB-IoT over GEO:

- Alt1: One or two SPS configurations—which may offer advantages in reducing more scheduling overhead but with difficulty on SPS configuration switch;

- Alt2: Conventional or enhanced dynamic scheduling way—which may provide better scheduling flexibility but with possible wasted resources or insufficient resources.

1.3 Support for more than two DRBs

Proposal 3: It’s suggested to introduce at least one more DRB for UP solution.

1.4 Enhancements for UP solution for supporting voice service

Proposal 4a: Support ROHC for voice over NB-IoT NTN and include RTP related ROHC profiles for PDCP configuration for UP solution.

Proposal 4b: Introduce establishment cause for voice call in EstablishmentCause-NB-r13.

- Reference

[1] RP-251867, New WI: Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4, vivo (moderator), RAN#108, (2025-06)

[2] RP-252473, Revised WID: Non-Terrestrial Networks (NTN) for Internet of Things(IoT) Phase 4, vivo (rapporteur), RAN#109, (2025-09)

Annex A: Background information

Annex A.1: ROHC operation modes and packet formats (RFC 3095)

The characteristics of three ROHC operation modes and their compression logic are roughly summarized as below:

| Mode | Characteristics | Figure for details of each state, state transitions, and compression logic |
| --- | --- | --- |
| U-mode | In RoHC U-mode (Unidirectional Mode), there is no feedback channel from the decompressor to the compressor. U-mode relies on a cycle of highly compressed packets (UO-0, UO-1-ID, UO-1-TS) for efficiency, interspersed with periodic context refresh packets (IR, IR-DYN) for robustness. This periodic refresh compensates for the lack of feedback, ensuring the decompressor can eventually resynchronize even after packet loss, at the cost of some bandwidth. |  |
| O-mode | Unlike U-mode, O-mode utilizes a feedback channel from the decompressor (DC) to the compressor (C).The decompressor primarily uses NACK (Negative Acknowledgment). If it cannot decompress a packet (e.g., due to context damage from lost packets), it sends a NACK to the compressor. The compressor can use highly efficient, context-dependent compression formats (like UOR-2) optimistically, assuming the channel is good. When a NACK is received, it knows the context is damaged and responds by sending a refresh packet (like IR or IR-DYN) to re-synchronize. Advantages over U-mode: Context repair is triggered immediately by a NACK, rather than waiting for a periodic refresh. |  |
| R-mode | R-mode provides the highest level of reliability and control. The compressor only advances to higher compression states (using more efficient formats) after receiving an ACK confirming the decompressor has received the necessary context updates. This ensures both ends are always in sync before aggressive compression is used. R-mode offers the highest reliability and is excellent for very poor links. R-mode is typically used in scenarios where maximum reliability is paramount and the reverse link has sufficient capacity for ACKs |  |

The packet formats primarily used in these ROHC operation modes are as follows:

| Mode | Mainly used packet formats |
| --- | --- |
| Common | - Initialization and Refresh (IR) packet: is mainly used for establishing and refreshing the decompression context, which is used for all the modes. - IR packets without static information (IR-DYN): is mainly used to update the dynamic parts of the context without resending static information, which is also used for all the modes. - Type 0 packets (e.g., UO-0, or UOR-0), type 1 packets (e.g., UO-1) and type 2 packets (e.g., UOR-2) are all packets carrying a compressed packet header. - A type 0 packet carries a packet header compressed according to a fixed pattern - Type 1, 2 packets are used when this pattern is broken. |
| U-mode | IR (Initialization and Refresh): The most critical packet for establishing and refreshing the decompression context. It contains the full static (e.g., IP addresses, port numbers) and dynamic (e.g., RTP SN, TS) header information, the Context ID (CID), and the Profile. It’s sent at the start of a flow and periodically throughout the session to prevent permanent desynchronization. IR-DYN (Initialization Dynamic): It updates the dynamic parts of the context without resending static information. It includes CID, Profile, CRC, and updated dynamic fields (like RTP SN, TS, IP-ID). It’s sent when dynamic values change significantly, serving as a lighter-weight refresh than a full IR. UO-0 (Unidirectional Optimization 0): It’s a simple, highly compressed format for in-order packets. It typically contains only the CID, a small difference in the Sequence Number (SN) and CRC. It’s used for regular data packets when the context is synchronized and packets arrive sequentially. UO-1-ID (Unidirectional Optimization 1 with IP-ID): It’s a compressed format that also encodes the IP-ID field. It typically contains CID, SN (difference), IP-ID (difference), R-bit, and CRC. It’s used when the IP-ID field needs to be compressed (e.g., for fragmentation). UO-1-TS (Unidirectional Optimization 1 with Timestamp): It’s a compressed format optimized for predictable RTP timestamp increments (common in voice). It typically contains CID, SN (difference), TS (difference), R-bit, and CRC. It’s highly efficient for voice traffic, compressing headers to 2-4 bytes by leveraging the regular pattern of timestamp growth. |
| O-mode | O-mode uses a superset of U-mode packets but introduces more efficient formats that leverage the feedback channel: IR (Initialization and Refresh): It establishes the initial context or performs a full refresh when requested (e.g., after a NACK). It has full static and dynamic header information, CID, Profile. For O-mode, it’s sent in response to a NACK or during initial setup. Less frequent than in U-mode due to faster NACK-triggered recovery. IR-DYN (Initialization Dynamic): It updates dynamic fields in the context. It typically contains CID, Profile, CRC, and updated dynamic header fields. For O-mode, it’s sent to refresh dynamic information, often in response to a NACK or when dynamic values change significantly. UOR-0 (U-mode/O-mode/R-mode 0): It’s a basic compressed format for O-mode. It contains CID, SN (difference), R-bit, and CRC. It’s a simple compressed packet, similar to UO-0 but used in bidirectional modes. UOR-1-ID / UOR-1-TS (U-mode/O-mode/R-mode 1 with IP-ID/Timestamp): It’s more advanced compressed formats for O-mode, handling IP-ID or RTP Timestamp compression. It includes CID, SN (diff), IP-ID/TS (diff), R-bit, CRC. It’s efficient compression for specific scenarios, used when the context is synchronized. UOR-2 (U-mode/O-mode/R-mode 2) - It’s the most common and efficient compressed format used in O-mode, especially for real-time traffic like voice/video. It’s extremely compact (typically 2-4 bytes) and contains a compressed SN, R-bit, CRC, and often a "First Byte" field that can carry the first byte of the payload (e.g., the RTP payload type) or other critical information. - Its efficiency relies on the feedback loop. If a UOR-2 packet is lost or corrupted and breaks decompression, the DC sends a NACK, and the C immediately responds with an IR or IR-DYN to fix the context. |
| R-mode | Like O-mode, R-mode requires a feedback channel from the decompressor (DC) to the compressor (C). It uses both ACK and NACK. - ACK: The DC sends an ACK to confirm it has successfully received and processed a context-updating packet (like IR, IR-DYN, or a packet with a "larger-than-expected" SN jump). This tells the C that the context is synchronized. - NACK: The DC sends a NACK if it cannot decompress a packet (context damaged) R-mode uses the same fundamental packet formats as U-mode and O-mode, but their usage and state transitions are governed by ACKs. IR (Initialization and Refresh): It establishes the initial context. For R-mode, it’s sent initially. The compressor waits for an ACK from the DC before transitioning to a state where it can use compressed formats like UOR-2. IR-DYN (Initialization Dynamic): It updates dynamic fields in the context. For R-mode, it’s sent when dynamic fields (like RTP timestamp base) need updating. The compressor typically waits for an ACK before resuming use of the highest compression efficiency formats. UOR-0, UOR-1-ID, UOR-1-TS: They are various levels of compressed packet formats. For R-mode, it’s used based on the current compression state, which is managed by ACKs. UOR-2 (U-mode/O-mode/R-mode 2): It’s the primary, highly efficient compressed format (2-4 bytes). For R-mode, It’s used extensively, but the compressor only uses it (or uses it with full confidence) after receiving ACKs confirming the context is synchronized. A NACK will trigger an IR/IR-DYN, and the C will wait for an ACK before returning to UOR-2. |