3GPP TSG-RAN WG2 Meeting #132	R2-2508189

# Dallas, USA, November 17 – 21, 2025

Agenda item:	10.2.3

Source:	Samsung

Title:	6G device types and services

Document for:	Discussion

1	Introduction

This paper discusses the support for diverse device types and Day-1 services in 6G.

2	Discussion

2.1	Device types

Starting with 5G, a wide variety of device types have emerged, and the 5G system has defined and implemented functionalities specifically tailored to support the diversity for device types. With the evolution to 6G, an even broader range of device types is anticipated. In principle, 6GR study aims to develop a scalable and forward compatible 6G radio design capable of supporting a wide range of device types. This involves establishing a common 6G radio framework that accommodates various device capabilities, ranging from IoT devices to high-performance eMBB UEs such as FWA devices. Such a framework ensures that the network remains adaptable to evolving use cases without requiring significant hardware redesigns

RAN2 can study 6GR design focused on RRC configuration and signalling improvements, taking into account the diversity of device types. This includes exploring modular RRC designs that optimized for various device types or verticals in a scalable manner. In this paper, we aim to discuss how diverse device types can effectively work with UE capability signalling.

In LTE, UE categories are defined to specify maximum downlink and uplink data rates. To accommodate additional downlink and uplink data rates, further UE categories were introduced in LTE. Since LTE Release 12, specific categories for IoT devices, such as LTE Cat 0, LTE-M, and LTE Cat 1bis, have been defined. By LTE Release 18, a total of 26 downlink categories and 26 uplink categories have been specified. Building on lessons learned from LTE, 5G NR adopts a distinct approach by using an equation based on the number of aggregated carriers within a UE’s supported bands or band combinations to calculate maximum downlink and uplink data rates, rather than defining specific UE categories. Additionally, 5G NR introduced a feature set framework to represent a collection of features associated with a particular band. However, the concept of features and feature sets in 5G NR is insufficient to specify discrete IoT devices.

We think that a UE category-like concept could offer certain marketing advantages. For example, LTE UE categories indicate the downlink and uplink data rates in LTE network. Moreover, IoT device specific categories in LTE allow for the representation of IoT features through corresponding UE categories. However, the LTE UE category has limitations in fully representing the supported features for all device types.

Observation 1: The LTE UE category has limitations in accurately representing features for all the device types.

If a UE category-like concept is necessary for clear device type classification in the 6G system, it is essential to establish a product-oriented device classification by anchoring device requirements to real-world product groups rather than abstract use cases. This approach ensures that physical layer, signalling, and architectural designs remain practical and scalable. Table 1 presents an example of the classification of 6G device types. As shown in Table 1, multiple 6G device types can be defined with distinctive key parameters to identify each device type’s requirements or capabilities. Note that the device type in Table 1 is illustrative, and the concrete classification for 6G device types must be specified through collaboration with RAN, RAN1, and RAN4.

Table 1 6G device types

| Parameter list | Type IV (FWA) | Type III (Smartphone) | Type II (Wearable) | Type I (IoT) | NTN (if applicable) |
| --- | --- | --- | --- | --- | --- |
| TX antenna | 1-4Tx (depending on FRs) (Note: 2TX can be baseline) | 1Tx baseline (for all FRs), up to 2Tx (optional) | 1Tx | 1Tx | 1Tx to 4 Tx (depending on PC). |
| RX antenna | 4, 6 or 8 RX (optional) | Below 1 GHz: 2RX, up to 4 RX (optional) Around 3.5 GHz: 4 RX, Around 7 GHz: 4RX, up to 6 RX (optional) Around 24 GHz : 2 RX, up to 4 RX (optional) | 1Rx | 1Rx and up to 2RX (optional) | Depending on device type |
| MIMO layer | Up to 4, 6 or 8 Layer | Up to 2 or 4 layer (depending on FR) | 1 layer | 1 or 2 layer (optional) | 1 layer |
| RF BW (max) | 400 MHz | 200 or [400] MHz | 20 MHz | 20 MHz | Depending on device type |
| Maximum Power class | 29 dBm [or 31 dBm] | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | [Depending on outcome of 5G work] |
| DL max modulation order | 1024QAM or [4096QAM] | 1024QAM or [4096QAM] | 64 QAM | 256 QAM | Depending on device type |
| UL max modulation order | 256QAM or [1024QAM] | 256QAM or [1024QAM] | 64 QAM | 64 QAM | Depending on device type |
| - Type I: This type spans from cost-sensitive industrial modules to ultra-low-power modules for IoT services. These devices will be deployed in massive scale and must operate under extremely tight cost and energy budgets. 6GR should provide simplified PHY/MAC designs, scalable addressing and identification mechanisms, and highly efficient paging/wake-up signalling. - Type II: Type II is to support wearables (e.g., smartwatches). Wearable devices represent a distinct category with unique constraints in terms of battery size, smallest form factor, and connectivity requirements. Unlike smartphones, wearables typically operate with reduced complexity (e.g., limited bandwidth, fewer antennas) while requiring robust coverage and efficient signalling to support continuous operation under low power. Wearable devices such as smartwatches face unique RF challenges due to their small form factor, including limited number of antenna, constrained PA/filter design, and strict power/thermal budgets. These factors typically result in higher error vector magnitude (EVM) compared to smartphones or IoT devices. Consequently, while generic IoT devices can rely on similar requirements as normal UE, wearable devices require a separate classification in 6G with mid-level modulation capability (e.g., up to 64 QAM) but with more relaxed EVM requirements than smartphones. This distinction justifies treating wearables as a device type separate from IoT in 6GR. - Type III: Smartphones will continue to be the most widely adopted 6G device type, serving as the baseline for feature-rich capabilities. Smartphones must support high spectrum flexibility, multi-antenna operation, and advanced AI-assisted resource management while maintaining strict energy efficiency and form-factor constraints. However, in practical smartphone form factors, achieving flexible multi-band frequency support is fundamentally constrained since adding more antennas and transmit chains is extremely difficult. The compact device architecture must already accommodate displays, batteries, cameras, and processors, leaving very limited space for RF front-end components. Each additional antenna requires its own matching network, filters, and often dedicated power amplifiers or switches, which not only increases board area but also raises power consumption and thermal challenges. As a result, the physical and electrical limitations of smartphone design make it unrealistic to scale antenna counts and RF chains in the same way as larger form-factor devices such as laptops or FWA. - Type IV: This is to support high capable UE for fixed Wireless Access (FWA) service. FWA devices will remain an essential category in 6G, providing broadband connectivity to households and enterprises. Compared to smartphones, FWA UEs typically allow less stringent size and power constraints, enabling support for high-gain antennas, multi-band reception, and advanced beam management. For FWA UEs, 6G standardization must ensure efficient support for wideband carrier aggregation, optimized mobility handling for semi-static installations, and seamless integration with wired backhaul networks. |  |  |  |  |  |

Along with device type classification, RAN2 can study a UE capability signalling framework that utilizes basic and essential capabilities associated with each device type to minimize signalling overhead for UE capability reporting. For example, if RAN2 defines basic and essential capabilities for each device type, the device type information alone can be reported in UE capability signalling instead of reporting individual parameters for these basic and essential capabilities. This approach would streamline the signalling process and provide a more structured framework for defining diverse device capabilities in 6G. The device type can be also used as a filter for UE Capability Enquiry to avoid unnecessary and excessive UE Capability report.

Observation 2: Along with device type classification and the identification of basic and essential capabilities, the UE capability signalling framework can be enhanced to determine UE capabilities for each device type.

Proposal 1: RAN2 is asked to study improvements to UE capability signalling mechanisms by leveraging device type based basic and essential capabilities.

2.2		6G Day-1 services

The study should aim to develop a single technology framework for 6G Radio (6GR) to support both existing and new services, addressing the deployment scenarios and requirements outlined in the RAN study on 6G scenarios and requirements [1]. The services identified in [1] include mobile broadband, immersive communication, massive communication (IoT), sensing, AI, voice, and regulatory services.

6G Radio protocol must adopt a unified framework that supports mobile broadband services and other services. While enabling broad vertical services support, the system must prioritize mobile broadband services to ensure high data rates, low latency, and a superior user experience. The mobile broadband service in 6GR should surpass the capabilities of the 5G system.

Observation 3: 6G radio protocol must meet the requirements of mobile broadband services as a high priority.

Given that the 6GR framework is expected to be built on a standalone architecture, voice service support must be guaranteed from Day-1. Therefore, the necessary 6G Radio protocols to enable voice service should be studied with high priority. For IoT or RedCap services, determining whether these services should be supported from Day-1 requires clear market justification.

Regarding new services such as AI and sensing, since RAN2 and other WGs are actively studying or planning to study their requirements and use cases, we recommend to wait for detailed requirements and specific use cases before deciding whether AI or sensing can be supported from Day-1. In this paper, the term “AI service” refers to applications such as AI agents or AI chatbots that utilize the 6G network. Use cases addressing AI for network—where AI technologies improve UE and NW performance and efficiency —are excluded from this context.

However, even though these services are introduced in later phases, 6GR must be designed in a scalable and forward-compatible manner to integrate these services within the unified framework without unnecessary separate solutions.

Observation 4: Voice service must be supported in 6G standalone architecture. For other services such as IoT, sensing and AI applications, clear market needs and use cases are required to justify their support from Day-1.

Based on the above observations, we propose:

Proposal 2: The 6GR study should prioritize mobile broadband and voice services from Day-1 while aiming for a common design that supports a wide range of services without requiring separate solutions.

3	Conclusion

Based on the discussion, we made the following observations:

Observation 1: The LTE UE category has limitations in accurately representing features for all the device types.

Observation 2: Along with device type classification and the identification of basic and essential capabilities, the UE capability signalling framework can be enhanced to determine UE capabilities for each device type.

Observation 3: 6G radio protocol must meet the requirements of mobile broadband services as a high priority.

Observation 4: Voice service must be supported in 6G standalone architecture. For other services such as IoT, sensing and AI applications, clear market needs and use cases are required to justify their support from Day-1.

Based on the observations, RAN2 is requested to discuss and agree the following proposals:

Proposal 1: RAN2 is asked to study improvements to UE capability signalling mechanisms by leveraging device type based basic and essential capabilities.

Proposal 2: The 6GR study should prioritize mobile broadband and voice services from Day-1 while aiming for a common design that supports a wide range of services without requiring separate solutions.

4	References

[1] TR 38.914 V0.2.0, 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on 6G Scenarios and Requirements; (Release 20)