3GPP TSG-RAN WG2 Meeting #132	R2-2508191

# Dallas, USA, November 17 – 21, 2025

Agenda item:	10.3.2.1

Source:	Samsung

Title:	RRC states and functions for 6G radio

Document for:	Discussion

1	Introduction

This paper is to discuss RRC functionality, states, and connection management for 6G Radio protocols.

2	Discussion

2.1		RRC functions

Regarding RRC services and functions for 6G RAN, the services and functions of RRC sublayer in 5G NR can serve as a starting point. In 5G NR [1], the main services and functions of the RRC include:

- Broadcast of System Information related to AS and NAS - Paging initiated by 5GC or NG-RAN - Establishment, maintenance and release of an RRC connection between the UE and NG-RAN including: - Addition, modification and release of carrier aggregation - Addition, modification and release of Dual Connectivity in NR or between E-UTRA and NR - Security functions including key management - Establishment, configuration, maintenance and release of Signalling Radio Bearers (SRBs) and Data Radio Bearers (DRBs) - Mobility functions including: - Handover and context transfer - UE cell selection and reselection and control of cell selection and reselection - Inter-RAT mobility - QoS management functions - UE measurement reporting and control of the reporting - Detection of and recovery from radio link failure - NAS message transfer to/from NAS from/to UE

Since the study on dual connectivity (DC) will be determined later, excluding the function “Addition, modification and release of Dual Connectivity in NR or between E-UTRA and NR”, the existing services and functions for the RRC in 5G NR can be considered a baseline for developing RRC functions in 6GR.

Observation 1: The services and functions of the 5G NR RRC can serve a starting point for discussing regarding 6G RRC services and functions.

This paper focuses on analyzing and discussing the services and functions of RRC, particularly RRC_IDLE and RRC_INACTIVE states, while considering potential enhancements in RRC states for 6GR.

2.2		RRC states

In 5G NR, three RRC states are defined–RRC_IDLE, RRC_INACTIVE, and RRC_CONNECTED–with specific functions and services associated with each state as below:

RRC_IDLE:

-	A UE specific DRX may be configured by upper layers

-	UE controlled mobility based on network configuration

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI

-	Monitors a Paging channel for CN paging using 5G-S-TMSI

-	Performs neighbouring cell measurements and cell (re-)selection

-	Acquires system information and can send SI request (if configured)

RRC_INACTIVE:

-	A UE specific DRX may be configured by upper layers or by RRC layer

-	UE controlled mobility based on network configuration

-	The UE stores the UE Inactive AS context

-	A RAN-based notification area is configured by RRC layer

-	5GC - NG-RAN connection (both C/U-planes) is established for UE

The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI

-	Monitors a Paging channel for CN paging using 5G-S-TMSI and RAN paging using fullI-RNTI

-	Performs neighbouring cell measurements and cell (re-)selection

-	Performs RAN-based notification area updates periodically and when moving outside the configured RAN-based notification area

-	Acquires system information and can send SI request (if configured).

RRC_CONNECTED:

-	The UE stores the AS context

-	Transfer of unicast data to/from UE

-	At lower layers, the UE may be configured with a UE specific DRX

-	For UEs supporting CA, use of one or more SCells, aggregated with the SpCell, for increased bandwidth

-	For UEs supporting DC, use of one SCG, aggregated with the MCG, for increased bandwidth

-	Network controlled mobility within NR and to/from E-UTRA

-	5GC - NG-RAN connection (both C/U-planes) is established for UE;

-	The UE:

-	Monitors Short Messages transmitted with P-RNTI over DCI, if configured

-	Monitors control channels associated with the shared data channel to determine if data is scheduled for it

-	Provides channel quality and feedback information

-	Performs neighbouring cell measurements and measurement reporting

-	Acquires system information

We propose maintaining these three RRC states in the 6G system. However, for 6G, it is essential to redefine and optimize the functions and services for the states, particularly by simplifying redundant functionalities across multiple states, such as UE mobility management and paging in both RRC_INACTIVE and RRC_IDLE.

Observation 2: The three RRC states in 5G NR can be reused in 6GR, but simplification of duplicate functions across states is necessary.

There may be concerns regarding the RRC_INACTIVE state, as its adoption has been limited so far. However, the RRC_INACTIVE state was introduced to enable the UE to remain in a low-power mode longer than in RRC_CONNECTED, while also reducing RRC connection setup latency compared to RRC_IDLE. These benefits remain relevant for 6GR, especially given the growing demand for low power consumption and reduced latency in emerging immersive and interactive applications. Therefore, improvements to the RRC_INACTIVE state should be studied to enhance its usability.

Observation 3: For 6GR, the RRC_INACTIVE state continues to offer benefits, including lower power consumption and reduced connection setup latency compared to both RRC_CONNECTED and RRC_IDLE.

Proposal 1: In 6G RAN, the three RRC states—RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE—should be retained for UE.

In 5G NR, faster RRC connection resumption is possible from the RRC_INACTIVE state because the Inactive AS context is stored and maintained at both UE and NG-RAN. This reduces signaling for RRC connection resumption procedures from the RRC_INACTIVE compared to resuming from RRC_IDLE. Additionally, thanks to the stored AS context, an RRC_INACTIVE UE can transmit or receive small data without requiring a full RRC connection resumption. However, excessive storage and maintenance of the AS context at both UE and RAN can impose significant overhead. In 5G NR, many configurations from RRCReconfiguration are stored as part of the Inactive AS context, even though not all of these configurations are necessary for connection resumption or small data transmission in the RRC_INACTIVE state. A study is needed to minimize overhead by storing only essential Inactive AS context for RRC_INACTIVE UEs in 6GR.

Observation 4: Excessive overhead associated with soring and maintaining the Inactive AS context for RRC_INACTIVE should be avoided.

    - For 6GR, UE mobility management, including paging and RNA update, should continue to be supported. In 5G NR, CN-based paging and CN involvement were required to address cases of paging failure (e.g., UE unreachability) resulting from RAN paging or scenarios where no Xn interface existed between gNBs. Due to these limitations in NG-RAN, mobility management and paging were duplicated in both RRC_INACTIVE state and RRC_IDLE state. In 6G system, UE mobility management procedures—including paging and RNA updates—should be enhanced to overcome these limitations identified in 5G. Achieving this will require collaboration with RAN3 and SA2.

Observation 5: Mobility management procedures in the RRC_INACTIVE state should be improved to address issues such as UE unreachability or the absence of interface between 6G-RANs, with inputs from RAN3 and SA2.

Proposal 2: RAN2 is asked to study enhancements to the RRC_INACTIVE state, including optimizing Inactive AS context configuration and improving UE mobility management (e.g., single paging).

Once the UE completes registration procedures transitioning from RRC_IDLE to RRC_CONNECTED, it can operate within either RRC_CONNECTED or RRC_INACTIVE under 6G-RAN whenever feasible. Mobility management for RRC_CONNECTED UEs can be handled through handover procedures, while mobility management for RRC_INACTIVE UEs can rely on paging by 6G-RAN and RNA update procedures. With enhanced UE mobility management mechanisms, the mobility management functionality for RRC_INACTIVE can be specified as a default feature without requiring CN-based paging.

Based on the above discussion, the functions and services of 6G RRC in each state can be characterised as follows:

-	RRC_IDLE:

-	PLMN selection

-	Cell selection

-	RRC_INACTIVE:

-	PLMN selection

-	Cell re-selection mobility

-	Paging is initiated by 6G-RAN

-	RAN-based notification area (RNA) is managed by 6G-RAN

-	DRX for paging configured by 6G-RAN

-	6GC – 6G-RAN connection is established for UE;

-	The UE Inactive AS context is stored in NG-RAN and the UE;

-	Transfer of unicast data and/or signalling to/from the UE over radio bearers configured for SDT

-	RRC_CONNECTED:

-	6GC – 6G-RAN connection is established for UE;

-	The UE AS context is stored in NG-RAN and the UE

-	NG-RAN knows the cell which the UE belongs to

-	Transfer of unicast data to/from the UE

-	Network controlled mobility including measurements

Considering the services and functions associated with each state, a scenario for RRC state transition in 6GR is illustrated in Figure 1. In this scenario, the UE transitions to RRC_IDLE due to specific events, such as a deregistration request from the network or the selection of a new PLMN.

Figure 1 RRC state transition scenario in 6G

Based on the above observations, we propose:

Proposal 3: RAN2 is asked to study methods to simply the services and functions related to UE mobility management in the RRC_IDLE state.

3	Conclusion

Based on the discussion, we made the following observations:

Observation 1: The services and functions of the 5G NR RRC can serve a starting point for discussing regarding 6G RRC services and functions.

Observation 2: The three RRC states in 5G NR can be reused in 6GR, but simplification of duplicate functions across states is necessary.

Observation 3: For 6GR, the RRC_INACTIVE state continues to offer benefits, including lower power consumption and reduced connection setup latency compared to both RRC_CONNECTED and RRC_IDLE.

Observation 4: Excessive overhead associated with soring and maintaining the Inactive AS context for RRC_INACTIVE should be avoided.

Observation 5: Mobility management procedures in the RRC_INACTIVE state should be improved to address issues such as UE unreachability or the absence of interface between 6G-RANs, with inputs from RAN3 and SA2.

Based on the observations, RAN2 is requested to discuss and agree the following proposals:

Proposal 1: In 6G RAN, the three RRC states—RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE—should be retained for UE.

Proposal 2: RAN2 is asked to study enhancements to the RRC_INACTIVE state, including optimizing Inactive AS context configuration and improving UE mobility management (e.g., single paging).

Proposal 3: RAN2 is asked to study methods to simply the services and functions related to UE mobility management in the RRC_IDLE state.

4	References

[1] TS 38.300