3GPP TSG-RAN WG2 Meeting #132	R2-2508205

Dallas, USA, Nov. 17th-21st, 2025

Agenda item:	8.12.3

Source:	Samsung

Title:	RIL S012, S017

WID/SID:	NR_MIMO_Ph5

Document for:	Discussion and Decision

# Introduction

In this paper, we discuss RIL S012 and S017.

# Discussion

## S012

As in the latest RAN1 RRC list R2-2508202, it has been updated to include qcl-info in each resource set in resourceForChannelCJTC according to RAN1 agreement.

Based on the RAN1 update, we propose to agree S012.

Proposal 1: Agree S012, i.e., add qcl-info for each resource set in resourcesForChannelCJTC-r19, which is similar to legacy resourcesForChannel.

## S017

Regarding the first change, as checked in TS 38.214 there is no consistent wording used:

- “trigger state” is used to refer to any trigger state for P/SP/AP-CSI in 5.2.1

- “aperiodic trigger state”, “CSI trigger state”, and “trigger state” are used here and there to refer to trigger state for AP-CSI in 5.2.1.5.1, and “aperiodic CSI trigger state” is used in 5.2.3

- “trigger state” is used to refer to trigger state for SP-CSI in 5.2.1.5.2, and “semi-persistent CSI trigger state” is used in 5.2.3

- “CSI trigger state” is used to refer to trigger state for UEI-CSI in 5.2.1.5.4.

Based on this, it should be okay to use “CSI trigger state” generally for any CSI in RRC, and make it explicit by using “CSI trigger state for AP-CSI” or “CSI trigger state for UE-initiated CSI” at specific places since a CSI trigger state is not shared between UEI-CSI report configuration and other CSI report configuration, as RAN1 specified below in TS 38.214.

5.2.1.5.4.1 UE Initiated CSI reporting … The UE does not expect that a CSI trigger state associated with CSI report configuration(s) configured with the higher layer parameter eventType is further associated with other CSI report configurations that are not configured with the higher layer parameter eventType.

Regarding the second change, we understand Ofinno’s comment has a valid point. We suggest to add a separate sentence for UEI CSI reporting as follows.

“Upon reception of the value associated with a trigger state, the UE will perform UE-initiated reporting on PUSCH in L1 according to all entries in the associatedReportConfigInfoList for that trigger state.”

Proposal 2: For RIL S017, adopt the following TP:

#### –	CSI-AperiodicTriggerStateList

The CSI-AperiodicTriggerStateList IE is used to configure the UE with a list of CSI trigger states for aperiodic CSI and UE-initiated CSI reporting. Each codepoint of the DCI field "CSI request" is associated with one trigger state (see TS 38.321 [3], clause 6.1.3.13). Upon reception of the value associated with a trigger state, the UE will perform measurement of CSI-RS, CSI-IM and/or SSB (reference signals) and aperiodic reporting on L1 according to all entries in the associatedReportConfigInfoList for that trigger state. Upon reception of the value associated with a trigger state, the UE will perform UE-initiated reporting on PUSCH in L1 according to all entries in the associatedReportConfigInfoList for that trigger state.

# Conclusion

Based on the discussion, we have the following proposal.

Proposal 1: Agree S012, i.e., add qcl-info for each resource set in resourcesForChannelCJTC-r19, which is similar to legacy resourcesForChannel.

Proposal 2: For RIL S017, adopt the following TP:

#### –	CSI-AperiodicTriggerStateList

The CSI-AperiodicTriggerStateList IE is used to configure the UE with a list of CSI trigger states for aperiodic CSI and UE-initiated CSI reporting. Each codepoint of the DCI field "CSI request" is associated with one trigger state (see TS 38.321 [3], clause 6.1.3.13). Upon reception of the value associated with a trigger state, the UE will perform measurement of CSI-RS, CSI-IM and/or SSB (reference signals) and aperiodic reporting on L1 according to all entries in the associatedReportConfigInfoList for that trigger state. Upon reception of the value associated with a trigger state, the UE will perform UE-initiated reporting on PUSCH in L1 according to all entries in the associatedReportConfigInfoList for that trigger state.

# References

- RP‑242394	WID revision: NR MIMO Phase 5.

- R2-250xxxx RIL review