3GPP TSG RAN WG2#132	R2-2508215

Dallas, USA, 17th - 21st, November 2025

Agenda Item		:	8.8.2

Source		:	CENC

Title		:	Discussion on downlink coverage enhancement

Document for		:	Discussion and Decision

1. Introduction

During RAN2#131 meeting, RAN2 made some progress with the following statements[1]:

RAN2#131 meeting agreements:

1.	RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements

2.	In connected mode the NW will only configure up to 4 STMC.

3.	UE Assistance Information message will include information to the NW to (re)configure the most relevant SMTCs for the area where the UE is located

4.	The procedure for UE assisted SMTC configuration in connected mode is follows:

-	For a UE indicating the relevant capability (FFS the name of the capability and it this is merged with some other capability) the network configures the UE via reconfiguration message with a list of reference locations and a parameter N, thereby configuring the UE to report the N closest reference locations (FFS whether also a distance threshold is indicated). If the list of reference locations is not provided, the list of reference locations provided in system information is reused.

-	The UE reports an indication of the N closest reference locations via UE assistance information, e.g. bitmap or list of indices of the locations.

-	The UE can report the N closest reference locations via the RRCReconfigurationComplete message.

-	The network configures the relevant SMTC(s). The maximum number of configured SMTCs is according to legacy UE capability in terms of SMTC support (same as legacy).

-	The UE provides an update on the set of the N closest reference locations, if changed, via UE assistance information. Upon receiving an update, the network reconfigures the relevant SMTC(s), if changed.

In this contribution, we will discuss the remaining issues related to the agreements

2. Discussion

Assistance information for UE to perform SMTC selection

In the last RAN2 meeting, RAN2 reconfirms the understanding that if the NW configures more than 4 STMCs the UE in idle/inactive the UE selects up to 4 STMCs (or less, depending on the UE capability) based on location and only considers them for measurements. At the same time, in connected mode the NW will only configure up to 4 STMC. Moreover, it was also decided in the previous meeting that there are two configuration methods when performing SMTC configuration:

- Alt 1: Reference location information of neighbor cells

- Alt 2: Serving cell’s SSB beam index

The location-based SMTC selection procedure requires UEs to support GNSS and UE in idle mode requires additional signaling interactions to obtain its location, which increases the power consumption and complexity of UE. Meanwhile, this solution is highly dependent on the availability of GNSS. Considering the GNSS information may be temporarily unavailable, UEs may fail to determine its distance to the neighbour cell.

Observation 1: The location-based SMTC selection is highly dependent on the availability of GNSS. However, for UEs with a fixed location, their positions remain unchanged, and obtaining their own positions only requires one interaction.

We introduce an SSB-index based SMTC selection procedure where each SMTC will be associated with an SSB index list of the serving cell. The UE can select the SMTC based on the geographical relationship between the SSB number of the beam it is in and the SSB number of the adjacent serving cell.

Observation 2: The selection based on SSB beam index for SMTC imposes requirements on network deployment. It is necessary to ensure that consistent SSB beam index and SMTC mapping are configured among UEs within the service cell during satellite coverage.

Proposal 1: RAN2 should support the SMTC selection method option, which is optional. In the idle state, UEs at fixed locations can adopt location-based SMTC selection. In the inactive/connected state, SMTC selection based on SSB index can be used.

Configuring assistance location information

Regardless of whether Option 1 and/or Option 2 and/or Option 1 and Option 2 is adopted as assistance location information, RAN2 should discuss how to associate location information with SMTC. Based on the previous meeting, a new SMTC list is to be introduced. Since SMTC4 can be composed of a group of PCIs, we suggest that the new SMTC list be configured together with the PCIs, and the network will associate the location information with the PCIs.

Proposal 2：Location information is associated with each PCI in the new SMTC list.

SMTC Configuration in Connected Mode

At the previous meeting, it was proposed that there were two options for the selection of SMTC in the connection mode:

- Alt 1: UE provide the closest N reference locations/neighbor cells to network.

- Alt 2: UE reports the selected SMTCs from configured SMTC set to network

From our point of view,, the UE can send its location to the network side through UEAssistanceInformation. The network side then determines the list of its areas based on the UE's location, and finely configures SMTC and gaps according to the coverage of each neighboring cell. It also sends the SMTC configuration to the UE through RRCReconfiguration.

Proposal 3：RAN2 should introduce a mechanism to assist the NW to configure SMTCs based on UAI in RRC_CONNECTED mode.

3. Conclusion

In this contribution, following statements are proposed:

Observation 1: The location-based SMTC selection is highly dependent on the availability of GNSS. However, for UEs with a fixed location, their positions remain unchanged, and obtaining their own positions only requires one interaction.

Observation 2: The selection based on SSB beam index for SMTC imposes requirements on network deployment. It is necessary to ensure that consistent SSB beam index and SMTC mapping are configured among UEs within the service cell during satellite coverage.

Proposal 1: RAN2 should support the SMTC selection method option, which is optional. In the idle state, UEs at fixed locations can adopt location-based SMTC selection. In the inactive/connected state, SMTC selection based on SSB index can be used.

Proposal 2：Location information is associated with each PCI in the new SMTC list.

Proposal 3：RAN2 should introduce a mechanism to assist the NW to configure SMTCs based on UAI in RRC_CONNECTED mode.

4. References

[1] R2-2506203 Report from Break-out session on NR-NTN and IoT-NTN.