3GPP TSG-RAN WG2 Meeting #132	                                                                                   R2-2508222

Dallas, USA, Nov. 17th - 21st

Source:	CATT

Title:	Discussion on remaining MAC issues

Agenda Item:    8.12.2

Document for:  Discussion and Decision

Introduction

In this contribution, we further discuss the remaining MAC issues suggested from the MAC Rapporteur [1]. More specially, the following issues are covered,

- UE behavior on sTAG TAT expiry for Type1 CG PUSCH and running TAT for PUCCH

- PDCCH monitor for mode-A UEIBR

Our proposals are summarized in Section 3. The conclusion of this paper is given in Section 4.

Discussion

[Issue-1]: UE behavior on sTAG TAT expiry for Type1 CG PUSCH and running TAT for PUCCH

The following open issue had been discussed in last meeting but still no consensus.

[Issue description]: As cross-CC UEI CSI reporting is supported, i.e., PUCCH for UEIRI and PUSCH for the actual report can be on different cells. For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be configured on different cells, thus, associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running, UE behaviour is not clear.

Now there are two major candidate solutions on the Table,

[Proposed Solution]: For mode-B UEI reporting, PUCCH and Type1 CG PUSCH can be associated with different TAGs. If the TAT (associated with a sTAG) for Type1 CG PUSCH is expired while the TAT for PUCCH is running, Option 1 (baseline from RAN2#131bis): UE releases the PUCCH if the configured PUCCH resource id is only associated to this type-1 CG PUSCH (i.e., not associated with any other UCI). Option 2 (to avoid any impact to other UCI transmission): UE does not release the PUCCH, UE does not transmit UEIRI on the PUCCH if UEI CSI report is triggered.

The favorers on Option 1 thought that it can be up to the NW implementation to configure the separated PUCCH resources for other UCI reporting, for example, P/SP CSI on PUCCH, HARQ feedback for SPS, SR. But for the dynamic DL scheduling, the UE determines the PUCCH resource set for HARQ feedback based on the size of UCI, and then further determines the PUCCH resource within the PUCCH resource set based on the PUCCH resource indicator in DCI. See TS 38.213:

9.2 UCI reporting in physical uplink control channel 9.2.1 PUCCH Resource Sets … If the UE provides HARQ-ACK information in a PUCCH transmission in response to detecting a DCI format scheduling a PDSCH reception or having associated HARQ-ACK information without scheduling a PDSCH reception, the UE determines a PUCCH resource with index rPUCCH, 0≤rPUCCH≤15, as rPUCCH=2⋅nCCE,0NCCE+2⋅∆PRI, where NCCE is a number of CCEs in a CORESET of a PDCCH reception with the DCI format, as described in clause 10.1, nCCE,0 is the index of a first CCE for the PDCCH reception, and ∆PRI is a value of the PUCCH resource indicator field in the DCI format. … If the UE transmits OUCI UCI information bits, that include HARQ-ACK information bits, the UE determines a PUCCH resource set to be - a first set of PUCCH resources with pucch-ResourceSetId = 0 if OUCI≤2 including 1 or 2 HARQ-ACK information bits and a positive or negative SR on one SR transmission occasion if transmission of HARQ-ACK information and SR occurs simultaneously, or - a second set of PUCCH resources with pucch-ResourceSetId = 1, if provided by higher layers, if 2<OUCI≤N2 where N2 is equal to maxPayloadSize if maxPayloadSize is provided for the PUCCH resource set with pucch-ResourceSetId = 1; otherwise N2 is equal to 1706, or - a third set of PUCCH resources with pucch-ResourceSetId = 2, if provided by higher layers, if N2<OUCI≤N3 where N3 is equal to maxPayloadSize if maxPayloadSize is provided for the PUCCH resource set with pucch-ResourceSetId = 2; otherwise N3 is equal to 1706, or - a fourth set of PUCCH resources with pucch-ResourceSetId = 3, if provided by higher layers, if N3<OUCI≤1706.

That means there will be a case where the UE receives a DCI dynamically scheduling a DL assignment, and determines a PUCCH resource to send the HARQ feedback. But unfortunately, before arrival of the PUCCH resource, the PUCCH resource for HARQ feedback is released due to the TAT expiry of type-1 CG PUSCH, if we go with Option 1. It seems this HARQ feedback for dynamic DL assignment cannot be well handled by the NW configuration. From this perspective, we can accept Option 2 to avoid potential impact to other UCI reporting.

Proposal 1 [Issue-1]: If the TAT associated with a sTAG for Type1 CG PUSCH is expired while the TAT for PUCCH is running, UE does not release the PUCCH and does not transmit UEIRI on the PUCCH if UEI CSI report is triggered.

[Issue-2]: PDCCH monitor for mode-A UEIBR

Another open issue is,

[Issue Description]: When DRX is configured, the Active Time for Serving Cells in a DRX group includes the time between transmsision of a UEIRI and receiving a PDCCH scheduling PUSCH for mode-A UEI CSI report: … - a PDCCH scheduling a mode-A UE-initiated CSI report on PUSCH has not been received after transmitting UE Initiated Report Indication on PUCCH (as specified in TS 38.214 [7]). If PDCCH for CSI report is never received (either the PDCCH is not scheduled by NW after the NW receives the UEIRI or the NW fails to receive the UEIRI on PUCCH), the UE will have to keep staying in DRX active time due to the condition being always met, causing unnecessary power consumption.

To solve above highlighted concern, another candidate option 2 is added,

[Proposed Solution]: For DRX, after transmitting UEIRI on PUCCH: Option 1: UE stays in active time until receiving PDCCH scheduling. Keep the current MAC spec. Option 2: UE stays in active time until the next PUCCH resource occasion. Update MAC spec.

During the post email discussion, it seems it is still controversy whether the UE continues reporting the UEIRI in the subsequent (non-)consecutive PUCCH resource. At least from our view, it is UE implementation to do one-shot UEIRI within one PUCCH resource or do multiple UEIRI using the consecutive PUCCH resource if not receiving the PDCCH upon arrival of the subsequent PUCCH resource, provided that the trigger condition is still satisfied.

Understanding 1: One-shot UEIRI

In this case, if we go with Option 1, we echo the concern on the power-consuming for Mode-A PDCCH monitoring due to miss of UEIRI by gNB or miss of PDCCH by UE. But Option 2 also has drawback if the NW configures a very short PUCCH period. This may cause early entering of the DRX inactive time, which also leads to the miss of PDCCH. No better solution to solve this concern but just rely on a NW to configure a suitable PUCCH period, in between which the DCI can schedule the DG PUSCH for Mode-A UCI reporting.

Understanding 2: multiple UEIRI using the consecutive PUCCH resource

If this is correct understanding, the consecutive UEIRI will prolong the DRX active time until receiving the PDCCH scheduling for UCI reporting. In this case, we think Option 1 is enough, since anyway UE will receive a PDCCH from sending UEIRI onwards.

However, as our understanding is up to UE implementation to choose either one-shot UEIRI or multiple UEIRI using the consecutive PUCCH resource, we can rely on a soft description as suggested by the rapporteur.

Proposal 2 [Issue-2]: RAN2 to confirm that after transmitting UEIRI on PUCCH, UE stays in active time until receiving PDCCH scheduling the mode-A UEI CSI report on PUSCH or until the next PUCCH occasion for UEIRI.

Conclusion

Based on the previous analysis in Section 2, our contributions are summarized as follows:

Proposal 1 [Issue-1]: If the TAT associated with a sTAG for Type1 CG PUSCH is expired while the TAT for PUCCH is running, UE does not release the PUCCH and does not transmit UEIRI on the PUCCH if UEI CSI report is triggered.

Proposal 2 [Issue-2]: RAN2 to confirm that after transmitting UEIRI on PUCCH, UE stays in active time until receiving PDCCH scheduling the mode-A UEI CSI report on PUSCH or until the next PUCCH occasion for UEIRI.

Reference

[1] R2-2508129, Report of Rel-19 MIMO MAC open issues