3GPP TSG-RAN WG2 Meeting #132                            R2-2508226

Dallas, USA, Nov.17th–21th, 2025

Agenda Item:	10.3.2.1

Source: 	Transsion Holdings

Title:  	Discussion on RRC Modelling and connection management in 6G

Document for:	Discussion and Decision

Introduction

In the last meeting, the RRC State Model and Spectrum Aggregation for 6G were discussed, and the following agreements were made[1]:

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements.   We can consider the pros/cons of different modelling, after having some discussions on the functionality.

Agreements on Spectrum aggregation

- 1	Study spectrum aggregation of multiple pieces of spectrum and understand the issues that need to be addressed.

- 2	Study how to use UL and DL spectrum more efficiently (e.g. decoupling of UL and DL, etc).  Understand the deployment scenarios and problems to address

In this contribution, we provide our views on these aspects to proceed the study item.

Discussion

RRC state model for 6G

In last meeting, the RRC state model for 6G was discussed, and the state of RRC_CONNECTION state and RRC_IDLE mode are widely regarded as as essential state as their distinct characteristics of energy consumption and latency and well commercialized in last two generation. While The RRC_INACTIVE introduced in 5G was quit questioned as its complexity and not widely enabled in the operator network. To determine whether the RRC_INACTIVE is needed for the 6G, here table below illustrates the characteristic of the three RRC state in 5G and give the typical applicable scenarios:

| characteristic | IDLE | INACTIVE | CONNECTED |
| --- | --- | --- | --- |
| PLMN selection; | Yes | Yes | - |
| Broadcast of system information | Yes | Yes | Yes |
| Mobility | Cell (re)selection RAU | Cell reselection RNAU | NW-controlled |
| UE Context | No | Yes | Yes |
| Paging | Yes (CN-initiated) | Yes (CN/RAN-initiated) | Yes (SI changes etc.) |
| Power saving | I-DRX、eDRX、LP-WUS、PSM | DRX、eDRX、LP-WUS | C-DRX |
| RAN-5GC connection | No | YES | Yes |
| UE-RAN connection | No | No(fast resume) | Yes |
| Data transfer | No | SDT | Yes(all type) |
| Suitable traffic scenario | Sporadic traffic with power saving requirement(e.g. low-end IoT traffic) | Mid-frequency traffic with one-shot small data(e.g. period heartbeat traffic) | Long or heavy traffic with latency requirement(e.g. XR traffic) |

- For sporadic traffic which is not frequent and most of these traffics maybe low-end IoT service which may send data over several hours or even days, obviously, for these traffic, it is delay-tolerance and more energy-efficient to put these UEs into IDLE mode when there are no data transmission. For small data traffic with mid-frequency, we think it is neither good to use the legacy 5G RRC_Idle mode as it is not efficient nor to use the legacy 5G RRC_CONNECTED state as it is not good for power saving. That is why the legacy 5G RRC_INACTIVE mode were introduced. For traffic which will last for long or latency requirement, it is obviously better to put these UEs into RRC_CONNECTED sate. Regarding the RRC state mode for 6G, we think it should supports meet the latency and energy consumption requirements of each specific traffic.

- Observation 1: The legacy 5G RRC_IDLE and RRC_CONNECTED mode can’t meet the requirements of the small data transmission with mid-frequency.

- Proposal 1:	 The RRC state mode for 6G should supports meeting latency and energy consumption requirements of each specific traffic.

- In last meeting there are some concerns about the legacy 5G RRC_INACTIVE MODE, the main concerns are mainly about the complex RNA configuration、requirement on Xn connectivity and   the benefit, but the characteristics of 5G RRC_INACTIVE including UE based mobility、UE context management and energy efficient for both UE and NW are considered to should be supported in 6G, the UE identification、RNA management and paging/wake-up are considered to need improvement. We think these concerns or issues to be improved are all related to the RNA(RAN-based Notification Area) concept. The introduction of RNA needs the RNA planning、complex Xn connection between the gNBs in RNA、the RAN paging、RNAU when the UE moves between the RNA and the UE context retrieve in the RNA. The RNA is mainly used for UE mobility management and context management and there are some duplication with the CN Tracking area. We think the RNA can be removed from RRC_INACTIVE mode and reuse the CN tracking area. For UE context retrieve, we can use the CN to relay the UE context or the UE context can be stored in the CN, in this way, the RNA planing、RAN paging、RNAU and Xn connectivity demand maybe not needed. The detailed procedure can be further discussed.

- Proposal 2:	 The RNA concept maybe not needed in 6G to simplify the legacy RRC_INACTIVE mode, the CN Tracking area and CN based UE context retrieve procedure can be used for UE mobility and context management.

- If there is no RNA concept in 6G, then we think the INACTIVE mode maybe have little difference with the RRC_IDLE mode except the UE context can be maintained in UE and the network, from this point of view we think there is no need to introduce a INACTIVE like mode but to enhance the legacy RRC IDLE mode to support UE context maintain and retrieve.

- Proposal 3:	 If RNA concept is not used in 6G, there is no need to introduce RRC_INACTIVE mode but to enhance the RRC_IDLE mode to support fast transition with UE context.

Spectrum Aggregation for 6G

6G cell model can be with more than one UL carriers and more than one DL carriers. This kind of 6G cell model can improve the paging capacity and RACH capacity to meet 6G IOT requirement. It is also benefit for network energy saving, e.g., the carrier can be shut down if service load is low.

- Proposal 4: 6G cell model with more than one UL carriers and more than one DL carriers is studied.

Conclusion

In this contribution we discussed RRC state model and spectrum aggregation for 6G and made the following proposals:

- Observation 1: The legacy 5G RRC_IDLE and RRC_CONNECTED mode can’t meet the requirements of the small data transmission with mid-frequency.

- Proposal 1:	 The RRC state mode for 6G should supports meeting latency and energy consumption requirements of each specific traffic.

- Proposal 2:	 The RNA concept maybe not needed in 6G to simplify the legacy RRC_INACTIVE mode, the CN Tracking area and CN based UE context retrieve procedure can be used for UE mobility and context management.

- Proposal 3:	 If RNA concept is not used in 6G, there is no need to introduce RRC_INACTIVE mode but to enhance the RRC_IDLE mode to support fast transition with UE context.

- Proposal 4: 6G cell model with more than one UL carriers and more than one DL carriers is studied.

Reference

- “Draft_RAN2_131bis_MeetingReport_v1.docx”