3GPP TSG-RAN WG2 Meeting #132										       R2-2508229

Dallas, USA, Nov. 17th-21st, 2025

Agenda Item:	10.2.3

Source: 	Sharp

Title:  	Design issues for 6GR study

Document for: 	Discussion

Introduction

RAN2 WG has started 6G Study Item (SI) progress since R2#131bis meeting. This document discusses key design issues of 6GR that should consider based on the agreements on RAN#109 meeting [1] and R2#131bis meeting [2].

Discussion

Network slicing and service awareness

In RAN#109 meeting [1], it is agreed that RAN design for the 6G Radio Access Technologies shall be designed to fulfil the following requirements:

- The 6G RAN architecture shall allow for the operation of network slicing.

- The 6G RAN architecture shall support enhanced service awareness in RAN.

However, it is till un-known how to support enhanced service awareness in RAN. Furthermore, RAN TSG also agreed that the study of 6GR services requirements includes, but not limited to, the following new and existing services [3]:

- Mobile Broadband

- Immersive Communication

- Immersive Communication

- Massive Communication (IoT)

- Sensing

- AI

- Voice

- Regulatory Services.

In R2#131 bis meeting, RAN2 also agreed to study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic) [3]. In 5G system, UE and RAN would exchange S-NSSAI (Single Network Slice Selection Assistance Information) list, which includes service-related information based on network slicing operation in 5GS. In order to identify a network slice end to end, the 5GS uses S-NSSAI. An S-NSSAI is comprised of:

-	A Slice/Service type (SST),

-	A Slice Differentiator (SD), which is optional information that complements the Slice/Service type(s) to differentiate amongst multiple Network Slices.

In addition, RAN and UE would exchange S-NSSAI list via broadcasting system information and UE-specific RRC signaling. Based on the agreement of RAN#109 meeting, it is expected that RAN and UE would still share service-related information via S-NSSAI exchange. Therefore, RAN2 is suggested to study how to support enhanced service awareness based on the operation of RAN slicing.

Observation 1: It is expected that RAN and UE would still share service-related information via S-NSSAI exchange.

Proposal 1: RAN2 shall study how to support enhanced service awareness for existing and new services based on the operation of RAN slicing, E.g., S-NSSAI exchange.

Slice-related mechanisms in New Radio

In 5G-advanced protocols [2], following slice-related mechanisms are introduced for the RAN and UE to prioritize slice operation:

A) NSAG information delivery and highest priority NSAG selection.

B) Slice-based cell reselection, which includes cell reselection priority information delivery (e.g., via SIB16 or RRCRelease message), allowed/excluded cell list management, and frequency prioritization for slicing.

C) RA prioritization for slicing, which includes random access resources selection, preamble power ramping adjustment, backoff indicator scaling while slicing is prioritized.

In 6GR study, RAN2 WG is suggested to study whether the existing slice-related mechanisms should be reused w/wo further enhancements.

Observation 2: In 5G-advanced, multiple slice-related mechanisms are introduced for the RAN and UE to prioritize slice operation.

Proposal 2: RAN2 WG shall study whether to reuse NR slice-related mechanisms including:

- NSAG information delivery and highest priority NSAG selection,

- Slice-based cell reselection, and

- RA prioritization for slicing.

RAN architecture

NR has flexible architecture options in terms of combination of LTE and NR entity/layers. However, it introduced a huge complexity in both standardization and development. In 6G, one or only a few promising options should be considered.

More specifically, NG-RAN is allowed to connect to either EPC or 5G Core. E-UTRAN (LTE) is also allowed to connect to either EPC or 5G Core. Those RAN-CN combination options introduced additional flexibility. For simplified 6G standards, it is reasonable to have a limit to only one RAN-CN architecture option. If 6G CN is standardized in SA, we can assume only 6G-RAN connected to 6G CN.

Observation 3: Multiple RAN-CN combination introduced a huge complexity in both standardization and development in RAN.

Proposal 3: RAN2 assumes 6GR/6G-RAN connects to only one core network type (e.g. 6G Core Network).

The 5G CU-DU split was designed to enable flexible C-RAN deployment by centralizing non-real-time functions (CU) and keeping real-time functions local for low latency (DU). However, this architectural promise has not materialized widely in commercial 5G deployments. The implementation complexity and high fronthaul requirements introduced by the CU-DU split often outweighed the theoretical gains in virtualization and resource pooling. Consequently, the overall real-world gain from the CU-DU split may be questionable. Given that 6G aims for massive simplification of RAN architecture, retaining the CU-DU split concept may be counterproductive.

To ensure the simplicity and efficiency of 6GR, the RAN2 6G study should not consider the impact or detailed concept of the CU-DU split unless RAN3 agree to support it. The study should strictly focus on defining essential RAN functionalities needed to meet fundamental 6G requirements from the RAN2 perspective.

Observation 4: Given that 6G aims for massive simplification of RAN architecture, retaining the CU-DU split concept may be counterproductive.

Proposal 4: RAN2 study does not assume CU-DU split in 6GR unless RAN3 agrees to support CU-DU split in 6GR.

Conclusion

Based on the discussion given above, RAN2 is requested to discuss the observations and agree the following proposals for the following 6GR study progress:

< Slicing and service awareness >

Observation 1: It is expected that RAN and UE would still share service-related information via S-NSSAI exchange.

Proposal 1: RAN2 WG shall study how to support enhanced service awareness for existing and new services based on the operation of RAN slicing, E.g., S-NSSAI exchange.

< NR slice-related mechanisms >

Observation 2: In 5G-advanced, multiple slice-related mechanisms are introduced for the RAN and UE to prioritize slice operation.

Proposal 2: RAN2 WG shall study whether to reuse NR slice-related mechanisms including:

- NSAG information delivery and highest priority NSAG selection,

- Slice-based cell reselection, and

- RA prioritization for slicing.

< RAN architecture >

Observation 3: Multiple RAN-CN combination introduced a huge complexity in both standardization and development in RAN.

Proposal 3: RAN2 assumes 6GR/6G-RAN connects to only one core network type (e.g. 6G Core Network).

Observation 4: Given that 6G aims for massive simplification of RAN architecture, retaining the CU-DU split concept may be counterproductive.

Proposal 4: RAN2 study does not assume CU-DU split in 6GR unless RAN3 agrees to support CU-DU split in 6GR.

References

[1] RAN#109 meeting chairman notes.

[2] R2#131bis meeting chairman notes.

[3] 3GPP TR 38.914 Study on 6G Scenarios and requirements