3GPP TSG-RAN WG2 Meeting #132	                                                             R2-2508234

Dallas, USA, Nov 17th – 21st, 2025

Source:	CATT

Title:	Considerations on 6GR QoS Framework

Agenda Item:	10.3.1.2

Document for:	Discussion and Decision

- Introduction

In RAN2#131bis, agreement on QoS is below:

Agreements on UP

4.	Study potential benefits and standardized mechanisms for applications/service-awareness in the RAN (e.g. immersive communications, AI mobile traffic).  Understand the issues and shortcoming with the current NR QoS.   Aim to look at a general framework

In this contribution, we share our views on 6G RAN-level QoS framework.

- Discussion

  - Traffic characterization of 6G

ITU recommended six usage scenarios for IMT-2030[1]: Immersive communications (IC), massive communications (MC), and HRLLC are extended based on the eMBB, eMTC, and URLLC scenarios from IMT-2020. In addition, integrated sensing and communications (ISAC), ubiquitous connectivity (UC) and integrated AI and communications are new scenarios.

Figure1. Usage scenarios for IMT-2030

For 6G QoS framework design, aims to meet the data transmission requirements of all traffics. Among the usage scenarios, IC and AI have distinct new traffic characteristics different from 5G traffics.

Immersive communication

5G XR has made valuable explorations in XR service awareness. 6G immersive communication has the similar and extended requirements on service awareness so as to guarantee the requirements of service continuity and improved user experience. For example, immersive XR inherits the packets formats of data burst, PDU sets and other service characteristics such as variable packet size. Accurate perception of service and packets characteristics (e.g. frame size distribution, frame interval, jitter, control input packet size distribution, packet-level priority) by RAN facilitates rational Uu configurations and optimal data scheduling. Conversely, under 5G, without knowing the traffic pattern, network may not be able to align the resource allocation with traffic arrival time and data size, etc. Therefore, 6G Day1 should support Service-Aware QoS management and scheduling.

Furthermore, the application layer may employ FEC (Forward Error Correction) coding to enhance transmission reliability and reduce retransmission latency. As noted in [2], error tolerance emerges as a potential new requirement for real time VR live service with deterministic user experience.

Table 9.11.1-1: Classification of immersive content Information; Sub-classification; Error-tolerant/non error-tolerant; Description. Audio (DL); Text information; Non error-tolerant; A person's voice can be converted into text and pitch.. Pitch information; Error-tolerant. Video (DL); Eye-tracking content; Non error-tolerant; The content of eye-tracking and fast- moving thing is much more important, while the relatively static information can be recovered by AI. Fast-moving object information; Non error-tolerant. Background information; Error-tolerant. Haptic (UL/DL); Haptic information; Non error- tolerant; Similar as described in TS 22.261 [14], the data size is small. When a communication link status is bad, the communication can transmit error tolerant data with high error rate or even no transmitting while still guaranteeing the low error loss rate for the non-error tolerant data, It hence greatly reduces the transmission burden while using the a proper algorithm (e.g. using AI model) to guarantee a deterministic user experience such as restore the missed content at the receiving end, which can achieve the same level of user experience (e.g. QoE) compared to a good communication link condition. NOTE: The Application (e.g. using the Grace algorithm) does not necessarily need to know the changed QoS for error tolerant data traffic before recovering the lossy content. Noting that the Error tolerant communication is at the cost of consuming much more energy consumption by the sending and receiving end, it is an option for deterministic user experience when the communication link status is bad but may not be always used. In other words, activating the “Error tolerant communication” is decided by application’s decision, taking into account the real time communication link status, energy consumption, and application layer capabilities. … [PR 9.11.6-4] Subject to operator policy and user consent, the 6G system shall provide a suitable means for communication with error tolerance to evaluate the high data transmission error rate and accept the data packet that fulfill the condition of high data transmission error rate (e.g. allow a percentage of error bits for a successfully delivered packet).

QoS can be dynamically adjusted to prioritize high-priority transmission for critical feature information (non error-tolerant packets) while relegating redundant information (error-tolerant packets) to lower-priority transmission. Additionally, under poor channel conditions, the transmitter may strategically drop error-tolerant packets to optimize overall performance.

Observation 1: Service Awareness is beneficial for addressing the data characteristics and service requirements of immersive communication, which impacts service transmission and scheduling.

Immersive communication has the characteristic of adaptive QoS. Its traffic patterns and characteristics dynamically evolve across different scenarios and applications. Immersive XR services exhibit diverse data attributes, including variable packet size, non-integer period, jitter and multiple streams. Furthermore, immersive services often comprise multiple media streams with distinct QoS requirements. These streams can be multiplexed within a single QoS flow, while their service characteristics may also undergo dynamic changes.

Error tolerance can also be considered as a requirement for adaptive QoS implementations.

In 5G, RAN nodes attend some QoS management such as alternative QoS for GBR and RAN feedback for adaptation of Burst Arrival Time and Periodicity. Looking ahead to 6G, RAN may need to establish tighter cooperation with SA2 to enable more efficient QoS management and parameter adjustments.

Observation 2: RAN2 should consider the impacts of adaptive QoS mechanisms for immersive services.

Multi-sensory telepresence and holographic communications further raise the requirements for multi-modal transmission especially for the time-synchronized data transmission. While multi-modality was addressed in 5G XR discussions, 5GS lacked native support - even for single-UE multi-modality scenarios. RAN faced challenges in accurately determining synchronization delay requirements for packet sequencing. Although application-layer solutions could approximately guarantee synchronized multi-modal data streams, a RAN-level approach is crucial for fine-grained management and maximizing system capacity for immersive communications. Looking ahead to 6G, Multi-modality support should be jointly addressed in both SA2 and RAN. RAN2 should closely track SA2 developments to enable RAN-level multi-modality solutions.

Observation 3: RAN2 can study RAN-level multi-modality solutions and coordination with SA2.

Overall, 6G immersive services present new challenges, include:

1) Service awareness and improved user experience;

2) Adaptive QoS management, including dynamic QoS adaptation performance requirements, QoS policy adjustments based on network conditions, and QoS adjustments at different granularities;

3) Synchronization requirements: Synchronizing multi-modal data streams of different media for each participant, as well as the data streams for coordinated participants.

Token communication

Beyond the immersive service characteristics mentioned above, AI introduces a new data dimension - tokens. As the machine-interpretable fundamental unit for LLM/VLM processing, tokenization serves as the foundation for model comprehension and language generation.

When AI data traverses networks, it manifests as token flows. Performance metrics should be evaluated at token granularity - for instance, the reliability of tokenized AI traffic should be assessed based on both the quantity and importance of successfully transmitted tokens.

For tokenized AI traffic, new QoS parameters may need to be introduced, such as TI (Token Importance), TER (Token Error Rate), etc.

These AI token-based QoS parameters should be systematically evaluated in both SA and RAN working groups.

Observation 4: Both SA and RAN should jointly address AI token-based QoS parameters (e.g., TI, TER) in their standardization efforts.

Proposals for traffic characterization

Based on the above analysis of IC and AI characteristics, RAN2 should study enhancements to service awareness and adaptive QoS mechanisms, including fine-grained QoS treatment and dynamic QoS adjustment.

Proposal 1: RAN2 study service awareness with fine-grained QoS treatment and adaptive QoS mechanisms with dynamic QoS adjustment.

In addition, error tolerance has been identified as a potential new requirement in SA1/SA2 discussions, applicable to both IC and AI services. RAN2 should explore enhancements to the QoS framework to enable error-tolerant capabilities.

Proposal 2: RAN2 study the following potential enhancements to address the error tolerance requirements in 6G:

- SDU discard policies.

- Retransmission mechanisms.

- Reordering and in-order delivery.

  - QoE

Traditional optimization approaches relying on QoS metrics have performance limitations in modern large-scale streaming environments. A persistent challenge remains in aligning user-level QoE with algorithmic optimization objectives. 5G network system supports measure QoE (QMC, QoE Measurement Collection) of the user, but the analysis is not too accurate because traffic statistics are used as the only approach. 5G RAN-visible QoE parameters include “Application Layer Buffer Level List” and “Playout Delay for Media Startup”.

To improve user experience, QoE prediction combined with real-time network status analysis should be leveraged to dynamically adapt QoS policies and optimize resource allocation strategies

Proposal 3: 6G should study RAN-visible QoE parameters and QoE prediction-driven RAN solutions.

  - QoS framework for 6GR

The 5G system introduced a flow-based QoS framework where the network performs flow-level QoS control. With Rel-18 enhancements, PDU sets were specifically introduced for XR services. This evolution extended RAN-controlled service characteristics beyond static per-flow information to now include burst-level and PDU set-level parameters. However, the current 5G QoS mechanism primarily addresses data transmission metrics (e.g., packet loss) rather than holistic user experience requirements.

Observation 5: 5G employs a flow-based QoS framework. QoS treatment granularity spans three levels: flow-level, burst-level, and PDU set-level.

Proposal 4: Maintain the flow-based QoS framework as the baseline for 6G.

Proposal 5: Continue supporting flow-level, burst-level, and PDU set-level QoS treatment in 6G.

6G must accommodate diverse data types (e.g., immersive communication and AI tokens as discussed in Section 2.1). QoS requirements may vary at per-packet or per-token granularity, meaning individual packets/tokens within a flow may require distinct QoS parameters or characteristics.

Proposal 6: RAN2 should evaluate whether 6G should support packet-level and/or token-level QoS treatment.

Proposal 7: Send LS to SA2 to evaluate the feasibility of following QoS related assumptions:

- Reuse of flow-based QoS framework in 6G.

- Retention of flow-level, burst-level, and PDU set-level QoS treatment.

- Introduction of packet-level and/or token-level QoS treatment.

- Provision of error tolerance to RAN.

- Conclusion

In this contribution, we provide our views on 6G QoS framework including below observations and proposals:

Observation 1: Service Awareness is beneficial for addressing the data characteristics and service requirements of immersive communication, which impacts service transmission and scheduling.

Observation 2: RAN2 should consider the impacts of adaptive QoS mechanisms for immersive services.

Observation 3: RAN2 can study RAN-level multi-modality solutions and coordination with SA2.

Observation 4: Both SA and RAN should jointly address AI token-based QoS parameters (e.g., TI, TER) in their standardization efforts.

Proposal 1: RAN2 study service awareness with fine-grained QoS treatment and adaptive QoS mechanisms with dynamic QoS adjustment.

Proposal 2: RAN2 study the following potential enhancements to address the error tolerance requirements in 6G:

- SDU discard policies.

- Retransmission mechanisms.

- Reordering and in-order delivery.

Proposal 3: 6G should study RAN-visible QoE parameters and QoE prediction-driven RAN solutions.

Observation 5: 5G employs a flow-based QoS framework. QoS treatment granularity spans three levels: flow-level, burst-level, and PDU set-level.

Proposal 4: Maintain the flow-based QoS framework as the baseline for 6G.

Proposal 5: Continue supporting flow-level, burst-level, and PDU set-level QoS treatment in 6G.

Proposal 6: RAN2 should evaluate whether 6G should support packet-level and/or token-level QoS treatment.

Proposal 7: Send LS to SA2 to evaluate the feasibility of following QoS related assumptions:

- Reuse of flow-based QoS framework in 6G.

- Retention of flow-level, burst-level, and PDU set-level QoS treatment.

- Introduction of packet-level and/or token-level QoS treatment.

- Provision of error tolerance to RAN.

- References

- Recommendation ITU-R M.2160-0 Framework and overall objectives of the future development of IMT for 2030 and beyond, ITU, 2023.11.

- 3GPP TR 22.870, Study on 6G Use Cases and Service Requirements, V0.4.0.