3GPP TSG-RAN WG2 Meeting #132	R2-2508236

Dallas, USA, Nov. 17th-21st

Agenda item: 10.3.1	.1

Source: Tejas Networks

Title: Optimization and Enhancements to F1 and NR UP Protocols for 6G

Document for: Discussion & Decision

# Introduction

In this contribution, we will point out a few issues related to user plane functionalities which are applicable to both TN and NTN deployments.

It has been agreed that consideration for deployment options other than standalone (SA) will be decided only after agreement is reached in RAN Plenary. Hence, the discussion that follows will be w.r.t to SA deployment option.

# 2	Some issues on user plane association

## 2.1	Mid-Haul Interface

For SA deployment model, two architectures are considered in 5G: monolithic architecture and dis-aggregated architecture. In dis-aggregated architecture, the network functions (NF) DU and CU are connected by F1 interface [2] which has both control and user plane functionalities.

From the product design and deployment point of view, F1 interface needs many optimizations. For example, there is a need to simplify the NR UP protocol [1] counterpart in 6G. Another example refers to strengthening of semantics of various IEs of Assistance Information Data (AID) PDU to allow for true interoperability. One more suggestion is to upgrade the DDDS PDU structure to help measure delay and / or jitter over the mid-haul.

We believe that study of the above issues will help for successful deployment of products based on 3GPP standards.

Proposal 1: Optimize and enhance F1 interface and NR UP protocol for 6G.

## 2.2	RLC

From protocol design standpoint, a general objective is to reduce protocol overhead incurred while transporting data over the air interface. In that context, we can see that both 4G and 5G have utilized two sequence number (SN) spaces: one at the PDCP layer (PDCP SN) and the other at the RLC layer (RLC SN). PDCP SN is aimed to provide in-sequence delivery during mobility while RLC SN serves to provide in-sequence delivery as well as reassembly of RLC segments at the receiver.

An argument in favor of existence of PDCP SNs is that they are the only means to provide in-sequence delivery during handover because RLC entity will be reset before handover. We believe that study should be conducted to understand how and when just one SN space can be sufficient to ensure in-sequence delivery during both handover and non-handover scenarios.

Proposal 2: Study to understand when and under what circumstances just one SN space is sufficient in the L2 protocol stack.

# Conclusion

Proposal 1: Optimize and enhance F1 interface and NR UP protocol for 6G.

Proposal 2: Study to understand when and under what circumstances just one SN space is sufficient in the L2 protocol stack.

# Reference

[1] 3GPP TS 38.425, `` NG-RAN; NR user plane protocol “

[2] 3GPP TS 38.473, `` NG-RAN; F1 Application Protocol (F1AP) “