3GPP TSG-RAN WG2 Meeting #132	R2-2508237

Dallas, USA, 17 – 21 November 2025

Agenda item:	10.2.3

Source:	Nokia

Title:	6G Services

WID/SID:	FS_6G_Radio - Release 20

Document for:	Discussion and Decision

# 1	Introduction

In this contribution, we discuss the support of some services in 6G (emergency services, fixed wireless access and AI services) and elaborate on the need for real-time monitoring of user plane performance.

# 2	Emergency Services

To minimize the need for fallback to 4G or 5G, basic telco services need to be supported in 6G. These includes emergency, critical and regulatory services such as emergency voice call, positioning and PWS. For instance, the US FCC 911 requirements define positioning accuracy for x/y location to be within 50 meters for 80% of the callers. The z-axis location accuracy is defined to be +/- 3 m for 80% of wireless 911 callers with z-axis capable handsets.

PWS will impact system information and paging design, while the extent to which positioning will impact 6GR depends on the accuracy requirements, the LCS architecture selected for 6G, and decision on reference signal design for 6GR RAT-dependent Positioning. Assuming the 5G LCS architecture and RAN architecture for UE Positioning are reused in 6G system, the protocols and procedures in basic positioning are expected to be similar to NR protocols. The simplest method is Enhanced Cell ID (E-CID) positioning, which refers to techniques which use additional UE measurements (DL) and/or 6G-NB measurements to improve the UE location estimate. In case more accurate positioning than E-CID positioning is needed, then the basic RAT-independent positioning methods supported in NR (not dependent on 3GPP reference signal design for positioning) may be available for 6G, e.g. A-GNSS, Barometric pressure sensor, WLAN, Bluetooth, and Terrestrial Beacon System.

In case the joint sensing and communication feature is supported, it may be possible to support basic positioning features under the joint sensing and communication/positioning given the strong interconnection between the sensing and positioning features. However, the design of reference signals (RS) and their associated procedures are anticipated to differ significantly from those used in NR positioning and hence has dependencies to RAN1 decisions on reference signal design.

Moreover, it is important to recognize the substantial wireless resource overhead required to attain high accuracy levels. 3GPP has assessed accuracy performance using reference signals with a bandwidth of 100 MHz, which represents the maximum bandwidth for FR1. However, it is noteworthy that such maximum bandwidth may not be always configured even for data communication except when the peak data rate is needed.

Proposal 1: 6GR shall be able to fulfil all regulatory requirements on emergency voice calls, positioning and PWS.

# 3	FWA

Fixed Wireless Access (FWA) in 2024 is the largest 5G use case after mobile broadband in terms of uptake and continues to attract wireless customers with the number of connections expected to grow by a factor 3 by 2030. With FWA becoming the primary traffic in operators’ networks, it is critical that on Day One of 6G, FWA as a primary service offered by operators can meet customer demand with a sustainable and profitable business model [RP-251983]. And while it is important to acknowledge that part of the success of FWA is that it relies on eMBB design, factoring in FWA requirements and characteristics from the start can only be beneficial. As a matter of fact, a more optimized version of FWA could reduce cost and enhances capacity & coverage by exploiting FWA CPE characteristics such as zero mobility, larger form factor supporting more RF components and larger arrays, external power supply… (all the unique RF and power characteristics, that stand out from all the other devices). Although this aspect was briefly discussed at the last meeting, since there was no agreement, we would like to reiterate how important it is to consider FWA devices.

Proposal 2: the specific characteristics of FWA devices (e.g. larger RF components, external power supply…) shall be made available to and taken into account by 6GR for an optimised support of FWA services.

# 4	AI Traffic

Every AI interaction, from generating text to analysing data, can rely on tokens [NVIDIA]:

-	Tokens are the small fragments into which data such as text, images, or audio are divided so that AI models can process and understand them. In text, for instance, the word “darkness” might be split into “dark” and “ness,” allowing the model to recognize and reuse patterns when it later encounters similar words like “brightness.” This process, called tokenization, transforms raw human input into structured digital units that models can interpret and learn from.

-	During training, models consume enormous quantities of tokens, often numbering in the billions or trillions. Each token helps the model learn how pieces of data relate to one another, and the more tokens it sees, the more capable it tends to become. After this initial training, models are fine-tuned on more specific datasets, such as legal or medical text, to strengthen their expertise in certain areas.

-	When a user interacts with a model during inference, their input is again broken down into tokens, processed, and then reconstructed into coherent words, images, or sounds.

-	From an economic perspective, tokens have become the currency of AI. They represent both the data and compute investment required to train models and the cost of using them. Many AI services charge based on how many tokens are processed.

Focusing on the inference phase, a number of contributions in both RAN1 and RAN2 have suggested RAN to be optimised for carrying tokens [R1-2505188] [R1-2505182] [R1-2505181] [R1-2507746] [R1-2507014] [R1-2507635] [R2-2506850] [R2-2506905] [R2-2506940] [R2-2507313].

While there is no doubt that for some AI servicestokens can be transmitted over the air for inference, several fundamental questions must be addressed before attempting to optimize the air interface for such traffic:

-	Token Usage: in the uplink text prompts are tokenized, and on the downlink, text responses are also tokenized. For other services, the presence of tokens is a lot less clear. For instance, for ChatGPT and Gemini, the UE can send video over WebRTC, as URLs or files. Images can be sent with base64 encoding encapsulated in JSON and none of the non-text uplink data is tokenized [OpenAI API] [Gemini API].

-	Transmission Overhead: tokens are typically small and exchanged in large numbers. Transmitting them individually would clearly be inefficient due to excessive overhead. Tokens should therefore be concatenated prior to transmission.

-	Encryption and Visibility: since most applications encrypt their traffic, token boundaries and contents may not be visible at the SDAP/PDCP layers. This implies the need for either a dedicated API to share traffic characteristics and/or the introduction of a new type of Traffic Flow Template (TFT).

-	Processing and Ordering Constraints: if tokens are processed strictly in sequence, reordering may not be possible. This could limit the ability to group or prioritize tokens based on their importance.

-	Dynamic Importance of Tokens: token importance may only become evident during inference, not at the tokenization stage. Consequently, the tokenizer on the transmitting side may be unable to determine importance in advance.

Given these unresolved questions, discussing tokenization aspects within RAN at this stage would be premature.  Furthermore, it's not just the volume of tokens that matters; it's the unique characteristics of AI traffic as a whole that can pose challenges for networks [Nokia]:

-	Bursty and unpredictable: one of the defining features of AI traffic is its burstiness. Unlike streaming a video, where data flows at a relatively constant rate, AI applications tend to generate short, intense bursts of data. For instance, every time you make a change with a picture editing tool, there's a spike in uplink traffic as your device sends the updated image to the cloud for processing. These spikes can reach up to tens of Mbps, but they only last for 1-2 seconds, making it essential for networks to allocate resources efficiently.

-	Real-time responsiveness: another crucial factor is latency. Low latency is critical for many AI applications, especially those involving real-time interactions like voice conversations. This puts additional pressure on networks to handle high volumes of data with lightning-fast responsiveness.

-	On-device processing vs. cloud: interestingly, different device manufacturers are taking varied approaches to AI processing, which directly impacts network traffic. Based on testing the latest commercial software of Apple, Google Pixel and Samsung devices, we discovered three approaches. Some devices favor on-device processing, which reduces network load but still requires significant uplink for non real-time cloud synchronization. Conversely, other devices heavily rely on cloud processing, generating more consistent network traffic. And finally, some take the middle ground, with some tasks processed on-device and others in the cloud. This diversity in approaches adds another layer of complexity to network planning and optimization.

Here also, having a clear characterisation of the traffic is crucial to design meaningful mechanisms in 6GR.

SA4 is expected to initiate a new study on Media Aspects for 6G Systems, which will explore the characteristics of AI-related traffic. Before SA4 concludes the study, having detailed discussion about the characteristisc of AI traffic in RAN WGs, especially on the notion tokens is highly premature. What seems safe to assume though is that 6GR should be designed to efficiently cope with highly dynamic traffic [RP-251125]] [R2-2508300].

Proposal 3: RAN WGs should wait for SA4 to conclude its study on Media Aspects for 6G Systems before initiating any discussion related to tokenization.

Proposal 4: 6GR shall be designed to efficiently cope with highly dynamic traffic.

# 5	Observability

The QoS framework for 6G services requires intelligence to deal with all the challenges and future scenarios foreseen in the coming decade, some of which have been discussed above. To achieve this, we have identified four key RAN capabilities: the identification of services, the characterisation of services, the monitoring of how service perform and the adjustments required to ensure that the QoS requirements of a given QoS flow or a group of QoS flows are fulfilled.

Here we would like to elaborate the monitoring further. The monitoring mechanisms currently defined in the QoE Measurement Collection (QMC) framework are good for network optimization and benchmarking purposes but not designed for immediate enhancement of a UE's user experience. The collection of real-time data is indeed for tracking user plane performance, such as fulfilment of a service-level agreement [RP-252227]. While data can be collected by different sources, the collection of real-time data and feedback at the UE ought to be addressed by RAN2.

Proposal 5: 6GR shall enable the monitoring of real-time performance of the user plane.

# 6	Conclusion

This document has proposed the following:

Proposal 1: 6GR shall be able to fulfil all regulatory requirements on emergency voice calls, positioning and PWS.

Proposal 2: the specific characteristics of FWA devices (e.g. larger RF components, external power supply…) shall be made available to and taken into account by 6GR for an optimised support of FWA services.

Proposal 3: RAN WGs should wait for SA4 to conclude its study on Media Aspects for 6G Systems before initiating any discussion related to AI traffic (including but not limited to tokenization).

Proposal 4: 6GR shall be designed to efficiently cope with highly dynamic traffic.

Proposal 5: 6GR shall enable the monitoring of real-time performance of the user plane.