3GPP TSG-RAN WG2 Meeting #132	                     		      R2-2508239

# Dallas, USA, Nov. 17th - 21st, 2025

Source:	CATT

Title:	Discussion on NCD-SSB for positioning

Agenda Item:	7.0.2.21

Document for:	Discussion and Decision

1 Introduction

There was a discussion on NCD-SSB at RAN2#131bis meeting but there is no conclusion yet.

R2-2507328	Correction on NCD-SSB Configuration for Positioning	China Telecom	CR	Rel-18	38.331	18.7.0	5524	-	F	NR_pos_enh2-Core

Needs to be revised for ASN.1 backward compatibility

Postponed

This paper will discuss whether NCD-SSB can be used for the spatial relationship of UL-SRS positioning.

2 Discussion

Spatial relationship of UL-SRS positioning has been introduced in Rel-16 which may include one of these signals:

- Serving cell Reference signal, e.g. SSB, NCD-SSB, CSI-RS, SRS;

- Neighbor cell SSB;

- DL-PRS.

SRS-SpatialRelationInfoPos-r16 ::=      CHOICE {

servingRS-r16                           SEQUENCE {

servingCellId                           ServCellIndex                                              OPTIONAL,   -- Need S

referenceSignal-r16                     CHOICE {

ssb-IndexServing-r16                    SSB-Index,

csi-RS-IndexServing-r16                 NZP-CSI-RS-ResourceId,

srs-SpatialRelation-r16                 SEQUENCE {

resourceSelection-r16                   CHOICE {

srs-ResourceId-r16                      SRS-ResourceId,

srs-PosResourceId-r16                   SRS-PosResourceId-r16

},

uplinkBWP-r16                           BWP-Id

}

}

},

ssb-Ncell-r16                           SSB-InfoNcell-r16,

dl-PRS-r16                              DL-PRS-Info-r16

}

SSB-Configuration-r16  ::=          SEQUENCE {

ssb-Freq-r16                     ARFCN-ValueNR,

halfFrameIndex-r16                  ENUMERATED {zero, one},

ssbSubcarrierSpacing-r16            SubcarrierSpacing,

ssb-Periodicity-r16                 ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2,spare1 }   OPTIONAL, -- Need S

sfn0-Offset-r16                     SEQUENCE {

sfn-Offset-r16                      INTEGER (0..1023),

integerSubframeOffset-r16           INTEGER (0..9)                                                 OPTIONAL  -- Need R

}                                                                                                      OPTIONAL, -- Need R

sfn-SSB-Offset-r16                  INTEGER (0..15),

ss-PBCH-BlockPower-r16              INTEGER (-60..50)                                                  OPTIONAL  -- Cond Pathloss

}

SSB-InfoNcell-r16  ::=              SEQUENCE {

physicalCellId-r16                  PhysCellId,

ssb-IndexNcell-r16                  SSB-Index                                                          OPTIONAL, -- Need S

ssb-Configuration-r16               SSB-Configuration-r16                                              OPTIONAL  -- Need S

}

DL-PRS-Info-r16  ::=                SEQUENCE {

dl-PRS-ID-r16                      INTEGER (0..255),

dl-PRS-ResourceSetId-r16           INTEGER (0..7),

dl-PRS-ResourceId-r16              INTEGER (0..63)                                                     OPTIONAL  -- Need S

}

It is clear that NCD-SSB of serving cell can be the Spatial RelationInfo for SRS Pos but NCD-SSB of neighbor cell cannot, according to the following table in TS 38.331:

Why NCD-SSB of neighbor cell cannot be the reference signal for SRS Pos?

NonCellDefiningSSB-r17 was introduced in Rel-17 for Redcap UE who has an RedCap-specific initial BWP that does not contain the CD-SSB or a dedicated BWP that does not contain the CD-SSB. This IE is configured to UE in BWP-DownlinkDedicated shown as below. However there is no mechanism that neighbor gNB can configure a BWP-DownlinkDedicated to the UE who belongs to the serving cell. So the NonCellDefiningSSB-r17 from neighbor cell as reference signal for SRS Pos is not feasible. On the other hand, the NCD-SSB may be UE-specific, so NCD-SSB of neighbor cell cannot be the reference signal for SRS Pos.

#### –	BWP-DownlinkDedicated

The IE BWP-DownlinkDedicated is used to configure the dedicated (UE specific) parameters of a downlink BWP.

BWP-DownlinkDedicated information element

-- ASN1START

-- TAG-BWP-DOWNLINKDEDICATED-START

BWP-DownlinkDedicated ::=           SEQUENCE {

pdcch-Config                        SetupRelease { PDCCH-Config }                                     OPTIONAL,   -- Need M

pdsch-Config                        SetupRelease { PDSCH-Config }                                     OPTIONAL,   -- Need M

sps-Config                          SetupRelease { SPS-Config }                                       OPTIONAL,   -- Need M

radioLinkMonitoringConfig           SetupRelease { RadioLinkMonitoringConfig }                        OPTIONAL,   -- Need M

...,

******skip irrelative parts*************************

dl-PPW-PreConfigToAddModList-r17    DL-PPW-PreConfigToAddModList-r17                                  OPTIONAL,   -- Need N

dl-PPW-PreConfigToReleaseList-r17   DL-PPW-PreConfigToReleaseList-r17                                 OPTIONAL,   -- Need N

nonCellDefiningSSB-r17              NonCellDefiningSSB-r17                                            OPTIONAL,   -- Need R

servingCellMO-r17                   MeasObjectId                                                  OPTIONAL -- Cond MeasObject-NCD-SSB

#### –	NonCellDefiningSSB

The IE NonCellDefiningSSB is used to configure a NCD-SSB to be used while the UE operates in an RedCap-specific initial BWP that does not contain the CD-SSB or a dedicated BWP that does not contain the CD-SSB.

Observation 1: NonCellDefiningSSB-r17 is configured via BWP-DownlinkDedicated which makes it UE specific. Consequently, utilizing the NonCellDefiningSSB-r17 of a neighbor cell as a reference signal for SRS Pos is not feasible.‌

Observation 2: Spatial RelationInfo for SRS Pos introduced in Rel-16 may include one of these signals:

- Serving cell Reference signal, e.g. SSB, NCD-SSB, CSI-RS, SRS;

- Neighbor cell SSB (but no NCD-SSB);

- DL-PRS.

All configurations of signals of serving cell have been provided to the target UE which are not configured again in srs-Config. Only the configuration of neighbor cell can be provided to UE in srs-Config.

So there is no need to provide any additional information of serving cell in srs-Config.

Proposal 1: Any additional information of serving cell (e.g. the NCD-SSB) is not provided to target UE. NCD-SSB of neighbor cell is not included in the Spatial RelationInfo for SRS Pos. Send an LS on this agreement to RAN1 for confirmation.

According to the SSB-InfoNCell field descriptions table, the field description of ssb-Ncell is not accurate because actually it includes the info of both serving cell and neighbor cell. So we propose to revise it :

SRS-SpatialRelationInfoPos field descriptions

ssb-Ncell This field indicates a SSB configuration from neighboring cell and the index of the SSB for a neighbour cell or NCD-SSB of the serving cell.

Proposal 2: RAN2 agree with the CR of Rel-19 in R2-2508240 which captures the following correction:

ssb-Ncell

This field indicates a SSB configuration from neighboring cell and the index of the SSB for a neighbour cell or NCD-SSB of the serving cell.

3 Conclusions

In this contribution, we have following observation and proposals on NCD-SSB:

Observation 1: NonCellDefiningSSB-r17 is configured via BWP-DownlinkDedicated which makes it UE specific. Consequently, utilizing the NonCellDefiningSSB-r17 of a neighbor cell as a reference signal for SRS Pos is not feasible.‌

Observation 2: Spatial RelationInfo for SRS Pos introduced in Rel-16 may include one of these signals:

- Serving cell Reference signal, e.g. SSB, NCD-SSB, CSI-RS, SRS;

- Neighbor cell SSB (but no NCD-SSB);

- DL-PRS.

Proposal 1: Any additional information of serving cell (e.g. the NCD-SSB) is not provided to target UE. NCD-SSB of neighbor cell is not included in the Spatial RelationInfo for SRS Pos. Send an LS on this agreement to RAN1 for confirmation.

Proposal 2: RAN2 agree with the CR of Rel-19 in R2-2508240 which captures the following correction:

ssb-Ncell

This field indicates a SSB configuration from neighboring cell and the index of the SSB for a neighbour cell or NCD-SSB of the serving cell.

4 Reference

- R2-2507704 Report from session on positioning and sidelink relay MediaTek

- R2-2507328	Correction on NCD-SSB Configuration for Positioning	China Telecom	CR	Rel-18	38.331	18.7.0

- 3GPP TS 38.331 Radio Resource Control (RRC) protocol specification (Release 18) V18.5.1