3GPP TSG-RAN WG2 Meeting #132	R2-2508264

Dallas, USA, Nov. 17th – 21st, 2025

Agenda Item:	8.6.2

Source: 	Fujitsu

Title:	Corrections to L1 event triggered measurement reporting

Document for:	Discussion and Decision

- Introduction

In last RAN2 meetings, some agreements on L1 event triggered measurement reporting are reached [1].

- The field ltm-CSI-ReportConfig-r19 within LTM-Candidate IE is changed to SetupRelease. - We clarify in field description that ltm-CSI-ReportConfig and ltm-CSI-ResourceConfig can only be configured within ServingCellConfig of the SpCell. (MAC-V03) If an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is modified or removed, RRC layer should trigger MAC layer to remove the measurement reporting entry, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId. (MAC-O02) The SR resource for (truncated) MR MAC CE has the same priority as the SR resource for BFR MAC CE. (MAC-O02) If the SR resource for (truncated) MR MAC CE overlaps with SR resource for BFR MAC CE (for an SCell or a BFD-RS set), it’s up to UE implementation which SR resource to be selected. Update the Note in the current MAC specification. (MAC-S02) Not agree to move beams from the BEAM_ENTERING_LIST and BEAM_LEAVING_LIST when they are reported in a truncated measurement report. Keep the current MAC spec. as it is. (MAC-H03) Proponent companies to comment the CR during post-meeting email discussion. Rapporteur will consider them accordingly. For MAC-H04 (2nd Part), take the below change as the starting point: Text in the current CR: 1> for each ltm-CSI-ReportConfigId included in the LTM-CSI-ReportConfig: is changed as: 1> for each ltm-CSI-ReportConfigId included in the SpCell for L1 measurement and event-triggered report:

This contribution will discuss remaining issues for L1 event triggered measurement reporting and provide our proposals.

- Discussions

Issue 1: definition of Event LTM2

In the latest rapporteur CR [2], the following changes are made to applicable beam for event LTM2 in clause 5.35.3.1:

The MAC entity shall: 1> for each ltm-CSI-ReportConfigId included in the SpCell for L1 measurement and event triggered measurement report: 2> if the corresponding ltm-ReportConfigType is set to eventTriggered and there is ltm-EventTriggeredReportContent configuration: 3> if the eventLTM2 is configured in the corresponding ltm-CSI-ReportConfigId: 4> if the associated LTM-CSI-ResourceConfig includes ltm-NZP-CSI-RS-ResourceSet: 5> consider the current beam of serving cell, i.e. the RS configured in the indicated TCI State of the SpCell, as defined in clause 5.1.5 in TS 38.214 [7], to be applicable. 4> else: 5> consider the current beam of serving cell, i.e. the SSB QCLed with the RS configured in the indicated TCI State of the SpCell, as defined in clause 5.1.5 in TS 38.214 [7], to be applicable.

We think that the same changes to the definition of event LTM2/ LTM3/LTM5 should be implemented in clause 5.35.3.2, 5.35.3.3 and 5.35.3.5. For example, the corresponding TP is shown below:

Ms is the beam measurement quantity of the serving cell based on SS/PBCH block or CSI-RS, not taking into account any offsets. The beam associated with this event is the current beam, i.e. corresponding to the RS configured in the indicated TCI state if the associated LTM-CSI-ResourceConfig includes ltm-NZP-CSI-RS-ResourceSet or the RS QCLed with the RS configured in the indicated TCI state in the serving cell as defined in clause 5.1.5 in TS 38.214 [7] otherwise.

Proposal 1: Align the description of serving RS for Event LTM2/3/5 to 5.35.3.1, i.e., to agree above TP for Event LTM2 in 5.35.3.2, Event LTM3 in 5.35.3.3 and Event LTM5 in 5.35.3.5.

Issue 2: UE behaviour when current beam is reconfigured

RAN2 has agreed that If an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is modified or removed, RRC layer should trigger MAC layer to remove the measurement reporting entry, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId. This has been implemented in the latest rapporteur CR [2], clause 5.35.1. With this agreement and above changes, the UE behaviour is clear when candidate beam configuration or reporting configuration is modified or removed.

In addition to candidate beams, current beam is associated with LTM events. In our opinion, similar handling needs to be applied if current RS is reconfigured by RRC. The following TP to clause 5.35.1 can be an example for the change:

For L1 measurement and event triggered L1 measurement reporting, if an ltm-CSI-ReportConfig or an ltm-CSI-ResourceConfig associated with that ltm-CSI-ReportConfig is removed or modified from the current UE configuration, as specified in TS 38.331 [5], or if current beam is reconfigured by RRC, the MAC entity shall: 1> remove the measurement reporting entry for the corresponding ltm-CSI-ReportConfigId from the MR_LIST, if included: 1> stop the periodical reporting timer, if running, and reset the associated information (e.g. TTT and variables defined in 5.35.3) for the corresponding ltm-CSI-ReportConfigId.

Proposal 2: If current beam is reconfigured by RRC, MAC layer will remove the measurement reporting entry, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId.

- Conclusion

This document discussed the remaining open issues for the L1 event triggered measurement report and made the following proposals:

Proposal 1: Align the description of serving RS for Event LTM2/3/5 to 5.35.3.1, i.e., to agree above TP for Event LTM2 in 5.35.3.2, Event LTM3 in 5.35.3.3 and Event LTM5 in 5.35.3.5.

Proposal 2: If current beam is reconfigured by RRC, MAC layer will remove the measurement reporting entry, stop the periodical reporting timer, and reset the associated information (e.g. timeToTrigger) for this ltm-CSI-ReportConfigId.

# Reference

- R2-2507701, “Report from session on R18 SL, R18/19 MOB, and R19 NES”, Session chair (Ericsson) Report, RAN2#131bis

- R2-2507728, “Miscellaneous corrections on MAC for Mob Ph4”, vivo (Rapporteur), RAN2#131bis