3GPP TSG-RAN WG2#132	R2-2508267

Dallas, USA, Nov 17th – 21st 2025

Source: 	Fujitsu

Agenda item:	9.3.3

Title:	Discussion on measurement event prediction

Document for:	Discussion and decision

# Introduction

The WID of Rel-20 Study on 6G Radio was approved in RAN#108 and updated in RAN#109 [1].

- Specify support for RRM measurement prediction and measurement event prediction for UE sided models [RAN2]: - Temporal domain prediction case A and case B, and Frequency domain prediction for co-located cases are supported. - L3 Cell level predication is supported Note 1: For UE sided model, spatial domain prediction is not supported. Note 2: For UE sided model, L3 beam level prediction is not supported. - For both RRM measurement prediction and measurement event prediction, specify signalling and protocol aspects to enable LCM functionality management for UE sided model including [RAN2]: - UE capability request and response procedure - Applicability report for both full and partial configuration approach - Inference configuration and inference report - Performance monitoring performed in UE or network side, based on which network can manage UE sided functionality - UE sided data collection - For measurement event prediction: - Measurement event A1~A6 are in the scope in this release Note3: Data transfer and model delivery for UE sided model are out of scope of this WI. And model related choices can be up to UE’s implementation. - For RRM measurement prediction, specify signalling and protocol aspects to enable LCM functionality management for network sided model including [RAN2]: - Inference input report for at least RRM sub-case 2 of intra-frequency temporal domain case A for L3 cell prediction - Inference input report for RRM sub-case 5 of all scenarios for L3 beam level prediction - Network sided data collection for all scenarios and all RRM sub-cases of both L3 cell level and L3 beam level prediction, at least - UE is configured to log RRM measurement results. - UE supports periodical logging and L3 event triggered logging. - UE can indicate logged data availability. Note 4: All scenarios in this objective refer to intra-frequency domain case A and case B, inter-frequency prediction and spatial domain prediction. Note5: The above objectives are to be based on the LCM framework for Rel-19 AI/ML for NR air interface WI as much as possible.

In this contribution, we would like to provide our views on measurement event prediction.

# Discussion

## UE capability on RRM measurement prediction

In Rel-19, measurement event prediction was studied, and the system level performance was evaluated by using the predicted measurement event under different handover models [2], and the performance difference can be identified between different handover models. If network treats the predicted measurement event as legacy event, and performs handover preparation and execution immediately when the measurement event is received, the performance will fully depend on the prediction accuracy. At the same time, too early HO may be triggered. With the awareness of the event prediction, NW may enhance the robustness of predicted event based HO and improve system level performance. For example, NW may start/stop the event prediction based on the KPI of HO performance, or require UE to report events, both the predicted and the actual detected, for joint consideration of HO decision. Therefore, the measurement event prediction by UE is required to be under NW control.

Proposal 1: NW has the control of UE performing measurement event prediction.

According to Rel-20 AI/ML for mobility WID [1], LCM functionality management related signalling for UE sided model needs to be specified for measurement event prediction, and event A1 ~ A6 are in the work scope. By following the legacy event report behaviour, when the event is predicted based on the legacy enter condition and time to trigger requirement, UE will keep on reporting the event periodically. Considering the prediction of the measurement event is intended to notify NW earlier than the real happening time, whether the TTT is still required or same as that of the legacy event needs to be further considered. In addition, NW may wait for the ground truth event associated to the predicted one to decide the HO execution. Before ground truth event is received, the corresponding predicted event reports will keep ongoing over the air interface, which may introduce too much signalling. Therefore, NW may provide trigger configuration for a predicted event report, which can be different from the condition for reporting the ground truth event.

Proposal 2: NW configures trigger condition for UE to report a predicted measurement event.

When NW considers the predicted event is trustable enough, NW may use the predicted event directly for HO decision, while NW may also trigger HO execution by using the ground-truth event along with the predicted event to make sure the HO is indeed necessary. Whether a ground-truth measurement event will be used depends on NW implementation, and UE has no idea about it. To avoid the unnecessary event report, NW may indicate UE whether to report a ground-truth measurement event when inference for measurement event has been started by the UE.

Proposal 3: NW configures whether to report a ground-truth measurement event when inference has been started.

And if both ground-truth measurement event and predicted event are required to be reported, NW may need to be aware whether an event report is due to UE prediction or UE measurement, then the NW can decide whether to start the HO execution, e.g., handover of model 1 and model 2 in [2]. Therefore, UE is required to indicate whether a measurement event report is triggered based on AI/ML prediction.

Proposal 4: UE indicates whether a measurement event report is triggered based on AI/ML prediction.

When UE sends a measurement event report triggered by a predicted event, by indicating when a predicted measurement event is foreseen to happen, NW will understand the reported event is a predicted one, then the NW may suspend the HO procedure to wait the ground truth event, or check whether the event enter condition can be fulfilled by the ground truth results. Therefore, we think including information of predicted time at which a measurement event will happen in predicted measurement event report can be considered.

Proposal 5: RAN2 to discuss including information of predicted time at which a measurement event will happen.

# Conclusion

In this paper we presented some views on measurement event prediction for AI mobility and have come up with the following proposals:

Proposal 1: NW has the control of UE performing measurement event prediction.

Proposal 2: NW configures trigger condition for UE to report a predicted measurement event.

Proposal 3: NW configures whether to report a ground-truth measurement event when inference has been started.

Proposal 4: UE indicates whether a measurement event report is triggered based on AI/ML prediction.

Proposal 5: RAN2 to discuss including information of predicted time at which a measurement event will happen.

# References

- RP-252899, Revised WI: Artificial Intelligence (AI)/Machine Learning (ML) for mobility in NR

- 3GPP TR 38.744, Study on Artificial Intelligence (AI)/Machine Learning (ML) for Mobility in NR