3GPP TSG-RAN WG2 Meeting #132	R2-2508275

Dallas, USA, 17th - 21th November 2025

Agenda Item:	8.9.1

Source:	Huawei, HiSilicon

Title:	Discussion on RAN1 LS on CB-Msg3 EDT

Document for: Discussion and Decision

1	Introduction

RAN2 agreed to support 2 MPDCCH narrowbands for CB-Msg4 monitoring and sent an LS to RAN1 for them to check the RAN1 specification impact [1]. In the reply LS [2], RAN1 confirms that there is on concerns with supporting two MPDCCH narrowbands, but RAN2 should decide on a mechanism to ensure the UE is monitoring a unique narrowband.

## Support narrow band MPDCCH configuration for CB-Msg3-EDT RAN2 ask RAN1 to check the specification impact in RAN1 to support 2 MPDCCH narrowbands for CB-Msg4 monitoring, and update the corresponding specification if needed. RAN1 response for support MPDCCH narrowband configuration for CB-Msg3-EDT: - From RAN1 perspective, introducing 2 MPDCCH narrowbands requires additional RAN1 standard effort. RAN1 would like to inform RAN2 that there are no concerns with supporting two MPDCCH narrowbands, but RAN2 should decide on a mechanism to ensure the UE is monitoring a unique narrowband.

This contribution discussed how to make sure UE monitors a unique narrowband.

2	Discussion

In legacy RACH procedure, it is possible that NW configures two MPDCCH narrow bands for RAR reception. The multiple MPDCCH narrowbands are configured in PRACH-Config to schedule Random Access Response. From UE perspective, it only needs to monitor one MPDCCH narrowband. Which narrowband (NBRAR) to monitor for MPDCCH RAR reception is defined in TS 36.213 as follows:

Table 6.2-E in TS 36.213: Narrowband () for MPDCCH RAR.

| Mapped Preamble Index | NBRAR |
| --- | --- |
| Mod (Preamble Index, 2) =0 | First narrowband configured by high layer parameter mpdcch-NarrowbandsToMonitor-r13 |
| Mod (Preamble Index, 2) =1 | Second narrowband configured by high layer parameter mpdcch-NarrowbandsToMonitor-r13 |

As can be seen from above, the narrowband (NBRAR) for MPDCCH RAR in legacy RACH procedure is determined by the preamble index. However, since there are no Msg1 and Msg2 in CB-Msg3-EDT, we need to define other means to determine which narrowband to monitor. As far as we can consider, there are two candidate solutions following the similar logic with the legacy procedure:

- Solution 1: determine based on UE ID

In CB-MSG3, UE ID will be carried to let the NW identify the UE. In this case, NW and UE can use the UE ID to assign a specific UE to one of the two narrow bands. Specifically, if mod (UE ID, 2) =0, UE selects the first MPDCCH narrowband configuration for CB-Msg4 monitoring. If mod (UE ID, 2) =1, UE selects the second MPDCCH narrowband configuration for CB-Msg4 monitoring.

- Solution 2: determine based on the index of CB-Msg3 resource within a transmission window

During CB-Msg3-EDT procedure, the UE will first select a Msg3 transmission window and then randomly select a PUSCH resource for CB-Msg3 transmission. Although there is no explicit index in the configuration CB-Msg3 PUSCH resource, both UE and NW know which CB-Msg3 resource the UE used to transmit the CB-Msg3, e.g., the first one or the second one and so on. This index can be used to determine which narrowband to monitor CB-Msg4 similar as legacy, i.e., using mod (resource index, 2) to determine.

Both solutions can achieve what RAN1 asks. So it is proposed RAN2 select one solution for this issue:

Proposal 1: RAN2 to down select one solution from the following to determine which narrowband to monitor for CB-Msg4 reception in case two narrowbands are configured：

- Solution 1: determine based on UE ID

- Solution 2: determine based on the index of CB-Msg3 resource within a transmission window

3	Conclusion

In this contribution, we discussed how UE can determine which narrowband to monitor for CB-Msg4 reception in case two narrowbands are configured and have the following proposal:

Proposal 1: RAN2 to down select one solution from the following to determine which narrowband to monitor for CB-Msg4 reception in case two narrowbands are configured：

- Solution 1: determine based on UE ID

- Solution 2: determine based on the index of CB-Msg3 resource within a transmission window

3	Reference

[1] R2-2506284 LS on power ramping and RRC configuration for CB-Msg3-EDT

[2] R1-2508114 Reply LS on power ramping and RRC configuration for CB-Msg3-EDT