3GPP TSG-RAN WG2 Meeting #132                                                                          R2-2508285

Dallas, USA, Nov. 17th - 21st 2025

Agenda item:	10.3.3.1

Source: 	ZTE Corporation, Sanechips

Title: 	Views on data transfer and AI/ML framework

Document for:	Discussion and Decision

Introduction

During RAN2#131bis meeting, the common user plane and control aspects were discussed and the following agreements were reached [1]:

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who are producers, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange; - Applicable Functionality Reporting; - Inference Configuration and Reporting; - Performance Monitoring Configuration and Reporting; - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

In this paper, we will discuss the data transfer, model transfer, AI/ML framework and use cases study. The general principles and design considerations are analyzed and presented.

Discussion

  - Data transfer

2.1.1 Potential use cases

In this section, we focus on various use cases for AI for network, sensing, SON/MDT and QoE respectively.

- AI/ML for network

With regard to AI for network, as shown in Figure 1, the following three data collection cases for offline AI/M model training can be considered:

- UE side model: UE may report AI training data towards UE server via RAN/CN/OAM for UE-side AI model training and receive the well-trained UE side AI/ML model from UE server via RAN/CN/OAM. Moreover, when multiple AI/ML models are available, more suitable AI/ML model can be selected and transferred to UE based on current situation.

- Network side model: For network-side model training, both RAN and OAM-centric data collection can be considered. The RAN-centric data collection means that RAN configure the UE to initiate/terminate the data collection procedure and RAN collect AI training data from UE for the network side model training. For the OAM-centric data collection, OAM provides the configuration via RAN needed for the UE to initiate/terminate the data collection. The AI training data collected from UE will be delivered by RAN to OAM. Upon completion of the network side model training, OAM will send the network side model to RAN.

- Two-sided model: RAN can collect AI training data from UE and UE receive the well-trained model from RAN.  For the network side training, both RAN and OAM-centric data collection and transfer can be considered. In addition, RAN/OAM/CN may also send data set toward UE/UE server for UE side training. For the UE side training, the AI training data report similar to UE side model training can be reused.

Figure 1 Data collection for offline AI/ML model training of AI for network scenarios

The data type and endpoint pair for these three offline AI/ML model training cases are listed in Table 1. Moreover, the potential data size and transmission interval are also listed. As we can see, the data size of AI training data ranges from several MB to tens of GB, which depends on the number of model parameters and varies greatly for different models. It should be noted that the training data may be collected from lots of UEs and span a long time duration. For the data size of AI/ML model, it depends on the implementation of different vendors, which ranges from tens of KB to hundreds of MB. Furthermore, there is no latency requirement for AI training data transfer but the latency requirement for AI/ML model transfer varies for different use cases.

- Sensing

Generally speaking, six sensing modes need to be supported from 6G Day1. To be specific, BS mono-static, BS-BS bi-static, BS-UE bi-static, UE-BS bi-static, UE mono-static, UE-UE bi-static (without sidelink), as shown in Figure 2. On the other hand, sensing can be used for two cases, i.e. sensing assisted communication and service-triggered sensing (e.g. UAV detection, environment detection, etc.).

Figure 2 Illustration of sensing modes

- Sensing assisted communication: RAN may collect the sensing measurement data from UE to assist the scheduling or mobility control. Here the sensing measurement data may be generated by UE via sensing mode of BS-UE bi-static and UE-UE bi-static.

- Service-triggered sensing: the data collection may be triggered by Sensing Function(SF) within the 6G network. As a matter of fact, it is not yet clear where the SF resides. The details depends on the architecture progress of SA2/RAN3. Nevertheless, SF may collect the sensing measurement from UE for the sensing modes of BS-UE bi-static, UE mono-static and UE-UE bi-static. Moreover, SF may collect the sensing measurement from BS for the sensing modes of  BS mono-static, BS-BS bi-static, and UE-BS bi-static. Based on the collected sensing measurement, SF processes the sensing data and then sends the sensing result to UE and or BS if necessary.

As shown in Figure 3, the data size for the raw sensing data, intermediate data at different split points of data processing and the final sensing result varies greatly. In order to reduce the data size to be reported, split sensing data processing can be considered where UE/RAN pre-processes the sensing measurement data. In this case, the intermediate sensing data and even sensing result can be reported by UE/RAN towards the sensing function. According to the study of NR ISAC, these sensing measurement data can be divided into 6 levels and the data size ranges from several GB to several KB as follows.

The following measurements are identified in the study of NR ISAC: - Level 1: Raw data, e.g. subcarrier-domain samples of the channel response from each antenna port (~several GB) - Level 2: Partial raw data, e.g. amplitude and phase profile of delay, Doppler, and/or angle (~hundreds of MB) - Level 3: Intermediate sensing results per detected path, e.g. Delay, Doppler, angle, and power per detected path (~tens of KB) - Level 4: Intermediate sensing results per detected point, e.g. {Position,velocity,power} or {Range,velocity,angle,power} per point (~ tens of KB) - Level 5: Final sensing results per detected object/target, e.g. {Position,velocity} (~several KB) - Level 6: Final sensing results per detected object/target, e.g. {Position,velocity,target ID} (~several KB) - SINR, RSRP from perspective of sensing

Figure 3 Data size at different split points of sensing data processing

It should be noted that for the service triggered sensing with UE based measurement, the transmission interval of sensing measurement depends on the speed of target. The short transmission interval(e.g. 32ms) is applicable for the high speed target detection while the long transmission interval is applicable for the low speed target detection. Moreover, the low reliability is needed for level 1~4 sensing data transfer while high reliability is needed for level 5-6 sensing data data transfer.

- SON/MDT

MDT introduced starting from 3GPP Rel-10, aims to enable operators to use UE to collect network measurement data, thereby reducing the need for traditional drive tests and achieving the goal of lowering operators’ network optimization and maintenance costs. There are following two modes for the MDT measurements:

- Logged MDT: UE logs measurements in idle mode, together with timestamps and, optionally, accurate location information. The reporting of the log to RAN can be done when the UE is in connected mode. The UE ﬁfirstly provides only an indication that MDT data is available. It is then up to the network to decide whether to retrieve the log at this point in time or later. Once retrieved, the MDT log is transferred to TCE.

- Immediate MDT: UE makes measurements in connected mode. Immediate MDT reuses the existing measurement configuration and reporting procedures for RRM purposes. The UE reports the measurements to the network periodically or event triggered. The measurements data is saved as trace file and is transferred to the TCE.

For both modes, the data size may range from tens to thousands of bytes. For the immediate MDT, the periodicity for the data report ranges between 120ms to 30min. High reliability and relaxed latency is assumed for the MDT data transmission.

- QoE

In 5G NR, the QoE function focuses on user-centric service experience by standardizing the collection, reporting, and utilization of application-layer experience data. To be specific, the QoE measurement collection is supported for the DASH, MTSI and VR service. Upon receiving the request from OAM or CN, RAN may configure UE to perform the QoE measurement and report. The collected application-layer QoE report is formatted in XML and transparent to RAN. RAN sends the QoE report to the MCE(Measurement Collector Entity).

Moreover, RAN visible QoE measurements can be configured at the UE by RAN, where a subset of configured QoE metrics is reported from the UE to the RAN as an explicit IE readable by the gNB. The RAN visible QoE measurements can be used by the RAN for network optimization purpose.

The data size and transmission interval for QoE report are captured in Table 1. As we can see, the maximum QoE report can be 144000B and it can be reported to RAN with segmentation (i.e. max 8000B for one segments) via RRC signalling. The transmission interval can be configured in seconds level. For the RAN visible QoE measurement, the available data size and transmission interval is smaller than that of application-layer QoE report.

RAN/OAM/CN may also send data set toward UE/UE server for UE side training.

Table 1 Summary of data transfer requirements in 6G of various use cases

| Scenario | Use cases | Data Type | Endpoint pairs | Data size | Transmission Interval | QoS |
| --- | --- | --- | --- | --- | --- | --- |
| AI/ML NOTE1 | UE-side model training | Training data | UE->UE server (via RAN, CN, or OAM) | Several MB to tens of GB NOTE2 | Upon network request | no latency requirement, medium/high reliability |
|  |  | UE-side model | UE server->UE (via RAN, CN, or OAM) | Tens of KB to hundreds of MB | Infrequent update | High latency, medium/high reliability |
|  | Network-side model training | Training data | 1)UE->RAN, 2)UE->RAN->OAM,RAN->OAM | Several MB to tens of GB NOTE2 | Upon network request | no latency requirement, medium/high reliability |
|  |  | Network-side model | OAM->RAN | Tens of KB to hundreds of MB | Infrequent update | High latency, medium/high reliability |
|  | Two-side model training | Training data | 1)UE->RAN, 2)UE->RAN->OAM, RAN->OAM | Several MB to tens of GB NOTE2 | Upon network request | no latency requirement, medium/high reliability |
|  |  | Data set for UE side training | 1)RAN/OAM/CN->UE, 2)RAN/OAM/CN->UE server | Tens of KB to hundreds of MB | Upon network request | High latency, medium/high reliability |
|  |  | UE-side model | - OAM->RAN->UE 2) UE server->UE | Tens of KB to hundreds of MB | Infrequent update | Medium or low latency, medium/high reliability |
| Sensing | Sensing assisted communication with UE based measurement | Sensing measurement | UE->RAN | Level 1-6 NOTE4 | 32ms~160ms or event triggered | Low latency, low reliability for level 1-4, high reliability for level 5-6 |
|  | Service-triggered sensing with UE based measurement | Sensing measurement | UE->SF NOTE3 | Level 1-6 NOTE4 | 32ms~160ms or event triggered NOTE5 |  |
|  | Service-triggered sensing with TRP based measurement | Sensing measurement | RAN->SF NOTE3 | Level 1-6 NOTE4 | ~80ms or event triggered |  |
| MDT | Logged measurement | Logged L3 measurement | UE->RAN->(OAM)->TCE | Tens to thousands of bytes NOTE6 | Upon network request | High latency, high reliability |
|  | Immediate measurement | M1~M9 measurement | UE->RAN->(OAM)->TCE | Tens to thousands of bytes | 120ms~30min or event trigger | High latency, high reliability |
| QoE | MTSI/DASH/VR | APP layer measurement | UE->RAN->MCE | Max 144000 bytes with segmentation | Seconds | High latency, high reliability |
|  |  | RAN visible measurement | UE->RAN | Hundreds of bytes | 120ms~1024ms | Low latency, high reliability |
| NOTE1: Only offline AI/ML model training use cases are captured in this table. More AI/ML use cases can be included if necessary. NOTE2: The training data depends on the number of model parameters which vary greatly for different models. Moreover, the training data may be collected from lots of UEs and span a long time duration. NOTE3: The location of SF is FFS. The details depends on the architecture progress of SA2/RAN. NOTE4: The following measurements are identified for NR ISAC. The data rate ranges from several GB to several KB. - Level 1: Raw data, e.g. subcarrier-domain samples of the channel response from each antenna port (~several GB) - Level 2: Partial raw data, e.g. amplitude and phase profile of delay, Doppler, and/or angle (~hundreds of MB) - Level 3: Intermediate sensing results per detected path, e.g. Delay, Doppler, angle, and power per detected path (~tens of KB) - Level 4: Intermediate sensing results per detected point, e.g. {Position,velocity,power} or {Range,velocity,angle,power} per point (~ tens of KB) - Level 5: Final sensing results per detected object/target, e.g. {Position,velocity} (~several KB) - Level 6: Final sensing results per detected object/target, e.g. {Position,velocity,target ID} (~several KB) - SINR, RSRP from perspective of sensing NOTE5: For the service triggered sensing with UE based measurement, the transmission interval of sensing measurement depends on the speed of target. The short transmission interval(e.g. 32ms) is applicable for the target detection in high speed while the long transmission interval is applicable for the target detection in low speed. NOTE6: The minimum memory size of MDT logged measurements for UE is 64KB(TS 38.306). |  |  |  |  |  |  |

Proposal 1	RAN2 to capture the comparison on data type, endpoint pairs, data size, transmission interval and QoS requirement (summarized in Table 1) of various use cases (e.g. AI/ML, sensing, MDT, QoE) to the TR as baseline for further unified data transfer study.

2.1.2 Data collection and transfer design

According to the analysis in Section 2.1.1, the endpoint pairs for AI/ML, sensing, MDT and QoE data transfer are more flexible than that of traditional user plane. As shown in Table 1, the endpoint pairs can be UE-UE server, UE-RAN, UE-OAM, UE-SF, UE-TCE, UE-MCE, RAN-OAM, RAN-SF, etc. For the endpoint pairs like UE-UE server, UE-OAM, UE-SF, UE-TCE, UE-MCE, the transferred data needs to be forwarded by the RAN, CN, and/or OAM. For example, the UE’s RSRP/RSRQ measurement may be collected for different uses cases, e.g. AI/ML model training and inference, sensing and MDT. It means that one data producer (e.g. UE) may be associated with multiple data consumers (e.g. RAN, OAM, SF, CN). These data consumers have overlapped requirements on the data to be collected(e.g. RSRP, RSRQ) with slight difference on the data granularity and transmission frequency. Based on this observation, it is better for RAN to have visibility of the detailed data to be collected and then aggregate these data collection requirements from multiple data consumers for different use cases. For the overlapped data collection, it can be merged into one unified data collection configuration and UE only need to report them once.  By doing so, the redundant data collection configurations and data reports can be avoided. Upon receiving the data report from UE, RAN can customize the data based on each data consumer’s requirements and then distribute the data towards the data consumers correspondingly.

Observation 1	One data producer(e.g. UE) may be associated with multiple data consumers (e.g. RAN, OAM, SF, CN). The data collection requirements from these data consumers may be overlapped with slight difference on data granularity and transmission frequency.

Proposal 2	It is suggested that RAN have visibility to the data collection requirements from multiple data consumers (e.g. RAN, OAM, SF, CN) for various use cases (e.g. AI/ML, sensing, MDT, QoE, etc.) and provide the UE with unified data collection and report configurations.

With regard to the data transfer between UE and RAN, the following design aspects should be considered:

- Radio bearer type:

- SRB: SRB is used for high priority signalling transmission. Only priority is associated with SRB. The differentiated QoS requirements for data traffic such as the latency, reliability and guaranteed data rate, are not supported by SRB. Moreover, the maximum size of UL and DL RRC signalling is 144kB and 45kB respectively, which can accommodate the large volume AI/ML or sensing data . Furthermore, the transmission interval of sensing data may be tens of ms, such frequent transmission via SRB may cause signalling storm.

- DRB: DRB is designed to support traffic transmission with different QoS requirements. Compared to the SRB, it is more suitable for the large volume data transmission. However, traditional DRB is associated with PDU session terminated at UPF which is not the case for the AI/ML/sensing/MDT/QoE data. Some enhancements should be considered if DRB is used for the data transfer terminated between UE and RAN.

- Data format:  For the data collected from UE, it is necessary to determine how to format these data. As we know, ASN.1 is widely used to define the data structure of RRC signalling. Suppose SRB is used, it is natural to reuse ASN.1 to format the AI/ML/sensing/MDT data. In 5G QoE, the APP layer measurement in QoE is delivered via RRC as a container and the container is an octet string formatted as an XML document. On the other hand, JSON is used for the application layer encoding of service based interface in 5GC. When it comes to the 6G data transfer, it is necessary to decide which data format should be used.

- Protocol: Suppose SRB is used for the data transfer, RRC signalling can be used to deliver the collected data. However, if DRB is used, it is necessary to further consider whether the collected data will be encapsulated as IP packet  or non IP packet.

Suppose IP packet is used, IP segmentation and IP based QoS flow mapping can be reused. However, for the non-IP packet, the packet segmentation/reassembly needs to be addressed. As we know, the maximum supported size of a PDCP SDU is 9000 bytes in 5G. Suppose the non-IP packet size is larger than 9000 bytes, it needs to be segmented on top of PDCP so as to fit the 9000 bytes limitation of PDCP.

- QoS handling: Suppose SRB is used for the data transfer, multiple SRBs with different priority may be configured. How to map different data typed to SRBs needs to be studied. On the other hand, for the DRB based data transfer, SDAP can be reused for the mapping from IP packet based QoS flow to DRB. How to map different non-IP packets to DRB needs to be studied, which depends on SA2’s work and should be discussed with SA2 jointly.

- Compression: Due to the large volume of AI training data, AI model and sensing measurement data, it is expected that compression can be supported. As far as we know, UDC is supported in PDCP for the uplink data  compression in 5G. It is necessary to investigate whether the UDC can be reused to support the DL data compression.

- Encryption/decryption: According to the 6G requirements in [2], security and privacy protection should be supported for data transfer. Generally speaking, the encryption/decryption mechanism defined in PDCP can be reused.

Based on the above analysis, DRB is more suitable for the large volume and frequent data transfer. RAN2 is suggested to use DRB as baseline for the data transfer with some enhancements. With regard to the data format and protocol design, more WGs should be involved considering that the data consumer may be RAN/CN/OAM/UE server. Joint discussions with RAN3/SA2/SA5 are needed.

Observation 2	Compared with SRB, DRB is more suitable for the large-volume and frequent data transfer. However, traditional DRB is associated with PDU session terminated at UPF and needs to be enhanced for data transfer.

Observation 3	The data format and protocol need to be aligned between data producer and data consumer. Considering that multiple data consumers are involved for the UE based data producer, cross-WGs discussion (e.g. RAN2/RAN3/SA2/SA5) is needed.

Proposal 3	RAN2 to reuse DRB as baseline for the unified data transfer with some enhancements (e.g. non-PDU session associated, enhanced QoS mapping and compression support, etc.). For the data format (e.g. ASN.1, JSON, XML) and protocol (IP vs. Non-IP) used for the data transfer, joint discussions with RAN3/SA2/SA5 are needed.

During the last meeting, it is discussed whether UE can reject requests for UE data collection for network use-cases based on UE internal considerations. Based on the prior art, there are different handling for different use cases as follows.

- Network-side data collection for AI/ML in Rel-19: After receiving the log measurement configuration, UE logs the L1 measurement. When AS layer memory for storing logged data becomes full or reaches an absolute threshold, UE indicates data availability to the network and network requests the UE to transmit the logged data to network. Moreover, when low power state is detected, the UE can indicate the lower power state to the network and network should release the UE data collection configuration for NW-side model.

- MDT in 3G/4G/5G: User consent needs to be collected and stored as part of the subscription data of UE. When selecting suitable UE for MDT data collection, RAN needs to take into account the user consent information received from AMF into account. Upon receiving the logged measurement configuration, the UE shall initiate the measurements unless it is power off or deregistered.

As agreed in RAN2#131bis meeting, one of the requirement of standardized data framework for AI/ML is that the user data privacy, anonymity and user consent should be respected. It means that getting user consent is a pre-requisite for RAN before initiating the AI/ML data collection of UE. Suppose user consent is available, RAN may select the UE based on the UE capability and the user consent indication from core network. In this case, the selected UE should not reject the data collection request from network. Upon exceptional cases (e.g. low power, run out of memory), it is reasonable for UE to indicate its status to RAN and it is up to RAN to decide whether to release the data collection configuration. Actually, in 5G, user consent is only supported in MDT and not applicable for the AI/ML and QoE scenario. When it comes to 6G, It is not yet clear whether and which data should be subject to user consent in 6G. RAN2 may wait for SA2’s progress on the user consent.

Proposal 4	If user consent is available for data collection, UE should not reject the data collection request from network. Upon exceptional cases (e.g. low power, run out of memory), UE may indicate its status to RAN and it is up to RAN to decide whether to release the data collection configuration. Whether and how to support user consent is up to SA2’s progress.

2.2 Model transfer

In Rel-19, the two-side model is studied in RAN 1 and it is determined that the inter-vendor coordination for two side model may require the model transfer/delivery from NW to UE. And then RAN2 has evaluated two possible solutions, one is OTA based solution, the other one is non-OTA solution:

- OTA based solution: RAN directly shares the UE part of two side model parameter with UE over the Uu interface.

- Non-OTA based solution: RAN/OAM/CN transfer the UE part of two side model parameter to the UE server, then UE server forwards the model parameter to the UE somehow before applying the model.

In 5GA, only non-OTA based solution is adopt since the the average model size is around 11.6MB. OTA based solution cannot transfer the data with such large size as the maximum data size for SRB signaling is only 45KB and there is no UP tunnel terminated between UE and RAN.

However, for two side model based LCM, UE needs to pair the UE part of two side model with the NW part of two side model that is applicable at NW side. Considering the UE storage for AI/ML models are quite limited,  UE is not able to download too many models in advance, In most case, UE may download the UE part of two-side model immediately when NW providing the applicable/available NW part of two side model, and then the delay requirement for such downloading is most important.

Observation 4	For model transfer of two side model, considering storage limitation, UE may download the UE part of two-side model when NW provides the applicable NW part of two-side model. In this scenario, the delay for such download should be as less as possible.

For non-OTA based solution which has been adopt by RAN2 in 5GA, the NW shares the model parameter to the UE server, and then the model shall be downloaded by UE from remote UE server when doing the pairing to the NW part of two side model, the delay caused by such download cannot be guaranteed. Given the delay caused by the model transfer between UE and RAN is much more short than UE to download it from remote UE server, we propose that:

Proposal 5	For two-side model that is located at UE and RAN respectively, RAN 2 to study the model transfer terminated between UE and RAN for inter-vendor collaboration.

2.3 AI/ML framework

According to the  AI/ML framework discussion in RAN2#131bis meeting, the following components should be taken into account for the study:

- UE AI/ML Capabilities Exchange;

- Applicable Functionality Reporting;

- Inference Configuration and Reporting;

- Performance Monitoring Configuration and Reporting;

UE AI/ML Capability

For UE AI/ML capability designed in 5GA, UE needs to report the following information to the network in general:

- Supported AI/ML functions

- APU/CPU related capability information

Regarding the supported AI/ML functionalities, it is not like the legacy UE capability which is deterministic once the UE device is produced. The newly supported AI/ML functions can be developed for a UE device in old generation which is mostly like the operator system. However, the currently static UE capability exchange method cannot reflect timely the development of the supported AI/ML functions for the UE device in the older generation since the current UE capability exchange is triggered by NW. As a result, some UE device in old generation cannot use the newly supported AI/ML functions until to the next round UE capability exchange, which may last a long time.

Regarding the xPU related capability information, the xPU resource pool for AI/ML functions (i.e. Y) and a unified xPU consumption quantity for an AI/ML based function (i.e. X) is provided in the UE capability. However, with those capability information, the xPU regulation for AI/ML function is too coarse, that is, the maximum number (i.e. M) of  activated inference configuration for the AI/ML based function is:

M = Y/X.

Such coarse xPU regulation method will reduce the xPU utilization efficiency when considering the following real deployments scenarios:

1) the xPU resource pool for AI/ML functions in AS layer may be varied as, normally, the xPU may be shared by AIML functionality/models in both app layer and AS layer.

2) the xPU consumption for different models for one AI/ML function may be different.

3) One AI/ML model can be activated for more than one inference configurations, which means, only one unit xPU consumption can support multiple activated inference configurations.

Observation 5 For the AI/ML capability information in 5GA, the newly supported AI/ML function can be developed by UE vendor for the UE of the olde generation; the indicated maximum xPU resource pools may be varied due to the xPU sharing for all AI/ML models in both app layer and AS layer, and the unified xPU consumption of each inference configuration for each AI function is not realistic at all.

Applicable Functionality Reporting

In 5GA, for AI/ML based beam management, there are two options for the applicability reporting:

- Option A: NW provides the full inference configuration in the CSI-MeasConfig  and UE provides the applicability information according to the received full inference configuration.

- Option B: NW provides the partial inference configuration in the OtherConfig and UE provide the applicability information according to the received partial inference configuration.

In option A, NW need to provide full inference configuration and UE can activate the inference automatically when the full inference configuration is reported as applicable which can reduce the AI/ML functionality activation delay. However, NW needs to blindly configuring full inference configuration which may be inapplicable, and NW then need to release them which increases the signalling overhead for the applicability reporting.

In option B, NW can just provide partial inference configuration for UE to determine which inference configuration is applicable, and then NW can formally configure the full inference configuration to the UE to activate the inference configuration. This option can prevent NW from blindly configuring the full inference configuration which saves the signalling overhead compare to option A, but the activation delay may be increased.

Compared between option A and option B, there is no much difference between each other from technical perspective, both of them share the same aiming. It is not necessary to have such duplicate methods for applicability reporting since the support of both option A and option B dramatically increases the specification complexity.

Observation 6 In 5GA, the applicability reporting has two options for triggering: Option A) full inference configuration in RRCReconfiguration; Option B) partial inference configuration in OtherConfig. The specification complexity is increased for supporting both options but there is no much differences between each other.

Inference Configuration and Reporting

In 5GA, when inference configuration for AI/ML CSI prediction/ beam management is received from NW side, the UE behaviour is like below:

1> To check the inference configuration is applicable or not

2> If applicable, send the applicable indication to the NW and activate the inference configuration immediately or waiting for the activation command from NW.

2> otherwise, UE sends an inapplicable indication to the NW and pending the inference configuration in RRC layer. In this case, NW need to release the inapplicable inference configuration by initiating another RRCReconfiguration procedure.

According to above procedure, it can be seen that, in 5GA, NW shall configure the inference configuration blindly regardless of the its applicability. It may cause the unnecessary RRC procedure if the blindly configured inference configuration is inapplicable. Besides, NW also needs to provide the corresponding CSI resources and/or signals for those inapplicable inference configurations in advance, which may cause the unnecessary reference signalling overhead.

Observation 7 In 5GA, the inference configuration will be blindly configured by the NW to the UE regardless of its applicability status which may cause unnecessary extra RRC procedure and reference signalling overhead for dealing with such inapplicable inference configurations.

Performance monitoring and Reporting

In Rel-19 5GA, only the NW-centric performance monitoring method is adopt :

One is that UE sends the ground truth value to the NW, and it is up to NW to calculate the metrics based on the received inference value and ground truth value and make the final decision on the model/functionality management (i.e. activation/deactivation/fallback).

The other one is that UE calculates the metrics according to the inference value and corresponding ground truth value and sends that metrics to the NW, and then NW make the final decision on the model/functionality management (i.e. activation/deactivation/fallback).

Even though the NW centric performance monitoring can guarantee the AI/ML model/functionalities to be fully under the NW control, the extra control delay is inevitable due to the monitoring measurement period and the lack of knowledge about UE additional conditions change at NW side, especially in the case of deactivation/fallback of AI function since UE may find the AI/ML model/functionality unsuitable earlier than NW. For example, the change of UE additional conditions (e.g. speed variation, barrier detection, movement detection, etc) has been detected, then UE fallback to the non-AI functions to guarantee the performance. However, in the current NW-centric performance monitoring, NW will detect this by evaluating the metrics change which may last several measurement periods and hence the performance degradation may happen in those periods.

Observation 8  In 5GA, only the NW-centric performance monitoring method is adopted, for which the delay caused by monitoring measurement period is inevitable, and the lack of knowledge of the UE additional conditions change by NW side also leads to the slower NW reaction to the model management.

The aim of AI/ML framework design in 6GR

Based on above observations in this subclause, the 6GR AI/ML framework shall avoid the issues mentioned in above observations, so the RAN2 study on 6GR AI/ML framework can focus on the following aspects:

- Regarding AI/ML capability exchange, how to deal with issue where the information contained in the AI/ML capability is dynamically varied, such as, the supported AI/ML functions development and xPU resources variation.

- Regarding the applicability reporting, there is no need to have two methods for one target, a unified applicability reporting solution is preferred for all use cases in 6GR.

- Regarding the inference configuration, avoid NW configuring the inapplicable inference configuration being configured to the UE.

- Regarding the performance monitoring, to study the UE-centric management method, especially the UE based fallback/deactivation, to reduce the control delay caused by NW-centric management method.

Proposal 6	RAN 2 to study the 6GR AI/ML framework by considering the following objectives: 1) AI/ML capability exchange: to address the issue of AI/ML capability information variation, such as: supported AI/ML functions development, xPU resources variation; 2) Applicability reporting: design one unified applicability reporting method for all use cases; 3) Inference configuration: avoid configuring the inapplicable inference configuration to the UE; 4) Performance monitoring: study the UE based management method, especially UE based deactivation or fallback mechanism.

2.4 AI/ML use case study guidance

For the AI/ML use case study in RAN2, the scope includes both use cases inherited from 5G—such as AI/ML-based mobility—and new use cases proposed by companies for 6G. Examples of these new use cases are:

- AI/ML-based traffic pattern prediction

- AI/ML based BSR

- AI/ML based C-DRX

- AI/ML-based PDCP duplication

- AI/ML-based QoS control

- AI/ML-based L2 parameter prediction

For each of these use cases, RAN2 should conduct a case-by-case analysis and for each use case, the proponent shall address the following aspects:

- Use case description: including the motivation, justification, etc.

- Performance gain: quantify the improvement achieved by the AI-based feature compared to non-AI solutions. This assessment will help determine whether the use case is worthwhile for standardization in 3GPP. However, in some aspects such as power saving, the gain is hard to be evaluated. So the methodology of the gain evaluation shall be contained in this aspect as well.

- Specification impact: evaluate the expected impact on 3GPP specifications for supporting the use case in 6G, such as the analysis of the components of the LCM framework, including:

-  Data Collection: Identify which measurements and/or measurement results should be used for model training, inference, and performance monitoring.

-  Model Training: Determine the training approach (e.g., offline, online, or federated learning) and assess whether online training can be network-controlled—and if so, how.

-  Model Inference: Define the input and output data required for inference.

-  Management: Establish methods to ensure environmental consistency between training and inference, define performance monitoring techniques, and specify AI/ML model control mechanisms.

Proposal 7	RAN 2 to conduct a case by case study on 6GR AI/ML use cases and the use case proponent shall provide the following aspects for the use case discussion: 1) Use case description, including the motivation, justification; 2) Performance gain, including the methodology for the gain evaluation; 3) Specification impact, including the evaluations based on each component of the LCM.

Conclusion

In this paper, we discussed the general principles and design considerations of the data transfer, model transfer, AI/ML framework and use cases study. We have the following observations and proposals:

Observation 1	One data producer(e.g. UE) may be associated with multiple data consumers (e.g. RAN, OAM, SF, CN). The data collection requirements from these data consumers may be overlapped with slight difference on data granularity and transmission frequency.

Observation 2	Compared with SRB, DRB is more suitable for the large-volume and frequent data transfer. However, traditional DRB is associated with PDU session terminated at UPF and needs to be enhanced for data transfer.

Observation 3	The data format and protocol need to be aligned between data producer and data consumer. Considering that multiple data consumers are involved for the UE based data producer, cross-WGs discussion (e.g. RAN2/RAN3/SA2/SA5) is needed.

Observation 4	For model transfer of two side model, considering storage limitation, UE may download the UE part of two-side model when NW provides the applicable NW part of two-side model. In this scenario, the delay for such download should be as less as possible.

Observation 5 For AI/ML capability information in 5GA, the newly supported AI/ML function can be developed by UE vendor for the UE of the olde generation; the indicated maximum xPU resource pools may be varied due to the xPU sharing for all AI/ML models in both app layer and AS layer, and the unified xPU consumption of each inference configuration for each AI function is not realistic at all.

Observation 6 In 5GA, the applicability reporting can be triggered by: Option A) full inference configuration in RRCReconfiguration; Option B) partial inference configuration in OtherConfig. The specification complexity is increased for supporting both options but there is no much differences between each other.

Observation 7 In 5GA, the inference configuration will be blindly configured by the NW to the UE regardless of its applicability status which may cause unnecessary extra RRC procedure and reference signalling overhead for dealing with such inapplicable inference configurations.

Observation 8  In 5GA, only the NW-centric performance monitoring method is adopted, for which the delay caused by monitoring measurement period is inevitable, and the lack of knowledge of the UE additional conditions change by NW side also leads to the slower NW reaction to the model management.

Proposal 1	RAN2 to capture the comparison on data type, endpoint pairs, data size, transmission interval and QoS requirement (summarized in Table 1) of various use cases (e.g. AI/ML, sensing, MDT, QoE) to the TR as baseline for further unified data transfer study.

Proposal 2	It is suggested that RAN have visibility to the data collection requirements from multiple data consumers (e.g. RAN, OAM, SF, CN) for various use cases (e.g. AI/ML, sensing, MDT, QoE, etc.) and provide the UE with unified data collection and report configurations.

Proposal 3	RAN2 to reuse DRB as baseline for the unified data transfer with some enhancements (e.g. non-PDU session associated, enhanced QoS mapping and compression support, etc.). For the data format (e.g. ASN.1, JSON, XML) and protocol (IP vs. Non-IP) used for the data transfer, joint discussions with RAN3/SA2/SA5 are needed.

Proposal 4	If user consent is available for data collection, UE should not reject the data collection request from network. Upon exceptional cases (e.g. low power, run out of memory), UE may indicate its status to RAN and it is up to RAN to decide whether to release the data collection configuration. Whether and how to support user consent is up to SA2’s progress.

Proposal 5	For two-side model that is located at UE and RAN respectively, RAN 2 to study the model transfer terminated between UE and RAN for inter-vendor collaboration.

Proposal 6	RAN 2 to study the 6GR AI/ML framework by considering the following objectives: 1) AI/ML capability exchange: to address the issue of AI/ML capability information variation, such as: supported AI/ML functions development, xPU resources variation; 2) Applicability reporting: design one unified applicability reporting method for all use cases; 3) Inference configuration: avoid configuring the inapplicable inference configuration to the UE; 4) Performance monitoring: study the UE based management method, especially UE based deactivation or fallback mechanism.

Proposal 7	RAN 2 to conduct a case by case study on 6GR AI/ML use cases and the use case proponent shall provide the following aspects for the use case discussion: 1) Use case description, including the motivation, justification; 2) Performance gain, including the methodology for the gain evaluation; 3) Specification impact, including the evaluations based on each component of the LCM.

Reference

- RAN2#131bis meeting chair minutes.

- 3GPP TR 22.870 v0.4.0, Study on 6G use cases and requirements, September, 2025.