3GPP TSG-RAN WG2 Meeting #132												R2-2508292

Dallas, USA, November 17th – 21st, 2025

Agenda Item: 10.3.3.1

Source: Sharp

Title: Discussion on Data transfer, Model Transfer, and AI/ML

Document for: Discussion

# Introduction

In RAN2#131bis meeting [1], RAN2 has made the related agreements in the agenda item 10.3.3 issue “Data Collection/Management/Transfer” and “AIML”. The agreements in “Data Collection/Management/Transfer” are as follows:

Agreements 1. Study the standardized data collection framework with these requirements as a baseline at least for AI/ML - The data collected is secured and data integrity and confidentiality for that data is ensured. - User data privacy, anonymity and user consent is respected. - The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose - This includes initiating, terminating, and fully managing data transfer. - MNO has full visibility for standardized data. - The design is future-proof and extendable. - The UE data collection should minimize impact to the UE battery, UE processing and memory utilization. - UE data collection should minimize impact to user traffic transmission and power saving features These requirements can apply to both UE and NW sided data collection. FFS if some don’t apply to both. 2. Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases. 3. Study termination points (i.e. understand who producers are, consumers of the data, data collection points. Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP). 4. Study model transfer/delivery requirements/functions

And the chair also mentioned the aspects in the RAN2#132 meeting as follows:

For next meeting companies can consider at least the following aspects: - Diverse types of data and its services/use case scenarios (e.g., AI/ML related data, sensing data, QoE, SON/MDT, etc); - For each type of data, study: - Applicable use case(s) - End point pairs (i.e., producer and consumer), including UE and RAN, UE and CN, RAN and CN, RAN and OAM. - Data size in a single reporting and total data size - Frequency of data reporting. - QoS requirements (e.g., latency, priority, GBR, packet error rate, etc);

In addition, the agreements in “AIML” are as follows:

Agreements on LCM Study 6G AI/ML LCM framework that supports at least the following functions/procedures: - UE AI/ML Capabilities Exchange. - Applicable Functionality Reporting. - Inference Configuration and Reporting. - Performance Monitoring Configuration and Reporting. - Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

This document discusses aspects of data transfer, model transfer, and AIML for 6GR.

# Discussion

It was noted in the 6G RAN-level SID that the 6GR shall leverage the 5G AI/ML framework and study further use cases of interest. In 5G-Advanced, the AI/ML models are one-sided, whereas two-sided models requiring coordination between the network and the UE may be developed in 6GR. To facilitate this, 6GR needs to study a unified-forward compatible architecture and LCM procedure, considering the potential advantages provided by one and two-sided models for future use cases. This requires that the protocol and architectures to support unified, yet forward compatible procedures are considered from the early stages of 6GR for forward-compatibility. Similar considerations can also be made for data collection and data management for 6GR AI/ML features. This approach, would ensure consistent and efficient operation across the network for 6G use cases

Observation 1: 6GR is expected to build on the 5G AI/ML framework while expanding the study toward additional use cases of interest.

Proposal 1: RAN2 to study a forward-compatible unified AI/ML framework for both one-sided and two-sided models, including LCM, data collection, and model management.

## Data collection and transfer of diverse types of data

In NR, NW-side data collection has been supported, and UE-side data collection has been studied but more work is still needed. In 6GR with two-sided models, additional considerations must be made for data collection and data transfer procedures for efficient utilization of network and UE resources, especially considering the expected UE power consumption and signalling overhead. For 6G, both NW-side data collection and UE-side data collection should be studied, and NW-side data collection can be prioritized.

Observation 2: In 6GR, both one and two-sided models require careful consideration of data collection and transfer procedures to balance efficient resource use with UE power consumption and signalling overhead.

Proposal 2: RAN2 to study both network-side and UE-side data collection for AI/ML, with network-side prioritized in early 6GR studies.

In RAN2#131bis, some companies proposed that RAN2 define data collection [2][3], which refers to the data provider (e.g., UEs or RAN nodes) collecting data with diverse data types (e.g., through measurements or sensing) based on configuration (e.g., measurement configuration). In order to fulfil multiple potential use cases in 6GR, we suggest that RAN2 study the accommodation of diverse types of input data (e.g., sensing data and vendor-specific non-standardized data) for AI/ML LCM. In addition, we also suggest that RAN2 further define the source of the collected data, meaning that data collection should include logging the data in memory and collecting the data through measurement and/or prediction via AI/ML model inference.

Proposal 3: RAN2 further clarify and define data collection under the AI/ML LCM framework to include:

- Logging of data in local memory for later use or transfer, and

- Collection of data obtained either through actual measurement or through prediction/inference performed by an AI/ML model.

Proposal 4: To deal with multiple potential use cases, RAN2 to study a framework to enable the following end point pairs, in which both the producer and consumer could be the UE and the RAN node (e.g., gNB):

- Single producer to single consumer

- Single producer to multiple consumers

- Multiple producers to single consumer

- Multiple producers to multiple consumers

In NR, AI/ML and SON/MDT share a common objective: improving network performance through intelligent optimization. These functions were standardized separately, and therefore each has its own reporting format and reporting events. To avoid unnecessary reporting and complex UE procedures, we believe 6GR study should be conducted comprehensively, taking into account reporting mechanisms other than AI/ML that have been considered for 5G NR. A unified approach in 6GR would help streamline signalling procedures and reduce overhead, especially as networks become increasingly dense and heterogeneous. By harmonizing reporting frameworks, more efficient data collection and decision-making across multiple optimization layers can be enabled. This would also facilitate better interoperability between legacy and future systems, ensuring smoother transitions and more robust performance enhancements. RAN2 needs to support consistent reporting across AI/ML and SON/MDT to avoid duplication and reduce UE overhead, reusing existing triggers/formats where possible.

For 6GR, an AI-native design is envisioned, supporting AI-driven intelligent interaction between the network and devices across different layers of the protocol stack. The study phase will aim to identify promising use cases and establish a flexible AI framework covering LCM and data aspects. It needs to be ensured that the existing features remain fully operable without relying solely on AI functions. For this, 6GR must ensure efficient switching between AI/ML-based and non-AI-ML-based procedures taking into account both UE and network side operations. In line with existing 3GPP assumption, AI/ML models themselves are not standardized, the focus is on signalling control, measurements and lifecycle handling to ensure testability and consistent behaviour.

Observation 3: In NR, both AI/ML and SON/MDT target network performance optimization but were standardized independently, resulting in separate reporting formats and events.

Observation 4: 6GR envisions an AI-native design that enables intelligent, AI-driven interaction between the network and devices across multiple protocol layers.

Proposal 5: RAN2 to study an integrated reporting framework that consolidates reporting for network optimization purposes.

## Coordination between data collection and AI/ML LCM

To minimize the impact of data collection on UE battery, processing, and memory utilization, we propose to study coordination between data collection and AI/ML LCM procedures. Such coordination would enable the network to align data collection activity with the AI/ML model lifecycle (e.g., activation, update, or deactivation), allowing the UE to collect and report data only when required. This approach improves energy efficiency and resource utilization while ensuring consistency with active AI/ML functionalities.

Proposal 6: RAN2 to study mechanisms enabling coordination between ongoing data collection sessions and AI/ML LCM (e.g., model testing, training, inference, or performance monitoring to activate, update, or fallback the model).

## Model transfer and delivery

In Rel 19, The AIML LCM framework introduced core functions including capability exchange, applicability reporting, inference configuration, performance monitoring, and functionality (de-)activation and fallback.

This contribution builds on the RAN2 #131bis agreements and proposes clarifications and extensions to facilitate unified data and model transfer procedures under the AI/ML LCM framework.

In RAN2#131bis [1], it was agreed to study model transfer/delivery requirements and functions as part of 6G study in the agenda item 10.3.3 issue Data Collection/Management/Transfer and AIML.

Following this, we propose to study detailed procedures for model transfer, update, and synchronization to enable efficient coordination between network entities and UE, as well as other relevant nodes. The study on model transfer and delivery, including analysis of existing mechanisms, identified gaps, and potential RAN impacts, has been conducted in RAN2 during the AI PHY study item phase. The outcome of this study provides the basis for understanding feasible solutions and procedural considerations. Therefore, these findings may be used as the baseline for subsequent study and specification work on model transfer and delivery procedures under the 6G AI/ML LCM framework.

Observation 5: Further study is needed on detailed procedures for model transfer, update, and synchronization to ensure efficient coordination between network and UE under the 6G AI/ML LCM framework

Proposal 7: The study conducted on AI/ML model transfer and delivery during the AI/ML PHY study item can be used as baseline for subsequent study and specification work on model transfer and delivery procedures under the 6G AI/ML LCM framework.

# Conclusion

Based on the discussion given above, RAN2 is requested to discuss the observations and agree the following proposals:

Data collection and transfer of diverse types of data

Observation 1: 6GR is expected to build on the 5G AI/ML framework while expanding the study toward additional use cases of interest.

Proposal 1: RAN2 to study a forward-compatible unified AI/ML framework for both one-sided and two-sided models, including LCM, data collection, and model management.

Observation 2: In 6GR, both one and two-sided models require careful consideration of data collection and transfer procedures to balance efficient resource use with UE power consumption and signalling overhead.

Proposal 2: RAN2 to study both network-side and UE-side data collection for AI/ML, with network-side prioritized in early 6GR studies.

Proposal 3: RAN2 further clarify and define data collection under the AI/ML LCM framework to include:

- Logging of data in local memory for later use or transfer, and

- Collection of data obtained either through actual measurement or through prediction/inference performed by an AI/ML model.

Proposal 4: To deal with multiple potential use cases, RAN2 to study a framework to enable the following end point pairs, in which both the producer and consumer could be the UE and the RAN node (e.g., gNB):

- Single producer to single consumer

- Single producer to multiple consumers

- Multiple producers to single consumer

- Multiple producers to multiple consumers

Observation 3: In NR, both AI/ML and SON/MDT target network performance optimization but were standardized independently, resulting in separate reporting formats and events.

Observation 4: 6GR envisions an AI-native design that enables intelligent, AI-driven interaction between the network and devices across multiple protocol layers.

Proposal 5: RAN2 to study an integrated reporting framework that consolidates reporting for network optimization purposes.

Coordination between data collection and AI/ML LCM

Proposal 6: RAN2 to study mechanisms enabling coordination between ongoing data collection sessions and AI/ML LCM (e.g., model testing, training, inference, or performance monitoring to activate, update, or fallback the model).

Model transfer and delivery

Observation 5: Further study is needed on detailed procedures for model transfer, update, and synchronization to ensure efficient coordination between network and UE under the 6G AI/ML LCM framework

Proposal 7: The study conducted on AI/ML model transfer and delivery during the AI/ML PHY study item can be used as baseline for subsequent study and specification work on model transfer and delivery procedures under the 6G AI/ML LCM framework.

# References

[1] Chair notes on RAN2#131bis meeting

[2] R2-2506801, Considerations on 6G data collection and data transfer, vivo, NTT DOCOMO, INC.

[3] R2-2507036, Discussion on Common User plane and Control plane for 6GR, HONOR