3GPP TSG-RAN WG2 #132	R2-2508293

Dallas, USA, 17 – 21 November 2025

Agenda item:		10.3.3.1

Title:		Discussion on Data Collection and AI/ML framework

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In the last RAN2 meeting (RAN2#131bis), the following agreements were reached for data collection for 6G:

Agreements

1.  Study the standardized data collection framework with these requirements as a baseline at least for AI/ML

-	The data collected is secured and data integrity and confidentiality for that data is ensured.

-	User data privacy, anonymity and user consent is respected.

-	The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE side data collection, without the need of Service Level Agreement (SLA) for this purpose

-	This includes initiating, terminating, and fully managing data transfer.

-	MNO has full visibility for standardized data.

-	The design is future-proof and extendable.

-	The UE data collection should minimize impact to the UE battery, UE processing and memory utilization.

-	UE data collection should minimize impact to user traffic transmission and power saving features

-	These requirements can apply to both UE and NW sided data collection.  FFS if some don’t apply to both.

2.	Study further requirements of other non-AI/ML use cases/services and understand if we can have more common requirements across different use cases.

3.	Study termination points (i.e. understand who are producers, consumers of the data, data collection points.  Understand requirements of use cases sensing, aI/ml, son/mdt, QoE etc) and protocol used for data transfer (e.g. IP vs. non-IP).

4.	Study model transfer/delivery requirements/functions

With regard to the life cycle management for AIML, the following agreements were reached in RAN2#131bis:

Agreements on LCM

Study 6G AI/ML LCM framework that supports at least the following functions/procedures:

-	UE AI/ML Capabilities Exchange;

-	Applicable Functionality Reporting;

-	Inference Configuration and Reporting

-	Performance Monitoring Configuration and Reporting;

-	Functionality (De-)Activation and Fallback/Switching to the AI/ML/non-AI/ML Functionality;

In this contribution, we further discuss the transfer of various type of data (including AI/ML data, sensing, etc.) and the general AI/ML framework.

# 2. Discussion

2.1	Requirements for Data collection

Within the 6G Radio study item, one of the objectives is to study data transfer design to support various types of data (e.g. AI/ML, Sensing, etc). The aspects related to data transfer of new types of data may be considered as neither UP nor CP data.

The new data types may include the AIML data collected for AIML training, AIML inference, or performance monitoring, sensing data collected from the sensing node, and the data to be collected for other purposes. It may also include the data to be transmitted from the network to the UE side, for example, the AIML model transfer, data set sharing and model parameter configuration. These data have different characteristics in terms of the transmission over the air. For example, a small amount of real time data may be required to transfer from the UE to the network for AIML model inference; a large amount of non real time data may be required to transfer from the UE to the network for the purpose of AIML model training. In addition, for sensing based object tracking, a large amount of real time sensing data transmission may be required to transfer from the sensing node (e.g. a UE) to the network in order to calculate the real-time location of the tracked object. It would be essential to clarify the transmission requirements for the collection of different types of data.

Proposal 1:  Data collection design should consider the following transmission requirements:

(1) a small amount of real time data may be required to transfer from the UE to the network for AIML model inference.

(2) a large amount of non real time data may be required to transfer from the UE to the network for AIML model training.

(3) a large amount of real time sensing data transmission may be required to transfer from the sensing node (e.g. a UE) to the network.

From the data consumer perspective, it may be different for different data types and different use cases. For example, for sensing data based location track, the consumer of the data may be a specific entity, Sensing Control Function (SCF) hosted by the Core network (with reference to the discussion at SA2 for 5G NR), which manages and orchestrates sensing services. For AIML model training, the consumer of the data may be 6G Base Station or OAM. For positioning, the data is collected by the LMF entity hosted by the Core network.

If the consumer of the collected data is different, the final termination point of the data collection will be not the same either. But from RAN2 perspective, it would be prominent to know if the Base Station is the first termination point of the data collection for the use case, since RAN2 mainly discuss the air interface based data collection procedure. For 6G study, RAN2 may primarily focus on the use cases that take the Base Station as the first termination point of the data collection.

Proposal-2: RAN2 to assume the Base Station as the first termination point of the data collection for 6G study.

2.2	Data collection framework

The concept of data collection is introduced in 5G which is used to collect AI/ML data for the purpose of e.g., AI/ML model training. When it comes to 6G, more and more AI/ML use cases will emerge. A huge amount of data are foreseen to be transferred among different NW entities. Furthermore, besides AI/ML, in 6G more use cases like sensing also requires such data transfer mechanism. Therefore, data collection framework is promising within 6G network.

It is noted that in 5G, data collection framework is discussed and designed per use case, and CP based solution (UE information procedure) is introduced for RAN domain data collection. However, such use case specific design has many drawbacks, for example, it is only for RAN domain, and some collected data could be duplicated among different use cases, e.g., L1 beam measurement results could be applied for both RAN1 led beam management use case and RAN2 led AI mobility use case, there is no need for UE to collect those data twice based on CSI configuration and RRM configuration. Furthermore, if data collection is designed based on use case specific, multiple signalling could be introduced in the end (e.g., some procedure for RAN domain transfer while other procedure for CN domain transfer), which increase the data collection complexity. Therefore, unified data collection framework should be considered.

Proposal 3: RAN2 should aim for a unified data collection framework for different 6G use cases which require data collection, e.g., AI/ML data, sensing data, etc.

Having said this, a unified data collection framework is required to include different use case data transfer which may have different requirements. These data may be characterized by large amount of data, with diverse latency requirements. For example, the latency requirement is low for the data transfer supporting AIML training, but for sensing based object tracking, real time sensing data transmission may be required to transfer from the sensing node (e.g. a UE) to the network in order to calculate the real-time position of the object. Therefore, priority management mechanism should be considered to determine which data transfer is urgent and which is not. Furthermore, since data collection procedure may occupy air interface transmission resource, it may impact legacy CP and UP transmission resource, therefore, the priority between data collection and legacy transmission resource should also be considered.

On the other hand, data collection also requires UE to perform amount of measurements, which costs a lot of UE power consumption. It is not expected that user experience is degraded by introducing data collection. Therefore, data collection framework should be designed with power consumption friendly. Furthermore, a lot of logged measurement results may be temporarily stored at UE-side with UE buffer/memory, the design should also be memory friendly.

Proposal 4: the following characteristics for data collection should be considered:

- priority between data transfer and other legacy transmission

- priority between different data collection use cases

- power consumption and memory utilization

As for transmission path/solution, we now have legacy CP and UP solution.

Before analysing different solution, we should figure out what characteristics that data collection has. In our understanding, this new type of data transfer may require large size or small size, may require low latency and without legacy consideration, may require RAN-side termination node or CN-side termination node. And it depends on different use case.

For CP solution, it is terminated between UE and RAN for reliable air inference transmission, and a single RRC message can only contain maximum 9KB data, thus segmentation is introduced to transfer more data, the legacy RRC message supports up to 16 segments in UL, which is maximum 144 KB data. If for 6G use case, large data size is required for data transfer, CP transmission size is not suitable then. Meanwhile, usually CP is designed for low latency transmission, but data transfer is not always required for low latency transmission (e.g., data for model training), this is not well matched.

For UP solution, it is terminated between UE and external data network across 5GC/UPF, governed by PDU session. The legacy UP protocol stack was designed to support IP based data transmission and managed by QoS flow based granularity. It should be noted that, the consumer of the data carried by the UP is located in the public data network. However the data collection procedure may be only collected for the 3GPP network internally, and sometimes it is only terminated at the RAN domain (xNB / OAM). In this case, the IP based data transmission and QoS flow based management may be not that essential.

Based on the above analysis, the legacy CP and UP solution may not be suitable for the design on data collection framework, at least it should be enhanced to support data collection.

On the other hand, since this is new type of data transfer, a new solution can be considered to deliberately transfer such data to fit all features that data collection requires. For example, new data plane can be introduced, this may or may not require a new protocol layer to manage the data transfer including RAN domain and CN domain, and to achieve a unified solution, data repository entity is also needed to collect/store/forward data for different use case. Apart from new data plane consideration, we believe that at least a new radio bearer other than SRB/DRB should be introduced to establish the air interface part for this new type of data.

The following table is to illustrate the above mentioned approaches for convenience.

Table-1: analysis of different data transfer approaches

| Approaches | Pros | Cons |
| --- | --- | --- |
| Control plane | 1. High priority / Low latency transmission 2. Reliable transmission | 1. It is terminated between UE and RAN 2. Size limitation (up to 144KB by UL segmentation) |
| User plane | 1. No size limitation 2. Flexible priority transmission based on QoS management | 1. It is terminated between UE and external data network across 5GC/UPF 2. The transmitted data is mainly for IP packets instead of data used in 3GPP internally 3. PDU session is required |
| New solution e.g., new data plane, new entity / protocol stack, new radio bearer | 1. The brand-new solution can be designed to well match data collection characteristics, e.g., various use cases, size, latency, termination point. 2. Facilitate unified data collection framework. | 1. should consider complexity of network framework 2. should consider practical development cost |

Proposal 5: RAN2 should study which solution to be adopted for data transfer:

- enhanced control plane

- enhanced user plane

- new solution (e.g., new data plane, new entity/protocol layer, new RB)

2.3	AI/ML general framework

In this section, we intend to discuss high-level aspects of the AI/ML general framework. In 5G NR, the AI/ML functions (e.g., air interface, mobility and NG-RAN) has been introduced as the use case specific manner. Even though those AI/ML functions are useful, the framework is largely independent. With this way, the specification support are designed per use case and new functionalities are added when a new use case is to be supported. Normally this approach makes the system complicated.

For 6G, it is expected that there will be many types of AI/ML applications and services. However, there will be still some different targets from the high-level. For example, one type of AI/ML is achieved within RAN domain, which may be similar to the use cases supported in the 5G NR. While, the other type of AI/ML is achieved in wider range or in the CN domain, which may be similar to use cases supported in the 5GC. On one hand, it would not be good to specify divergent framework like use case specific like 5G NR. On the other hand, it would not be well justified to make a single framework for all types of the AI/ML to be expected for 6G. Therefore, we expect that RAN domain specific single framework would be beneficial with necessary coordination with the CN side.

Proposal 6: RAN2 to aim to support a single framework for RAN domain specific AI/ML applications and services in 6G.

One possible approach to achieve the single framework for RAN domain specific AI/ML may be to introduce a new AI/ML node (or entity) specific to AI/ML for air interface and mobility which can be located in RAN domain. Such new AI/ML node will manage the AI/ML Life Cycle Management (LCM) including data collection, model training/monitoring, and/or inference operation. The new AI/ML node may also manage AI computing/storage resource scheduling, if introduced in 6G.

Proposal 7: RAN2 to study the possibility of introducing a new AI/ML node/entity in RAN domain to enable the single framework for RAN domain specific AI/ML.

Note that if the new AI/ML node/entity is to be introduced in the RAN domain, this should also manage a part of the transfer of various data types as discussed in section 2.1.

For the discussion of 6G AIML, the AIML use cases discussed at 5G NR should be the starting point. With this said, we think AI based mobility should be studied by RAN2 for 6G based on the progress made by 5G NR for that topic. In addition, it would be beneficial to clarify if additional AIML use cases should be studied for 6G in Rel-20 in RAN2.

# 3. Conclusion

In this contribution we discussed both data collection and general AI/ML framework. We made the following proposals:

Proposal 1: Data collection design should consider the following transmission requirements:

(1) a small amount of real time data may be required to transfer from the UE to the network for AIML model inference.

(2) a large amount of non real time data may be required to transfer from the UE to the network for AIML model training.

(3) a large amount of real time sensing data transmission may be required to transfer from the sensing node (e.g. a UE) to the network.

Proposal 2: RAN2 to assume the Base Station as the first termination point of the data collection for 6G study.

Proposal 3: RAN2 should aim for a unified data collection framework for different 6G use cases which require data collection, e.g., AI/ML data, sensing data, etc.

Proposal 4: the following characteristics for data collection should be considered:

- priority between data transfer and other legacy transmission

- priority between different data collection use cases

- power consumption and memory utilization

Proposal 5: RAN2 should study which solution to be adopted for data transfer:

- enhanced control plane

- enhanced user plane

- new solution (e.g., new data plane, new entity/protocol layer, new RB)

Proposal 6: RAN2 to aim to support a single framework for RAN domain specific AI/ML applications and services in 6G

Proposal 7: RAN2 to study the possibility of introducing a new AI/ML node/entity in RAN domain to enable the single framework for RAN domain specific AI/ML.

# References

[1] RP-252912, Revised SID: Study on 6G Radio, NTT DOCOMO, CMCC, AT&T, Vodafone

[2] TR38.843

[3] R2-2507374 Transfer of various data and AIML framework