3GPP TSG-RAN WG2 #132	R2-2508294

Dallas, USA, 17-21 November 2025

Agenda item:		10.3.3.3

Title:		Additional Security Aspects for 6G

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In last RAN2 meeting in Prague (RAN2#131bis), extensive discussions were taken with regard to the MAC CE based security for 6G radio. An LS to SA3 on security was approved in R2-2507945 based on the post RAN2 email discussion for this aspect.

However, other than MAC layer based security, there are additional aspects for 6G security consideration including false base station, system information, etc. This contribution discusses additional security aspects for the design of 6G radio air interface.

# 2. Discussion

2.1 False Base Station

In the SID on Security for the 6G System (SP-251132, approved in SA#109), there is a clear justification description on the consideration of cyberattacks and malicious abuse. For example, In the description about the justification, it is said that “From the first release of 6G, 6G security needs to consider cyberattacks and malicious abuse.”

Additionally, During the study on 6G use cases and service requirements carried out by 3GPP SA1, a use case was being studied on UE selecting a base station after assessing its legitimacy in TR22.870 (with the latest version in v0.4.1). The majority of the efforts in the mobile communication system security has been focused on delivering secure communication once connected, whereas security related to bootstrapping those connections are relatively less explored. A UE can interact with a Base Station without assessing its legitimacy, which opens way for diverse attacks. False Base Station (FBS) attacks are one such example and its potential to cause active and passive impacts are an alarming concern, worldwide. Addressing fundamental security issues is important and can entail a system level reappraisal of risk and expectations. This is most feasible to undertake at the beginning of a new mobile generation. Attacks mounted by a FBS are a major security flaw in the existing communication system. There is an important opportunity to address this concern in 6G wireless systems.

By increasing the level of security between the radio network entities and the UE during the initial network selection procedure, False Base Station (FBS) attacks can be prevented at earlier stages of communication establishment. One such example is the introduction of an efficient Base Station authenticity verification mechanism such that the UE can verify the authenticity of the Base Station prior to starting the 6G registration procedure.

Proposal-1: RAN2 to study 6G AS layer security for the UE to be able to distinguish a False Base Station from an authentic Base Station from the first release.

2.2 Security for System information

In legacy wireless systems specified by 3GPP, the system information (SI) (e.g. MIB/SIBs) over the air interface, carries important parameters for UE’s initial access and system acquisition.

In Release-18, SA3 carried out the study for security protection of system information. The study is about investigating if and how a new protection mechanism could be introduced against over-the-air attackers who broadcast rogue SI messages or replay previously captured SI messages as-is (without modification). Since SI messages are broadcast messages meant for all UEs, it is not apparent that an integrity and replay protection is strictly necessary. Nevertheless, in general, an integrity and replay protected SIs could add security value by at least making it difficult for over-the-air attackers to succeed in using a rogue SI or a previously captures SI at a later time, e.g., to lure UEs using SI messages with incorrect neighbouring cells, and to send self-crafted or old PWS messages.

According to the previous study made SA3, lack of protection of SI could potentially have following impacts in some cases:

-	DoS attack on UE

-	Rogue services

In Release-18, a basic solution was identified by SA3 for SI protection in TR33.809, which is to have gNB provide the hash of the MIB/SIBs to the UE after it has established an AS security context. For the initial access, the AS security mode command is the first message that is integrity protected by the gNB, hence is used to carry the hash values of the MIB/SIBs. PCI (Physical Cell Identity) is additionally included in the calculation of the MIB/SIB hash. The UE determines whether it has read the correct MIB/SIBs by comparing the gNB provided hash values with the locally computed ones. If hash mismatch is detected, the UE requests the concerned MIB/SIBs. This solution does not require any additional key provisioning and setup procedure but only requires transporting the hashes of MIB/SIBs using secure RRC signalling. However, that solution can only support the UE that is in the RRC-Connected state.

Proposal-2: RAN2 to study 6G AS layer security to protect system information and mitigate threat from DoS attack and Rogue services.

For the study of 6G system information security, it should provide a means to ensure a UE in any RRC state is able to determine the authenticity of system information obtained from a cell.

Proposal-3: RAN2 to study security mechanism to ensure a UE in any RRC state is able to determine the authenticity of system information obtained from a 6G cell.

In the last RAN2 meeting, when the discussion was taken for the security protection of MAC CE, some companies raised concerns on the protection of all of the MAC CEs. The reason behind is some MAC CEs are quite frequent and the provision of security may lead to tremendous overhead. We believe the same discussion should apply to the security protection of system information. It would be prominent to classify the potential 6G MIBs and SIBs in terms of frequency and overhead criticality. For example, the MIB and SIB1 are periodically and frequently broadcasted. Some SI message(s) are quite stable. But some other SI message(s) are changed very often. If a digital signature (e.g. the HASH value of the content of the SIB) is transmitted together with the SIB message for the purpose of security verification, there will be a severe overhead over the air interface if the SIB are periodically and frequently broadcasted to the UE. With this said, it may be beneficial to discuss which part of the system information should be subject to the security protection.

Proposal-4: RAN2 to study which part of the system information should be subject to the security protection for 6G radio.

It should be noted that 3GPP SA3 has studied the PWS security during Release-12 for 4G LTE and the result was captured in TR 33.969, in which the security mechanisms are used to protect the Warning notifications message from Public Warning System against false base station. Based on such study, the following distinct challenges can be noted for the study of security protection for system information in 6G radio:

a)	Key management. It is because of heterogenous trust-boundaries, and diverse regulations (or requirements) per countries (or regions);

b)	Time synchronization. It is because of difficulty to achieve fairly acceptable time synchronization between one gNB and other gNBs, and between UEs and gNBs;

c)	Signaling complexity. It is because of restrictive signaling expected from UEs in RRC_IDLE.

From RAN2 perspective, it is proposed that the time synchronization and signalling complexity should be considered for the study of security protection for system information.

Proposal-5: the time synchronization and signalling complexity should be considered by RAN2 for the study of security protection for system information in 6G radio.

2.3 Security for Data Plane

As discussed in our companion contribution for agenda item 10.3.3.2, a new data plane is proposed to handle the traffic management for various types of data (e.g. AI/ML, Sensing, etc).

The security key for “Data Plane” may reuse the same security key as 6G User Plane. For example, when the sensing data is carried by PDCP layer, the sensing data is subject to ciphering like normal data. Or alternatively, the security key for “Data Plane” may use a different security key from 6G User Plane. A specific sensing data ciphering is used (e.g. a specific ciphering key is derived and used for new type of data transmission).  As the key derivation architecture is managed by SA3 Work Group, SA3 may need to be informed during the study if a security key for “Data Plane” is required.

Proposal-6: RAN2 to collaborate with SA3 for the security aspects for 6G “Data Plane” via a liaison when appropriate.

# 3. Conclusion

In this contribution the following proposals are concluded:

Proposal-1: RAN2 to study 6G AS layer security for the UE to be able to distinguish a False Base Station from an authentic Base Station from the first release.

Proposal-2: RAN2 to study 6G AS layer security to protect system information and mitigate threat from DoS attack and Rogue services.

Proposal-3: RAN2 to study security mechanism to ensure a UE in any RRC state is able to determine the authenticity of system information obtained from a 6G cell.

Proposal-4: RAN2 to study which part of the system information should be subject to the security protection for 6G radio.

Proposal-5: the time synchronization and signalling complexity should be considered by RAN2 for the study of security protection for system information in 6G radio.

Proposal-6: RAN2 to collaborate with SA3 for the security aspects for 6G “Data Plane” via a liaison when appropriate.

# References

[1] TR33.809

[2] R2-2507945 LS to SA3 on 6G security

[3] R2-25nnnnn Transfer of various data and AIML framework

[4] 3GPP TR 22.870 V0.4.1 (2025-10-24), Study on 6G Use Cases and Service Requirements; Stage 1 (Release 20)