3GPP TSG-RAN WG2 Meeting #132	R2-2508296

Dallas, US, November 17th – 21st, 2025

Agenda Item:	10.3.3.2

Source: 	OPPO

Title:            Discussion on energy efficiency in 6G

Document for:	Discussion and Decision

Introduction

In RAN#108 meeting, a new SID on 6GR [1] was approved. According to the SID, one of the requirements for 6G study is energy efficiency and energy saving both for network and device.

In RAN2#131 meeting, network and UE energy efficiency was preliminarily discussed, with the following agreement made.

- For next meeting companies are encouraged to identify what UE/NW power saving features can be considered and improved in 6G while avoiding specifying multiple features for the same purpose. Focus on solutions that have RAN2 impacts.

In this contribution, we further discuss energy efficiency in 6G, and present our views.

Discussion

  - Lessons from 5G

In 5G Day-1, no dedicated features were designed aiming for UE and network energy saving, resulting in poor performance with respective to power consumption for commercial 5G UE devices. Then, UE power saving was studied since Rel-16, while network energy saving related features were introduced since Rel-18, with multiple features introduced so far. The UE power saving and NES features in 5G are summarized in table 1 and table 2, respectively.

Table1 Summary of UE power saving features in 5G

| Standard release | UE power saving feature |
| --- | --- |
| Rel-15 | Time domain: i-DRX, C-DRX Frequency domain: SCell (de-)activation, BWP switching |
| Rel-16 | Time domain: DCP for C-DRX, cross slot scheduling, dual DRX group for FR1+FR2 CA Frequency domain: dormant BWP for SCell Spatial domain: Maximum MIMO layer adaptation Measurement: RRM relaxation for RRC idle/inactive state UEs UAI for UE power saving |
| Rel-17 | Time domain: PEI for i-DRX, eDRX, provision of potential TRS/CSI-RS occasion(s) available to idle/inactive state UEs, SSSG switching, PDCCH skipping Measurement: RLM/BFD relaxation, RRM relaxation for stationary RedCap UEs |
| Rel-18 | Longer eDRX for RRC inactive state UEs |
| Rel-19 | LP-WUS for both RRC Idle/inactive and RRC connected state UEs |

Table2 Summary of NES features in 5G

| Standard release | NES feature |
| --- | --- |
| Rel-15 | Configurable SSB period on-demand OSI, validity area for SIB |
| Rel-18 | Time domain: Cell DTX/DRX for RRC connected state UEs Frequency domain: SSB-less SCell for inter-band CA and co-located cells CSI enhancements for adaptations in spatial and power domains Enhancements on CHO procedure for NES cell(s) |
| Rel-19 | Time domain: on-demand SSB for SCell, on-demand SIB1, SSB periodicity adaptation, paging adaptation, PRACH adaptation |

Based on above summary, it’s observed that although many 5G energy saving features are useful, the way those features were introduced (i.e., independent features related to power saving introduced in later release after 5G Day-1) result in some drawbacks:

- Overlapped design for the same purpose: e.g., DCP and LP-WUS for RRC connected state UEs, both of which target for DRX active time reduction so as to reduce UE’s unnecessary PDCCH monitoring. Another example is PEI and LP-WUS for RRC idle/inactive state UEs, both of which target for UE power consumption reduction due to false paging alarms.

- Complicated specification and co-existence due to multiple features: some efforts were made on whether features can work together or not, e.g., the co-existence of PEI and LP-WUS. Somehow, more specification work is the consequence, but without any benefits to real deployment.

- Backward compatibility: e.g., cell DTX/DRX is only appliable to RRC connected state UEs, while those key parts impacts power saving, e.g., the random-access procedure, SSB transmission, paging, and system information broadcasting were not touched, leading to limited the NES gain.

- Very limited commercialization in real field: almost all the dedicated features for power saving were introduced after 5G Day-1, causing less support for these features from both UE and network, which limits their commercialization, e.g., when a Rel-19 UE supporting LP-WUS access a legacy (Rel-17 or Rel-18) cell, or if a legacy UE accesses a Rel-19 cell supporting LP-WUS, the UE cannot benefit from LP-WUS. Another reason is that UE power saving and NES are designed independently, resulting in a lack of mutual benefits to implement these features. Lastly, the product line may wait and want to see more advanced features, to be introduced in later release, while keep evaluating the features in current release.

In 6G, in order to ensure better power saving performance than 5G for both UE and network, and to facilitate the successful commercialization, energy saving features should be supported from 6G Day-1.

- In 5G, there are multiple features designed for the same target, which result in some drawbacks, e.g., over-designing, complicated specification, and very limited commercialization.

- In 5G, many useful dedicated energy-saving features were introduced after 5G Day-1. Due to the challenges with backward compatibility and limited commercialization, the energy saving gain is not maximized.

- In 5G, UE power saving and NES are designed independently, resulting in a lack of mutual benefits to implement these features.

- 6G supports energy efficiency from Day-1. Energy saving features should be designed in a coordinated and unified manner to avoid functional redundancy and to prevent independent UE and network power saving design.

  - UE power saving

In 5G, a UE in RRC idle state needs to wake up every i-DRX cycle to monitor paging. UE does not know whether it is paged by network until it decodes paging message. In scenarios with low paging probability, the UE would wake up unnecessarily in most i-DRX cycles. In order to reduce UE power consumption due to false paging alarms, PEI and LP-WUS have been introduced in Rel-17 and Rel-19, respectively, both of which can provide subgroup based wake-up indication on whether to monitor its associated PO. So for RRC idle state UEs, we now have two types of wake-up signalling, i.e., PEI and LP-WUS. Both features are aiming for the same purpose, i.e., trying to improve energy efficiency for RRC idle state UEs. Although PEI and LP-WUS can be used together, however, it’s hard to say, more power saving gain can be obtained if we have both two features in the real field.

- In 5G, PEI and LP-WUS are introduced in two dependent releases, both of which are aiming for the same target, i.e., reduce RRC idle state UEs’ power consumption due to false paging alarms.

For RRC connected state UEs, the situation is very similar. As a foundational feature inherited from LTE, C-DRX is supported in NR Rel-15. Then, DCP and LP-WUS were introduced in Rel-16 and Rel-19, respectively, both of which aims to reduce UE’s unnecessary PDCCH monitoring on top of C-DRX. Since functionalities of LP-WUS and DCP are fully redundant, it is specified that network will not configure LP-WUS and DCP for a UE simultaneously.

- In 5G, DCP and LP-WUS are introduced in two dependent releases, both of which are aiming for the same target, i.e., reduce RRC connected state UE’s DRX active time.

As an effective energy saving tool, the wake-up signalling feature should be supported in 6G, for both RRC idle state UEs and RRC connected state UEs. And the wake-up scheme should be simplified compared to 5G, i.e., there is no need to support multiple wake-up signalling simultaneously in 6G. Instead, a single wake-up signal is sufficient. In our view, LP-WUS is an effective mechanism for UE power saving, which can reduce UE power consumption and latency by being able to monitor the WUS with a LR such that the UE’s MR can be kept in a sleep state. The main drawback of LP-WUS in 5G is that it requires UE to equipped with a separate low power radio, which may increase the UE cost. Another issue is that it may only support partial coverage of the cell due to the degraded link budget of LP-WUS/LP-SS. How to design the wake-up signalling and how to ensure/improve its coverage performance is up to RAN1 study. Form RAN2 perspective, we can focus on the wake-up related procedure.

- For both RRC idle state and RRC connected state, try to focus on a single/unified wake-up scheme. The signalling design is up to RAN1, while RAN2 may focus on the wake-up signalling related procedure.

In 5G, paging can be used for the network to reach UEs in RRCidle stateand in RRC inactive state upon DL data arrival through paging message, and to notify UEs in all RRC states of system information change and ETWS/CMAS notifications through short message. For DL data arrival triggered paging, if UE receives a P-RNTI scrambled PDCCH in its PO, the UE further decodes PDSCH scheduled by the PDCCH and processes the paging message. If its UE ID is included in the pagingRecordList in paging message, the UE initiates RRC connection establishment. For paging triggered by system information change or ETWS/CMAS notification, when UE receives paging PDCCH, UE could be aware of whether there will be system information change and/or ETWS/CMAS notification through systemInfoModification and etwsAndCmasIndication, respectively, with 1 bit for each in short message. If the systemInfoModification bit in short message is set to 1, the UE (re-)acquires SIB1, checks the validity for each SIB, and then acquires the SIB(s) for the one(s) considered as invalid from the start of the next modification period. If the etwsAndCmasIndication bit in short message is set to 1, the UE immediately (re-)acquires SIB1 and acquires SIBs for PWS according to the si-SchedulingInfo in SIB1.

- In NR, paging may be triggered by DL data arrival or system information change and ETWS/CMAS notifications.

In 5G, for RRC idle state UEs, both PEI and LP-WUS are used to wake up UE for paging monitoring. For both features, UE does not know the purpose of the paging (i.e., whether paging is triggered by DL data arrival or it is intended for system information change and ETWS/CMAS notification) until it receives paging.

- In NR, when a UE is waken up by PEI/LP-WUS, the UE always monitors PO firstly, regardless of the trigger event.

Table 3 wake-up signalling based notification

| Paging trigger event | UE behaviour if there is no paging trigger indication in wake-up signaling | UE behaviour if there is paging trigger indication in wake-up signaling |
| --- | --- | --- |
| DL Data Arrival | Monitor/receive PEI/LP-WUS -> monitor/receive paging -> Initiate RRC connection establishment | Monitor/receive PEI/LP-WUS -> monitor/receive paging -> Initiate RRC connection establishment |
| System Information Change | Monitor/receive PEI/LP-WUS -> monitor/receive paging -> (re-)acquires SIB1 -> check validity for each SIB -> acquires invalid SIB(s) | Monitor/receive PEI/LP-WUS -> (re-)acquires SIB1 -> check validity for each SIB -> acquires invalid SIB(s) |
| ETWS/CMAS Notification | Monitor/receive PEI/LP-WUS -> monitor/receive paging -> (re-)acquires SIB1 -> acquires SIBs for PWS | Monitor/receive PEI/LP-WUS -> (re-)acquires SIB1 -> acquires SIBs for PWS |

Based on above, we propose that in 6G, RAN2 studies more efficient mechanism to notify paging/system-information-change/ETWS-CMAS-indication to further reduce UE power consumption, e.g., UE could be aware of whether there is paging and/or system information change and/or ETWS-CMAS-indication via wake-up signalling reception, rather than being mandated to read PO to differentiate the cases.

- For RRC idle state UEs, RAN2 studies early notifying scheme of paging purpose by designing more efficient wake-up scheme, so that the UE can be indicated to skip some of the paging monitoring due to paging purpose. RAN2 further check with RAN1 on the wake-up signalling feasibility.

For RRC connected state UEs, in 5G, UE have C-DRX, DCP and LP-WUS, all of which are used to control UE’s PDCCH monitoring. C-DRX requires UE to wake up in every DRX cycle, even there is no scheduling for both DCP and LP-WUS, while DCP and LP-WUS can enable UE to monitor PDCCH in a more flexible way. Currently, both DCP and LP-WUS can only be configured on top of C-DRX. DCP and LP-WUS option 1-1 are used to indicate UE whether to monitor PDCCH in the next DRX onduration. For LP-WUS option 1-2, LP-WUS is monitored by UE with a periodicity independent of DRX cycle. In this option, scheduling delay would depend on the LP-WUS monitoring periodicity rather than C-DRX cycle. In this sense, C-DRX seems duplicated when LP-WUS option 1-2 is used. In 6G, if the wake up signalling can be designed such that monitoring for it is sufficiently power efficient, we can study further enhancement on wake-up procedure for RRC CONNECTED UEs, e.g., wake-up operation without DRX.

- For RRC connected state UEs, DRX and wake-up scheme are somehow overlapped, both of which are used to control UE’s PDCCH monitoring.

- For RRC CONNECTED state UEs, RAN2 studies more simplified UE power saving features, e.g., support wake-up signal operation without DRX.

  - Network energy saving

In 5G, to facilitate reducing gNB transmission/reception activity time, cell DTX/DRX was introduced in Rel-18. A UE can be configured with a periodic cell DTX/DRX pattern. During non-active periods of cell DTX/DRX, certain channels/signals (i.e., PDCCH in selected cases, SPS and LP-WUS for DL, CG and SR for UL) are not received/transmitted on the cell by the UE. As an effective network energy saving feature, cell DTX/DRX should be supported in 6G, with following potential enhancements.

- Applying cell DTX/DRX to both RRC connected state and RRC idle state

In 5G, due to the backward compatibility concern, cell DTX/DRX is only appliable to RRC connected state UEs, while RRC idle state procedure, such as random access, paging, system information broadcasting are not touched. In 6G, without backward compatibility constraints, cell DTX/DRX can be applied to both RRC connected and RRC idle states. When applying it to RRC idle state, it would affect SSB, paging, and RACH. Based on the ongoing RAN1 discussions regarding clustered SSB, paging, and RACH, if the final design of these channels enables their periods to align with the DTX/DRX cycle, then from our perspective, the application of cell DTX/DRX for RRC idle state could be fully up to network configuration. Since RAN1 is actively promoting this topic, we can wait for further progress from RAN1.

- Support more flexible cell DTX/DRX active time

In NR, once cell DTX/DTX is configured and activated, both UE and network need to wake up every DRX cycle, even if there is no transmission in the cell DTX/DRX active duration. On the other hand, the static cell DTX/DRX on-off pattern presents a challenge for delay-sensitive service. For example, if UE triggers a SR due to UL data arrival of a logical channel with low latency requirement, or if gNB receives DL data of a delay-sensitive traffic in the Cell DRX/DTX non-active time, since either SR or PDCCH for new transmission are prohibited during the Cell DRX/DTX non-active period and shall be postponed at least to the next Cell DRX/DTX active period, the data transmission may suffer significant latency. In 6G, we should study to optimize Cell DTX/DRX mechanism for energy efficiency while guaranteeing transmission latency, e.g. support UE or network triggered cell DTX/DRX adaptation via wake-up signalling.

- In 5G, due to backward compatibility constraints, cell DTX/DRX is only appliable to RRC connected state UEs.

- In 5G, Cell DTX/DRX enables network energy saving by suspending certain transmission/reception for RRC connected state UEs during the Cell DTX/DRX non-active periods, which presents challenges for latency-sensitive services.

- In 6G, further enhance cell DTX/DRX to 1) applying cell DTX/DRX to both RRC connected state and RRC idle state UEs, 2) support UE or network triggered cell DTX/DRX adaptation

In NR, the on-demand mechanism was introduced to reduce always-on system transmissions in the time domain, aiming to improve power efficiency for both the UE and the network. On-demand SIB transmission is supported for all SIB messages, including SIB1 and OSI. However, due to backward compatibility constraints, OD-SIB1 is currently only supported in NES cells with assistance from another cell. In 6G, where backward compatibility is no longer a limiting factor, the feasibility of OD-SIB1 for standalone cells can be explored to further enhance power-saving benefits.

- On-demand system information is supported in NR for both SIB1 and other system information, reducing the need for always-on transmission and reception in the time domain. This mechanism helps achieve power savings for both the network (NW) and user equipment (UE).

- For 6G system information energy efficiency, RAN2 studies scheme to support on-demand SIB1 for both single-layer and multiple-layer frequency scenario, taking 5G on-demand OSI mechanism as baseline.

  - Cooperated UE power saving

Currently, UEs within cell coverage perform independent operations (e.g., individual channel measurements, control signalling monitoring). However, in certain scenarios such as a personal area network, multiple UEs belonging to the same user, e.g., a smartphone and several smart IoT devices, are located in close proximity and they experience somehow similar channel conditions. If these devices can operate in a coordinated manner, redundant radio activities for some UEs can be eliminated, thereby conserving their battery power. For example, several UEs forms a cooperated UE group, if a UE helps other UEs in the same UE group to monitor paging and receive system information, it enables other UEs to sleep longer for power saving. Another example is that one UE performs measurement, and the measurement results may be shared with or applied to other UEs in the same cooperated UE group. Some of these functionalities, e.g., assisted paging monitoring have been supported in 5G sidelink relay. However, sidelink relay primarily addresses coverage issue for remote UEs out-of-coverage, which differs from the use case of cooperated UE power saving. Such cooperated UE power saving can be designed in a way that they remains transparent to network, i.e., network still take them as independent UEs. But if cooperated power saving is desired, i.e., one UE helps other UEs for power saving, some kind of information exchange between UEs would be needed, e.g., by non-3GPP interfaces.

- When multiple UEs are in close proximity, they often perform independent operations (e.g., individual channel measurements, control signalling monitoring), which leads to unnecessary power drain.

- Cooperative UEs can eliminate redundant radio activities for some UEs, thereby conserving their battery power.

- RAN2 studies UE cooperated power saving mechanisms to further improve UE energy efficiency.

Conclusion

Based on the discussion in section2, we make the following observations:

- In 5G, there are multiple features designed for the same target, which result in some drawbacks, e.g., over-designing, complicated specification, and very limited commercialization.

- In 5G, many useful dedicated energy-saving features were introduced after 5G Day-1. Due to the challenges with backward compatibility and limited commercialization, the energy saving gain is not maximized.

- In 5G, UE power saving and NES are designed independently, resulting in a lack of mutual benefits to implement these features.

- In 5G, PEI and LP-WUS are introduced in two dependent releases, both of which are aiming for the same target, i.e., reduce RRC idle state UEs’ power consumption due to false paging alarms.

- In 5G, DCP and LP-WUS are introduced in two dependent releases, both of which are aiming for the same target, i.e., reduce RRC connected state UE’s DRX active time.

- In NR, paging may be triggered by DL data arrival or system information change and ETWS/CMAS notifications.

- In NR, when a UE is waken up by PEI/LP-WUS, the UE always monitors PO firstly, regardless of the trigger event.

- For RRC connected state UEs, DRX and wake-up scheme are somehow overlapped, both of which are used to control UE’s PDCCH monitoring.

- In 5G, due to backward compatibility constraints, cell DTX/DRX is only appliable to RRC connected state UEs.

- In 5G, Cell DTX/DRX enables network energy saving by suspending certain transmission/reception for RRC connected state UEs during the Cell DTX/DRX non-active periods, which presents challenges for latency-sensitive services.

- On-demand system information is supported in NR for both SIB1 and other system information, reducing the need for always-on transmission and reception in the time domain. This mechanism helps achieve power savings for both the network (NW) and user equipment (UE).

- When multiple UEs are in close proximity, they often perform independent operations (e.g., individual channel measurements, control signalling monitoring), which leads to unnecessary power drain.

- Cooperative UEs can eliminate redundant radio activities for some UEs, thereby conserving their battery power.

And we give the following proposal:

- 6G supports energy efficiency from Day-1. Energy saving features should be designed in a coordinated and unified manner to avoid functional redundancy and to prevent independent UE and network power saving design.

- For both RRC idle state and RRC connected state, try to focus on a single/unified wake-up scheme. The signalling design is up to RAN1, while RAN2 may focus on the wake-up signalling related procedure.

- For RRC idle state UEs, RAN2 studies early notifying scheme of paging purpose by designing more efficient wake-up scheme, so that the UE can be indicated to skip some of the paging monitoring due to paging purpose. RAN2 further check with RAN1 on the wake-up signalling feasibility.

- For RRC CONNECTED state UEs, RAN2 studies more simplified UE power saving features, e.g., support wake-up signal operation without DRX.

- In 6G, further enhance cell DTX/DRX to 1) applying cell DTX/DRX to both RRC connected state and RRC idle state UEs, 2) support UE or network triggered cell DTX/DRX adaptation

- For 6G system information energy efficiency, RAN2 studies scheme to support on-demand SIB1 for both single-layer and multiple-layer frequency scenario, taking 5G on-demand OSI mechanism as baseline.

- RAN2 studies UE cooperated power saving mechanisms to further improve UE energy efficiency.

Reference

[1] RP-251881   New SID: Study on 6G Radio