3GPP TSG-RAN WG2 Meeting#132                               	R2-2508301

Dallas, USA, Nov 17th – 21st, 2025

Agenda item:	10.2.2

Source: 	Google

Title: 	Discussion on the harmonized 6GR for TN and NTN

Document for:	Discussion and Decision

# Introduction

Initially approved at RAN#108 and subsequently updated at RAN#109, the 6G-Radio study item outlines core objectives for the 6G radio protocol architecture [1]. These objectives explicitly include the goal of achieving a harmonized 6G radio design that integrates both TN and NTN.

The detailed objectives of the study are: - Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

In this paper, we outline a strategic approach for achieving the harmonized 6G radio design that integrates both TN and NTN. The paper begins by analyzing existing 5G NR-NTN features to identify and shortlist those essential for 6G Day-1 implementation. Furthermore, we present several proposals aimed at equipping the 6G system with more comprehensive support for TN-NTN mobility (surpassing 5G system) and establishing enhanced access control mechanisms specifically tailored for cells with extended coverage, such as GEO cells.

# Discussion

## Day-1 features and TN-NTN common design

The dream of true global connectivity, extending the reach of high-speed internet and reliable communication to every corner of the Earth, is rapidly moving toward reality through NTN utilizing satellite constellations. However, communicating with devices on the ground UE via orbiting satellites is fundamentally different from terrestrial network and introduces a unique set of physical and logistical challenges. The vast distance and constant motion inherent to satellite systems dictate three primary physical and operational challenges:

1. Very long and high-variant propagation delay

Unlike terrestrial cellular networks where the signal path is relatively short, signals traveling to and from satellites especially those in GEO, must traverse tens of thousands of kilometers. This results in a very long propagation delay, which significantly impacts the performance of delay-sensitive applications and protocols. Besides, the delay is high-variant, particularly in LEO and MEO systems. When a satellite moves across the sky relative to a UE fixed on the ground, the path length constantly changes, causing the signal travel time to fluctuate rapidly. This dynamic delay complicates the accurate timing and synchronization necessary for the communication between the UE and the network.

2. Frequent handover and cell-switch

Satellites, especially in fast-moving LEO constellations, can only serve a UE for a limited time before moving out of range. This necessitates frequent handover. For a UE in the idle mode, the fast-moving LTE constellations lead to frequent cell-switches (i.e., cell reselections) from one cell to another. The network must minimize the impacts of these handovers and cell-switches (e.g., by precisely triggering the HO/cell-switch, or by reducing the HO/cell-switch latency), otherwise the user experience in using the NTN will be compromised.

3. Large Doppler Shift

Due to the extremely high velocities of LEO and MEO satellites, the relative motion between the satellite transmitter/receiver and the ground-based UE causes a significant frequency shift known as the Doppler effect. This large Doppler shift must be precisely predicted and compensated by the UE at least in the UL direction. Uncorrected Doppler shift can disrupt the frequency synchronization of the wireless link, severely degrading signal quality, frame alignment, and overall communication reliability. The magnitude of this shift is constantly changing as the satellite's movement relative to the UE changes from approaching to receding, demanding sophisticated determination of the UE’s and satellite’s position.

The key challenges of the NTN communication are: 1) very long and high-variant propagation delay, 2) frequent handover and cell-switch, and 3) large Doppler shift.

The 5G NTN base station provides the UE with satellite assistance information (e.g., satellite ephemeris, common TA, Kmac, epoch time) of the concerned cell, which make it possible for the UE to calculate the distance and the change of the distance between the UE and the concerned satellites, provided that the UE’s GNSS position information is also available. The aforementioned challenges become manageable when the distance and the change of distance between the UE and the concerned satellites is known to the UE.

However, the current procedure for acquiring essential satellite assistance information via SIB19 in NR-NTN creates a delay for NTN-capable UE during cell access. To minimize the cell access time and enhance mobility performance for NTN-capable UEs, the critical satellite assistance information currently contained in SIB19 must be moved to, or referenced within,SIB1. Integrating or referencing the critical NTN parameters within SIB1 ensures that the UE obtains all necessary information in a single, mandatory acquisition step, and establishes a first step toward the goal of having an integrated TN-NTN system design.

The satellite assistance information (e.g., ephemeris, common TA, Kmac, epoch time) of the serving cell is included in the essential SIB (e.g., SIB1).

Addressing the negative impacts of aforementioned challenges requires a strong foundation. Therefore, the suite of UE-centric enhancements and protocol optimizations developed for 5G NTN (summarized below) is considered critical and shall be repurposed as the essential Day-1 feature set to ensure the initial stability and functional readiness of 6G NTN systems.

HARQ Feedback Disabling: To address the excessively long latency that renders standard HARQ timing ineffective and inefficient, the capability to disable HARQ feedback is essential. This moves the focus to repetition and higher layer retransmissions (e.g., RLC AM) better suited for high-latency channels.

Time Alignment (TA) Compensation by the UE: To account for the large and time-variant propagation delay and ensure uplink timing is correct, the UE must be equipped to autonomously calculate and apply TA compensation. This offloads the precise timing burden from the network.

Frequency Compensation by the UE: To counter the large and dynamic Doppler shift, the UE is required to implement frequency compensation to accurately shift its transmission frequency, ensuring the signal arrives at the gNB/satellite receiver within the acceptable frequency window.

TA Report and Koffset adjustment: Even with the UE TA compensation, the network still needs to track timing evolution. The UE shall report the calculated TA (for service-link) along with a network-configured common TA (for feeder-link) and a TA correction. This allows the network to be aware of the genuine RTT and schedule uplink traffic according to the RTT.

Time-/Location-Based Mobility: To address the issue of frequent and reactive handovers caused by high satellite mobility, the system will leverage precise satellite ephemeris data and UE location to implement predictive mobility procedures. This minimizes service interruption.

GNSS-Resilient Operation (Rel-20): Given that GNSS signals can be unavailable (e.g., indoors or jammed), the NTN system must support GNSS-resilient operation for critical functions like timing, location, and mobility management. This ensures service continuity even when precise positioning data is not available.

Day-1 features for 6G NTN includes: 1) HARQ feedback disabling, 2) TA compensation by the UE, 3) frequency compensation by the UE, 4) TA report and Koffset adjustment, 5) time-/location-based mobility, and 6) GNSS-resilient operation.

The above day-1 NTN features provide solutions based on enhanced UE autonomy in timing/frequency management and robust mobility management. These solutions are not exclusive to satellites; they address generic challenges present in TN under conditions like extended cell range or high-speed UE traversal. To maximize system versatility, the 6G system should allow the flexibility to configure these essential NTN features selectively within challenging TN environments.

Day-1 features for 6G NTN can be also configured in a TN cell when needed.

## Comprehensive support for TN-NTN mobility

Cell type identification is critical for the decision-making in the cell reselection and handover procedure. The UE can determine whether to prioritize TN or NTN neighbors based on service policy, expected propagation delay, or expected link quality. In order to improve the mobility efficiency, the UE should be able to know the cell type (TN or NTN) of a cell upon acquiring the cell identity of the cell through measurement. This can be achieved by leveraging the approach used in 5G NR-NTN. Specifically, the serving cell reveals the cell type of neighbor cells by including their cell identities within the satellite assistance information provided in the system information.

UE should be able to know the cell type (TN or NTN) of a cell upon acquiring the cell identity of the cell through measurement.

A core design objective of the 6G radio is the integration of TN and NTN components. Ensuring UEs have the necessary information to access the integrated environment is a foundational step toward achieving this objective. In 5G, TN cells broadcast satellite assistance information in system information, allowing NTN-capable UEs to facilitate measurements of NTN neighbor cells. Conversely, 5G NTN cells broadcast TN area information in system information, which UEs leverage to reduce power consumption while measuring TN neighbor cells. Adopting the same information exchange methodology used in 5G NR-NTN can ensure seamless and efficient mobility across the integrated 6G architecture.

Same as in 5G, 6G TN cells can broadcast the satellite assistance information in system information, and 6G NTN cells can broadcast TN area information in system information.

Nowadays, operators are increasingly deploying satellite access or implementing roaming agreements with other operators for satellite access.  Because NTNs are often not confined to geopolitical boundaries, it is common for an NTN to have a PLMN ID that differs from the PLMN IDs for TNs. It is also likely that there are multiple TN cells operated by different TN operators within the coverage of the same NTN cell, and these TN operators have a roaming agreement with the same satellite operator owning the NTN cell. In such a setup, the NTN cell (with an NTN-specific PLMN) may have to support all the UEs registered to these TN operators, and hence have to broadcast the information of the TN frequencies and the associated TN areas used by these TN operators.

To enable further UE energy saving, we propose that a 6G NTN cell can indicate, via system information, the PLMN ID(s) of a TN frequency together with the carrier frequency information of the TN frequency. With this information, the UE is able to determine whether to perform measurements on a TN frequency, not only based on whether the UE is close to the deployment of that TN frequency, but also based on whether the UE is able to access the PLMN operating that TN frequency.

The PLMN ID(s) of a TN frequency can be provided in the system information, by an NTN cell.

## Location-specific access control

In large satellite cells, such as those provided by GEO satellites, network coverage can span multiple distinct geographical areas. When one of these areas becomes densely populated, a massive number of users may attempt to connect to the network simultaneously, causing localized network congestion. The current standard response is to enable cell-wide congestion control mechanisms like the cell barring or the UAC.  Because conventional congestion control mechanisms are configured on a cell-wide basis, a UE located in a part of the cell not experiencing localized congestion is subject to the same congestion control parameters as a UE located within the congested area.  For example, a UE in a non-congested geographic area (such as a rural area) may be barred from accessing a network due to congestion occurring hundreds of kilometers away in a different geographic area within that cell, which may create fairness issues.

Crucially, a satellite's cell coverage often spans multiple countries with disparate regulations and may serve various MNOs contracted by the same SNO. Therefore, implementing independent access control configuration for each country and contracted MNO is essential to prevent network interference, ensure regulatory compliance, and maintain equitable service operation.

Cell barring and UAC configuration in 6G systems shall be designed to enable location-specific control, restricting their applicability to designated geographic area(s).

# Conclusion

In this paper, we discuss a strategic approach for achieving the harmonized 6G radio design that integrates both TN and NTN. Based on the discussion and observations made in the paper, we respectfully ask RAN2 to discuss and consider the following proposals.

Day-1 features and TN-NTN common design

Observation 1	The key challenges of the NTN communication are: 1) very long and high-variant propagation delay, 2) frequent handover and cell-switch, and 3) large Doppler shift.

Proposal 1	The satellite assistance information (e.g., ephemeris, common TA, Kmac, epoch time) of the serving cell is included in the essential SIB (e.g., SIB1).

Proposal 2	Day-1 features for 6G NTN includes: 1) HARQ feedback disabling, 2) TA compensation by the UE, 3) frequency compensation by the UE, 4) TA report and Koffset adjustment, 5) time-/location-based mobility, and 6) GNSS-resilient operation.

Proposal 3	Day-1 features for 6G NTN can be also configured in a TN cell when needed.

Comprehensive support for TN-NTN mobility

Proposal 4	UE should be able to know the cell type (TN or NTN) of a cell upon acquiring the cell identity of the cell through measurement.

Proposal 5	Same as in 5G, 6G TN cells can broadcast the satellite assistance information in system information, and 6G NTN cells can broadcast TN area information in system information.

Proposal 6	The PLMN ID(s) of a TN frequency can be provided in the system information, by an NTN cell.

Location-specific access control

Proposal 7	Cell barring and UAC configuration in 6G systems shall be designed to enable location-specific control, restricting their applicability to designated geographic area(s).

# Reference

RP-252912, Revised SID: Study on 6G Radio, NTT Docomo, CMCC, AT&T, Vodafone, Beijing, CN.

3GPP TR 38.914, v0.2.0, Study on 6G Scenarios and Requirements, RAN#109.