3GPP TSG-RAN WG2 Meeting #132	R2-2508312

Dallas, USA, 17 – 21 November 2025

Agenda item:	8.10.2

Source:	Nokia

Title:	[N066] [N067] RIL Corrections

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

This paper provides background information and TP for RIL comments N066 and N067.

# 2	Discussion

## 2.1	SHR availability for RACH-less LTM [N066]

Since SHR availability indication cannot be included in the RRCReconfigurationComplete message that finalizes the LTM cell switch, RAN2 agreed that this should be signaled in the next RRCReconfigurationComplete message that the UE sends (which may or not be related to a reconfigurationWithSync execution).

This solution, however, in case of multiple subsequent LTM cell switches, makes it likely that the SHR will be overwritten and lost before the indication is sent. Therefore, the current solution is suboptimal.

Observation 1: The current solution for SHR availability indication is suboptimal and can lead to the loss of SHR content in the case of subsequent LTM cell switches.

We propose to fix this problem by updating the UE behavior in case of RACHless LTM cell switch. More specifically, the UE would check SHR triggering conditions after receiving the cell switch command but before sending RRCReconfigurationComplete. If the checks for the SHR triggering conditions are met, the UE would add a pre-emptive availability flag to the RRCReconfigurationComplete message. As soon as the RRCReconfigurationComplete message is successfully delivered to the network, the UE can generate the SHR and deliver it to the network as per today’s specifications.

Proposal 1: RAN2 to agree the updated UE behavior to fix the SHR availability in case of RACH-less LTM.

As for the implementation of the solution, a simple way to capture it in the specifications is adding a note stating that in case of RACHless LTM cell switch, the UE checks the SHR triggering conditions just after cell switch command is received.

Proposal 2: Add the following NOTE in section 5.3.5.3 Reception of an RRCReconfiguration by the UE:

2>	if the RRCReconfiguration is applied due to an LTM cell switch execution:

3>	submit the RRCReconfigurationComplete message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

NOTE X:	In case of RACHless LTM cell switch, SHR generation is immediately triggered as specified in clause 5.7.10.6 before generating RRCReconfigurationComplete message.

## 2.2	Missing choWithCandidateSCGInfo in case of RLF shortly after successful completion of CHO with candidate SCGs [N067]

MRO optimizes the timing of the handover based on mobility failure types referring to mistimed triggering like Too Late Handover, Too Early Handover or Handover to Wrong Cell. In case of CHO with candidate SCGs (CwcS) the timing of execution depends on two conditions and, therefore, the chronological order of their fulfillment is important for a correct root cause analysis and finally resulted in the introduction of the IE choWithCandidateSCGInfoList in the RLF report, where the fulfillment of all configured pairs is logged.

But this information is only logged, if the UE is configured for CHO with candidate SCGs (CwcS), i.e., the UE possesses condReconfigIds with two execution conditions condExecutionCond and condExecutionCondPSCell.

Those entries are in UE’s VarConditionalReconfig when UE has been configured with CwcS, i.e., when the RLF occurs during the monitoring phase having at most only one condition fulfilled (referring to a too late CwcS execution provided that the reestablishCellId ≠ failedPCellId) or when both conditions are fulfilled but the CwcS execution itself fails (i.e., in case of HOF).

However, those condReconfigIds referring to CwcS will not exist for an RLF occurring shortly after the successful CwcS completion (case 8a from the agreed RAN3 failure scenarios), since all mobility configurations related to previous source PCell are released and UE starts in the new serving PCell without any conditional configuration, and without CwcS configuration, the new IE choWithCandidateSCGInfoList will not be logged.

Observation 2: The current specification does not specify logging of the information about the chronological fulfilment of the two execution conditions of the preceding executed CHO with candidate SCGs in case of RLF shortly after performing a successful CHO with candidate SCG.

RLFs shortly after handover are identified as mobility failures provided that the time was too short to allow for adequate reaction like new handover. Therefore, those failures could be, for instance, interpreted as a “CwcS execution to wrong PCell”, if the UE was reestablishing to a third PCell. Even though a small timeConnFailure would categorize this RLF as “CwcS execution to wrong PCell”, it cannot be properly analyzed without the information of the chronological fulfilment of the two execution conditions, since the new choWithCandidateSCGInfoList is not logged in the RLF report for this case.

Observation 3: Without the information of choWithCandidateSCGInfoList a proper MRO for failure cases where RLF occurs shortly after successful CHO with candidate SCGs completing (case 8a from the agreed RAN3 failure scenarios) is not possible.

With reception of the RRCReconfiguration message for CwcS configuration the UE starts monitoring the fulfilment of the two conditions for each condReconfigId. Thus, from UE implementation perspective this information about the chronological fulfilment of the two execution conditions is available. The monitoring stops with RLF or with execution. In case of execution, the UE will also keep the information for the HOF case. For the RLF shortly after successful completion of CwcS the information needs to be kept few seconds further (in the range of timeConnFailure) to be added. Thus, it can be concluded that UE would have the information of chronological condition fulfillment of preceding, but the current specification does not include the logging this information in the new IE choWithCandidateSCGInfoList.

Proposal 3: RAN2 agrees that the information about the chronological fulfilment of the two execution conditions of the last executed CHO with candidate SCG (which is captured in choWithCandidateSCGInfoList) in RLF report is missing for the cases where RLF occurred shortly after a CHO with candidate SCG.

Proposal 4: RAN2 agrees that amendment of section 5.3.10.5 is made if the last handover of the UE was a CHO with candidate SCG (i.e., lastHO-Type is set to choWithCandidateSCG) and UE was not configured with CHO with candidate SCG when RLF occurred, then the UE logs the information about of the last executed CHO with candidate SCG in choWithCandidateSCGInfoList. The text proposal is in the Annex of this paper.

# 3	Conclusion

This document has made the following observation, and proposal for SHR availability for RACH-less LTM [N066]:

Observation 1: The current solution for SHR availability indication is suboptimal and can lead to the loss of SHR content in the case of subsequent LTM cell switches.

Proposal 1: RAN2 to agree the updated UE behavior to fix the SHR availability in case of RACH-less LTM.

Proposal 2: Add the following NOTE in section 5.3.5.3 Reception of an RRCReconfiguration by the UE:

2>	if the RRCReconfiguration is applied due to an LTM cell switch execution:

3>	submit the RRCReconfigurationComplete message via SRB1 embedded in NR RRC message ULInformationTransferMRDC as specified in clause 5.7.2a.3.

NOTE X:	In case of RACHless LTM cell switch, SHR generation is immediately triggered as specified in clause 5.7.10.6 before generating RRCReconfigurationComplete message.

This document has made the following observations, and proposals for RLF shortly after successful completion of CHO with candidate SCGs [N067]:

Observation 2: The current specification does not specify logging of the information about the chronological fulfilment of the two execution conditions of the preceding executed CHO with candidate SCGs in case of RLF shortly after performing a successful CHO with candidate SCG.

Observation 3: Without the information of choWithCandidateSCGInfoList a proper MRO for failure cases where RLF occurs shortly after successful CHO with candidate SCGs completing (case 8a from the agreed RAN3 failure scenarios) is not possible.

Proposal 3: RAN2 agrees that the information about the chronological fulfilment of the two execution conditions of the last executed CHO with candidate SCG (which is captured in choWithCandidateSCGInfoList) in RLF report is missing for the cases where RLF occurred shortly after a CHO with candidate SCG.

Proposal 4: RAN2 agrees that amendment of section 5.3.10.5 is made if the last handover of the UE was a CHO with candidate SCG (i.e., lastHO-Type is set to choWithCandidateSCG) and UE was not configured with CHO with candidate SCG when RLF occurred, then the UE logs the information about of the last executed CHO with candidate SCG in choWithCandidateSCGInfoList. The text proposal is in the Annex of this paper.

# Annex: Text proposal for N067

Start of Changes

#### 5.3.10.5	RLF report content determination

The UE shall determine the content in the VarRLF-Report as follows:

<text omitted>

2>	if the UE supports RLF-Report for conditional handover with candidate SCG and if the last executed RRCReconfiguration message including reconfigurationWithSync was concerning conditional handover with candidate SCG:

3>	set lastHO-Type to choWithCandidateSCG;

3>	if the UE was in NR-DC when the RLF was declared and if the UE is not configured for CHO with candidate SCG:

4>	include in cho-WithCandidateSCGInfoList the values for each entry of condReconfigList in the MCG VarConditionalReconfig stored at the time of the last executed handover including both condExecutionCond and condExecutionCondPSCell of the previous executed message including reconfigurationWithSync, where the values are to be set as follows:

5>	if all triggering events of both condExecutionCond and condExecutionCondPSCell of the concerned entry of condReconfigList are fulfilled:

6>	set firstFulfilledConfig to cho if condExecutionCond was fulfilled first or cpc if condExecutionCondPSCell was fulfilled first;

6>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

5>	else if all triggering events of only one of the condExecutionCond or condExecutionCondPSCell of the concerned entry of condReconfigList is fulfilled:

6>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled;

6>	set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition;

2>	set the nrFailedPCellId in failedPCellId to the global cell identity and tracking area code, if available, and otherwise to the physical cell identity and carrier frequency of the target PCell of the failed handover or a failed LTM cell switch;

<text omitted>

End of Changes