3GPP TSG-RAN WG2 Meeting #132	R2-2508329

Dallas, Texas, USA, 17th – 21st November, 2025

Agenda item:	8.9.3

Source:	Samsung

Title:	Other corrections for IoT NTN

WID/SID:	IoT_NTN_Ph3-Core

Document for:	Discussion and Decision

# Introduction

In RAN#107 the latest WID for IoT NTN Phase 3 was agreed [1], where Capacity enhancements for uplink:

- Support of Capacity enhancements for uplink - Study then specify, if beneficial, enhancements to enable multiplexing of multiple UEs (e.g. up to the min of 4 and the maximum allowed by the existing UL and DL signalling) in a single 3.75 kHz or 15 kHz subcarrier via orthogonal cover codes (OCC) for NPUSCH format 1 and NPRACH [RAN1, RAN2, RAN4] - Multi-tone support for 15 kHz SCS should also be considered - Specify necessary signalling, if needed - Update RF requirements accordingly, if needed Note: Impact of impairment shall be taken into account - Study and specify, if beneficial the following enhancements to reduce the necessary uplink and downlink signaling to complete an Early Data Transmission (EDT) transaction [RAN2]: - Msg3 transmission without msg1/ Random Access Response (RAR) - Efficient delivery (reduced overhead) of msg4 / RRCEarlyDataComplete - Study and specify RRM requirement, if identified [RAN4]

In this contribution, we provide some corrections on idle mode, MAC and capabilities.

# Idle mode corrections

## On S&F list use in idle mode operation

In RAN2#131 meeting the following was agreed:

The S&F mode indication (i.e., sf-OperationMode) and the S&F mode transition time (i.e., t-ModeSwitching) of the neighbor satellite are signaled in SIB33 per neighbor satellite.

A Rel-19 UE may deprioritize the neighbor cells operating in the S&F mode (FFS on the detailed specification impact, i.e. whether a note is sufficient or some normative text. To be considered in the post meeting CR review)

And based upon this, the following note was captured:

NOTE: 	If the highest ranked cell or best cell according to absolute priority reselection rules is a cell operating in store and forward mode the UE may deprioritise or not consider this cell as candidate cell for cell reselection and may continue considering other cells not operating in store and forward mode for cell reselection.

The main issue with making the text in the note to normative text seems to be that “deprioritise” may need to be defined. However, we think that this can be resolved by clarifying that what deprioritise means is up to UE implementation.

Proposal 1: Make the NOTE “If the highest ranked cell or... “ normative text and a note that “deprioritisation” is up to UE implementation.

Proposal 2: Agree text proposal on 36.304 in Annex A.

# MAC corrections

## Corrections related to timing alignment timer

Normally, an uplink PUCCH or PUSCH transmission never occurs if a UE does not have the timeAlignmentTimer running. This is document in the MAC in as follows:

The MAC entity shall not perform any uplink transmission on a Serving Cell, except the Random Access Preamble transmission and transmissions corresponding to a PUR-RNTI, when the timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running. Furthermore, when the timeAlignmentTimer associated with the pTAG is not running, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble transmission on the SpCell.

However, for CB-Msg3-EDT, the UE will be performing an uplink transmission on a serving cell when the timeAlignmentTimer is not running. Therefore we propose:

Proposal 3: Introduce correction to clarify that for CB-Msg3-EDT, the UE can perform uplink tranmission without the timeAlignmentTimer running.

Proposal 4: Agree Text proposal in Annex B1.

## AS RAI reporting

On AS RAI In RAN2#131 the following was agreed:

Based on NW indication, it shall be possible for the UE to indicate during CB-Msg3-EDT procedure whether DL data following the UL data in CB-Msg3 is expected or not. RAI will be reused for this.

In the last meeting the following was discussed:

- CQI and AS RAI reporting in Msg3

R2-2507242	Issues on early termination of CB-Msg3-EDT without RRC message	Google	discussion	Rel-19	IoT_NTN_Ph3-Core

Observation 1	NB-IoT UEs are not allowed to transmit DCQR using “DCQR and AS RAI MAC CE” in CB-Msg3, which may hinder these UEs from informing the network of whether DL data following the UL data in CB-Msg3 is expected or not (i.e., may conflict with the RAN2#131 agreement).

Proposal 1	NB-IoT UEs are allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3, when rai-ActivationEnh is configured in SIB2-NB.

-	QC thinks this is already possible. Google thinks this is only clear for connected mode

-	Nokia and Nordic think we could use the NAS procedure.

Come back in the next meeting to decide whether NB-IoT UEs are allowed to transmit the “DCQR and AS RAI MAC CE” in CB-Msg3, when rai-ActivationEnh is configured in SIB2-NB

In the currently endorsed CR, it is made clear that the AS RAI in Msg3 is only supported for eMTC. The background is that AS RAI was only agreed to be sent over NAS for NB-IoT due to the CP solution being the majority supported solution. In principle, it may make sense to also only allow for AS RAI over NAS for CB-Msg3-EDT, which would mean that AS RAI is not introduced for NB-IoT CB-Msg3-EDT. However, since regenerative architecture is introduced for Release 19, where the latency to the core network may be very large, then the AS RAI indication in NAS may not be particularly useful. Therefore it can make sense to introduce AS RAI in CB-Msg3-EDT for NB-IoT. However, care should be taken to ensure that this is only introduced for CB-Msg3, as otherwise there may be backwards compatibility issues. For this purpose, we think that a new capability without signalling would be useful.

Proposal 5: AS RAI for NB-IoT CB-Msg3 is introduced (not for legacy Msg3).

Proposal 6: Introduce new capability without signalling for AS RAI for NB-IoT CB-Msg3.

Proposal 7: Agree Text proposal in Annex B2.

# Capabilities-related correction

## Support of acceptable cell in TN/NTN

One open issue is on whether acceptable cell shall be considered support only in NTN or both in TN and NTN. In principle and according to the WID, it may be support if there is no further work required.

However, situation in TN and NTN is different and the acceptable cell camping feature is tricky. This is because current terrestrial networks are very unlikely to be able to support PWS as they would be tailored to IoT services. The tricky part is that a UE may camp on a cell as an acceptable cell without the knowledge of cell that is being camped on, or for the cells that are part of the UEs PLMN. Thus in order to support this, there would need to be a way for a network to allow for UEs to camp or not to camp as an acceptable on an NB-IoT cell. One way to allow it is for the network to indicate whether it supports PWS broadcasting or not, and if it does the UE is can camp on the cell as an acceptable cell.

Proposal 8: Acceptable cell camping on a TN cell can be allowed if there is a way of preventing UEs to camp on the cell as an acceptable cell.

Proposal 9: Introduce a network indication of support of PWS broadcasting. If indicated, a UE may camp on the cell as an acceptable cell.

# Conclusion

In this contribution we discussed other corrections for IoT NTN:

Proposal 1: Make the NOTE “If the highest ranked cell or... “ normative text and a note that “deprioritisation” is up to UE implementation.

Proposal 2: Agree text proposal on 36.304 in Annex A.

Proposal 3: Introduce correction to clarify that for CB-Msg3-EDT, the UE can perform uplink tranmission without the timeAlignmentTimer running.

Proposal 4: Agree Text proposal in Annex B1.

Proposal 5: AS RAI for NB-IoT CB-Msg3 is introduced (not for legacy Msg3).

Proposal 6: Introduce new capability without signalling for AS RAI for NB-IoT CB-Msg3.

Proposal 7: Agree Text proposal in Annex B2.

Proposal 8: Acceptable cell camping on a TN cell can be allowed if there is a way of preventing UEs to camp on the cell as an acceptable cell.

Proposal 9: Introduce a network indication of support of PWS broadcasting. If indicated, a UE may camp on the cell as an acceptable cell.

# Reference

RP-250472, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 3, Mediatek, RAN#107, Incheon, ROK, March 2025.

# Annex A

-------------------------------------- Based on 36.304 --------------------------------------

#### 5.2.4.4	Cells with cell reservations, access restrictions or unsuitable for normal camping

For the highest ranked cell (including serving cell) according to cell reselection criteria specified in clause 5.2.4.6, for the best cell according to absolute priority reselection criteria specified in clause 5.2.4.5, the UE shall check if the access is restricted according to the rules in clause 5.3.1.

If that cell and other cells have to be excluded from the candidate list, as stated in clause 5.3.1, the UE shall not consider these as candidates for cell reselection. This limitation shall be removed when the highest ranked cell changes.

If the highest ranked cell or best cell according to absolute priority reselection rules is an intra-frequency or inter-frequency cell which is not suitable for a CN type due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency, as candidates for reselection for the CN type for a maximum of 300s. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is an inter-RAT cell which is not suitable due to being part of the "list of forbidden TAs for roaming" or belonging to a PLMN which is not indicated as being equivalent to the registered PLMN, the UE shall not consider this cell and other cells on the same frequency as candidates for reselection for a maximum of 300s. In case of UTRA further requirements are defined in the TS 25.304 [8]. In case of NR further requirements are defined in the TS 38.304 [38]. If the UE enters into state any cell selection, any limitation shall be removed. If the UE is redirected under E-UTRAN control to a frequency for which the timer is running, any limitation on that frequency shall be removed.

If the highest ranked cell or best cell according to absolute priority reselection rules is a CSG cell which is not suitable due to not being a CSG member cell, the UE shall not consider this cell as candidate for cell reselection but shall continue considering other cells on the same frequency for cell reselection.

If the highest ranked cell or best cell according to absolute priority reselection rules is a cell operating in store and forward mode the UE may deprioritise or not consider this cell as candidate cell for cell reselection and may continue considering other cells not operating in store and forward mode for cell reselection.

NOTE: 	How to deprioritise a cell in store and forward mode is up to UE implementation.

-------------------------------------- Based on 36.304 --------------------------------------

# Annex B

## B1

-------------------------------------- Based on 36.321 --------------------------------------

5.2	Maintenance of Uplink Time Alignment

The MAC entity has a configurable timer timeAlignmentTimer per TAG. The timeAlignmentTimer is used to control how long the MAC entity considers the Serving Cells belonging to the associated TAG to be uplink time aligned, as specified in TS 36.331 [8].

The MAC entity shall:

-	when a Timing Advance Command MAC control element is received and if a NTA has been stored or maintained with the indicated TAG:

-	except when the received Timing Advance Command MAC control element is addressed with a PUR-RNTI:

-	apply the Timing Advance Command for the indicated TAG;

-	start or restart the timeAlignmentTimer associated with the indicated TAG.

-	when a Timing Advance Command is received in a Random Access Response message for a serving cell belonging to a TAG:

-	if the UE is configured with pur-Config (see TS 36.331 [8]) and if a NTA has been stored or maintained and no temporary NTA has been stored:

-	store current NTA as temporary NTA (see clause 5.4.7.2).

-	if the Random Access Preamble was not selected by the MAC entity:

-	apply the Timing Advance Command for this TAG;

-	start or restart the timeAlignmentTimer associated with this TAG.

-	else, if the timeAlignmentTimer associated with this TAG is not running:

-	apply the Timing Advance Command for this TAG;

-	start the timeAlignmentTimer associated with this TAG;

-	when the contention resolution is considered not successful as described in clause 5.1.5, stop timeAlignmentTimer associated with this TAG.

-	else:

-	ignore the received Timing Advance Command.

-	when a Timing Advance Command is received in a successful CMR for a serving cell belonging to a TAG:

-	apply the Timing Advance Command for this TAG.

-	when the MAC entity is configured with rach-Skip or rach-SkipSCG:

-	apply timing advance value indicated by targetTA in rach-Skip or rach-SkipSCG for the pTAG;

-	start the timeAlignmentTimer associated with this TAG.

-	when a timeAlignmentTimer expires:

-	if the timeAlignmentTimer is associated with the pTAG:

-	flush all HARQ buffers for all serving cells;

-	notify RRC to release PUCCH/SPUCCH for all serving cells;

-	notify RRC to release SRS for all serving cells;

-	for NB-IoT, notify RRC to release all dedicated resources for SR;

-	clear any configured downlink assignments and uplink grants;

-	consider all running timeAlignmentTimers as expired;

-	else if the timeAlignmentTimer is associated with an sTAG, then for all Serving Cells belonging to this TAG:

-	flush all HARQ buffers;

-	notify RRC to release SRS;

-	notify RRC to release PUCCH/SPUCCH, if configured;

-	clear any configured downlink assignments and uplink grants.

-	upon indication from upper layers to start timeAlignmentTimer, if a NTA has been stored or maintained with the indicated TAG:

-	start or restart the timeAlignmentTimer associated with the indicated TAG.

When the MAC entity stops uplink transmissions for an SCell due to the fact that the maximum uplink transmission timing difference (as described in clause 7.9.2 of TS 36.133 [9]) or the maximum uplink transmission timing difference the UE can handle between TAGs of any MAC entity of the UE is exceeded, the MAC entity considers the timeAlignmentTimer associated with the SCell as expired.

The MAC entity shall not perform any uplink transmission on a Serving Cell, except the Random Access Preamble transmission, transmissions corresponding to a PUR-RNTI and transmissions for the CB-Msg3-EDT procedure, when the timeAlignmentTimer associated with the TAG to which this Serving Cell belongs is not running. Furthermore, when the timeAlignmentTimer associated with the pTAG is not running, the MAC entity shall not perform any uplink transmission on any Serving Cell except the Random Access Preamble transmission on the SpCell.

The MAC entity shall not perform any sidelink transmission which is performed based on UL timing of the corresponding serving cell and any associated SCI transmissions when the corresponding timeAlignmentTimer is not running.

NOTE:	A MAC entity stores or maintains NTA upon expiry of associated timeAlignmentTimer, where NTA is defined in TS 36.211 [7]. The MAC entity applies a received Timing Advance Command MAC control element and starts associated timeAlignmentTimer also when the timeAlignmentTimer is not running.

-------------------------------------- Based on 36.321 --------------------------------------

## B2

-------------------------------------- Based on R2-2507656 --------------------------------------

5.25	Transmission of Downlink Channel Quality Report

The MAC entity of a BL UE or UE in enhanced coverage may be configured by upper layers to report DL channel quality in Msg3. DL channel quality in Msg3 in RRC_CONNECTED is not reported.

If the UE is a BL UE or UE in enhanced coverage or an NB-IoT UE, a Downlink Channel Quality Report (DCQR) shall be triggered if any of the following events occur:

-	DCQR Command MAC control element is received, in which case the DCQR is referred below to as "Regular DCQR";

-	for BL UE or UE in enhanced coverage, transmission of DCQR in (CB-)Msg3 is configured by upper layers in mpdcch-CQI-Reporting, and for NB-IoT transmission of DCQR in CB-Msg3 is configured by upper layers in rai-ActivationEnh, in which case DCQR is referred below to as "Msg3 DCQR".

If any type of DCQR has been triggered:

-	start performing DL channel quality measurements according to TS 36.133 [9].

If "Regular DCQR" has been triggered:

-	if an uplink grant has been received on the PDCCH for MAC entity's C-RNTI:

-	instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19;

-	cancel the triggered "Regular DCQR".

If "Msg3 DCQR" has been triggered:

-	if an uplink grant has been received on the PDCCH for MAC entity's RA-RNTI or if the uplink grant is for CB-Msg3-EDT:

-	if the allocated resources can accommodate a DCQR and AS RAI MAC control element plus its subheader as a result of logical channel prioritization:

-	instruct the Multiplexing and Assembly procedure to generate a DCQR and AS RAI MAC control element as defined in clause 6.1.3.19;

-	else if the uplink grant is not for EDT:

-	if configured by upper layers in mpdcch-CQI-Reporting, use R and F2 fields in the MAC PDU subheader, to transmit the measurement outcome, as defined in clause 6.2.1;

-	cancel the triggered "Msg3 DCQR".

<Skip>