3GPP TSG-RAN WG2 Meeting #132	R2-2508332

Dallas, USA, 17th – 21st Nov, 2025

Agenda item:	9.7.2

Source:	Samsung

Title:	On CP vs UP solution and other issues for voice over NB-IoT NTN

WID/SID:	IoT_NTN_Ph4

Document for:	Discussion and Decision

# Introduction

In RAN#108 the WID for IoT NTN Phase 4 was agreed [1], where the objective is the following:

The aim of this work item is to specify the enhancement of NB-IoT-NTN to support IMS voice call over GSO with the following objectives: - Down-select between CP and UP solutions for voice support over NB-IoT-NTN until RAN#110 [RAN2] - Coordination with SA is required - The objective can be updated with further details based on the outcome of the above down-selection - Support of semi-persistent scheduling for NB-IoT-NTN for DL and UL data transmission for voice packets [RAN2, RAN1] - Support of necessary modifications to RRC connection setup procedure for NB-IoT-NTN [RAN2] - Support of necessary modifications for emergency call for voice over NB-IoT-NTN [RAN2] - Study and if feasible, specify UE transmit power higher than PC1 (e.g. up to 37dBm) for NB-IoT-NTN [RAN4] - Note: The enhancements to support voice over NB-IoT-NTN via GSO in this work item may be also applicable to NGSO cases without additional specification enhancements - Note: Coordination with SA4 is expected

In this contribution, we have an initial discussion on voice over NB-IoT NTN.

# Discussion

## UP vs CP solution for voice support

- The Control Plane CIoT optimization was introduced in 3GPP Release 13 for NB-IoT and eMTC. This enhancement simplifies connection and data transmission procedures compared to previous releases. The UE does not need to establish security or data radio bearers in order to send data, and as a result data can be transmitted at a much earlier stage. This especially brings benefits for scenarios and deployments where the devices are transmitting infrequent and small amount of data.

- Some features of the CP CIoT optimization:

- Data can be transmitted in RRCConnectionSetupComplete-NB message

- Data is transmitted over NAS in DL/ULInformationTransfer-NB

- AS security does not need to be established and is thus not mandatory to be supported

- UE cannot be RRC Reconfigured

- No data radio bearer beyond SRB1bis (nor other radio bearers) are established

- The CP CIoT optimization is mandatory for NB-IoT devices and optional for eMTC. As a result, the majority of NB-IoT deployments are using CP CIoT.

- The UP CIoT optimization was also introduced for 3GPP Release 13 (and also for 5G NR in Release 15 – although this is not the term used). This enhancement mainly simplifies and makes the connection establishment procedure more efficient. It allows a network to suspend an RRC connection

- All-in-all, it is clear that NB-IoT was never intended to support voice, so the best way to compare UP vs CP would be to discuss the con of each case.

- Some of the mentioned cons of the CP solution are the following:

- CP solution not originally designed to support voice traffic. The CP solution mainly targets very short connections whereby the UE sends a small amount of data and is then released. In this sense the CP solution is very efficient as it minimizes the number of messages until the data payload is sent. However, the UP solution relies on how data, and by extension voice traffic, has been configured and transmitted since 3G/4G times. The need to change this for the CP solution would present a challenge for network operators. Furthermore, several questions emerge for adapting the voice over the CP solution. For instance, with a new SRB1, does the CP solution UE now have to support LCP? Several more such detailed questions exist, and we believe that there may be other hidden issues that have yet to come to light.

- CP solution does not support RRC Reconfiguration. Since the CP solution does not support AS security, there is no integrity protection available for RRC messages. This means that it would be a very large security flaw if RRC Connection Reconfiguration was supported, so it was agreed to not be supported, which means that the UE may only be configured via Msg4 (RRCConnectionSetup). For voice traffic, this would be a very serious problem, as this would mean that the UE would have to be configured for voice already in Msg4. These types of configurations would for instance include SPS or any new SRBs.

- Observation 1: The main cons of CP solution is that it was never designed to support voice traffic and that there is no support for RRC reconfiguration.

- Observation 2: The UP solution relies on how voice traffic has been configured and transmitted since 3G/4G times – changing this presents a challenge for network operators.

- Some of the mentioned cons of the UP solution are the following:

- UP solution is not widely supported. So far the UP solution has not had as much traction in the market place compared to the CP solution, which is most likely due to the traffic types that have been supported in networks so far. In cases where larger payloads have been needed, eMTC has typically been the option.

- UP solution implies IP and RoHC, which may exhibit performance issues. One of the main cons that have been discussed is that the UP solution implies that IP solution is used and that RoHC applied. The issue with RoHC is that the compression state may change over time, which may unexpectedly cause issues as the latency in a geostationary NTN is of an RTT of 500 ms. However, RoHC is deployed in satellite networks. For instance, RoHC was discussed in RAN2#105 during the Rel-16 study item and the following was noted:

R2-1901796 Discussion on ROHC for NTN LG Electronics Inc. discussion FS_NR_NTN_solutions Proposal. The ROHC should not be configured in NTN. - Ericsson asks what is the problem, propagation delay or packet drop. Thales explains that we are already using this feature in satellite systems so we should not disable => Noted

- There are two important considerations for RoHC. The first is that the link budgets in a realistic deployment for the voice solution will have to be good enough to allow for a certain amount overprovisioning. The second consideration is that while RoHC decompression may occur at some points, the question is whether this will be the major issue for the quality of a voice service. More likely the key point of quality of a voice service is that the packet loss rate is acceptable. If a large number of packets are lost, then the key issue is not that the RoHC itself has decompressed, the key issue that would degrade the voice quality is rather that a large number of packets have been lost. We therefore believe that the issue of RoHC decompression is not as big of a problem as stated in some contributions.

- Observation 3: The main cons of UP solution is that it has not been widely deployed so far.

- Observation 4: The issue of RoHC decompression for the UP solution has been overstated.

- Also, whether to specify voice over IP or not is not a decision that RAN2 should take. If for instance non-IP is specified, then the RoHC decompression issue may not be as severe.

- Observation 5: It is not up to RAN2 whether to specify voice over IP for NB-IoT NTN.

- With all of these considerations, we believe that RAN2 has sufficient understanding of the cons of both CP and UP solution to make a decision. And given the drawbacks and the re-designing of the CP solution required, we strongly believe that RAN2 should decide to go with UP solution for voice over NB-IoT NTN.

Proposal 1: RAN2 confirms that UP solution is supported for voice over NB-IoT NTN.

- As this is an important decision, we believe that RAN, SA2, SA3 and CT1 shall be informed of our decision.

Proposal 2: Send LS to inform RAN, SA2, SA3 and CT1.

### Co-existence between UP for voice and CP for data

- As mentioned above, even though UP solution is selected for voice, the CP solution is still a more efficient solution for sending small amounts of data. And given that there already are IoT NTN CP devices and network currently being deployed, it is likely that future NB-IoT NTN networks would support both UP and CP, where UP is used for voice and CP is used for data. We think that the co-existence between these solutions should be addressed.

- Proposal 3: RAN2 to address the co-existence between using UP for voice and CP solution for small payloads.

## Semi-persistent scheduling

One of the key solutions to make voice efficient is the introduction of semi-persistent scheduling. Semi-persistent scheduling was introduced for NB-IoT, but only for delivering Buffer Status reports in the uplink. This is can be useful but is clearly not sufficient for voice support, which is why the WID states that DL and UL SPS shall be supported.

For 4G LTE, SPS is configured via RRC, and activated via DCI with a specific RNTI. In 5G NR, there are two types of Configured Grant (Type 1 and Type 2), where in Type 1 the RRC message itself activates SPS and in Type 2 it works similar to 4G LTE. For NB-IoT we believe that it would be suitable to follow 4G LTE, i.e Type 2.

Proposal 4: Downlink/Uplink SPS is activated using DCI as in 4G LTE SPS.

In order to make the reconfiguration faster, one method that has been introduced in LTE Rel-14, is the support of multiple SPS configurations, which can be switched by signaling a configuration index in the DCI. In order to save the number of bits in DCI that may need to be repurposed, a maximum of two configurations indices may be configured and switched in-between. RAN1 may need to confirm whether there are any bits available to be reused for the configuration index in the DCI.

Proposal 5: Introduce multiple SPS configurations that can be switched using an SPS configuration index in the DCI.

Proposal 6: The maximum number of SPS configurations is limited to 2.

## Voice connection establishment

When a normal 5G UE initiates a voice call, the UE indicates in the establishment cause that an MO voice call has been initiated. In order to prepare the network, in particular an NB-IoT network, it is important that the network is made aware that a voice call is initiated. This is so that the network can begin reserve resources for the NB-IoT UE while the voice call is being setup and initiated in the core network. This is needed because the number of UEs that the network can allocate for IoT traffic and for voice services would likely vastly differ.

For CP solution, an establishment cause in RRCConnectionRequest-NB would at least be required, while for UP solution an establishment/resume cause would need to be introduced in RRCConnectionRequest and RRCConnectionResumeRequest. However, for NB-IoT the resume cause reuses the establishment cause for RRC setup.

Proposal 7: At least introduce a new establishment cause for MO voice.

# Conclusion

In this contribution, we discussed issues related to voice over NB-IoT NTN:

- Observation 1: The main cons of CP solution is that it was never designed to support voice traffic and that there is no support for RRC reconfiguration.

- Observation 2: The UP solution relies on how voice traffic has been configured and transmitted since 3G/4G times – changing this presents a challenge for network operators.

- Observation 3: The main cons of UP solution is that it has not been widely deployed so far.

- Observation 4: The issue of RoHC decompression for the UP solution has been overstated.

- Observation 5: It is not up to RAN2 whether to specify voice over IP for NB-IoT NTN.

Proposal 1: RAN2 confirms that UP solution is supported for voice over NB-IoT NTN.

Proposal 2: Send LS to inform RAN, SA2, SA3 and CT1.

- Proposal 3: RAN2 to address the co-existence between using UP for voice and CP solution for small payloads.

Proposal 4: Downlink/Uplink SPS is activated using DCI as in 4G LTE SPS.

Proposal 5: Introduce multiple SPS configurations that can be switched using an SPS configuration index in the DCI.

Proposal 6: The maximum number of SPS configurations is limited to 2.

Proposal 7: At least introduce a new establishment cause for MO voice.

# Reference

RP-251867, Revised WID on Non-Terrestrial Networks (NTN) for Internet of Things (IoT) Phase 4, Vivo, RAN#108, Prague, Czechia, June 2025.