3GPP TSG-RAN WG2 Meeting #132	R2-2508348

Dallas, USA, 17th - 21st November 2025

Source: 	Vodafone

Title: 		Suspend/Resume as an alternative to RRC Inactive

Document for: decision

Agenda Item: 10.3.2.1

Introduction

During RAN2#131bis, the discussion on RRC state design started and the following was captured in the meeting minutes

Agreements

Continue study

-	Idle mode –

-	Connected mode –

-	Inactive mode or sub-state: Should support at least on these functions: UE based mobility (e.g. cell selection/reselection), UE context and identification and energy efficient for both UE and NW.  Further study fast transitions, data transfer, further improvements to UE identification, and RNA management and paging/wake-up simplifications/enhancements. We can consider the pros/cons of different modelling, after having some discussions on the functionality.

This contribution aims to analyze the advantages and disadvantages of RRC Inactive Mode in NR and, proposes that the R13 EPS / R16 5GC Suspend/Resume procedure is investigated as an alternative approach.

Discussion

RRC Inactive Mode was introduced in the Rel 15 “main drop” (c.f. StandAlone”) as a mandatory UE feature that was optional for the network to use. It reduces the time needed for the UE’s Inactive to Active transition. However, in our understanding, the feature is not widely deployed; it adds system complexity by duplicating CN functionality in the RAN; and is complex to maintain (e.g. features in new 3GPP Releases lead to updates to the NGAP “Core Network Assistance Information for RRC INACTIVE” IE).

TS 38.300 provides e.g. the following message flow:

For example, document R2-2507270 highlights several shortcomings that have contributed to limited deployments.
Taking this into account, we would like to outline the key points we believe are important to consider moving forward:

- In IDLE mode, the UE’s paging area is handled by the AMF in the Core Network (CN), using the Registration Area.
In INACTIVE mode, the UE’s  RAN paging area is handled by the RAN and in addition the CN paging area is handled by the AMF (Note that the RAN Notification Area boundary shall not cross the Core Network Registration Area boundary – this is because, if the UE is out of coverage and misses RAN paging for CN originated signalling, the Core Network SHALL put the UE into CM-IDLE state while the UE remains in Inactive state.

- Paging (re)transmission strategies and timers need to be co-ordinated in gNBs and AMF, e.g. to ensure timely delivery of MT voice calls. This necessitates the implementation of paging functionality in both CN and RAN.

- During the transition from INACTIVE to CONNECTED mode, if the UE establishes a connection with a different gNB than the one it was previously connected to before entering INACTIVE mode, the context retrieval procedure is triggered. This procedure operates over the Xn interface.
In real-world deployments, the Xn interface is not always available between gNBs from different (or even the same) vendors, which can lead to limitations in context retrieval.

- If the UE reselects to a different RAT (e.g., from NR to LTE), the CN releases the NGAP connection and the UE context in the gNB is deleted. This issue is particularly relevant during the rollout of a new RAT when Inter-RAT mobility may occur frequently. This impacts the benefit of the Inactive feature in the Xth generation for a large proportion of the time, as either mobility to generation “X-1” is frequent, or, mobility to generation”X+1” is frequent.

- The design of several features such as eDRX (Extended Discontinuous Reception) and SDT (Small Data Transmission) has taken RRC INACTIVE mode into account, resulting in a more complex system architecture.

Observation 1: The limited adoption of the RRC INACTIVE state is not only due to the delayed availability of standalone 5G architecture in many networks, but also because its practical use is constrained by configuration complexities and frequent IDLE-mode inter-RAT mobility.

Alternative approaches:

In release 13 EPS/and Release 16 5GCore the suspend/resume procedure was introduced. This procedure allows to suspend the RAN-CN connection and store the context of the UE in the gNB, but it does not move paging functionalities into RAN and does not require to configure RAN areas.

TS 23.401 shows the EPC message flows see 5.3.4A and 4.3.5A. The corresponding ones for 5GCore are in sections 5.31.18 of TS 23.501 and 4.8.1.2 and 4.8.2.3 of TS 23.502. For Idle-Active transition, we believe that the latency is the same, but further optimizations are not precluded.

23.401:

23.502:

While this procedure may not address all challenges associated with RRC INACTIVE, it avoids the complexity inherent in that state.
It offers a practical starting point for exploring ways to reduce delays in transitioning from IDLE to CONNECTED mode without introducing significant system or configuration overhead

Conclusion

Proposal: RAN 2 should investigate the suspend/resume procedure as a potential method to reduce the transition delay from IDLE to CONNECTED mode and to simplify the 6GR UE and to simplify the overall 6G system.